3GPP TSG-RAN5 Meeting #109	R5-256250

# Dallas, USA, 17th Nov. – 21st Nov. 2025

Title:							MU discussion on OBW for FR2 UL MIMO

Source:			Anritsu

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

In previous meetings, OBW MU for FR2 UL MIMO was discussed [1] – [4]. However, the discussion was not concluded due to its low testability in FR2c with CBW = 400 MHz. This paper presents our analysis of OBW MU for FR2c, CBW = 400 MHz, UL MIMO.

# 2.	Discussion

For the possible minimum UL level, the MPR value corresponding to CP-OFDM QPSK Outer_Full should be considered, because CP-OFDM is the mandatory modulation type in UL MIMO, as clarified in [5]. As a result, the possible minimum UL level for UL MIMO is 9.7 dBm in FR2c with CBW = 400 MHz [1]. Since this level is 3.5 dB lower than that for SISO, OBW MU for MIMO could be larger than that for SISO.

Regarding MBW, Keysight clarified that 1.1 * CBW is too small because the spectrum emission mask allows relatively high UE emissions up to ΔfOOB = 10% [3]. Therefore, MBW ≥ 1.2 * CBW should be considered to avoid this issue.

Based on the above assumptions for UL level and MBW, we analyzed OBW MU for each MBW/CBW ratio as shown below. The simulation model described in Annex A of [6] and the data sets provided in [7] were used for the analysis.

Table 1: OBW MU for each MBW/CBW in UL MIMO, FR2c, CBW = 400 MHz

| MBW/CBW | OBW MU |
| --- | --- |
| 1.20 | 1.8%CBW |
| 1.25 | 5.9%CBW |
| 1.30 | > 10%CBW |

Since 5.9%CBW or > 10%CBW are too large for OBW MU, we proposed using MBW = 1.20 * CBW with OBW MU = 1.8%CBW.

Proposal 1: Adopt MBW = 1.2 * CBW and MTSU = 1.8%CBW for OBW in UL MIMO, FR2c, CBW = 400 MHz, PC3, NTC, IFF, max device size ≤ 40 cm.

# 3.	Conclusion

RAN5 is asked to endorse the following proposals.

Proposal 1: Adopt MBW = 1.2 * CBW and MTSU = 1.8%CBW for OBW in UL MIMO, FR2c, CBW = 400 MHz, PC3, NTC, IFF, max device size ≤ 40 cm.

# 4.	References

[1] R5-247165, “MU discussion on OBW for FR2 UL MIMO”, Anritsu, RAN5#105

[2] R5-247337, “On the MU and TT for SISO and MIMO test cases”, Rohde & Schwarz, RAN5#105

[3] R5-253527, “FR2 MU - PC3 analysis for OBW UL MIMO test”, Keysight Technologies, RAN5#107

[4] R5-253804, “FR2 MU - PC3 analysis for OBW UL MIMO test”, Keysight Technologies UK Ltd, RAN5#108

[5] TR 38.905, “Derivation of test points for radio transmission and reception; User Equipment (UE) conformance test cases”, V19.1.0 (2025-09)

[6] R5-211946, “On FR2 OBW MU”, Anritsu, RAN5#90-e

[7] R5-198175, “	Discussion on AP#84.22 Tx Spectrum Shape for FR2 OBW MU”, Qualcomm Inc., RAN5#85