3GPP TSG-RAN5 Meeting #109 	R5-256260

# Dallas, USA, 17th Nov. 2025 - 21st Nov. 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 2019 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction to Reference sensitivity for FR1 2CC EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are editorial errors in reference sensitivity for inter-band EN-DC within FR1 (2CC). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In Table 7.3B.2.0.3.5.1-1, invisible border for DC_3A_n77A was corrected to visible border. - In Table 7.3B.2.3.4.2.1-6, test settings titles were corrected for consistency with others. - In Table 7.3B.2.3.4.2.1-6, text style for DC_30A_n77A was corrected. - In Table 7.3B.2.3.4.2.1-6, unnecessary space was removed from DC_48A_n66A. - In Table 7.3B.2.3.5-0, UL/DL CBW for DC_2A_n71A was corrected. - In Table 7.3B.2.3.5-0, unnecessary visible border for DC_12A_n66A was corrected to invisible border. - In Table 7.3B.2.3.5-0, the row positions of DC_28A_n40A and DC_28A_n71A were corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editorial errors will remain in the test spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.0.3, 7.3B.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3B.2.0.3	Inter-band EN-DC within FR1

<<Unchanged sections skipped>>

7.3B.2.0.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for PC3 EN-DC in NR FR1 involving two bands

Table 7.3B.2.0.3.5.1-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| <<Unchanged sections skipped>> |  |  |  |  |  |  |  |
| DC_3A_n77A DC_3A_n77(2A) DC_3A_SUL_n77A-n80A DC_3A_n78A DC_3A-SUL_n78A-n80A DC_3C_n78A | 3 | 1740 | 5 | 25 | 1835 | 26 | IMD23 |
|  | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
|  | 3 | 1765 | 5 | 25 | 1860 | 8.0 | IMD43 |
|  | n77, n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| <<Unchanged sections skipped>> |  |  |  |  |  |  |  |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Void NOTE 5: Void. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: Reserved. NOTE 10: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 - 4100 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 11: For a UE which supports this band combination only when the Band n78 frequency range restriction of 3400 - 3800 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

Editor's note:	The minimum conformance requirements for the following test points are missing in 7.3B.2.0.3.

- DC_40A_n78A (REFSENS_ENDC_2) in Table 7.3B.2.3.4.2.1-6

- DC_48A_n66A (REFSENS_ENDC_1) in Table 7.3B.2.3.4.2.1-6

Editor’s Note: Test configuration table and test requirements in clauses 7.3B.2.1.4.1 and 7.3B.2.1.5 alignment with TP analysis in TR 38.905 (V19.0.0) clause 4.3.3.1 is FFS.

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC within FR1 with 2 DL CCs.

7.3B.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

LTE anchor agnostic approach is not applied.

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.5B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS 36.521-1 [10] Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG, respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG, respectively.

For configurations without any reference sensitivity exception, in a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. For configurations with reference sensitivity exception, in an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

<<Unchanged tables skipped>>

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB" and "E-UTRA NRB " columns |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  |  |  |  |  |  | Lowest supported SCS |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  |  |  | SCG -NR |  |  |  |  |  |
|  | Band | Range |  |  |  | NRB |  |  |  | Band |  | Range | NRB |  |  |
|  | UL MOD | DL MOD |  |  |  | CH BW |  | DLalloc / UL alloc |  | UL MOD |  | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |  |
| <<Unchanged tables skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for a DC_3A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 | Mid |  |  |  |  |  |  |  | n77 |  | UL/DL 3495 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK | 10 MHz | All RBs / 0 |  |
| 28 | 3 | Low |  |  |  |  |  |  |  | n77 |  | Low |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK | 10 MHz | All RBs / 0 |  |
| 33 | 3 | Mid |  |  |  |  |  |  |  | n77 |  | UL/DL 3495 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 48 | 3 | Low |  |  |  |  |  |  |  | n77 |  | Low |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 55 | 3 | Mid |  |  |  |  |  |  |  | n77 |  | UL/DL 3685.005 |  |  |  |
|  | N/A | QPSK |  |  |  | 5 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_2 |  |
| 65 | 3 | Mid |  |  |  |  |  |  |  | n77 |  | UL/DL 3685.005 |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_2 |  |
| 79 | 3 | Low |  |  |  |  |  |  |  | n77 |  | High |  |  |  |
|  | N/A | QPSK |  |  |  | 5 MHz |  | All RBs/0 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_2 |  |
| 89 | 3 | Low |  |  |  |  |  |  |  | n77 |  | High |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  | All RBs/0 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_2 |  |
| 94 | 3 | UL 1740 / DL 1835 |  |  |  |  |  |  |  | n77 |  | UL/DL 3574.995 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 104 | 3 | UL 1765 / DL 1860 |  |  |  |  |  |  |  | n77 |  | UL/DL 3435 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n78A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 | Mid |  |  |  |  |  |  |  | n78 |  | UL/DL 3495 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK | 10 MHz | All RBs / 0 |  |
| 28 | 3 | Low |  |  |  |  |  |  |  | n78 |  | Low |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK | 10 MHz | All RBs / 0 |  |
| 33 | 3 | Mid |  |  |  |  |  |  |  | n78 |  | UL/DL 3495 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 48 | 3 | Low |  |  |  |  |  |  |  | n78 |  | Low |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 55,19 | 3 | Mid |  |  |  |  |  |  |  | n78 |  | UL/DL 3685.005 |  |  |  |
|  | N/A | QPSK |  |  |  | 5 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_2 |  |
| 65,19 | 3 | Mid |  |  |  |  |  |  |  | n78 |  | UL/DL 3685.005 |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_2 |  |
| 79,19 | 3 | Low |  |  |  |  |  |  |  | n78 |  | High |  |  |  |
|  | N/A | QPSK |  |  |  | 5 MHz |  | All RBs/0 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_2 |  |
| 89,19 | 3 | Low |  |  |  |  |  |  |  | n78 |  | High |  |  |  |
|  | N/A | QPSK |  |  |  | 20 MHz |  | All RBs/0 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_2 |  |
| 94 | 3 | UL 1740 / DL 1835 |  |  |  |  |  |  |  | n78 |  | UL/DL 3574.995 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 104 | 3 | UL 1765 / DL 1860 |  |  |  |  |  |  |  | n78 |  | UL/DL 3435 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for a DC_5A_n78A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 | UL 840 |  |  |  |  |  |  |  | n78 |  | UL/DL 3360 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 23 | 5 | UL 840 |  |  |  |  |  |  |  | n78 |  | UL/DL 3360 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK | 10 MHz | All RBs / 0 |  |
| 38 | 5 | UL 840 |  |  |  |  |  |  |  | n78 |  | UL/DL 3560.01 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 44 | 5 | UL 844/DL 889 |  |  |  |  |  |  |  | n78 |  | UL/DL 3421.005 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 55 | 5 | Mid |  |  |  |  |  |  |  | n78 |  | UL/DL 3525.99 |  |  |  |
|  | N/A | QPSK |  |  |  | 5 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_2 |  |
| 69 | 5 | High |  |  |  |  |  |  |  | n78 |  | MID |  |  |  |
|  | N/A | QPSK |  |  |  | 5 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_2 |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for a DC_8A_n77A (PC3) / DC_8A_n78A (PC2 and PC3) Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 | UL Mid |  |  |  |  |  |  |  | n77, n78 |  | UL/DL 3590.01 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK | 10 MHz | All RBs / 0 |  |
| 23 | 8 | UL Mid |  |  |  |  |  |  |  | n77, n78 |  | UL/DL 3590.01 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 38 | 8 | UL Mid |  |  |  |  |  |  |  | n77, n78 |  | UL/DL 3520.005 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK | 100 MHz | All RBs / 0 |  |
| 44,19 | 8 | UL 897.5 / DL 942.5 |  |  |  |  |  |  |  | n77, n78 |  | UL/DL 3634.995 |  |  |  |
|  | QPSK | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for a DC_30A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 30 | UL 2310 / DL 2355 |  |  |  |  |  |  | n77 |  |  | UL/DL 3487.5 |  |  |  |
|  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK | 10 MHz |  | All RBs / REFSENS_ENDC_4 |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for a DC_48A_n66A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 48 |  |  | UL/DL 3555 |  |  |  |  | n66 |  |  | High |  |  |  |
|  | N/A |  |  | QPSK |  | 5 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / REFSENS_ENDC_1 |
| 23 | 48 |  |  | UL/DL 3555 |  |  |  |  | n66 |  |  | High |  |  |  |
|  | N/A |  |  | QPSK |  | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / REFSENS_ENDC_1 |
| 38 | 48 |  |  | UL/DL 3600 |  |  |  |  | n66 |  |  | High |  |  |  |
|  | N/A |  |  | QPSK |  | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / REFSENS_ENDC_1 |
| 44 | 48 |  |  | UL/DL 3630 |  |  |  |  | n66 |  |  | UL 1715 / DL 2115 |  |  |  |
|  | QPSK |  |  | QPSK |  | 20 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / REFSENS_ENDC_4 |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for a DC_71A_n2A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 71 |  | UL 665.5 |  |  |  |  |  | n2 |  |  | DL 1987.5 |  |  |  |
|  | QPSK |  | QPSK |  |  | 5MHz | All RBs / REFSENS_ENDC_1 |  | N/A |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / 0 |
| 25 | 71 |  | Low |  |  |  |  |  | n2 |  |  | UL 1858.5 |  |  |  |
|  | N/A |  | QPSK |  |  | 5 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / REFSENS_ENDC_2 |
| 35 | 71 |  | Low |  |  |  |  |  | n2 |  |  | UL 1881 |  |  |  |
|  | N/A |  | QPSK |  |  | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / REFSENS_ENDC_2 |
| 49 | 71 |  | High |  |  |  |  |  | n2 |  |  | UL 1881 |  |  |  |
|  | N/A |  | QPSK |  |  | 20 MHz | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_71A_n66A Configuration (PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 71 |  | UL 675 / DL 629 |  |  |  |  |  | n66 |  |  | UL1750 / DL 2150 |  |  |  |
|  | QPSK |  | QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / REFSENS_ENDC_4 |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-1. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-1 for PC3 and table 7.3B.2.0.3.2-1a for PC2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-1 for PC3 and table 7.3B.2.0.3.4-1a for PC2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception. The test point is for UL PC3 only. NOTE 4: Test ID with 2UL intermodulation exception. The test point is only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided. The test point is for UL PC3 only. NOTE 9: Test ID with UL receiving harmonic mixing exception avoided NOTE 10: Void NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. NOTE 15: The test points are executed only when UEs support simultaneous Rx/Tx capability as indicated in Table A.4.3.2B.2.3.1-2 of TS 38.508-2. NOTE 16: This test ID is for UL PC2 only. NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 - 4100 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 18: For a UE which supports this band combination only when the Band n78 frequency range restriction of 3400 - 3800 MHz in Japan applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 19: This test ID is for UL PC3 only. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations affected by reference sensitivity exceptions, when test points without NOTE 8, 9 and 11 in Table 7.3B.2.3.4.2.1-6 are tested, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-0, 7.3B.2.3.5-0a, Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-3a, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation for PC3 EN-DC, cross band isolation for PC2 EN-DC, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

For test points with NOTE 8, 9 or 11 in Table 7.3B.2.3.4.2.1-6, reference sensitivity test requirements are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC, and Table 7.3.5-1 in TS 36.521-1 [10] for E-UTRA CC.

Reference sensitivity test requirements for test points in Table 7.3B.2.3.4.2.1-0 are specified in Table 7.3.2.5-1 in TS 38.521-1 [8].

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable two, three, four and five bands operation.

Table 7.3B.2.3.5-0: Reference sensitivity test requirement for PC3 EN-DC

| EN-DC configuration | Test ID | E-UTRA/NR band | UL/DL CBW (MHz) | REFSENS test requirement (dBm) | Exception type |
| --- | --- | --- | --- | --- | --- |
| <<Unchanged rows skipped>> |  |  |  |  |  |
| DC_2A_n71A | 1 | 2 | 5 | REFSENS + 1.7 | UL harmonic exception |
|  |  | n71 | 5 | REFSENS |  |
|  | 2 | 2 | 5 | REFSENS | Harmonic mixing |
|  |  | n71 | 20 | REFSENS + 15.6 |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |
| DC_12A_n66A | 1 | 12 | 5 | REFSENS | UL harmonic exception |
|  |  | n66 | 5 | REFSENS + 10 |  |
|  | 2 | 12 | 5 | REFSENS | UL harmonic exception |
|  |  | n66 | 40 | REFSENS + 3.1 |  |
|  | 3 | 12 | 5 | REFSENS | UL harmonic exception |
|  |  | n66 | 40 | REFSENS | avoided |
| <<Unchanged rows skipped>> |  |  |  |  |  |
| DC_26A_n41A | 1 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n41 | 100 | REFSENS + 2.7 |  |
|  | 2 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n41 | 10 | REFSENS + 10.3 |  |
|  | 3 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n41 | 10 | REFSENS | avoided |
|  | 4 | 26 | 5 | REFSENS + 15.6 | Dual uplink operation |
|  |  | n41 | 10 | REFSENS |  |
|  | 5 | 26 | 5 | REFSENS + 23.8 | Harmonic mixing |
|  |  | n41 | 10 | REFSENS |  |
|  | 6 | 26 | 15 | REFSENS + 19 | Harmonic mixing |
|  |  | n41 | 10 | REFSENS |  |
|  | 7 | 26 | 15 | REFSENS | harmonic mixing |
|  |  | n41 | 10 | REFSENS | avoiding |
| DC_26A_n77A | 1 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS + 3.1 |  |
|  | 2 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS | avoided |
|  | 3 | 26 | 5 | REFSENS + 11.1 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_26A_n78A | 1 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS + 3 |  |
|  | 2 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS | avoided |
|  | 3 | 26 | 5 | REFSENS + 11.1 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_26A_n79A | 1 | 26 | 15 | REFSENS + 22.2 | Harmonic mixing |
|  |  | n79 | 10 | REFSENS |  |
|  | 2 | 26 | 15 | REFSENS | Harmonic mixing |
|  |  | n79 | 10 | REFSENS | avoiding |
| DC_28A_n5A | 1 | 28 | 5 | REFSENS + 17.5 | Cross band isolation |
|  |  | n5 | 20 | REFSENS |  |
| DC_28A_n40A | 1 | 28 | 5 | REFSENS + 37.8 | Harmonic mixing |
|  |  | n40 | 10 | REFSENS |  |
|  | 2 | 28 | 20 | REFSENS + 30.3 | Harmonic mixing |
|  |  | n40 | 10 | REFSENS |  |
|  | 3 | 28 | 5 | REFSENS | Harmonic mixing |
|  |  | n40 | 10 | REFSENS | avoiding |
| DC_28A_n71A | 1 | 28 | 20 | REFSENS | Cross band isolation |
|  |  | n71 | 5 | REFSENS + 5.7 |  |
|  | 2 | 28 | 5 | REFSENS + 6.5 | Cross band isolation |
|  |  | n71 | 20 | REFSENS |  |
| DC_28A_n77A | 1 | 28 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS + 2.9 |  |
|  | 2 | 28 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS | avoided |
|  | 3 | 28 | 5 | REFSENS + 5.5 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 4 | 28 | 5 | REFSENS + 31 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |
| NOTE 1: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1a and Table 7.3.2.5-1b of TS 38.521-1 [8] for 2 antenna port NR band, Table 7.3.2.5-2a and Table 7.3.2.5-2b of TS 38.521-1 [8] for 4 antenna port NR band. |  |  |  |  |  |

<<End of change>>