3GPP TSG-RAN5 Meeting #109	R5-256357

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5502 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of EMM test case 9.2.3.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A UE operating in CS/PS Mode 1 of operation disables the E-UTRA capability as specified in TS 24.301, clause 5.5.3.3.4.3 and may start a timer (T3402) for re-enabling E-UTRA capability. Depending on the UE configuration, if UTRA/GERAN cell (according to TP3) is selected after ~26-28secs, UE may initiate additional Routing Area Update procedure after enabling E-UTRA capability according to TS 24.301, clause 4.5. TS 24.301, clause 5.5.3.3.4.3: If the tracking area updating attempt counter is equal to 5: … - a UE operating in CS/PS mode 1 of operation with "IMS voice not available" without a persistent EPS bearer context shall attempt to select GERAN or UTRAN radio access technology and proceed with appropriate MM or GMM specific procedures and disable the E-UTRA capability (see clause 4.5); TS 24.301, clause 4.5: As an implementation option, the UE may start a timer for enabling E-UTRA when the UE disables E-UTRA capability. The UE should memorize the identity of the PLMNs where E-UTRA capability was disabled. On expiry of this timer: - if the UE is in Iu mode or A/Gb mode and is in idle mode as specified in 3GPP TS 24.008 [13], the UE should enable the E-UTRA capability; - if the UE is in Iu mode or A/Gb mode and an RR connection exists, the UE shall delay enabling E-UTRA capability until the RR connection is released; |  |  |  |  |  |  |  |  |
| Summary of change: |  | Increased T3402 timer value to 60 secs from 30 secs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

##### 9.2.3.2.3	Combined tracking area update / Successful for EPS services only / MSC temporarily not reachable

9.2.3.2.3.1	Test Purpose (TP)

(1)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating' or 'Combined TA/LA updating with IMSI attach' and having the tracking area updating attempt counter set to the value less than four }

ensure that {
  when { UE receives a TRACKING AREA UPDATE ACCEPT message with the EPS update result set to 'TA updated' and the EMM cause set to 'MSC temporarily not reachable' or 'Network failure' }

then { UE sends TRACKING AREA UPDATE REQUEST message after T3411 expiry }

}

(2)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating with IMSI attach', having the tracking area updating attempt counter set to four and operating in CS/PS mode 2 of operation }

ensure that {
  when { UE receives a TRACKING AREA UPDATE ACCEPT message with the EPS update result set to 'TA updated' and the EMM cause set to 'MSC temporarily not reachable' or 'Network failure' }

then { UE sends TRACKING AREA UPDATE REQUEST message after T3402 expiry }

}

(3)

with { UE having sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Combined TA/LA updating with IMSI attach', having the tracking area updating attempt counter set to four and operating in CS/PS mode 1 of operation }

ensure that {
  when { UE receives a TRACKING AREA UPDATE ACCEPT message with the EPS update result set to 'TA updated' and the EMM cause set to 'MSC temporarily not reachable' or 'Network failure'}

then { UE attempts to select GERAN or UTRAN radio access technology }

}

9.2.3.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.3.4.3, TS 24.301, clause 4.5.

[TS 24.301, clause 5.5.3.3.4.3]

Apart from the actions on the tracking area updating attempt counter, the description for tracking area for EPS services as specified in subclause 5.5.3.2.4 shall be followed. In addition, the following description for location updating for non-EPS services applies.

...

#16	(MSC temporarily not reachable); or

#17	(Network failure)

The UE shall stop timer T3430 if still running. The tracking area updating attempt counter shall be incremented, unless it was already set to 5.

If the tracking area updating attempt counter is less than 5:

-	the UE shall start timer T3411, shall set the EPS update status to EU1 UPDATED and shall enter state EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM. When timer T3411 expires the combined tracking area updating procedure indicating "combined TA/LA updating with IMSI attach" is triggered again.

If the tracking area updating attempt counter is equal to 5:

-	a UE operating in CS/PS mode 2 of operation shall start timer T3402, shall set the EPS update status to EU1 UPDATED and shall enter state EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM. When timer T3402 expires the combined tracking area updating procedure indicating "combined TA/LA updating with IMSI attach" is triggered again;

-	a UE operating in CS/PS mode 1 of operation with "IMS voice not available" shall attempt to select GERAN or UTRAN radio access technology and proceed with appropriate MM or GMM specific procedures. The UE shall disable the E-UTRA capability (see subclause 4.5).

...

[TS 24.301, clause 4.5]

When the UE supporting the A/Gb and/or Iu mode together with the S1 mode needs to stay in A/Gb or Iu mode, in order to prevent unwanted handover or cell reselection from UTRAN/GERAN to E-UTRAN, the UE shall disable the E-UTRA capability.

-	The UE shall not set the E-UTRA support bits of the MS Radio Access capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12a), the E-UTRA support bits of Mobile Station Classmark 3 IE (see 3GPP TS 24.008 [13], subclause 10.5.1.7) and the ISR support bit of the MS network capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12) in the ATTACH REQUEST message and the ROUTING AREA UPDATE REQUEST message after it selects GERAN or UTRAN;

-	the UE shall use the same value of the EPC capability bit of the MS network capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12) in the ATTACH REQUEST message and the ROUTING AREA UPDATE REQUEST message;

-	the UE NAS layer shall indicate the access stratum layer(s) of disabling of the E-UTRA capability ; and

-	the UE shall change the MS network capability IE (see 3GPP TS 24.008 [13], subclause 10.5.5.12) in the ATTACH REQUEST message and the ROUTING AREA UPDATE REQUEST message after it selects GERAN or UTRAN if any capability bit is subject to change after disabling of the UE's E-UTRA capability.NOTE:	The UE can only disable the E-UTRAN capabilities when in EMM-IDLE mode.

The UE shall enable the E-UTRA capability again in the following cases:

- 	the UE mode of operation changes from CS/PS mode 1 of operation to CS/PS mode 2 of operation;

-	the UE mode of operation changes from PS mode 1 of operation to PS mode 2 of operation; or

-	the UE powers off and powers on again.

...

9.2.3.2.3.3	Test description

9.2.3.2.3.3.1	Pre-test conditions

System Simulator:

-	cell A and cell B;

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (belongs to LAI-1 and RAI-1, home PLMN) is configured;

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, cell 24 (belongs to LAI-1 and RAI-1, home PLMN) is configured;

-	System information combination 10a as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

UE:

-	the UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.3.2.3.3.2	Test procedure sequence

The sequence is executed for execution counter k = 1, 2.

Table 9.2.3.2.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Set the cell type of Cell A to the "Serving cell". Set the cell type of Cell B to the "non-Suitable cell". Set the cell type of Cell 5 or Cell 24 to the "non-Suitable cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 0A | The UE is switched on or the USIM is inserted or the UE is powered on. | - | - | - | - |
| 0B-0M | Steps 2-13 of the generic test procedure in TS 36.508 subclause 4.5.2.3 is performed. NOTE: The UE performs an ATTACH procedure | - | - | - | - |
| 0N | The SS transmits an ATTACH ACCEPT message | <- | ATTACH ACCEPT |  |  |
| 0O-0Qb1 | Steps 16-18b1 of the generic test procedure in TS 36.508 subclause 4.5.2.3 is performed. NOTE: The UE completes the ATTACH procedure and the RRC connection is released. |  |  |  |  |
| 1 | Set the cell type of Cell A to the "non-Suitable cell". Set the cell type of Cell B to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 2 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | Void | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. Note: T3411 is started on the UE | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 6 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 7 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: The step 8 to 13 shall be repeated 3 times. | - | - | - | - |
| 8 | Wait for 10s for T3411 to expire. | - | - | - | - |
| 9 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message at the time when T3411 expired? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 10 | Void | - | - | - | - |
| 11 | Void | - | - | - | - |
| 12 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 13 | The SS releases the RRC connection. | - | - | - | - |
| 13A1 | Set the cell type of cell 5 or cell 24 to ''Suitable neighbour cell'' and wait for 10s for T3411 to expire. | - | - | - | - |
| 13A2 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message at the time when T3411 expired? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 13A3 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 14-19 | Void | - | - | - | - |
| - | EXCEPTION: Steps 20a1 to 20b17 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place according to UE mode of operation. | - | - | - | - |
| 20a1 | IF the UE is configured to operate in CS/PS mode 2 THEN the SS releases the RRC connection. Note: T3402 is started on the UE. | - | - | - | - |
| 20a2 | The SS waits for 60 sec for T3402 to expire. | - | - | - | - |
| 20a3 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 20a4 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 20a5 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 20a6 | The SS releases the RRC connection. | - | - | - | - |
| 20a7 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 20a8 describes behaviour that depends on the UE capability. |  |  |  |  |
| 20a8 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 20b1-20b16 | Void | - | - | - | - |
| 20b17 | IF the UE is configured in CS/PS mode 1 of operation with “IMS voice not available” THEN perform actions specified in Table 9.2.3.2.3.3.2-2 | - | - | - | - |
| 21 | Void | - | - | - | - |

Table 9.2.3.2.3.3.2-2: CS/PS mode 1 of operation behaviour

| - | EXCEPTION: Step 1a1 to 1a2 are optional and depend on UE implementation. They shall be executed if the UE sends a DETACH REQUEST message during [2] seconds. The "lower case letter" identifies a step sequence that takes place if a particular condition specified is met. | - | - | - | - |
| --- | --- | --- | --- | --- | --- |
| 1a1 | UE sends a DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 1a2 | The SS transmits a DETACH COMPLETE message. | <-- | DETACH COMPLETE | - | - |
| 2 | The SS releases the RRC connection. | - | - | - | - |
| - | The following messages are to be observed on Cell 5 or Cell 24 unless explicitly stated otherwise. | - | - | - | - |
| - | EXCEPTION: Step 3a1 to 3b5 are optional and depend on UE implementation. The "lower case letter" identifies a step sequence that takes place if a particular condition specified in the first step is met. | - | - | - | - |
| 3a1 | IF UE has not sent a DETACH REQUEST in step 1a1 AND sends a ROUTING AREA UPDATE REQUEST message during [2] seconds THEN check contents of ROUTING AREA UPDATE REQUEST. | --> | ROUTING AREA UPDATE REQUEST | 3 | P |
| - | EXCEPTION: step 3a1Aa1 to 3a1Aa2 take place if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN | - | - | - | - |
| 3a1Aa1 | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message on Cell 24 | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 3a1Aa2 | The UE transmits an AUTHENTICATION AND CIPHERING RESPONSE message on Cell 24 | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 3a2 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 3a3 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 3b1 | ELSE Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 3 | P |
| - | EXCEPTION: Step 3b2 to 3b3 are performed if UE sent a DETACH REQUEST in step 1a1 | - | - | - | - |
| 3b2 | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 3b3 | The UE transmits an AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 3b4 | The SS transmits an ATTACH ACCEPT message. | <-- | ATTACH ACCEPT | - | - |
| 3b5 | The UE transmits an ATTACH COMPLETE message. | --> | ATTACH COMPLETE | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| - | Exception: Steps 4AAa1 to 4AAa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 4AAa1 | IF UE has sent ROUTING AREA REQUEST at step 3a1 AND pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 4AAa1a1- 4AAa1a4a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 4AAa1a5 | SS releases RRC Connection | - | - | - | - |
| 4AAa1a6 | SS Stops 6s timer | - | - | - | - |
| 4AAa1b1 | The 6s timer expires | - | - | - | - |
| 4A | The UE may attempt the generic procedures for IMS signalling defined in 36.508 4.5A.3A (for cell 5) or 4.5A.3B (for cell 24) | - | - | - | - |
| 5 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 6 describes behaviour that depends on the UE capability. |  |  |  |  |
| 6 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST message | --> | DETACH REQUEST | - | - |

9.2.3.2.3.3.3	Specific message contents

Table 9.2.3.2.3.3.3-0: Message ATTACH ACCEPT (step 0N, Table 9.2.3.2.3.3.2-1)

| Derivation Path: TS 36.508, Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS network feature support | 0000 0000'B | IMS voice over PS session in S1 mode not supported |  |

Table 9.2.3.2.3.3.3-1: TRACKING AREA UPDATE ACCEPT (step 5, Table 9.2.3.2.3.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| GUTI | GUTI-2 |  |  |  |
| EPS update result | 000 | "TA updated" |  |  |
| EMM cause | '0001 0000'B for k=1 or '0001 0001'B for k=2 | MSC temporarily not reachable for k=1 Network failure for k=2 |  |  |
| EPS network feature support | 0000 0000'B | IMS voice over PS session in S1 mode not supported |  |  |

Table 9.2.3.2.3.3.3-2: TRACKING AREA UPDATE REQUEST (step 9 and 20a3, Table 9.2.3.2.3.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | '010'B | Combined TA/LA updating with IMSI attach |  |

Table 9.2.3.2.3.3.3-3: TRACKING AREA UPDATE ACCEPT (step 12, and 13A3 Table 9.2.3.2.3.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update result | 000 | "TA updated" |  |
| GUTI | Not present |  |  |
| EMM cause | '0001 0000'B for k=1 or '0001 0001'B for k=2 | MSC temporarily not reachable for k=1 Network failure for k=2 |  |
| T3402 | ‘00100001’B | 1 minute |  |
| EPS network feature support | 0000 0000'B | IMS voice over PS session in S1 mode not supported |  |

Table 9.2.3.2.3.3.3-4: TRACKING AREA UPDATE ACCEPT (step 20a4, Table 9.2.3.2.3.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | Not present |  |  |
| Location area identification | LAI-1 |  |  |
| MS identity | TMSI-1 |  |  |

Table 9.2.3.2.3.3.3-5: Message ROUTING AREA UPDATE REQUEST (step 3a1, Table 9.2.3.2.3.3.2-2)

| Derivation path: 36.508 table 4.7B.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE network capability | Not Present | UE does not support S1 mode any more |  |

Table 9.2.3.2.3.3.3-6: Message ATTACH REQUEST (step 3b1, Table 9.2.3.2.3.3.2-2)

| Derivation path: 36.508 table 4.7B.2-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE network capability | Not Present | UE does not support S1 mode any more |  |

Table 9.2.3.2.3.3.3-7: DETACH REQUEST (step 1a1, Table 9.2.3.2.3.3.2-2)

Derivation path: 36.508 table 4.7.2-11, condition EPS_only

==============End of change==============