3GPP TSG RAN5 Meeting #109								R5-256361

Dallas, USA, 17 – 21 Nov 2025

Agenda item:	5.3.32.10

Source:	Huawei, HiSilicon

Title:	Discussion on E-UTRA uplink configuration for intra-band ENDC MSD testing

Document for:	Discussion

Background

In RAN5#108, there was an incoming LS [1] from RAN4 on E-UTRA uplink configuration for intra-band ENDC MSD testing. This contribution provides our view on the LS.

1. Overall Description:

RAN4 would like to share with RAN5 the agreements reached to address concerns that some E-UTRA uplink configurations specified as “N/A” for the intra-band contiguous EN-DC MSD requirements may be problematic for conformance testing. An example of such uplink configuration is illustrated in Table 1 for DC_(n)3AA.

Table 1: Example of E-UTRA “N/A” uplink configuration for intra-band contiguous EN-DC MSD - Table 7.3B.2.1-1.

| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | 6.2 | FDD |
|  | n3 | 1770.0 | 30 | 10 (RBstart = 150) | 1865.0 | N/A |  |

RAN4 has made the following agreements:

- E-UTRA uplink carrier frequency “Fc (UL)”: RAN4 recommends configuring the E-UTRA Fc (UL) according to the E-UTRA band’s default Tx-Rx frequency separation specified in TS 36.101 Table 5.7.4-1;

- E-UTRA uplink RB allocation “Lcrb”: RAN4 recommends configuring the E-UTRA UL RB allocation according to the UL configuration specified in TS 36.101 Table 7.3.1-2 of the corresponding E-UTRA DL CBW;

- E-UTRA transmission power: RAN4 recommends configuring the E-UTRA transmit power as anchor agnostic, or preferably at minimum output power.

2. Actions:

To RAN5

ACTION: 	RAN4 respectfully asks RAN5 to take the above into consideration.

Discussion

The LS was based on discussion paper R4-2503242 [2]. The discussion paper explains that the DL REFSENS conformance testing will be impacted by the UL transmission on bands where the UL configurations is set to ‘N/A’ in the MSD table. To avoid this unexpected impact, the LS is sent to RAN5 to recommend some configurations which RAN4 believes minimize the impact from UL transmission.

Observation 1: The intention of the LS is to avoid UL interference from bands with ‘N/A’ UL configuration.

Following sections discuss the status of UL transmission in RAN5 test cases, including PUSCH, PUCCH, and SRS.

PUSCH

In RAN5 test case 7.3B.2.1 REFSESE for Intra-band Contiguous EN-DC in TS 38.521-3, the test configurations are captured in two tables for configurations with MSD exception and without MSD exception respectively. Currently DC_(n)3AA has not yet introduced to RAN5 specification. If the configuration is to be introduced by interested company, it’s expected the UL RB allocation of LTE band 3 will be set to N/A according to the normal practice of RAN5, which means there is no LTE PUSCH scheduling during the DL REFSENS measurement.

SRS

To test the REFSENS of DC_(n)3AA, LTE anchor agnostic approach can’t apply due to there is MSD exception. The default LTE SRS configuration is in section 4.8 of TS 38.521-3 as copied below. LTE SRS will not be transmitted during REFSENS measurement according to the configuration.

Table 4.8-2: PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| soundingRS-UL-ConfigDedicated | Not present |  | RBC |
| } |  |  |  |

PUCCH

Similar as SRS, the default LTE PUCCH configuration for CQI report could be found in section 4.8 of TS 38.521-3. LTE PUCCH for CQI report will not be transmitted during REFSENS measurement according to the configuration.

Table 4.8-1: CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | NOT PRESENT |  |  |
| cqi-ReportPeriodic | NOT PRESENT |  |  |
| } |  |  |  |

Observation 2: According to current RAN5 specification, there is no PUSCH/SRS/PUCCH for CQI transmission on LTE bands.

Since the LTE PDSCH needs to be scheduled and measured, the LTE PUCCH will be transmitted to report the HARQ-ACK information. The band 3 is FDD band, and the HARQ-ACK is sent by the UE on subframe n when the PDSCH is received on subframe n-4. The PDSCH retransmission will be received on subframe n+4 if needed.

Observation 3: The impact from PUCCH for HARQ-ACK transmission needs to be addressed.

To avoid LTE UL interference, the LTE PDSCH transmission could be configured as discontinuously, e.g. the figure below. With this specific configuration, the measurement of LTE/NR PDSCH is never interfered by LTE UL transmission, but the test time will be doubled.

Another option is keeping the current scheduling pattern, while configuring the LTE PUCCH transmit power to the minimum output power following RAN4’s recommendation. This option might increase the risk of PUCCH decoding error, and needs further evaluation from TE vendors.

Proposal 1: RAN5 to consider following options for handling PUCCH for HARQ-ACK transmission.

- Option 1: The LTE PDSCH transmission is scheduled as discontinuously so that the PUCCH transmission will not interfere the PDSCH measurement. The test time is doubled.

- Option 2: The LTE PUCCH transmission is set to the minimum output power. The PUCCH decoding performance needs double check.

Consideration on TDD band

For TDD bands, there is a step in the initial condition of 7.3B.2.1 REFSENS for intra-band contiguous EN-DC:

11.	When E-UTRA and NR bands are TDD, ensure the same uplink-downlink configuration by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

This step ensures the same UL/DL configuration on E-UTRA and NR bands, therefore the uplink in either band will not impact the DL REFSENS performance.

Observation 4: No change is needed for TDD band due to the same UL/DL configuration on E-UTRA and NR bands.

Summary

Based on the discussion above, we have following observations and proposals for RAN5 to consider:

Observation 1: The intention of the LS is to avoid UL interference from bands with ‘N/A’ UL configuration.

Observation 2: According to current RAN5 specification, there is no PUSCH/SRS/PUCCH for CQI transmission on LTE bands.

Observation 3: The impact from PUCCH for HARQ-ACK transmission needs to be addressed.

Proposal 1: RAN5 to consider following options for handling PUCCH for HARQ-ACK transmission.

- Option 1: The LTE PDSCH transmission is scheduled as discontinuously so that the PUCCH transmission will not interfere the PDSCH measurement. The test time is doubled.

- Option 2: The LTE PUCCH transmission is set to the minimum output power. The PUCCH decoding performance needs double check.

Observation 4: No change is needed for TDD band due to the same UL/DL configuration on E-UTRA and NR bands.

Reference

[1] R5-253716, LS on E-UTRA uplink configurations for intra-band ENDC MSD , TSG WG RAN4, RAN5#108

[2] R4-2503242, Clarifications on valid 1UL E-UTRA configuration for EN-DC intra-band contiguous MSD, Skyworks Solutions, RAN4#114-bis