3GPP TSG-RAN5 Meeting #109	R5-256297

# Dallas, US, 17th – 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3657 | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates in PDSCH config for eRedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Table 5.4.2.0-26 defines PDSCH resource allocation type = Type 0 for all Demod test cases. However, for eRedCap CQI and PMI test cases, the number of RB that should be configured in the PDSCH according to DL RMC definition can not be achieved using a PDSCH resource allocation type 0 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include new table for PDSCH-Config to configure PDSCH resource allocation type = Type 1 in section 5.4.2.4 for eRedCap CQI reporting test cases. Update Table 5.4.2.5-18 for PDSCH-Config to include resource allocation type = Type 1 for eRedCap PMI test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DL RMC defined for eRedCap CQI and eRedCap PMI test cases will not be configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.4, 5.4.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Dependencies with RAN5 CR R5-256294 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 5.4.2.4	Message contents for CQI reporting requirements

## <<< Skip unchanged sections >>>

CSI-ResourceConfig

Table 5.4.2.4-18: CSI-ResourceConfig for CSI Acquisition

| Derivation Path: Table 5.4.2.0-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | 1 |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 1 | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

PDSCH-Config

Table 5.4.2.4-19: PDSCH-Config

| Derivation Path: Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | eRedCap |
| } |  |  |  |

#### 5.4.2.5	Message contents for PMI reporting requirements

## <<< Skip unchanged sections >>>

PDSCH-Config

Table 5.4.2.5-18: PDSCH-Config

| Derivation Path: Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | eRedCap |
| aperiodic-ZP-CSI-RS-ResourceSetsToAddModList | Not present |  | 2Tx, 4Tx, 8Tx |
|  | aperiodic-ZP-CSI-RS-ResourceSetsToAddModList |  | 16Tx, 32Tx |
| p-ZP-CSI-RS-ResourceSet | p-ZP-CSI-RS-ResourceSet |  | 2Tx, 4Tx, 8Tx |
|  | Not present |  | 16Tx, 32Tx |
| } |  |  |  |

## <<< END OF CHANGES >>>