3GPP TSG-RAN5 Meeting #109	R5-256651

# Dallas, US, Nov.17th – 21st 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5229 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Cov Enh phase 2 MAC test case for DCI 0_2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Cov Enh phase 2 test case for DCI 0_2 7.1.1.3.25 Correct Handling of UL grant / Dynamic waveform switch / DCI format 0_2, needs to be added as per work plan |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added cov enh phase 2 new test case 7.1.1.3.25 Correct Handling of UL grant / Dynamic waveform switch / DCI format 0_2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing test coverage impacting interoperability |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table 7.1.1.3.24.3.3-4: Physical layer parameters for DCI format 0_1 (Step 3, 7, 11 and 15, Table 7.1.1.3.24.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.1.2-1 with condition DWS |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Transform precoder indicator | A value of "0" indicates that transform precoder is enabled, i.e. DFT-s-OFDM waveform is used. | “0” | Step 3, Step 11 |
|  | A value of "1" indicates that transform precoder is disabled, i.e. CP-OFDM waveform is used. | “1” | Step 7, Step 15 |

##### 7.1.1.3.25 	Correct Handling of UL grant / Dynamic waveform switch / DCI format 0_2

7.1.1.3.25.1	Test Purpose (TP)

(1)

with { UE is in RRC_connected state and is configured with transformPrecoder = disabled and dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 in active BWP }

ensure that {

when { UE receives an DCI format 0_2 with Transform precoder indicator = 0 }

then { UE transmits PUSCH with DFT-s-OFDM waveform }

}

(2)

with { UE is in RRC_connected state and is configured with msg3-transformPrecoder not configured, transformPrecoder not configured and dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 in active BWP }

ensure that {

when { UE receives an DCI format 0_2 with Transform precoder indicator = 0 }

then { UE transmits PUSCH with DFT-s-OFDM waveform }

}

(3)

with { UE is in RRC_connected state and is configured with transformPrecoder = enabled and dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 in active BWP }

ensure that {

when { UE receives an DCI format 0_2 with Transform precoder indicator = 1 }

then { UE transmits PUSCH with CP-OFDM waveform }

}

(4)

with { UE is in RRC_connected state and is configured with msg3-transformPrecoder = enabled, transformPrecoder not configured and dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 in active BWP }

ensure that {

when { UE receives an DCI format 0_2 with Transform precoder indicator = 1 }

then { UE transmits PUSCH with CP-OFDM waveform }

}

7.1.1.3.25.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212, clause 7.3.1.1.3 and TS 38.214 clause 6.1.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.212, clause 7.3.1.1.3]

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

…

-	Transform precoder indicator - 0 or 1 bit

-	1 bit if the higher layer parameter dynamicTransformPrecoderFieldPresenceDCI-0-2 is configured to 'enabled' and if the UE is configured to monitor DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI, where the bit value of 0 indicates that transform precoder is enabled and the bit value of 1 indicates that transform precoder is disabled. For a DCI format 0_2 with CRC scrambled by CS-RNTI and the value indicated by new data indicator field is 0, or for a DCI format 0_2 with CRC scrambled by SP-CSI-RNTI, the bit is reserved.

-	0 bit otherwise.

…

[TS 38.214, clause 6.1.3]

...

For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI:

…

-	If the DCI with the scheduling grant was not received with DCI format 0_0

-	If the DCI with the scheduling grant was received with DCI format 0_1 or 0_2 with CRC scrambled by C-RNTI, MCS-RNTI, or CS-RNTI with NDI=1 and if the UE is configured with a higher layer parameter dynamicTransformPrecoderIndicationDCI-0-1 in pusch-Config for DCI format 0_1 or dynamicTransformPrecoderIndicationDCI-0-2 in pusch-Config for DCI format 0_2 and the higher layer parameter is set to 'enabled',

-	the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the Transform precoder indicator field in the DCI with the scheduling grant.

…

-	If resourceAllocation in pusch-Config for DCI format 0_1 or resourceAllocationDCI-0-2 in pusch-Config for DCI format 0_2 is set to resourceAllocationType0, or if the resource allocation is set to resource allocation type 0 according to the DCI configuration as described in clauses 7.3.1.1.2 and 7.3.1.1.3 of [6, TS 38.212], or if dmrs-Type in DMRS-UplinkConfig is set to ‘type 2’ for this PUSCH transmission, the UE does not expect that the Transform precoder indicator field in the DCI with the scheduling grant indicates that transform precoding is enabled.

-	If the UE is configured with the higher layer parameter dmrs-TypeEnh in DMRS-UplinkConfig, and if the scheduling grant indicates that transform precoding is enabled for the scheduled PUSCH transmission, the UE ignores the higher layer parameters dmrs-TypeEnh in DMRS-UplinkConfig, if configured, for the DM-RS transmission of the scheduled PUSCH transmission.

-	Otherwise,

-	If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter.

-	If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder.

For PUSCH transmission with a configured grant

-	If the UE is configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter.

-	If the UE is not configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder.

…

7.1.1.3.25.3	Test description

7.1.1.3.25.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0.

7.1.1.3.25.3.2	Test procedure sequence

Same Test procedure sequence as in table 7.1.1.3.24.3.2-1, except UL DCI format 0_2 is used in place of 0_1.

7.1.1.3.25.3.3	Specific message contents

Table 7.1.1.3.25.3.3-1: RRCReconfiguration (Step 1, 5, 9 and 13, Table 7.1.1.3.25.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) Table 7.1.1.3.25.3.3-2 | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) Table 7.1.1.3.25.3.3-2 | NR |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.25.3.3-2: CellGroupConfig (Table 7.1.1.3.25.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
| reconfigurationWithSync | Not present |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 7.1.1.3.25.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.25.3.3-3: ServingCellConfig (Table 7.1.1.3.25.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 with condition DLBWP_add and ULBWP_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 1 entry |  |  |
| BWP-Uplink[1] SEQUENCE { |  | entry 1 |  |
| bwp-Common SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| msg3-transformPrecoder | Not present |  | Step 1, Step 5, Step 9 |
|  | enabled |  | Step 13 |
| } |  |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| transformPrecoder | disabled |  | Step 1 |
|  | enabled |  | Step 5 |
|  | Not present |  | Step 9, Step 13 |
| dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.25.3.3-4: Physical layer parameters for DCI format 0_2 (Step 3, 6, 11 and 14, Table 7.1.1.3.25.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.1.3-1 with condition DWS and TRANSFORM_PRECODER_ENABLED |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Transform precoder indicator | A value of "0" indicates that transform precoder is enabled, i.e. DFT-s-OFDM waveform is used. | “0” |  |

Table 7.1.1.3.25.3.3-5: Physical layer parameters for DCI format 0_2 (Step 2, 7, 10 and 15, Table 7.1.1.3.25.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.1.3-1 with condition DWS |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Transform precoder indicator | A value of "1" indicates that transform precoder is disabled, i.e. CP-OFDM waveform is used. | “1” |  |