3GPP TSG-RAN WG5 Meeting #109	R5-256941

# Dallas, US, 17 - 21 Nov, 2025

Title:	Discussion on work plan for NR_channel_raster_enh-UEConTest

Agenda item:	6.3.35.3

WI/SI:	NR_channel_raster_enh-UEConTest

Release:	Rel-18

Source:	China Telecom

Document for:	Discussion

# 1 Background

This contribution will discuss the work plan for WI NR_channel_raster_enh-UEConTest.

# 2 Discussion

The major change in the parent WI (NR_channel_raster_enh-Core) under RAN4 is the introduction of the enhanced channel raster for FR1 bands that currently use a 100 kHz channel raster. Also, a new UE capability signalling is introduced to indicate the support of this new feature.

As for the work in RAN5, we believe most of the tasks relate to the modifications in 38.508-1. Given the large number of bands involved, which makes it like a "basket WI". So, we have the work plan like this:

Observation 1: The primary RAN5 task entails updating the test frequency table in TS 38.508-1 to incorporate the enhanced channel raster.

Proposal 1: To this end, we should enumerate all impacted bands in the work plan, as illustrated in the prior example.

The parent RAN4 work item introduced the enhanced channel raster and an associated UE capability, without defining any other new performance requirements. Based on agreements from the online discussion, it was confirmed that only the reference sensitivity test case in clause 7.3 is needed. This is due to the potential configuration of the channel bandwidth close to the lower or upper edge of the carrier. Consequently, a normative note is necessary to verify that UEs supporting the enhanced channel raster function correctly in this scenario.

Table 7.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1[x] |  |  | Low range, Mid range, High range (NOTE 4) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest Lowest UL / Lowest DL, Lowest UL / Highest DL (NOTE 3) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: Additional test points selected according to asymmetric channel bandwidths specified in clause 5.3.6. DL channel bandwidth shall be selected first. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. NOTE 5: In a band where UE supports 4Rx but not supports 8Rx, the test needs to be repeated with only 2Rx antennas connected and the other antennas terminated. In a band where UE supports 8Rx, the test needs to be repeated with only 4Rx antennas connected and the other antennas terminated, and the test needs to be repeated with only 2Rx antennas connected and the other antennas terminated. NOTE x: In a band where UE supports enhanced channel raster, the test needs to be repeated using enhanced channel raster specified in TS 38.508-1 subclause 4.3.1 for low range and high range with highest channel bandwidth. |  |  |  |  |

Proposal 2: It is proposed to have one reference sensitivity test case (single carrier) for this feature by introducing a note to the existing reference sensitivity test cases for the verification of UEs supporting enhanced channel raster.

# 3 Conclusion

Observation 1: The primary RAN5 task entails updating the test frequency table in TS 38.508-1 to incorporate the enhanced channel raster.

Proposal 1: To this end, we should enumerate all impacted bands in the work plan.

Proposal 2: It is proposed to have one reference sensitivity test case (single carrier) for this feature by introducing a note to the existing reference sensitivity test cases for the verification of UEs supporting enhanced channel raster.