3GPP TSG-RAN5 Meeting #109			R5-256982

# Dallas, USA, 17th Nov 2025 – 21st Nov 2025

Agenda item:	5.3.43.3

Source: 	Qualcomm Inc

Title: 	discussion on satellite visibility with sat prop model

Document for:	Discussion and Endorsement

## 1.	Introduction

RAN4 introduced Rel19 satellite propagator model as part of the Rel19 WI on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN [1]. This paper discusses the potential issues related to maintaining satellite visibility during the entire test case with the Rel19 satellite propagator model implemented in the test equipment.

## 2.	Discussion

Current mode of operation for existing Rel17/18 Demodulation and RRM test requirements defined in 38.133 [2] and 38.101-5 [3] under the WIC NR_NTN_solutions and NR_NTN_enh is, there is a satellite position signalled to the UE via SIB19 ephemeris. The satellite ephemeris is not changed across SIB19 intervals. So it is emulating a fixed satellite snapshot scenario which is unrealistic for the UE, but has been used to enable the NR-NTN conformance testing so far.

For a real UE deployed in the field, once it decodes SIB19 ephemeris will start predicting the satellite positions until it again decodes SIB19 before the ulSyncValidity timer expires to maintain synchronization with the satellite. Between the successive SIB19 decodes, UE adjusts its UL by pre-compensating the delay and doppler it estimates based on predicted satellite position and its current position.

In order to facilitate the NR-NTN conformance testing using the existing methodology, UE’s position is signalled via preconfigured means and UE is required to disable its satellite trajectory prediction. Also the satellite position is chosen such that, it is within UE’s visibility. With this approach, there is no orbit emulation needed and no concerns regarding maintaining satellite visibility to the UE throughout the test duration. But as commented above, it is an unrealistic way of NR-NTN testing.

With the Rel19 satellite propagator model introduced for verifying Demodulation and RRM requirements, satellite orbit emulation will cause the satellite visibility to change for the UE depending on the time elapsed during the orbit emulation. Figure 1 below illustrates that. UE’s ground location is assumed to be fixed. A satellite’s orbit is emulated by the Test equipment and the SIB19 ephemeris signalled to the UE every X ms as per the orbit emulation. Additionally Test equipment needs to apply the delay and doppler corresponding to the emulated satellite orbit to the generated DL signals. For NGSO satellites, the satellite visibility typically in the order of few minutes.

Figure 1: satellite visibility related aspects

Observation1: While designing the test procedure to verify Demod and RRM requirements using the Rel19 satellite propagator model, maintaining satellite visibility for the UE right from attach to the end of the test case is critical.

In 38.508-1[4], there are LEO600 orbit and LEO1200 orbit. All the LEO1200 orbits belong to the same satellite at different time instance and elevation. Similarly for various LEO600 orbits.

LEO1200 SIB19 ephemeris

Table 5.6.2.1-3: SIB19- NR NTN Ephemeris information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

Table 7.6.2.1-3a: SIB19- NR NTN serving cell Ephemeris information (stateVectors format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2935552 |  |  |
| positionY-r17 | 4895460 |  |  |
| positionZ-r17 | 1188458 |  |  |
| velocityVX-r17 | 8506 |  |  |
| velocityVY-r17 | -23314 |  |  |
| velocityVZ-r17 | 118253 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: stateVectors ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-1 in TS38.133 [13]. |  |  |  |

Table 7.6.2.1-3c: SIB19- NR NTN neighbour cells Ephemeris information (stateVectors format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: TS 38.508-1 [12], Table 4.6.2-18C, Table 7.6.3.1-1 and Table 7.6.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-NeighCellConfigList-r17 SEQUENCE (SIZE(1..maxCellNTN-r17)) OF NTN-NeighCellConfig-r17{ | 1 entry for test cases requiring only one neighbour cell 2 entries in test cases requiring two neighbour cells |  | 2CELLS OR 3CELLS |
| NTN-NeighCellConfig-r17 [1] SEQUENCE { |  | entry 1 | 2CELLS or 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2470752 |  |  |
| positionY-r17 | 3858366 |  |  |
| positionZ-r17 | 3624634 |  |  |
| velocityVX-r17 | 31542 |  |  |
| velocityVY-r17 | -67516 |  |  |
| velocityVZ-r17 | 94597 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NTN-NeighCellConfig-r17 [2] SEQUENCE { |  | entry 2 | 3CELLS |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 150.02 degrees, one-way delay equal to 6.79 ms and Doppler equal to -17.65 ppm. Neighbour cell 2 Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-1 in TS38.133 [13]. |  |  |  |

LEO600 SIB19 ephemeris

Table 5.6.2.1-2: SIB19- NR NTN Ephemeris information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 |  |  |
| positionY-r17 | 4151097 |  |  |
| positionZ-r17 | 2263034 |  |  |
| velocityVX-r17 | 21970 |  |  |
| velocityVY-r17 | -48164 |  |  |
| velocityVZ-r17 | 114109 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm |  |  |  |

For the UE location specified in 38.508-1 [4], for LEO1200 orbit anchored at current UTC time (at the time of writing this paper), there is atleast one visibility pass per day spanning typically from 6-8 minutes. Similar visibility pass duration is observed if a different geographic UE location is chosen.

For the LEO1200 orbit defined in RAN5 spec, one can expect 6-8 minutes of satellite visibility.

For LEO600, one can expect 4-5 minutes of satellite visibility.

Proposal1: RAN5 to define a LEO600 orbit that maximizes the visibility ( ~ 5 minutes) for elevation of 30 deg. LEO1200 orbit that maximizes the visibility (~ 8 minutes) for elevation of 30 deg. It is not precluded to lower the elevation angle if longer visibility is desired for the test ensuring RAN4 side conditions are met.

Now, the question is whether 6-8 minutes of satellite visibility time is sufficient to run all the Demodulation and RRM test cases currently defined in 38.521-5 [5] and 38.533 [6] spec.

To clarify, start to finish encompasses the below

- TE setup time for the test until SIB19 is sent. Orbit emulation time starts with SIB19 transmission

- Wait for UE attach

- TE setup time to configure actual test conditions (NTN-TDL channel model, test SNR etc

- KPI measurement time

Based on our analysis, the various test cases can be bucketed into following categories.

Category1: the entire test case from start to finish can be completed within 6 minutes. For example PDSCH demodulation test cases

Category2: entire test case from start to finish spanning multiple iterations requires more than 6-8 minutes. But single iteration run is within 6 minutes. For eg, idle mode reselection test cases

Category3:  entire test case from start to finish requires more than 6-8 minutes and the test cannot be split into multiple parts/iterations.

For category 1 test cases, there are no issues regarding maintaining satellite visibility for the entire duration of the test.

Observation2: For category 1 test cases, there are no issues regarding maintaining satellite visibility for the entire duration of the test.

For category 2 test cases, our proposal is to power on/off the UE between iterations so that the same orbit emulation can be rerun across UE power cycles. This will ensure satellite visibility for each iteration.

Observation3: For category 2 test cases, satellite visibility can be maintained per each iteration by power cycling the device between iterations.

For category 3 test cases, Based on our analysis, there isn’t any test case currently defined in [5] or [6] that exceeds the 6-8 minutes visibility time. If such a test case comes up in future release, one option is to explore triggering satellite handover(s) during the test. During handover duration, KPI measurement will need to be stopped. All this requires careful consideration and needs to be handled case by case.

Observation4: There are currently no category 3 test cases currently defined in Demodulation or RRM spec. Such test cases need to be handled on a case-by-case basis by providing detailed analysis on how to ensure satellite visibility throughout the test.

Proposal2: RAN5 to develop the RRM (atleast one iteration) test cases with Rel19 satellite propagator channel model with the assumption that entire test from start (SIB19 start time) to the end of the measurements can be run with full satellite visibility. Further investigate for DEMOD if there is any issue related to satellite visibility .

Next, we analyse what information to be included to ensure the device under test can be consistently tested across various test platforms.

For satellite orbit emulation, the UE location (ground observer), satellite information, orbit emulation start time and minimum elevation mask is all that is needed.

For all tests, minimum elevation mask of 30 degrees is required. Fixed UE location (s) can be provided corresponding to different geographical regions as needed.

For orbit emulation start time, any start time in the past can potentially be used. Key consideration is how recent is the orbit emulation time w.r.t current time. As the difference in time becomes larger, the satellite visibility and pass timing can completely change.

For the SIB19 ephemeris generation for the test, following options were discussed. Other options are not precluded.

Option1: Live orbit emulation with pre-determined orbit emulation epoch time

Option2: Using a predetermined date & time, generate the entire Ephemeris file with values provided every 640ms granularity. Also indicate the calculated delay/doppler for each ephemeris set that TE needs to apply to the DL signal during the test.

If Ephemeris is generated every 640ms, RAN4 requirement is to do interpolation between delay and doppler points every 1ms

Option3: Using a predetermined date & time, generate the entire ephemeris file with values provided with 1ms granularity.

Indicate the calculated delay/doppler for each ephemeris set that TE needs to apply to the DL signal during the test.

Proposal3: RAN5 to take into consideration the outcome of the RAN4 Rel19 satellite motion based time varying NTN  channel model to specify all the parameters (for eg UE location, min elevation, satellite information (PVT/Orbital) including initial ephemeris information, orbit emulation epoch time) in 38.508-1.

## 3.	Conclusion

Observation1: While designing the test procedure to verify Demod and RRM requirements using the Rel19 satellite propagator model, maintaining satellite visibility for the UE right from attach to the end of the test case is critical.

Proposal1: RAN5 to define a LEO600 orbit that maximizes the visibility ( ~ 5 minutes) for elevation of 30 deg. LEO1200 orbit that maximizes the visibility (~ 8 minutes) for elevation of 30 deg. It is not precluded to lower the elevation angle if longer visibility is desired for the test ensuring RAN4 side conditions are met.

Observation2: For category 1 test cases, there are no issues regarding maintaining satellite visibility for the entire duration of the test.

Observation3: For category 2 test cases, satellite visibility can be maintained per each iteration by power cycling the device between iterations.

Observation4: There are currently no category 3 test cases currently defined in Demodulation or RRM spec. Such test cases need to be handled on a case-by-case basis by providing detailed analysis on how to ensure satellite visibility throughout the test.

Proposal2: RAN5 to develop the RRM (atleast one iteration) test cases with Rel19 satellite propagator channel model with the assumption that entire test from start (SIB19 start time) to the end of the measurements can be run with full satellite visibility. Further investigate for DEMOD if there is any issue related to satellite visibility.

Proposal3: RAN5 to take into consideration the outcome of the RAN4 Rel19 satellite motion based time varying NTN  channel model to specify all the parameters (for eg UE location, min elevation, satellite information (PVT/Orbital) including initial ephemeris information, orbit emulation epoch time) in TS 38.508-1.

## 4.		References

[1]	Rel19 WI UID 1032086, WIC NR_IoT_NTN_req_test_enh-Perf,    Perf. Part: Enhanced requirements and conductive test methodology for NR NTN and IoT NTN

[2]	TS 38.133 v19.2.0

[3]	TS 38.101-5 v19.2.0

[4]	TS 38.508-1 v19.1.1

[5]	TS 38.521-5 v18.7.1

[6]	TS 38.533 v19.0.0