3GPP TSG-RAN5 Meeting #108	R5-253750r1

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5034 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Idle mode TC for NR channel BW less than 5MHz for FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To apply for NR channel BW less than 5MHz for FR1, some Idle mode test cases shall be updated based on TS 38.331 and TS 36.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SIB4 and SIB24 were updated for the impacted Idle mode TC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.4.3.3, 6.1.2.5.3.3, 6.2.2.1.3.3, 6.2.2.2.3.3, 6.2.3.1.3.3, 6.2.3.2.3.3, 6.2.3.3.3.3, 6.2.3.4.3.3, 6.2.3.5.3.3, 6.2.3.6.3.3, 6.2.3.7.3.3, 6.2.3.8.3.3, 6.2.3.10.3.3, 6.2.3.11.3.3 and 6.4.3.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: updated the SIB4 and SIB24 referring to TS 38.508-1 and TS 36.508. removed MFBI TC 6.1.2.23 as it is not applicable to less than 5 MHz. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

6.1.2.4.3.3	Specific message contents

Table 6.1.2.4.3.3-1: Conditions for specific message contents in Tables below

| Condition | Explanation |
| --- | --- |
| NR Cell 1 | This condition applies to system information transmitted on NR Cell 1. |
| NR Cell 10 | This condition applies to system information transmitted on NR Cell 10. |
| BW3 | This condition applies to system information transmitted on NR cell with NR channel BW less than 5 MHz for FR1. |

Table 6.1.2.4.3.3-2: SIB2 of NR Cell 1 and NR Cell10 (preamble and all steps, Table 6.1.2.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE{ |  |  |  |
| cellReselectionPriority | 1 |  | NR Cell 1 |
|  | 5 |  | NR Cell 10 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.4.3.3-3: SIB4 of NR Cell 1 and NR Cell 10 (preamble and all steps, Table 6.1.2.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| InterFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq | Same downlink ARFCN as used for NR Cell 10 |  | NR Cell 1 |
|  | Same downlink ARFCN as used for NR Cell 1 |  | NR Cell 10 |
|  | 250 |  | BW3 |
| frequencyBandList | MultiFrequencyBandListNR-SIB |  |  |
|  | MultiFrequencyBandListNR-SIB with condition LessThan5MHz |  | BW3 |
| t-ReselectionNR | 7 | seconds |  |
| threshX-HighP | 10 | 20 dB |  |
| cellReselectionPriority | 5 |  | NR Cell 1 |
|  | 1 |  | NR Cell 10 |
| } |  |  |  |
| } |  |  |  |
| InterFreqCarrierFreqList-v1800 | Not present |  |  |
| InterFreqCarrierFreqList-v1800 SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1800 { | 1 entry |  | BW3 |
| InterFreqCarrierFreqInfo-v1800[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq-r18 | Same downlink ARFCN as used for NR Cell 10 |  | NR Cell 1 |
|  | Same downlink ARFCN as used for NR Cell 1 |  | NR Cell 10 |
| frequencyBandList-r18 SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfo { | 1 entry |  |  |
| NR-MultiBandInfo[1] SEQUENCE { |  | entry 1 |  |
| freqBandIndicatorNR | Same band as used for NR Cell 10 |  | NR Cell 1 |
|  | Same band as used for NR Cell 1 |  | NR Cell 10 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[Skip unchanged text]

6.1.2.5.3.3	Specific message contents

Table 6.1.2.5.3.3-1: Conditions for specific message contents in Tables below

| Condition | Explanation |
| --- | --- |
| NR Cell 1 | This condition applies to system information transmitted on NR Cell 1. |
| NR Cell 10 | This condition applies to system information transmitted on NR Cell 10. |
| BW3 | This condition applies to system information transmitted on NR cell with NR channel BW less than 5 MHz for FR1. |

Table 6.1.2.5.3.3-2: SIB1 of NR Cell 1 (preamble and all steps, Table 6.1.2.5.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon SEQUENCE{ |  |  |  |
| uplinkConfigCommon SEQUENCE{ |  |  |  |
| FrequencyInfoUL SEQUENCE { |  |  |  |
| p-Max | 33dBm |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.5.3.3-3: SIB2 of NR Cell 1 and NR Cell10 (preamble and all steps, Table 6.1.2.5.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE{ |  |  |  |
| threshServingLowP | 9 | 18 dB | NR Cell 1 |
| cellReselectionPriority | 5 |  | NR Cell 1 |
|  | 1 |  | NR Cell 10 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.5.3.3-4: SIB4 of NR Cell 1 and NR Cell 10 (preamble and all steps, Table 6.1.2.5.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| InterFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq | Same downlink ARFCN as used for NR Cell 10 |  | NR Cell 1 |
|  | Same downlink ARFCN as used for NR Cell 1 |  | NR Cell 10 |
|  | 250 |  | BW3 |
| frequencyBandList | MultiFrequencyBandListNR-SIB |  |  |
|  | MultiFrequencyBandListNR-SIB with condition LessThan5MHz |  | BW3 |
| p-Max | 33dBm |  | NR Cell 1 |
| t-ReselectionNR | 7 | seconds |  |
| threshX-HighP | 14 | 28 dB |  |
| threshX-LowP | 6 | 12 dB |  |
| cellReselectionPriority | 1 |  | NR Cell 1 |
|  | 5 |  | NR Cell 10 |
| } |  |  |  |
| } |  |  |  |
| InterFreqCarrierFreqList-v1800 | Not present |  |  |
| InterFreqCarrierFreqList-v1800 SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1800 { | 1 entry |  | BW3 |
| InterFreqCarrierFreqInfo-v1800[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq-r18 | Same downlink ARFCN as used for NR Cell 10 |  | NR Cell 1 |
|  | Same downlink ARFCN as used for NR Cell 1 |  | NR Cell 10 |
| frequencyBandList-r18 SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfo { | 1 entry |  |  |
| NR-MultiBandInfo[1] SEQUENCE { |  | entry 1 |  |
| freqBandIndicatorNR | Same band as used for NR Cell 10 |  | NR Cell 1 |
|  | Same band as used for NR Cell 1 |  | NR Cell 10 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[Skip unchanged text]

6.2.2.1.3.3	Specific message contents

Table 6.2.2.1.3.3-1: SIB5 of NR Cell 1(preamble and all steps, Table 6.2.2.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 1 entry |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink EUTRA ARFCN as E-UTRA Cell 1 used |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.1.3.3-2: SystemInformationBlockType24 of EUTRA Cell 1 (preamble and all steps, Table 6.2.2.1.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

Table 6.2.2.1.3.3-3: SIB1 for NR Cell 1 (step 5, Table 6.2.2.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellReservedForOtherUse | true | The UE shall treat this cell as if cell status is "barred" |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.1.3.3-4: REGISTRATION REQUEST (step 4, Table 6.2.2.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.9.9.2.3-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS NAS message container | TRACKING AREA UPDATE REQUEST message | See Table 6.2.2.1.3.3-5 |  |

Table 6.2.2.1.3.3-5: TRACKING AREA UPDATE REQUEST (Table 6.2.2.1.3.3-4)

Derivation Path: TS 38.508-1 [4], Table 4.9.9.2.3-2 with condition Mapped EPS security context.

[Skip unchanged text]

6.2.2.2.3.3	Specific message contents

Table 6.2.2.2.3.3-1: SIB5 of NR Cell 1(preamble and all steps, Table 6.2.2.2.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { |  |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink EUTRA ARFCN as E-UTRA Cell 1 used |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.2.3.3-2: SystemInformationBlockType24 of EUTRA Cell 1 (preamble and all steps, Table 6.2.2.2.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

Table 6.2.2.2.3.3-3: SystemInformationBlockType1 for E-UTRA Cell 1 (step 5, Table 6.2.2.2.3.2-3)

| Derivation Path: TS 36.508-1 [7], Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellBarred | barred |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.2.3.3-4: REGISTRATION REQUEST (step 4, Table 6.2.2.2.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.9.9.2.3-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS NAS message container | TRACKING AREA UPDATE REQUEST message | See Table 6.2.2.2.3.3-5 |  |

Table 6.2.2.2.3.3-5: TRACKING AREA UPDATE REQUEST (Table 6.2.2.2.3.3-4)

Derivation Path: TS 38.508-1 [4], Table 4.9.9.2.3-2 with condition Mapped EPS security context.

[Skip unchanged text]

6.2.3.1.3.3	Specific message contents

Table 6.2.3.1.3.3-1: SystemInformationBlockType3 of EUTRA Cell 1 (preamble and all steps, Table 6.2.3.1.3.2-3)

| Derivation Path: TS 36.508 [7], Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 10 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.1.3.3-2: SystemInformationBlockType24 of EUTRA Cell 1 (preamble, Table 6.2.3.1.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Downlink NR ARFCN of SSB for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| cellReselectionPriority-r15 | 3 |  |  |
| threshX-High-r15 | 20 | 40 dB |  |
| threshX-Low-r15 | 10 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionNR-r15 | 7 |  |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Downlink NR ARFCN of SSB for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

Table 6.2.3.1.3.3-3: SIB2 of NR Cell 1(preamble and all steps, Table 6.2.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLowP | 10 | 20 dB |  |
| cellReselectionPriority | 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.1.3.3-4: SIB5 of NR Cell 1(preamble and all steps, Table 6.2.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 1 entry |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink EUTRA ARFCN as E-UTRA Cell 1 used |  |  |
| cellReselectionPriority | 5 |  |  |
| threshX-High | 10 | 20 dB |  |
| threshX-Low | 20 | 40 dB |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionEUTRA | 7 |  |  |
| } |  |  |  |

Table 6.2.3.1.3.3-5: Paging (step 8, Table 6.2.3.1.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| systemInfoModification | True |  |  |
| } |  |  |  |

Table 6.2.3.1.3.3-6: SystemInformationBlockType1 of EUTRA Cell 1 (step 7A, Table 6.2.3.1.3.2-3)

| Derivation Path: TS 36.508 [7], Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| } |  |  |  |

Table 6.2.3.1.3.3-7: SystemInformationBlockType24 of EUTRA Cell 1 (step 7A, Table 6.2.3.1.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| cellReselectionPriority-r15 | 5 |  |  |
| threshX-High-r15 | 10 | 20 dB |  |
| threshX-Low-r15 | 20 | 40 dB |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionNR-r15 | 7 | 7 Seconds |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

Table 6.2.3.1.3.3-8: TRACKING AREA UPDATE REQUEST (Step 7, Table 6.2.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.9.7.2.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '010'B | Combined TA/LA updating | combined_TA_LA |
|  | '000'B | TA updating | TA_only |
| NOTE: The message shall be integrity protected using the 5GS security context available in the UE. |  |  |  |

| Condition | Explanation |
| --- | --- |
| TA_only | This condition applies if the UE is configured to initiate EPS attach or if explicitly specified. |
| combined_TA_LA | This condition applies if the UE is configured to initiate combined EPS/IMSI attach or if explicitly specified. |

Table 6.2.3.1.3.3-9: REGISTRATION REQUEST (Step 12, Table 6.2.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-6. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS NAS message container | TRACKING AREA UPDATE REQUEST message | See Table 6.2.3.1.3.3-10 |  |

Table 6.2.3.1.3.3-10: TRACKING AREA UPDATE REQUEST (Table 6.2.3.1.3.3-9)

Derivation Path: TS 38.508-1 [4], Table 4.9.9.2.3-2 with condition Mapped EPS security context

[Skip unchanged text]

6.2.3.2.3.3	Specific message contents

Table 6.2.3.2.3.3-1: SystemInformationBlockType1 for E-UTRA Cell 1 (preamble and all steps, Table 6.2.3.2.3.2-3)

| Derivation path: TS 36.508-1 [7], Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -20 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.2.3.3-2:SystemInformationBlockType3 for E-UTRA Cell 1 (preamble and all steps, Table 6.2.3.2.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| q-QualMin-r9 | -20 |  |  |
| threshServingLowQ-r9 | 12 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.2.3.3-3:SystemInformationBlockType24 for E-UTRA Cell 1 (preamble, Table 6.2.3.2.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Downlink NR ARFCN of SSB for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| cellReselectionPriority-r15 | 3 |  |  |
| threshX-Q-r15 SEQUENCE { |  |  |  |
| threshX-HighQ-r15 | 20 |  |  |
| threshX-LowQ-r15 | 5 |  |  |
| } |  |  |  |
| q-QualMin-r15 | -25 |  |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionNR-r15 | 7 |  |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Downlink NR ARFCN of SSB for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

Table 6.2.3.2.3.3-4:SIB1 for NR Cell 1 (preamble and all steps, Table 6.2.3.2.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-QualMin | -25 | -25dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.2.3.3-5:SIB2 for NR Cell 1 (preamble and all steps, Table 6.2.3.2.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLowQ | 26 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.2.3.3-6:SIB5 for NR Cell 1 (preamble and all steps, Table 6.2.3.2.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 1 entry |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink EUTRA ARFCN as E-UTRA Cell 1 used |  |  |
| cellReselectionPriority | 5 |  |  |
| threshX-Q SEQUENCE { |  |  |  |
| threshX-HighQ | 12 |  |  |
| threshX-LowQ | 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionEUTRA | 7 |  |  |
| } |  |  |  |

Table 6.2.3.2.3.3-7:SystemInformationBlockType24 for E-UTRA Cell 1 (step 7, Table 6.2.3.2.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Downlink NR ARFCN of SSB for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| cellReselectionPriority-r15 | 5 |  |  |
| threshX-Q-r15 SEQUENCE { |  |  |  |
| threshX-HighQ-r15 | 5 |  |  |
| threshX-LowQ-r15 | 20 |  |  |
| } |  |  |  |
| q-QualMin-r15 | -25 |  |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionNR-r15 | 7 |  |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Downlink NR ARFCN of SSB for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

Table 6.2.3.2.3.3-8: Paging for E-UTRA Cell 1 (step 8, Table 6.2.3.2.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | True |  |  |
| etws-PrimaryNotificationIndication | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 6.2.3.2.3.3-9: SystemInformationBlockType1 for E-UTRA Cell 1 (step 9, Table 6.2.3.2.3.2-3)

| Derivation path: TS 36.508-1 [7], Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 |  |  |
| } |  |  |  |

Table 6.2.3.2.3.3-10: REGISTRATION REQUEST (Step 11, Table 6.2.3.2.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-6. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS NAS message container | TRACKING AREA UPDATE REQUEST message | See Table 6.2.3.2.3.3-11 |  |

Table 6.2.3.2.3.3-11: TRACKING AREA UPDATE REQUEST (Table 6.2.3.2.3.3-10)

Derivation Path: TS 38.508-1 [4], Table 4.9.9.2.3-2 with condition Mapped EPS security context

[Skip unchanged text]

6.2.3.3.3.3	Specific message contents

Table 6.2.3.3.3.3-1: SIB2 of NR Cell 1(preamble and all steps, Table 6.2.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLowP | 10 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.3.3.3-2: SIB5 of NR Cell 1(preamble and all steps, Table 6.2.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 1 entry |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink EUTRA ARFCN as E-UTRA Cell 1 used |  |  |
| cellReselectionPriority | 5 |  |  |
| threshX-High | 10 | 20 dB |  |
| threshX-Low | 20 | 40 dB |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionEUTRA | 7 | 7 Seconds |  |
| } |  |  |  |

Table 6.2.3.3.3.3-3: SystemInformationBlockType3 of EUTRA Cell 1 (preamble and all steps, Table 6.2.3.3.3.2-3)

| Derivation Path: TS 36.508 [7], Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 10 | 20 dB |  |
| } |  |  |  |

Table 6.2.3.3.3.3-4: SystemInformationBlockType24 of EUTRA Cell 1 (preamble and all steps, Table 6.2.3.3.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| cellReselectionPriority-r15 | 3 |  |  |
| threshX-High-r15 | 20 | 40 dB |  |
| threshX-Low-r15 | 10 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionNR-r15 | 7 | 7 Seconds |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

Table 6.2.3.3.3.3-5: SIB5 of NR Cell 1(Step 7A, Table 6.2.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 1 entry |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink EUTRA ARFCN as E-UTRA Cell 1 used |  |  |
| cellReselectionPriority | 3 |  |  |
| threshX-High | 20 | 40 dB |  |
| threshX-Low | 10 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionEUTRA | 7 | 7 Seconds |  |
| } |  |  |  |

Table 6.2.3.3.3.3-6: SIB1 of NR Cell 1 (step 7A, Table 6.2.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| si-SchedulingInfo | SI-SchedulingInfo |  |  |
| } |  |  |  |

Table 6.2.3.3.3.3-7: SI-SchedulingInfo (si-SchedulingInfo in Table 6.2.3.3.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-173 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI-SchedulingInfo ::= SEQUENCE { |  |  |  |
| schedulingInfoList SEQUENCE (SIZE(1..maxSI-Message)) OF SchedulingInfo { | 1 entry |  |  |
| SchedulingInfo[1] SEQUENCE { |  | entry 1 |  |
| sib-MappingInfo SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo { | 1 entry |  |  |
| SIB-TypeInfo[1] SEQUENCE { |  | entry 1 |  |
| type | sibType5 |  |  |
| valueTag | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.3.3.3-8: TRACKING AREA UPDATE REQUEST (Step 3, Table 6.2.3.3.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.9.7.2.3-1 with condition First-N1-to-S1 = TRUE

Table 6.2.3.3.3.3-9: TRACKING AREA UPDATE REQUEST (Step 13, Table 6.2.3.3.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.9.7.2.3-1 with condition First-N1-to-S1 = FALSE

Table 6.2.3.3.3.3-10: REGISTRATION REQUEST (Step 6, Table 6.2.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-6. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS NAS message container | TRACKING AREA UPDATE REQUEST message | See Table 6.2.3.3.3.3-11 |  |

Table 6.2.3.3.3.3-11: TRACKING AREA UPDATE REQUEST (Table 6.2.3.3.3.3-10)

| Derivation Path: TS 36.508 [2], Table 4.7.2-27. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '000'B | TA updating |  |
| "Active" flag | '0'B | No bearer establishment requested |  |
| NAS key set identifier |  |  |  |
| NAS key set identifier | the eKSI value for the current mapped EPS security context |  |  |
| TSC | '1'B | mapped security context |  |

[Skip unchanged text]

6.2.3.4.3.3	Specific message contents

Table 6.2.3.4.3.3-1: SIB1 for NR Cell 1 (preamble and all steps, Table 6.2.3.4.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-QualMin | -25 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-2: SIB2 for NR Cell 1 (preamble and all steps, Table 6.2.3.4.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLowQ | 9 |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-3: SIB5 NR Cell 1 (preamble, Table 6.2.3.4.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 1 entry |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink E-UTRA ARFCN for E-UTRA Cell 1 |  |  |
| threshX-Q SEQUENCE { |  |  |  |
| threshX-HighQ | 20 |  |  |
| threshX-LowQ | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionEUTRA | 7 |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-4: SystemInformationBlockType1 for E-UTRA Cell 1 (preamble and all steps, Table 6.2.3.4.3.2-3)

| Derivation path: TS 36.508-1 [7], Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -20 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-5: SystemInformationBlockType3 for E-UTRA Cell 1 preamble and all steps, Table 6.2.3.4.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| q-QualMin-r9 | -20 |  |  |
| threshServingLowQ-r9 | 26 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-6: SystemInformationBlockType24 for E-UTRA Cell 1 (preamble and all steps, Table 6.2.3.4.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Downlink NR ARFCN of SSB for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| cellReselectionPriority-r15 | 5 |  |  |
| threshX-Q-r15 SEQUENCE { |  |  |  |
| threshX-HighQ-r15 | 10 |  |  |
| threshX-LowQ-r15 | 20 |  |  |
| } |  |  |  |
| q-QualMin-r15 | -25 |  |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionNR-r15 | 7 |  |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Downlink NR ARFCN of SSB for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

Table 6.2.3.4.3.3-7: SIB5 NR Cell 1 (step 7, Table 6.2.3.4.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 1 entry |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink E-UTRA ARFCN for E-UTRA Cell 1 |  |  |
| cellReselectionPriority | 5 |  |  |
| q-QualMin | -20 |  |  |
| threshX-Q SEQUENCE { |  |  |  |
| threshX-HighQ | 12 |  |  |
| threshX-LowQ | 20 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionEUTRA | 7 |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-8: SIB1 of NR Cell 1 (step 9, Table 6.2.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-QualMin | -25 |  |  |
| } |  |  |  |
| si-SchedulingInfo | SI-SchedulingInfo |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-9: SI-SchedulingInfo (si-SchedulingInfo in Table 6.2.3.4.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-173 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI-SchedulingInfo ::= SEQUENCE { |  |  |  |
| schedulingInfoList SEQUENCE (SIZE(1..maxSI-Message)) OF SchedulingInfo { | 1 entry |  |  |
| SchedulingInfo[1] SEQUENCE { |  | entry 1 |  |
| sib-MappingInfo SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo { | 1 entry |  |  |
| SIB-TypeInfo[1] SEQUENCE { |  | entry 1 |  |
| type | sibType5 |  |  |
| valueTag | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.4.3.3-10: REGISTRATION REQUEST (Step 5, Table 6.2.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-6. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS NAS message container | TRACKING AREA UPDATE REQUEST message | See Table 6.2.3.4.3.3-11 |  |

Table 6.2.3.4.3.3-11: TRACKING AREA UPDATE REQUEST (Table 6.2.3.4.3.3-10)

Derivation Path: TS 38.508-1 [4], Table 4.9.9.2.3-2 with condition Mapped EPS security context

[Skip unchanged text]

6.2.3.5.3.3	Specific message contents

Table 6.2.3.5.3.3-1: SIB2 of NR Cell 1(preamble and all steps, Table 6.2.3.5.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLowP | 5 | 10dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.5.3.3-2: SIB5 of NR Cell 1(preamble and all steps, Table 6.2.3.5.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 1 entry |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink EARFCN as E-UTRA Cell 1 used |  |  |
| cellReselectionPriority | 5 |  |  |
| threshX-High | 13 | 26 dB |  |
| threshX-Low | 5 | 10 dB |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionEUTRA | 7 | 7 Seconds |  |
| } |  |  |  |

Table 6.2.3.5.3.3-3: SystemInformationBlockType3 for EUTRA Cell 1 (preamble and all steps, Table 6.2.3.5.3.2-3)

| Derivation Path: TS 36.508 [7], Table 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 5 | 10dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.5.3.3-4: SystemInformationBlockType24 for EUTRA Cell 1 (preamble and all steps, Table 6.2.3.5.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| cellReselectionPriority-r15 | 3 |  |  |
| threshX-High-r15 | 20 | 40dB |  |
| threshX-Low-r15 | 10 | 20dB |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionNR-r15 | 7 | 7 Seconds |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

Table 6.2.3.5.3.3-5 RRCRelease (step1, Table 6.2.3.5.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| cellReselection Priorities SEQUENCE { |  |  |  |
| freqPriorityListEUTRA | Not present |  | Step1 |
| freqPriorityListEUTRA SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA { | 1 entry |  |  |
| FreqPriorityListEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink ARFCN of E-UTRA Cell 1 |  |  |
| cellReselectionPriority | 5 |  | Step5 |
|  | 3 |  | Step10 |
|  | 3 |  | Step14 |
| } |  |  |  |
| } |  |  |  |
| freqPriorityListNR SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR { | 1 entry |  |  |
| FreqPriorityNR[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink SSB ARFCN of NR Cell 1 |  |  |
| cellReselectionPriority | 4 |  | Step1 |
|  | 4 |  | Step5 |
|  | 4 |  | Step10 |
|  | 4 |  | Step14 |
| } |  |  |  |
| } |  |  |  |
| t320 | Not present |  | Step1 |
|  | Not present |  | Step5 |
|  | Not present |  | Step10 |
|  | min5 | 5 minutes | Step14 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.5.3.3-6: REGISTRATION REQUEST (Step 8 and Step13, Table 6.2.3.5.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-6. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS NAS message container | TRACKING AREA UPDATE REQUEST message | See Table 6.2.3.5.3.3-7 |  |

Table 6.2.3.5.3.3-7: TRACKING AREA UPDATE REQUEST (Table 6.2.3.5.3.3-6)

Derivation Path: TS 38.508-1 [4], Table 4.9.9.2.3-2 with condition Mapped EPS security context

[Skip unchanged text]

6.2.3.6.3.3	Specific message contents

Table 6.2.3.6.3.3-1: SystemInformationBlockType3 of EUTRA Cell 1 (preamble and all steps, Table 6.2.3.6.3.2-3)

| Derivation Path: TS 36.508 [7], Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 10 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.6.3.3-2: SystemInformationBlockType24 of EUTRA Cell 1 (preamble and all steps, Table 6.2.3.6.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| cellReselectionPriority-r15 | 5 |  |  |
| threshX-High-r15 | 20 | 40 dB |  |
| threshX-Low-r15 | 10 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionNR-r15 | 7 | 7 Seconds |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

Table 6.2.3.6.3.3-3: SIB2 of NR Cell 1(preamble and all steps, Table 6.2.3.6.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLowP | 10 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.6.3.3-4: SIB5 of NR Cell 1(preamble and all steps, Table 6.2.3.7.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 1 entry |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink EUTRA ARFCN as E-UTRA Cell 1 used |  |  |
| threshX-High | 20 | 40 dB |  |
| threshX-Low | 10 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionEUTRA | 7 | 7 Seconds |  |
| } |  |  |  |

Table 6.2.3.6.3.3-5: RRCConnectionRelease message for EUTRA Cell 1 (step 1 of Table 6.2.3.6.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| IdleModeMobilityControlInfo SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1.. maxFreq)) OF FreqPriorityEUTRA { | 1 entry |  |  |
| freqPriorityListEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | DownLink ARFCN-Value of E-UTRA cell 1 |  |  |
|  | maxEARFCN |  | Band > 64 |
| cellReselectionPriority | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| t320 | Not present |  |  |
| freqPriorityListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR-r15 { | 1 entry |  |  |
| FreqPriorityNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Same Downlink SSB ARFCN as used for NR Cell 1 |  |  |
| cellReselectionPriority-r15 | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension |  |  | Band > 64 |
| nonCriticalExtension |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | DownLink ARFCN-Value of E-UTRA cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[Skip unchanged text]

6.2.3.7.3.3	Specific message contents

Table 6.2.3.7.3.3-1: SIB2 of NR Cell 1 (preamble and all steps, Table 6.2.3.7.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | 3 | 6 dB |  |
| threshServingLowP | 3 | 6 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.7.3.3-2: SIB5 of NR Cell 1(preamble and all steps, Table 6.2.3.7.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 1 entry |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink EUTRA ARFCN as E-UTRA Cell 1 used |  |  |
| cellReselectionPriority | 5 |  |  |
| threshX-High | 10 | 20 dB |  |
| threshX-Low | 20 | 40 dB |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionEUTRA | 7 | 7 Seconds |  |
| } |  |  |  |

Table 6.2.3.7.3.3-3: SystemInformationBlockType3 of EUTRA Cell 1 (preamble and all steps, Table 6.2.3.7.3.2-3)

| Derivation Path: TS 36.508 [7], Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 3 | 6 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.7.3.3-4: SystemInformationBlockType24 for EUTRA Cell 1 (preamble and all steps, Table 6.2.3.7.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| cellReselectionPriority-r15 | 3 |  |  |
| threshX-High-r15 | 15 | 30 dB |  |
| threshX-Low-r15 | 15 | 30 dB |  |
| } |  |  |  |
| t-ReselectionNR-r15 | 7 | 7 Seconds |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

[Skip unchanged text]

6.2.3.8.3.3	Specific message contents

Table 6.2.3.8.3.3-1: SystemInformationBlockType3 of EUTRA Cell 1 (preamble and all steps, Table 6.2.3.8.3.2-3)

| Derivation Path: TS 36.508 [7], Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 3 | 6 dB |  |
| threshServingLow | 3 | 6 dB |  |
| } |  |  |  |

Table 6.2.3.8.3.3-2: SystemInformationBlockType24 of EUTRA Cell 1 (preamble and all steps, Table 6.2.3.8.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| cellReselectionPriority-r15 | 5 |  |  |
| threshX-High-r15 | 10 | 20 dB |  |
| threshX-Low-r15 | 20 | 40 dB |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionNR-r15 | 7 | 7 Seconds |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Same downlink SSB ARFCN as used for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

Table 6.2.3.8.3.3-3: SIB2 of NR Cell 1 (preamble and all steps, Table 6.2.3.8.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLowP | 3 | 6 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.8.3.3-4: SIB5 of NR Cell 1(preamble and all steps, Table 6.2.3.8.3.2-3)

| Derivation Path: TS 38.508 [6], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 1 entry |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Downlink EUTRA ARFCN as E-UTRA Cell 1 used |  |  |
| cellReselectionPriority | 3 |  |  |
| threshX-High | 15 | 30 dB |  |
| threshX-Low | 15 | 30 dB |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionEUTRA | 7 | 7 Seconds |  |
| } |  |  |  |

[Skip unchanged text]

6.2.3.10.3.3	Specific message contents

Table 6.2.3.10.3.3-1: SystemInformationBlockType1 for E-UTRA cell 1 (preamble and all steps, Table 6.2.3.10.3.2-3)

| Derivation Path: TS 36.508 [7], Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| schedulingInfoList-v12j0 SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE { | 2 entries | It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |  |
| SchedulingInfo-v12j0[1] SEQUENCE { |  | entry 1 |  |
| SIB-MappingInfo-v12j0 | Not present |  |  |
| } |  |  |  |
| SchedulingInfo-v12j0[2] SEQUENCE { |  | entry 2 |  |
| SIB-MappingInfo-v12j0 SEQUENCE (SIZE (1..maxSIB-1)) OF SIB-Type-v12j0 { | 1 entry |  |  |
| SIB-Type-v12j0[1] | sibType24-v1530 | entry 1, SIB24 is mapped into the SI message scheduled via the second entry of schedulingInfoList (without suffix) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.10.3.3-2: SystemInformationBlockType3 of E-UTRA Cell 1 (preamble and all steps, Table 6.2.3.10.3.2-3)

| Derivation Path: TS 36.508 [7], Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 10 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.10.3.3-3: SystemInformationBlockType24 of E-UTRA Cell 1 (preamble and all steps, Table 6.2.3.10.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Downlink NR ARFCN of SSB for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| cellReselectionPriority-r15 | 3 |  |  |
| threshX-Low-r15 | 10 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Downlink NR ARFCN of SSB for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

[Skip unchanged text]

6.2.3.11.3.3	Specific message contents

Table 6.2.3.11.3.3-1: SystemInformationBlockType1 for E-UTRA Cell 1 (Preamble and all steps)

| Derivation Path: TS 36.508 [7], Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| schedulingInfoListExt-r12 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfoExt-r12 { | 1 entry |  |  |
| SchedulingInfoExt-r12[1] SEQUENCE { |  | entry 1 |  |
| si-Periodicity-r12 | rf64 |  |  |
| SIB-MappingInfo-v12j0 SEQUENCE (SIZE (1..maxSIB-1)) OF SIB-Type-v12j0 { | 1 entry |  |  |
| SIB-Type-v12j0[1] | sibType24-v1530 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.11.3.3-2: SystemInformationBlockType3 of E-UTRA Cell 1 (preamble and all steps , Table 6.2.3.11.3.2-3)

| Derivation Path: TS 36.508 [7], Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 10 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.11.3.3-3: SystemInformationBlockType24 of E-UTRA Cell 1 (preamble and all steps, Table 6.2.3.11.3.2-3)

| Derivation path: TS 36.508 [7], Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Downlink NR ARFCN of SSB for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| cellReselectionPriority-r15 | 3 |  |  |
| threshX-Low-r15 | 10 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Downlink NR ARFCN of SSB for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

[Skip unchanged text]

6.4.3.1.3.3	Specific message contents

Table 6.4.3.1.3.3-1: SIB2 of NR Cell 1 (preamble and all steps, Table 6.4.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLowP | 10 | 20 dB |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.4.3.1.3.3-2: SIB5 of NR Cell 1 (preamble and all steps, Table 6.4.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA { | 2 entries |  |  |
| CarrierFreqEUTRA[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq | Same downlink ARFCN as used for E-UTRA Cell 1 |  |  |
| cellReselectionPriority | 6 |  |  |
| } |  |  |  |
| CarrierFreqEUTRA[2] SEQUENCE { |  | entry 2 |  |
| carrierFreq | Same downlink ARFCN as used for E-UTRA Cell 3 |  |  |
| cellReselectionPriority | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.4.3.1.3.3-3: SystemInformationBlockType24 for E-UTRA Cell 1 (preamble and all steps, Table 6.4.3.1.3.2-3)

| Derivation path: TS 36.508 [7] table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Same downlink ARFCN as used for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| cellReselectionPriority-r15 | 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Same downlink ARFCN as used for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

Table 6.4.3.1.3.3-4: TRACKING AREA UPDATE REQUEST (Step 3, Table 6.4.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.9.7.2.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '010'B | Combined TA/LA updating with IMSI attach | combined_TA_LA |
|  | '000'B | TA updating | TA_only |
| NOTE: The message shall be integrity protected using the 5GS security context available in the UE. |  |  |  |

Table 6.4.3.1.3.3-5: TRACKING AREA UPDATE REQUEST (Step 10, Table 6.4.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.9.7.2.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '001'B | Combined TA/LA updating | combined_TA_LA |
|  | '000'B | TA updating | TA_only |
| UE radio capability information update needed | Not Check | UE radio capability information update needed |  |
| NOTE: The message shall be integrity protected using the 5GS security context available in the UE. |  |  |  |

| Condition | Explanation |
| --- | --- |
| TA_only | This condition applies if the UE is configured to initiate EPS attach or if explicitly specified. |
| combined_TA_LA | This condition applies if the UE is configured to initiate combined EPS/IMSI attach or if explicitly specified. |

Table 6.4.3.1.3.3-6: REGISTRATION REQUEST (Step 6, Table 6.4.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-6. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS NAS message container | TRACKING AREA UPDATE REQUEST message | See Table 6.4.3.1.3.3-7 |  |

Table 6.4.3.1.3.3-7: TRACKING AREA UPDATE REQUEST (Table 6.4.3.1.3.3-6)

Derivation Path: TS 38.508-1 [4], Table 4.9.9.2.3-2 with condition Mapped EPS security context

Table 6.4.3.1.3.3-8: SystemInformationBlockType24 for E-UTRA Cell 3 (preamble and all steps, Table 6.4.3.1.3.2-3)

| Derivation path: TS 36.508 [7] table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15 { | 1 entry |  |  |
| CarrierFreqNR-r15[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r15 | Same downlink ARFCN as used for NR Cell 1 |  |  |
|  | 250 |  | LessThan5MHz |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | NR Operating band of NR Cell 1 | entry 1 |  |
|  | 200 | entry 1 | LessThan5MHz |
| } |  |  |  |
| threshX-High-r15 | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| carrierFreqListNR-v1810 | Not present |  |  |
| carrierFreqListNR-v1810 SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-v1810 { | 1 entry |  | LessThan5MHz |
| CarrierFreqNR-v1810[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r18 | Same downlink ARFCN as used for NR Cell 1 |  |  |
| multiBandInfoList-r18 SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF FreqBandIndicatorNR-r15 { | 1 entry |  |  |
| FreqBandIndicatorNR-r15[1] | Same band as used for NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LessThan5MHz | This condition applies if using a NR neighbouring celll with channel BW less than 5 MHz for FR1. |

[End of change]