3GPP TSG-RAN WG5 Meeting #108	R5-254249

# Bengaluru, India, 25th Aug 2025 – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1546 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RF test frequencies and conditions for variable Tx-Rx spacing for NB-IoT for NTN bands 253, 254 and 255 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar, Viasat |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2025-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test frequency for flexible Tx-Rx separation for NB-IoT RefSens were introduced only for Band 256 in RAN5#107 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds test frequencies for Bands 253, 254 and 255 to support conformance of flexible Tx-Rx separation for NB-IoT NTN UEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance of flexible Tx-Rx separation for NTN UEs cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1.1.253, 8.1.3.1.1.254, 8.1.3.1.1.255 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: i) Updated the T-doc number on the coversheet; Correct the EARFCN for Band 255 midrang from 338786 to 228786 |  |  |  |  |  |  |  |  |

8.1.3.1.1.253	NB-IoT FDD reference test frequencies for operating band 253

Table 8.1.3.1.1.253-1: NB-IoT standalone Test frequencies for operating band 253

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 261270 | 0 | 1668.1 | 228502 | 0 | 1518.1 |
| Mid Range | 261304 | 0 | 1671.5 | 228536 | 0 | 1521.5 |
| High Range | 261338 | 0 | 1674.9 | 228570 | 0 | 1524.9 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.253-2: NB-IoT standalone Test frequencies for operating band 253 with flexible Tx-Rx frequency separation

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1,2 | 261338 | 0 | 1674.9 | 228502 | 0 | 1518.1 |
| Mid Range1,3 | 261289 | 0 | 1670.0 | 228511 | 0 | 1519.0 |
| High Range1,2 | 261270 | 0 | 1668.1 | 228570 | 0 | 1524.9 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing NOTE 2: Low Range is based on the smallest Tx-Rx separation while High Range is based on the largest Tx-Rx separation. NOTE 3: The Mid Range is based on Tx-Rx separation of -151 MHz and has been specified for signalling tests. |  |  |  |  |  |  |

<<< NEXT CHANGE >>>

8.1.3.1.1.254	NB-IoT FDD reference test frequencies for operating band 254

Table 8.1.3.1.1.254-1: NB-IoT standalone Test frequencies for operating band 254

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 261340 | 0 | 1610.1 | 228572 | 0 | 2483.6 |
| Mid Range | 261422 | 0 | 1618.3 | 228654 | 0 | 2491.8 |
| High Range | 261503 | 0 | 1626.4 | 228735 | 0 | 2499.9 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.254-2: NB-IoT standalone Test frequencies for operating band 254 with flexible Tx-Rx frequency separation

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1,2 | 261503 | 0 | 1626.4 | 228572 | 0 | 2483.6 |
| Mid Range1,3 | 261389 | 0 | 1615.0 | 228586 | 0 | 2485.0 |
| High Range1,2 | 261340 | 0 | 1610.1 | 228735 | 0 | 2499.9 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing NOTE 2: Low Range is based on the smallest Tx-Rx separation while High Range is based on the largest Tx-Rx separation. NOTE 3: The Mid Range is based on Tx-Rx separation of 870 MHz and has been specified for signalling tests. |  |  |  |  |  |  |

<<< NEXT CHANGE >>>

8.1.3.1.1.255	NB-IoT FDD reference test frequencies for operating band 255

Table 8.1.3.1.1.255-1: NB-IoT standalone Test frequencies for operating band 255

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 261505 | 0 | 1626.6 | 228737 | 0 | 1525.1 |
| Mid Range | 261674 | 0 | 1643.5 | 228906 | 0 | 1542.0 |
| High Range | 261843 | 0 | 1660.4 | 229075 | 0 | 1558.9 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.255-2: NB-IoT standalone Test frequencies for operating band 255 with flexible Tx-Rx frequency separation

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1,2 | 261843 | 0 | 1660.4 | 228737 | 0 | 1525.1 |
| Mid Range1,3 | 261529 | 0 | 1640 | 228786 | 0 | 1530.0 |
| High Range1,2 | 261505 | 0 | 1626.6 | 229075 | 0 | 1558.9 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing NOTE 2: Low Range is based on the smallest Tx-Rx separation while High Range is based on the largest Tx-Rx separation. NOTE 3: The Mid Range is based on Tx-Rx separation of -110 MHz and has been specified for signalling tests. |  |  |  |  |  |  |

END OF CHANGES >>