3GPP TSG-RAN WG5 Meeting #109	R5-255525r1

DALLAS, USA, 17th – 21st Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4148 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex F correction for Rel-17 sidelink relay test cases including TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test Tolerance for Rel-17 sidelink relay test cases is missing in Annex F |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test Tolerance parameters have been added to Annex F |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test tolerance parameters will remain not recorded in Annex F and test cases cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.7, F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.903 CR 1084, 1085 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT. Depends on R4-2520155 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Change test case number from 9.1.7.1.1 to 9.1.7.1 to align test case numbering between RAN4 and RAN5 |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

E.7	Cell configuration mapping for NR sidelink test cases in Chapter 9

Table E.7-1 defines the cell configuration mapping for NR PC5 – NR SA FR1 Uu con-current test cases (serving cell in FR1) in chapter 9 of this test specification.

Table E.7-1: Cell configuration mapping for NR PC5 – NR SA FR1 Uu con-current RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 9.1.1.3 | NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source | NR Cell 1 |  |  |  |  |
| 9.1.2.1 | NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source | NR Cell 1 |  |  |  |  |
| 9.1.5.1 | NR SA FR1 congestion control measurement for concurrent operation | NR Cell 1 |  |  |  |  |
| 9.1.6.1 | NR SA FR1 interruption to WAN due to NR sidelink communication | NR Cell 1 |  |  |  |  |
| 9.1.6.3 | NR SA FR1 Interruptions at NR sidelink discovery configuration | NR Cell 1 |  |  |  |  |
| 9.1.7.1 | NR SA FR1 Selection / Reselection of relay UE for U2N case | NR Cell 1 |  |  |  |  |

Table E.7-2 defines the cell configuration mapping for NR PC5 – NR SA FR2 Uu con-current test cases (serving cell in FR2) in chapter 9 of this test specification.

Table E.7-2: Cell configuration mapping for NR PC5 – SA FR2 con-current RRM testing

FFS

< Unchanged sections omitted >

F.1.1.2	Measurement of RRM requirements

…

Table F.1.1.2-8: Maximum test system uncertainty for RRM requirements for FR1 NR sidelink test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 9.1.1.1 NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source | Sidelink signal transmit timing relative to GNSS timing: ±192 Tc | Tc is the basic timing unit defined in TS 38.211 [7], Tc = 1/(480000 x 4096) seconds |
| 9.1.1.2 NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source | Sidelink signal transmit timing relative to SyncRef UE: ±128 Tc for SL SCS = 30kHz Average Noc: ±1.5 dB Average Ês1 / Noc: ±0.3 dB Meas PRB Noc: ±1.5 dB Meas PRB Ês1 / Noc: ±0.3 dB | Tc is the basic timing unit defined in TS 38.211 [7], Tc = 1/(480000 x 4096) seconds Noc is the AWGN absolute power on PC5 carrier. Ês1 / Noc is the ratio of SyncRef UE 1 signal / AWGN Meas PRB is the measurement PRB for PSBCH-RSRP #RBJ to RBJ+10 |
| 9.1.1.3 NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source | Sidelink signal transmit timing relative to downlink timing: ±192 Tc for DL SCS = 15kHz and SL SCS = 30kHz ±160 Tc for DL SCS = 30kHz and SL SCS = 30kHz Average Noc: ±1.5 dB Average Ês1 / Noc: ±0.3 dB Meas PRB Noc: ±1.5 dB Meas PRB Ês1 / Noc: ±0.3 dB | Tc is the basic timing unit defined in TS 38.211 [7], Tc = 1/(480000 x 4096) seconds Noc is the AWGN absolute power on Uu carrier. Ês1 / Noc is the ratio of Cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 9.1.2.1 NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source | Average Noc: ±1.5 dB Average Ês1 / Noc: ±0.3 dB Meas PRB Noc: ±1.5 dB Meas PRB Ês1 / Noc: ±0.3 dB | Noc is the AWGN absolute power on Uu carrier. Ês1 / Noc is the ratio of Cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 9.1.2.2 NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source | Average Noc: ±1.5 dB Average Ês1 / Noc: ±0.3 dB Meas PRB Noc: ±1.5 dB Meas PRB Ês1 / Noc: ±0.3 dB | Noc is the AWGN absolute power on PC5 carrier. Ês1 / Noc is the ratio of SyncRef UE 1 signal / AWGN Meas PRB is the measurement PRB for PSBCH-RSRP #RBJ to RBJ+10 |
| 9.1.3.1 NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source | Average Noc: ±1.5 dB Average Êsi / Noc: ±0.3 dB, i=1,2,3 Meas PRB Noc: ±1.5 dB Meas PRB Êsi / Noc: ±0.3 dB, i=1,2,3 | Noc is the AWGN absolute power on PC5 carrier. Êsi / Noc is the ratio of SyncRef UE i signal / AWGN, i = 1,2,3 Meas PRB is the measurement PRB for PSBCH-RSRP #RBJ to RBJ+10 |
| 9.1.3.2 NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source | Average Noc: ±1.5 dB Average Êsi / Noc: ±0.3 dB, i=1,2 Meas PRB Noc: ±1.5 dB Meas PRB Êsi / Noc: ±0.3 dB, i=1,2 | Noc is the AWGN absolute power on PC5 carrier. Êsi / Noc is the ratio of SyncRef UE i signal / AWGN, i = 1,2 Meas PRB is the measurement PRB for PSBCH-RSRP #RBJ to RBJ+10 |
| 9.1.4.1 NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection | Average Noc: ±1.5 dB Average Êsi / Noc: ±0.3 dB, i=0,1,...,49 Meas PRB Noc: ±1.5 dB Meas PRB Êsi / Noc: ±0.3 dB, i=0,1,...,49 | Noc is the AWGN absolute power on PC5 carrier. Êsi / Noc is the ratio of active sidelink UE i signal / AWGN, i = 0,1,...,49 Meas PRB is the measurement PRB for SL-RSRP #RBJ to RBJ+599 |
| 9.1.4.2 NR SA FR1 L1 SL-RSRP measurement for resource pre-emption | Average Noc: ±1.5 dB Average Ês1 / Noc: ±0.3 dB, Meas PRB Noc: ±1.5 dB Meas PRB Ês1 / Noc: ±0.3 dB, | Noc is the AWGN absolute power on PC5 carrier. Ês1 / Noc is the ratio of active sidelink UE 1 signal / AWGN, Meas PRB is the measurement PRB for SL-RSRP #RBJ to RBJ+119 |
| 9.1.4.3 NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation | Average Noc: ±1.5 dB Average Êsi / Noc: ±0.3 dB, i=0,1,...,129 Meas PRB Noc: ±1.5 dB Meas PRB Êsi / Noc: ±0.3 dB, i=0,1,...,129 | Noc is the AWGN absolute power on PC5 carrier. Êsi / Noc is the ratio of active sidelink UE i signal / AWGN, i = 0,1,...,129 Meas PRB is the measurement PRB for SL-RSRP #RBJ to RBJ+119 |
| 9.1.5.1 NR SA FR1 congestion control measurement for concurrent operation | Average Noc: ±1.5 dB Average Êsi / Noc: ±0.3 dB, i=0,1,2,3 Meas PRB Noc: ±1.5 dB Meas PRB Êsi / Noc: ±0.3 dB, i=0,1,2,3 | Noc is the AWGN absolute power on PC5 carrier. Êsi / Noc is the ratio of active sidelink UE i signal / AWGN, i = 0,1,2,3 Meas PRB is the measurement PRB for SL-RSSI #RBJ to RBJ+119 |
| 9.1.5.2 NR SA FR1 congestion control measurement for PC5-only operation | Same as 9.1.5.1 | Same as 9.1.5.1 |
| 9.1.6.1 NR SA FR1 interruption to WAN due to NR sidelink communication | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 9.1.6.3 NR SA FR1 Interruptions at NR sidelink discovery configuration | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 9.1.7.1 NR SA FR1 Selection / Reselection of relay UE for U2N case | Average Noc1: ±1.5 dB Average Êsi / Noc: ±0.3 dB, i=1,2 Average Noc2: ±1.5 dB Average Ês1/ Noc: ±0.3 dB, Meas PRB Noc1: ±1.5 dB Meas PRB Êsi / Noc: ±0.3 dB, i=1,2 Meas PRB Noc2: ±1.5 dB Meas PRB Uu Ês0 / Noc: ±0.3 dB, | Noc1 is the AWGN absolute power on PC5 carrier. PC5 Ês / Noc is the ratio of sidelink relay UE i signal / AWGN, i=1,2 Noc2 is the AWGN absolute power on Uu carrier. Ês0 / Noc is the ratio of NR cell 1 signal / AWGN Meas PRB is the measurement PRB for SL-RSRP #RBJ to RBJ+599 Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |

< Unchanged sections omitted >

F.1.3.2	Measurement of RRM requirements

…

Table F.1.3.2-6: Derivation of test requirements for FR1 NR sidelink RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 9.1.1.1 NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source | Sidelink transmit timing error limit: ±12*64 Tc | ±3*64 Tc | ±15*64 Tc |
| 9.1.1.2 NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source | Sidelink transmit timing error limit: ±8*64 Tc Noc = -95 dBm/30 kHz Es1/Noc = 3 dB | ±2*64 Tc 0dB 0dB | ±10*64 Tc Noc = -95 dBm/30 kHz Es1/Noc = 3 dB |
| 9.1.1.3 NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source | Uu Test Configuration 1, 2 |  |  |
|  | Sidelink transmit timing error limit: ±12*64 Tc Noc = -98 dBm/15 kHz Es1/Noc = 3 dB | ±3*64 Tc 0dB 0dB | ±15*64 Tc Noc = -98 dBm/15 kHz Es1/Noc = 3 dB |
|  | Uu Test Configuration 3 |  |  |
|  | Sidelink transmit timing error limit: ±10*64 Tc Noc = -98 dBm/15 kHz Es1/Noc = 3 dB | ±3*64 Tc 0dB 0dB | ±13*64 Tc Noc = -98 dBm/15 kHz Es1/Noc = 3 dB |
| 9.1.2.1 NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source | Uu Test Configuration 1, 2 |  |  |
|  | During T1: Noc = -110 dBm/15 kHz Es1/Noc = 4.50 dB During T2: Noc = -110 dBm/15 kHz Es1/Noc = -4.50 dB During T3: Noc = -110 dBm/15 kHz Es1/Noc = 4.50 dB In signalling: syncTxThreshIC = -110.00 dBm/SCS | -1.55dB 3.10dB -1.55dB 0dB -1.55dB 3.10dB 0dB | During T1: Noc = -111.55 dBm/15 kHz Es1/Noc = 7.60 dB During T2: Noc = -111.55 dBm/15 kHz Es1/Noc = -4.50 dB During T3: Noc = -111.55 dBm/15 kHz Es1/Noc = 7.60 dB syncTxThreshIC = -110.00 dBm/SCS |
|  | Uu Test Configuration 3 |  |  |
|  | During T1: Noc = -110 dBm/15 kHz Es1/Noc = 4.50 dB During T2: Noc = -110 dBm/15 kHz Es1/Noc = -4.50 dB During T3: Noc = -110 dBm/15 kHz Es1/Noc = 4.50 dB In signalling: syncTxThreshIC = -110.00 dBm/SCS | -4.55dB 3.10dB -4.55dB 0dB -4.55dB 3.10dB 0dB | During T1: Noc = -114.55 dBm/15 kHz Es1/Noc = 7.60 dB During T2: Noc = -114.55 dBm/15 kHz Es1/Noc = -4.50 dB During T3: Noc = -114.55 dBm/15 kHz Es1/Noc = 7.60 dB syncTxThreshIC = -110.00 dBm/SCS |
| 9.1.2.2 NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source | During T1: Noc = -98 dBm/30 kHz Es1/Noc = 5.50 dB During T2: Noc = -98 dBm/30 kHz Es1/Noc = -3.50 dB During T3: Noc = -98 dBm/30 kHz Es1/Noc = 5.50 dB In signalling: syncTxThreshOoC = -100.00 dBm/SCS | -1.35dB 0dB -4.55dB 0dB -1.35dB 0dB 0dB | During T1: Noc = -99.35 dBm/30 kHz Es1/Noc = 5.50 dB During T2: Noc = -102.55 dBm/30 kHz Es1/Noc = -3.50 dB During T3: Noc = -99.35 dBm/30 kHz Es1/Noc = 5.50 dB syncTxThreshOoC = -100.00 dBm/SCS |
| 9.1.3.1 NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source | During T1: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = -infinity Es3/Noc = -infinity During T2: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0 dB Es3/Noc = -infinity During T3: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0 dB Es3/Noc = 3 dB | 0dB 0.35dB N/A N/A 0dB 0dB 0.35dB N/A 0dB 0dB 0dB 0.35dB | During T1: Noc = -95 dBm/30 kHz Es1/Noc = 0.35 dB Es2/Noc = -infinity Es3/Noc = -infinity During T2: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0.35 dB Es3/Noc = -infinity During T3: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0 dB Es3/Noc = 3.35 dB |
| 9.1.3.2 NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source | During T1: Noc = -95 dBm/30 kHz Es1/Noc = -infinity Es2/Noc = -infinity During T2: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = -infinity During T3: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0 dB | 0dB N/A N/A 0dB 0.35dB N/A 0dB 0dB 0.35dB | During T1: Noc = -95 dBm/30 kHz Es1/Noc = -infinity Es2/Noc = -infinity During T2: Noc = -95 dBm/30 kHz Es1/Noc = 0.35 dB Es2/Noc = -infinity During T3: Noc = -95 dBm/30 kHz Es1/Noc = 0 dB Es2/Noc = 0.35 dB |
| 9.1.4.1 NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection | During T1: Noc = -110 dBm/30 kHz Esi/Noc = 10 dB, i = 20~29 Esi/Noc = 0 dB, i = 0~19, 30~49 During T2: Noc = -121 dBm/30 kHz Esi/Noc = 10 dB, i = 20~29 Esi/Noc = 20 dB, i = 0~19, 30~49 In signalling: SL-Thres-RSRP = -106 dBm/SCS | 0dB 10.55dB 0.35dB 0dB 0dB 10.55dB 6dB | During T1: Noc = -110 dBm/30 kHz Esi/Noc = 20.55 dB, i = 20~29 Esi/Noc = 0.35 dB, i = 0~19, 30~49 During T2: Noc = -121 dBm/30 kHz Esi/Noc = 10 dB, i = 20~29 Esi/Noc = 30.55 dB, i = 0~19, 30~49 In signalling: SL-Thres-RSRP = -100 dBm/SCS |
| 9.1.4.2 NR SA FR1 L1 SL-RSRP measurement for resource pre-emption | During T1: Noc = -infinity Es1/Noc = -infinity During T2: Noc = -100 dBm/30 kHz Es1/Noc = 5 dB In signalling: SL-Thres-RSRP = -106 dBm/SCS | N/A N/A 0dB 0dB 0dB | During T1: Noc = -infinity Es1/Noc = -infinity During T2: Noc = -100 dBm/30 kHz Es1/Noc = 5 dB In signalling: SL-Thres-RSRP = -106 dBm/SCS |
| 9.1.4.3 NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation | During T1: Noc = -103 dBm/30 kHz Esi/Noc = 12dB, i = 0~29 Esi/Noc = 2 dB, i = 30~49 Esi/Noc = 25.50 dB, i = 50~64, 85~99 Esi/Noc = 12 dB, i = 65~84 Esi/Noc = -infinity, i = 100~129 During T2: Noc = -103 dBm/30 kHz Esi/Noc = -infinity, i = 0~29 Esi/Noc = 2 dB, i = 30~49 Esi/Noc = 25.5 dB, i = 50~64, 85~99 Esi/Noc = 12 dB, i = 65~84 Esi/Noc = 25.5dB, i = 100~129 In signalling: SL-Thres-RSRP = -96 dBm/SCS | -7.05dB 2.1dB 0dB 7.1dB 2.1dB N/A -7.05dB N/A 0dB 7.1dB 2.1dB 7.55dB -6dB | During T1: Noc = -110.05 dBm/30 kHz Esi/Noc = 14.1 dB, i = 0~29 Esi/Noc = 2 dB, i = 30~49 Esi/Noc = 32.60 dB, i = 50~64, 85~99 Esi/Noc = 14.1 dB, i = 65~84 Esi/Noc = -infinity, i = 100~129 During T2: Noc = -110.05 dBm/30 kHz Esi/Noc = -infinity, i = 0~29 Esi/Noc = 2 dB, i = 30~49 Esi/Noc = 32.60 dB, i = 50~64, 85~99 Esi/Noc = 14.1 dB, i = 65~84 Esi/Noc = 33.05dB, i = 100~129 In signalling: SL-Thres-RSRP = -102 dBm/SCS |
| 9.1.5.1 NR SA FR1 congestion control measurement for concurrent operation | During T1: Noc = -103 dBm/30kHz Esi/Noc = 4.35 dB, i = 0,1,2,3 During T2: Noc = -103 dBm/30kHz Esi/Noc = 10.32 dB, i = 0,1,2,3 In signalling: threshS-RSSI-CBR = -74 dBm/subchannel | -1.55dB 0dB -1.55dB 3.30dB 0dB | During T1: Noc = -104.55 dBm/30kHz Esi/Noc = 4.35 dB, i = 0,1,2,3 During T2: Noc = -104.55 dBm/30kHz Esi/Noc = 13.62 dB, i = 0,1,2,3 In signalling: threshS-RSSI-CBR = -74 dBm/subchannel |
| 9.1.5.2 NR SA FR1 congestion control measurement for PC5-only operation | Same as 9.1.5.1 | Same as 9.1.5.1 | Same as 9.1.5.1 |
| 9.1.6.1 NR SA FR1 interruption to WAN due to NR sidelink communication | N/A | N/A | N/A |
| 9.1.6.3 NR SA FR1 Interruptions at NR sidelink discovery configuration | N/A | N/A | N/A |
| 9.1.7.1 NR SA FR1 Selection / Reselection of relay UE for U2N case | Test Config 1,2: During T1: Noc1 = -97 dBm/30 kHz Noc2 = -97dBm/15 kHz Es0/Noc = 10.5 dB Es1/Noc = -infinity Es2/Noc = -infinity During T2: Noc1 = -97 dBm/30 kHz Noc2 = -97dBm/15 kHz Es0/Noc = 1.5 dB Es1/Noc = -infinity Es2/Noc = -infinity During T3 and T4: Noc1 = -97 dBm/30 kHz Noc2 = -97dBm/15 kHz Es0/Noc = 1.5 dB Es1/Noc = 10.5 dB Es2/Noc = -infinity During T5: Noc1 = -97 dBm/30 kHz Noc2 = -97dBm/15 kHz Es0/Noc = 1.5 dB Es1/Noc = -1.5 dB Es2/Noc = 10.5 dB Test Config 3: During T1: Noc1 = -97 dBm/30 kHz Noc2 = -94 dBm/30 kHz Es0/Noc = 10.5 dB Es1/Noc = -infinity Es2/Noc = -infinity During T2: Noc1 = -97 dBm/30 kHz Noc2 = -94dBm/30 kHz Es0/Noc = 1.5 dB Es1/Noc = -infinity Es2/Noc = -infinity During T3 and T4: Noc1 = -97 dBm/30 kHz Noc2 = -94dBm/30 kHz Es0/Noc = 1.5 dB Es1/Noc = 10.5 dB Es2/Noc = -infinity During T5: Noc1 = -97 dBm/30 kHz Noc2 = -94dBm/30 kHz Es0/Noc = 1.5 dB Es1/Noc = -1.5 dB Es2/Noc = 10.5 dB | 3.7dB 3.7dB 3.7dB -0.54dB 1.19dB 3.7dB 3.7dB | Test Config 1,2: During T1: Noc1 = -97 dBm/30 kHz Noc2 = -97dBm/15 kHz Es0/Noc = 14.2 dB Es1/Noc = -infinity Es2/Noc = -infinity During T2: Noc1 = -97 dBm/30 kHz Noc2 = -97dBm/15 kHz Es0/Noc = 1.5 dB Es1/Noc = -infinity Es2/Noc = -infinity During T3 and T4: Noc1 = -97 dBm/30 kHz Noc2 = -97dBm/15 kHz Es0/Noc = 1.5 dB Es1/Noc = 14.2 dB Es2/Noc = -infinity During T5: Noc1 = -97 dBm/30 kHz Noc2 = -97dBm/15 kHz Es0/Noc = 1.5 dB Es1/Noc = -1.5 dB Es2/Noc = 14.2 dB Test Config 3: During T1: Noc1 = -97 dBm/30 kHz Noc2 = -94.54 dBm/30 kHz Es0/Noc = 11.69 dB Es1/Noc = -infinity Es2/Noc = -infinity During T2: Noc1 = -97 dBm/30 kHz Noc2 = -94dBm/30 kHz Es0/Noc = 1.5 dB Es1/Noc = -infinity Es2/Noc = -infinity During T3 and T4: Noc1 = -97 dBm/30 kHz Noc2 = -94dBm/30 kHz Es0/Noc = 1.5 dB Es1/Noc = 14.2 dB Es2/Noc = -infinity During T5: Noc1 = -97 dBm/30 kHz Noc2 = -94dBm/30 kHz Es0/Noc = 1.5 dB Es1/Noc = -1.5 dB Es2/Noc = 14.2 dB |