3GPP TSG-RAN5 Meeting #109	R5-255679r1

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5155 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new UAV test case 8.1.3.5.12 for event A4H2 for Intra-frequency Neighbour |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UAV_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The measurement report trigger event A4H2 for Aerial UE has been proposed in TS 38.331, but the TC is still missing in TS38.523-1. Simulated aerial heights previously defined may not work with +/-90m allowance in TC 8.1.3.5.9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added measurement configuration control and reporting of Event A4H2 for Aerial UE. Simulated aerial heights are updated. Hysteresis, Threshold values are updated in ReportConfigNR for TC 8.1.3.5.9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The event A4H2 for Aerial UE can not be tested. TC 8.1.3.5.9will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.5.12 (new), 8.1.3.5.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0633, 5119 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Correcting Value of Simulated aerial height above sea level in Table 8.1.3.5.12.3.2-1 and 8.1.3.5.9.3.2-1. Correcting Value of h2-Threshold-r18 and h2-Hysteresis-r18 in Table 8.1.3.5.12.3.3-3 and h1-Threshold-r18 and h1-Hysteresis-r18 in 8.1.3.5.9.3.3-3. Deleting FR2 of ReportConfigNR in Table 8.1.3.5.12.3.3-3 and 8.1.3.5.9.3.3-3.. |  |  |  |  |  |  |  |  |

==============First change==============

8.1.3.5.12	NR UAV / Measurement configuration control and reporting / Event A4H2 / Measurement of Neighbour NR cell / Intra-frequency measurements

8.1.3.5.12.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and intra-frequency measurement configured for event A4H2 based periodical reporting}

ensure that {

when { Neighbour cell becomes better than absolute threshold1 and the Aerial UE altitude becomes lower than absolute threshold2 minus hysteresis}

then { UE sends MeasurementReport with correct measdID for event A4H2 }

}

(2)

with { UE in RRC_CONNECTED state and intra-frequency measurement configured for event A4H2 }

ensure that {

when { Neighbour cell becomes better than absolute threshold1 and the Aerial UE altitude becomes higher than the absolute threshold2 plus hysteresis}

then { UE stops sending MeasurementReport}

}

(3)

with { UE in RRC_CONNECTED state and intra-frequency measurement configured for event A4H2 }

ensure that {

when { Neighbour cell becomes lower than absolute threshold1 and the Aerial UE altitude becomes higher than the absolute threshold2 plus hysteresis }

then { UE does not sending MeasurementReport}

}

8.1.3.5.12.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.5.3, 5.5.4.1, 5.5.4.26 and 5.5.5.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA, CPC, or subsequent CPAC), or upon execution of an LTM cell switch:

…

1>	if the RRCReconfiguration message includes the aerial-Config:

2>	(re)configure the aerial parameters in accordance with the included aerial-Config;

[TS 38.331, clause 5.5.4.1]

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	else if the reportType is set to eventTriggered and if the eventId is set to eventD1 or eventD2 or eventH1 or eventH2 and if the entering condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

initiate the measurement reporting procedure, as specified in 5.5.5;

…

[TS 38.331, clause 5.5.4.26]

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A4H2-1 and condition A4H2-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A4H2-3 or condition A4H2-4, i.e. at least one of the two, as specified below, is fulfilled.

Inequality A4H2-1 (Entering condition 1)

Mn + Ofn + Ocn – Hys1 > Thresh1

Inequality A4H2-2 (Entering condition 2)

Ms + Hys2 < Thresh2

Inequality A4H2-3 (Leaving condition 1)

Mn + Ofn + Ocn + Hys1 < Thresh1

Inequality A4H2-4 (Leaving condition 2)

Ms – Hys2 > Thresh2

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Hys1 is the hysteresis parameter for this event (i.e. a4-Hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a4-Threshold as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h2-Hysteresis as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. h2-Threshold as defined within reportConfigNR for this event).

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys1 are expressed in dB.

Thresh1 is expressed in the same unit as Mn.

Ms, Hys2, Thresh2 are expressed in meters.

[TS 38.331, clause 5.5.5.1]

Figure 5.5.5-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to the network. The UE shall initiate this procedure only after successful AS security activation.

The UE shall, for each entry in the VarMeasReportList:

1>	if the eventH1 or eventH2 is configured in the corresponding reportConfig associated with the measId that triggered the measurement reporting:

2>	for all the entries in the VarMeasReportList for which the measurement reporting procedure was triggered and the corresponding reportConfig is configured with the same eventID and simulMultiTriggerSingleMeasReport set to true:

3>	except for the entry corresponding to the event for which the difference between the configured altitude threshold and the altitude of the UE is the smallest, remove all other measurement reporting entries from the VarMeasReportList, if any, and stop the associated periodical reporting timer(s), if running;

1>	else if the eventA3H1 or eventA3H2 or eventA4H2 or eventA4H2 or eventA5H1 or eventA5H2 is configured in the corresponding reportConfig associated with the measId that triggered the measurement reporting:

2>	for all the entries in the VarMeasReportList associated with the same measObjectNR for which the measurement reporting procedure was triggered and the corresponding reportConfig is configured with the same eventID and simulMultiTriggerSingleMeasReport set to true:

3>	except for the entry corresponding to the event for which the difference between the configured altitude threshold and the altitude of the UE is the smallest, remove all other measurement reporting entries from the VarMeasReportList, if any, and stop the associated periodical reporting timer(s), if running;

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

. . .

1>	if the reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5, or eventB1, or eventB2, or eventA3H1, or eventA3H2, or eventA4H2, or eventA4H2, or eventA5H1, or eventA5H2:

2>	if the UE is in NE-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListEUTRA-SCG to include an entry for each E-UTRA SCG serving frequency with the following:

4>	include carrierFreq of the E-UTRA serving frequency;

4>	set the measResultServingCell to include the available measurement quantities that the UE is configured to measure by the measurement configuration associated with the SCG;

4>	if reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

5>	set the measResultServFreqListEUTRA-SCG to include within measResultBestNeighCell the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	if reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5, or eventA3H1, or eventA3H2, or eventA4H2, or eventA4H2, or eventA5H1, or eventA5H2:

2>	if the UE is in NR-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListNR-SCG to include for each NR SCG serving cell that is configured with servingCellMO, if any, the following:

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes rsType:

5>	if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report;

...

1>	if the includeAltitudeUE is set to true in the corresponding reportConfig for this measId:

2>	set the altitudeUE to include the altitude of the UE;

8.1.3.5.12.3	Test description

8.1.3.5.12.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell, NR Cell 2 is the intra-frequency neighbour cell of NR Cell 1.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

- 1 GNSS simulator

UE:

-	None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.3.5.12.3.2	Test procedure sequence

Table 8.1.3.5.12.3.2-1 illustrates the simulated aerial height above sea level at various time instants of the test execution along with the downlink power levels to be applied for NR Cell 1 and NR Cell 2 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configurations marked "T1", "T2" and "T3" are to be applied subsequently at the point indicated in the Main behaviour description in Table 8.1.3.5.12.3.2-2.

Table 8.1.3.5.12.3.2-1: Time instances of cell power level and parameter changes for NR Cell 1 and NR Cell 2 in conducted test environment

| Time | Parameter | Unit | Value (Note 1) | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark (Note 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Simulated aerial height above sea level (Ms) | m | 700 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -91 | Simulated aerial height above sea level (Ms) and Power levels are such that neither of the entry conditions A4H2-1 and A4H2-2 are satisfied: A4H2-1:Mn + Ofn + Ocn – Hys1 > Thresh1 A4H2-2: Ms + Hys2 < Thresh2 |
| T1 |  |  | 300 |  |  | -85 | -79 | Simulated aerial height above sea level (Ms) and Power levels are such that both the entry conditions A4H2-1 and A4H2-2 are satisfied: A4H2-1:Mn + Ofn + Ocn – Hys1 > Thresh1 A4H2-2: Ms + Hys2 < Thresh2 |
| T2 |  |  | 1100 |  |  | -85 | -79 | Simulated aerial height above sea level (Ms) and Power levels are such that Leaving Condition conditions A4H2-4 is satisfied: Ms – Hys2 > Thresh2 |
| T3 |  |  | 1100 |  |  | -85 | -93 | Simulated aerial height above sea level (Ms) and Power levels are such that Leaving Condition conditions A4H2-3 and A4H2-4 is satisfied: A4H2-3: Mn + Ofn + Ocn + Hys1 < Thresh1 A4H2-4: Ms – Hys2 > Thresh2 |
| Note 1: An allowance of +/-90m has been used to generate the aerial height values. This allowance takes into account both the UE measurement accuracy and the test tolerances in the simulation. Note 2: Value of Hys correspond to values of h2-Hysteresis-r18 in Table 8.1.3.5.12.3.3-3. Value of Thresh corresponds to value of h2-Threshold-r18 in Table 8.1.3.5.12.3.3-3. |  |  |  |  |  |  |  |  |

Table 8.1.3.5.12.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger the UE to reset UTC time in order that the UE accepts the time generated by the GNSS simulator and also uses only GNSS for position calculation in the subsequent test procedure, as defined in TS 38.509 [25] clause 5.6.3. (Note1) | - | - | - | - |
| 2 | The SS configures the GNSS simulator as step 1 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate a location with the height above sea level given for time T0 in Table 8.1.3.5.12.3.2-1. | - | - | - | - |
| 3 | The SS transmits a RRCReconfiguration message including MeasConfig to setup intra NR measurement and reporting for event A4H2. | <-- | RRCReconfiguration | - | - |
| 4 | The UE transmits a RRCReconfigrationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 5 | The SS configures GNSS simulator as step 2 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level and readjusts the cell-specific reference signal to row "T1" in Table 8.1.3.5.12.3.2-1. | - | - | - | - |
| 6 | Check: Does the UE transmit a MeasurementReport message to report event A4H2 with the measured aerial height value? | --> | MeasurementReport | 1 | P |
| - | EXCEPTION: Step 7 below is repeated until 3 MeasurementReport messages are received from the UE and Interval between two MeasurementReport is same as the IE reportInterval configured in measConfig. | - | - | - | - |
| 7 | Check: Does the UE transmit a MeasurementReport message to report event A4H2 with the measured aerial height value? | --> | MeasurementReport | 1 | P |
| 8 | The SS configures GNSS simulator as step 2 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level and readjusts the cell-specific reference signal to row "T2" in Table 8.1.3.5.12.3.2-1. | - | - | - | - |
| 9 | Wait and ignore MeasurementReport messages for 50s (as detailed in TS 36.508 [7] Table 4.12.2-1) to allow simulated change of aerial height value to be measured by the UE. | - | - | - | - |
| 10 | Check: Does the UE transmit a MeasurementReport message within the next 10s? | - | - | 2 | F |
| 11 | The SS configures GNSS simulator as step 2 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level and readjusts the cell-specific reference signal to row "T3" in Table 8.1.3.5.12.3.2-1. | - | - | - | - |
| 12 | Wait for 50s (as detailed in TS 36.508 [7] Table 4.12.2-1) to allow simulated change of aerial height value to be measured by the UE. | - | - | - | - |
| 13 | Check: Does the UE transmit a MeasurementReport message within the next 10s? | - | - | 3 | F |
| Note 1: The UTC time reset may be performed by MMI or AT command (+CUTCR). |  |  |  |  |  |

8.1.3.5.12.3.3	Specific message contents

Table 8.1.3.5.12.3.3-1: RRCReconfiguration (step 3, Table 8.1.3.5.12.3.2-2)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR_MEAS

Table 8.1.3.5.12.3.3-2: MeasConfig(Table 8.1.3.5.12.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 | MeasObjectIdNR-f1 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR SEQUENCE { |  |  |  |
| ssbFrequency | ssbFrequency IE equals the ARFCN for NR Cell 2 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA4H2 | Table 8.1.3.5.12.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.5.12.3.3-3: ReportConfigNR-EventA4H2(Table 8.1.3.5.12.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A4H2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA4H2-r18 SEQUENCE { |  |  |  |
| a4-Threshold-r18 CHOICE { |  |  |  |
| rsrp | 70 | -86dBm | FR1 |
| } |  |  |  |
| a4-Hysteresis-r18 | 2 | 1 dB |  |
| h2-Threshold-r18 | 500 (500m) |  |  |
| h2-Hysteresis-r18 | 5 (5m) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.5.12.3.3-4: MeasurementReport (step 6 and 7, Table 8.1.3.5.12.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry | Report NR Cell 1 |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | Physical layer cell identity of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
|  | Not checked |  | pc_ss_SINR_Meas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF OF MeasResultNR { | 1 entry | Report NR neighbour cell |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| altitudeUE-r18 | (-420..10000) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

==============Next change==============

8.1.3.5.9	NR UAV / Measurement configuration control and reporting / Event A4H1 / Measurement of Neighbour NR cell / Intra-frequency measurements

<Unchanged Sections Omitted>

Table 8.1.3.5.9.3.2-1: Time instances of cell power level and parameter changes for NR Cell 1 and NR Cell 2 in conducted test environment

| Time | Parameter | Unit | Value (Note 1) | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark (Note 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Simulated aerial height above sea level (Ms) | m | 700 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -91 | Simulated aerial height above sea level (Ms) and Power levels are such that neither of the entry conditions A4H1-1 and A4H1-2 are satisfied: A4H1-1:Mn + Ofn + Ocn – Hys1 > Thresh1 A4H1-2: Ms – Hys2 > Thresh2 |
| T1 |  |  | 1100 |  |  | -85 | -79 | Simulated aerial height above sea level (Ms) and Power levels are such that both the entry conditions A4H1-1 and A4H1-2 are satisfied: A4H1-1:Mn + Ofn + Ocn – Hys1 > Thresh1 A4H1-2: Ms – Hys2 > Thresh2 |
| T2 |  |  | 300 |  |  | -85 | -79 | Simulated aerial height above sea level (Ms) and Power levels are such that Leaving Condition conditions A4H1-4 is satisfied: A4H1-4: Ms + Hys2 < Thresh2 |
| T3 |  |  | 300 |  |  | -85 | -93 | Simulated aerial height above sea level (Ms) and Power levels are such that Leaving Condition conditions A4H1-3 and A4H1-4 is satisfied: A4H1-3: Mn + Ofn + Ocn + Hys1 < Thresh1 A4H1-4: Ms + Hys2 < Thresh2 |
| Note 1: An allowance of +/-90m has been used to generate the aerial height values. This allowance takes into account both the UE measurement accuracy and the test tolerances in the simulation. Note 2: Value of Hys correspond to values of h1-Hysteresis-r18 in Table 8.1.3.5.9.3.3-3. Value of Thresh corresponds to value of h1-Threshold-r18 in Table 8.1.3.5.9.3.3-3. |  |  |  |  |  |  |  |  |

<Unchanged Sections Omitted>

Table 8.1.3.5.9.3.3-3: ReportConfigNR-EventA4H1(Table 8.1.3.5.9.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A4H1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA4H1-r18 SEQUENCE { |  |  |  |
| a4-Threshold-r18 CHOICE { |  |  |  |
| rsrp | 70 | -86dBm | FR1 |
| } |  |  |  |
| a4-Hysteresis-r18 | 2 | 1 dB |  |
| h1-Threshold-r18 | 900 (900m) |  |  |
| h1-Hysteresis-r18 | 5 (5m) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |

<Unchanged Sections Omitted>

==============End of change==============