3GPP TSG-RAN WG5 Meeting #109	R5-255686r1

DALLAS, USA, 17th – 21st Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3611 | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SL-PRS-ResourcePool default configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to 38.212, clause 8.3.1.2: - Source ID – 12 or 24 bits determined by higher layer parameter sl-SRC-ID-LenDedicatedSL-PRS-RP, as defined in clause 16.4A of [5, TS 38.213]. - Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserveDedicatedSL-PRS-RP is configured to 3, as defined in clause 8.2.4.2A of [6, TS 38.214]. sl-MaxNumPerReserveDedicatedSL-PRS-RP is related to the time resource assignment field in SCI format 1-B. sl-SRC-ID-LenDedicatedSL-PRS-RP determine the length of source ID.So these two IEs need to be configured in SL-PRS-ResourcePool. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Set sl-MaxNumPerReserveDedicatedSL-PRS-RP to n2. - Set sl-SRC-ID-LenDedicatedSL-PRS-RP-r18 to n24. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 4.6.6-17B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Corrected the spec current version to 19.1.1 |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

–	SL-PRS-ResourcePool

Table 4.6.6-17B: SL-PRS-ResourcePool

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-PRS-ResourcePool-r18 ::= SEQUENCE { |  |  |  |  |
| sl-PRS-PSCCH-Config-r18 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| sl-TimeResourcePSCCH-DedicatedSL-PRS-RP-r18 |  | n2 |  |  |
| sl-FreqResourcePSCCH-DedicatedSL-PRS-RP-r18 |  | n10 |  |  |
| sl-DMRS-ScrambleID-DedicatedSL-PRS-RP-r18 |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-StartRB-SubchannelDedicatedSL-PRS-RP-r18 |  | 0 | start RB of BWP |  |
| sl-FilterCoefficient-r18 |  | fc4 |  |  |
| sl-ThreshS-RSSI-PRS-CBR-r18 |  | 0 | actual value is -112 dBm |  |
| sl-RB-Number-r18 |  | 10 |  |  |
| sl-TimeResource-r18 |  | 1111111100 | First 8 of every 10 logical slots |  |
|  |  | 0000000011 | Last 2 of every 10 logical slots | EXCEPTIONAL |
| sl-PosAllowedResourceSelectionConfig-r18 |  | c3 |  |  |
| sl-PRS-ResourceReservePeriodList-r18 SEQUENCE (SIZE (1..16)) OF SL-ReservationPeriodAllowedDedicatedSL-PRS-RP-r18 { |  | 1 entry |  |  |
| SL-ReservationPeriodAllowedDedicatedSL-PRS-RP-r18[1] CHOICE { |  |  | entry 1 |  |
| sl-ResourceReservePeriod1-r18 |  | ms0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-PRS-ResourcesDedicatedSL-PRS-RP-r18 SEQUENCE (SIZE (1..12)) OF SL-PRS-ResourceDedicatedSL-PRS-RP-r18 { |  | 1 entry |  |  |
| SL-PRS-ResourceDedicatedSL-PRS-RP-r18[1] SEQUENCE { |  |  | entry 1 |  |
| sl-PRS-ResourceID-r18 |  | 0 |  |  |
| sl-NumberOfSymbols-r18 |  | 4 |  |  |
| sl-CombSize-r18 |  | n4 |  |  |
| sl-PRS-starting-symbol-r18 |  | 4 |  |  |
| sl-PRS-comb-offset-r18 |  | 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-PRS-PowerControl-r18 |  | Not present |  |  |
| sl-SensingWindowDedicatedSL-PRS-RP-r18 |  | ms100 |  |  |
| sl-TxPercentageDedicatedSL-PRS-RP-List-r18 SEQUENCE (SIZE (8)) OF SL-TxPercentageDedicatedSL-PRS-RP-Config-r18 { |  | 8 entries |  |  |
| SL-TxPercentageDedicatedSL-PRS-RP-Config-r18 [k, k=1..8] SEQUENCE { |  |  | entry k |  |
| sl-TxPercentageDedicatedSL-PRS-RP-r18 |  | k |  |  |
| sl-Priority-DedicatedSL-PRS-RP |  | p50 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-SCI-basedSL-PRS-TxTriggerSCI1-B-r18 |  | Not present |  |  |
| sl-NumSubchannelDedicatedSL-PRS-RP-r18 |  | floor(n/m) | (NOTE 1, 2) |  |
| sl-SubchannelSizeDedicatedSL-PRS-RP-r18 |  | n10 |  |  |
| sl-MaxNumPerReserveDedicatedSL-PRS-RP-r18 |  | n2 |  |  |
| sl-NumReservedBitsSCI1B-DedicatedSL-PRS-RP-r18 |  | 2 |  |  |
| sl-SRC-ID-LenDedicatedSL-PRS-RP-r18 |  | n24 |  |  |
| sl-CBR-PriorityTxConfigDedicatedSL-PRS-RP-List-r18 |  | Not present |  |  |
| sl-TimeWindowSizeCBR-DedicatedSL-PRS-RP-r18 |  | slot100 |  |  |
| sl-TimeWindowSizeCR-DedicatedSL-PRS-RP-r18 |  | slot1000 |  |  |
| sl-CBR-CommonTxDedicatedSL-PRS-RP-List-r18 SEQUENCE { |  |  |  |  |
| sl-CBR-RangeDedicatedSL-PRS-RP-List-r18 SEQUENCE (SIZE (1..maxCBR-ConfigDedSL-PRS-1-r18)) OF SL-CBR-LevelsDedicatedSL-PRS-RP-r18 { |  | 1 entry |  |  |
| SL-CBR-LevelsDedicatedSL-PRS-RP-r18[1] SEQUENCE (SIZE (0..maxCBR-LevelDedSL-PRS-1-r18)) OF SL-CBR-Dedicated-SL-PRS-RP-r18 { |  | 2 entries | entry 1 |  |
| SL-CBR-Dedicated-SL-PRS-RP-r18[1] |  | 50 | entry 1 0<=CBR<0.5 |  |
| SL-CBR-Dedicated-SL-PRS-RP-r18[2] |  | 100 | entry 2 0.5<=CBR<1 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-CBR-SL-PRS-TxConfigList-r18 SEQUENCE (SIZE (1..maxNrofSL-PRS-TxConfig-r18)) OF SL-CBR-SL-PRS-TxConfig-r18 { |  | 1 entry |  |  |
| SL-CBR-SL-PRS-TxConfig-r18[1] SEQUENCE { |  |  | entry 1 |  |
| sl-PRS-CR-Limit-r18 |  | 10000 | CR limit = 100% |  |
| sl-PRS-MaxTx-power-r18 |  | Not present |  |  |
| sl-PRS-MaxNum-Transmissions-r18 |  | 5 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-PriorityThreshold-UL-URLLC-r18 |  | Not present |  |  |
| sl-PriorityThreshold-r18 |  | Not present |  |  |
| sl-SelectionWindowListDedicatedSL-PRS-RP-r18 SEQUENCE (SIZE (8)) OF SL-SelectionWindowConfigDedicated-SL-PRS-RP-r18 { |  | 8 entries |  |  |
| SL-SelectionWindowConfigDedicated-SL-PRS-RP-r18 [k, k=1..8] SEQUENCE { |  |  | entry k |  |
| sl-PRS-Priority-r18 |  | k |  |  |
| sl-PRS-SelectionWindow-r18 |  | n10 |  |  |
| } |  |  |  |  |
| sl-Thres-RSRP-ListDedicatedSL-PRS-RP-r18 SEQUENCE (SIZE (64)) OF SL-PRS-ThresRSRP-r18 { |  | 64 entries |  |  |
| INTEGER[k, k=1..64] |  | 0 | entry k |  |
| } |  |  |  |  |
| sl-PreemptionEnableDedicatedSL-PRS-RP-r18 |  | Not present |  |  |
| sl-Thres-RSRP-ListDedicatedSL-PRS-RP-r18 SEQUENCE (SIZE (64)) OF SL-PRS-ThresRSRP-r18 { |  | 64 entries |  |  |
| SL-PRS-ThresRSRP-r18 |  | 0 | entry k |  |
| } |  |  |  |  |
| sl-PreemptionEnableDedicatedSL-PRS-RP-r18 |  | Not present |  |  |
| } |  |  |  |  |
| Note 1: n is the bandwidth of active SL BWP (in RB). n is determined in TS 38.508-1 [30] Table 4.3.1.0D-1 and Table 4.3.1.0D-2 for FR1 and FR2 respectively. Note 2: m is subchannel size configured by sl-SubchannelSizeDedicatedSL-PRS-RP-r18. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EXCEPTIONAL | For exceptional resource pool |