3GPP TSG-RAN5 Meeting #109	R5-255783r1

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5167 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to N3Access MM test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_Ph1-CT_n3GPPA-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Incorrect step numbers are referenced in generic UE registration procedure. - The wrong generic procedure is referenced. - EPC message Attach Request is referenced. - Prefix ‘5GMM/5GSM:’ is missing for most NAS messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected step numbers referenced in generic UE registration procedure. - Corrected references to generic procedures. - Removed reference to Attach Request message. - Added the prefix ‘5GMM/5GSM:’ to all NAS messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases cannot be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.1, 9.2.5.1.4, 9.2.6.1.1, 9.2.6.2.1, 9.2.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: Added change 4. |  |  |  |  |  |  |  |  |

### 9.2.5	Registration

#### 9.2.5.1	Initial Registration

##### 9.2.5.1.1	Initial registration / Success / 5G-GUTI reallocation, Last visited TAI

9.2.5.1.1.1	Test Purpose (TP)

(1)

with { the UE has no valid 5G-GUTI but available SUCI and switched off }

ensure that {

when { the UE is switched on }

then { the UE sends a REGISTRATION REQUEST message including the SUCI in the 5GS mobile identity IE }

}

(2)

with { the UE is 5GMM-REGISTERED state with assigned 5G-GUTI and last visited registered TAI and switched off }

ensure that {

when { the UE is switched on }

then { the UE sends a REGISTRATION REQUEST message including the 5G-GUTI assigned previously in the 5GS mobile identity IE and the last visited registered TAI }

}

9.2.5.1.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501 clauses 5.5.1.2.2 and 5.5.1.2.4. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.1.2.2]

The UE initiates the registration procedure for initial registration by sending a REGISTRATION REQUEST message to the AMF, starting timer T3510. If timer T3502 is currently running, the UE shall stop timer T3502. If timer T3511 is currently running, the UE shall stop timer T3511.

During initial registration the UE handles the 5GS mobile identity IE in the following order:

…

b)	if the UE holds a valid 5G-GUTI that was previously assigned, over 3GPP access or non-3GPP access, by the same PLMN with which the UE is performing the registration, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE;

c)	if the UE holds a valid 5G-GUTI that was previously assigned, over 3GPP access or non-3GPP access, by an equivalent PLMN, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE;

d)	if the UE holds a valid 5G-GUTI that was previously assigned, over 3GPP access or non-3GPP, by any other PLMN, the UE shall indicate the 5G-GUTI in the 5GS mobile identity IE;

e)	if a SUCI is available the UE shall include the SUCI in the 5GS mobile identity IE; and

If the SUCI is included in the 5GS mobile identity IE and the timer T3519 is not running, the UE shall start timer T3519 and store the value of the SUCI sent in the REGISTRATION REQUEST message. The UE shall include the stored SUCI in the REGISTRATION REQUEST message while timer T3519 is running.

…

If the last visited registered TAI is available, the UE shall include the last visited registered TAI in the REGISTRATION REQUEST message.

[TS 24.501, clause 5.5.1.2.4]

The 5G-GUTI reallocation shall be part of the initial registration procedure. During the initial registration procedure, if the AMF has not allocated a new 5G-GUTI by the generic UE configuration update procedure, the AMF shall include in the REGISTRATION ACCEPT message the new assigned 5G-GUTI together with the assigned TAI list.

9.2.5.1.1.3	Test description

9.2.5.1.1.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27

UE:

-	None.

Preamble:

-	The UE is in state Switched OFF (state 0W-B) according to TS 38.508-1 [4].

9.2.5.1.1.3.2	Test procedure sequence

Table 9.2.5.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-8 | UE establishes an IPsec SA and trigger 5GMM Registration procedure by executing steps 1 to 7 of Table 4.5.2.2-3 in TS 38.508-1 [4]. | - | - | - | - |
| 9 | SS transmits an REGISTRATION REJECT message with the 5GMM cause IE setting as “Illegal ME”. NOTE 1: 5G-GUTI-1 should be deleted, then UE has no valid 5G-GUTI but available SUCI now. | <-- | 5GMM: REGISTRATION REJECT | - | - |
| 10 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |
| 11 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 12 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 13-23 | Steps 1-11a1 of Table 4.5.2.2-3 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | 1 | P |
| 24 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 25 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 26-35 | Steps 1-11a1 of Table 4.5.2.2-3 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | 2 | P |

9.2.5.1.1.3.3	Specific message contents

Table 9.2.5.1.1.3.3-1: Message REGISTRATION REJECT (step 9, Table 9.2.5.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GMM cause | '0000 0011'B | Illegal UE |  |

Table 9.2.5.1.1.3.3-2: Message REGISTRATION REQUEST (step 15, Table 9.2.5.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type | '001'B | Initial registration |  |
| 5GS mobile identity | SUCI | The SUCI of UE |  |

Table 9.2.5.1.1.3.3-3: Message REGISTRATION ACCEPT (step 21, Table 9.2.5.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5G-GUTI | 5G-GUTI-2 |  |  |

Table 9.2.5.1.1.3.3-4: Message REGISTRATION REQUEST (step 34, Table9.2.5.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type | '001'B | Initial registration |  |
| 5GS mobile identity | 5G-GUTI-2 |  |  |
| Last visited registered TAI | TAI-1 | N3GPP TAI |  |

<End of modified section>

<Start of next modified section>

##### 9.2.5.1.4	Initial registration / Rejected / Congestion / Abnormal cases / T3346

9.2.5.1.4.1	Test Purpose (TP)

(1)

with { The UE has sent initial REGISTRAION REQUEST message }

ensure that {

when { UE receives a REGISTRATION REJECT with cause #22 (Congestion) with T3346 included and the UE is NOT configured for High Priority Access }

then { UE does not start the Initial registration until T3346 expires }

}

(2)

with { The UE has received initial REGISTRATION REJECT with T3346 included }

ensure that {

when { upon expiry of T3346 }

then { UE starts the Initial registration procedure }

}

(3)

with { The UE has received initial REGISTRATION REJECT with T3346 included }

ensure that {

when { the timer T3346 is running and the UE needs to perform initial registration for emergency services }

then { UE starts the Initial registration procedure }

}

9.2.5.1.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 5.5.1.2.5 and 5.5.1.2.7 and TS 24.301, clause 5.5.1.2.5. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.1.2.5]

If the initial registration request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

If the initial registration request is rejected due to general NAS level mobility management congestion control, the network shall set the 5GMM cause value to #22 "congestion" and assign a back-off timer T3346.

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

#3	(Illegal UE); or

#6	(Illegal ME).

…

#22	(Congestion).

If the T3346 value IE is present in the REGISTRATION REJECT message and the value indicates that this timer is neither zero nor deactivated, the UE shall proceed as described below; otherwise it shall be considered as an abnormal case and the behaviour of the UE for this case is specified in subclause 5.5.1.2.7.

The UE shall abort the initial registration procedure, set the 5GS update status to 5U2 NOT UPDATED and enter state 5GMM-DEREGISTERED.ATTEMPTING-REGISTRATION.

The UE shall stop timer T3346 if it is running.

If the REGISTRATION REJECT message is integrity protected, the UE shall start timer T3346 with the value provided in the T3346 value IE.

If the REGISTRATION REJECT message is not integrity protected, the UE shall start timer T3346 with a random value from the default range specified in 3GPP TS 24.008 [12].

The UE stays in the current serving cell and applies the normal cell reselection process. The initial registration procedure is started if still needed when timer T3346 expires or is stopped.

If the UE is operating in single-registration mode, the UE shall handle the EMM parameters EMM state, EPS update status, and attach attempt counter as specified in 3GPP TS 24.301 [15] for the case when the EPS attach request procedure is rejected with the EMM cause with the same value.

…

Other values are considered as abnormal cases. The behaviour of the UE in those cases is specified in subclause 5.5.1.2.7.

[TS 24.501, clause 5.5.1.2.7]

The following abnormal cases can be identified:

a)	Timer T3346 is running.

The UE shall not start the registration procedure for initial registration unless:

1)	the UE is a UE configured for high priority access in selected PLMN;

2)	the UE needs to perform the registration procedure for initial registration for emergency services; or

3)	the UE receives a DEREGISTRATION REQUEST message with the "re-registration required" indication.

The UE stays in the current serving cell and applies the normal cell reselection process.

NOTE 1:	It is considered an abnormal case if the UE needs to initiate a registration procedure for initial registration while timer T3346 is running independent on whether timer T3346 was started due to an abnormal case or a non-successful case.

[TS 24.301, clause 5.5.1.2.5]

…

#22	(Congestion);

If the T3346 value IE is present in the ATTACH REJECT message and the value indicates that this timer is neither zero nor deactivated, the UE shall proceed as described below; otherwise it shall be considered as an abnormal case and the behaviour of the UE for this case is specified in subclause 5.5.1.2.6.

The UE shall abort the attach procedure, reset the attach attempt counter, set the EPS update status to EU2 NOT UPDATED and enter state EMM-DEREGISTERED.ATTEMPTING-TO-ATTACH.

The UE shall stop timer T3346 if it is running.

If the ATTACH REJECT message is integrity protected, the UE shall start timer T3346 with the value provided in the T3346 value IE.

If the ATTACH REJECT message is not integrity protected, the UE shall start timer T3346 with a random value from the default range specified in 3GPP TS 24.008 [13].

The UE stays in the current serving cell and applies the normal cell reselection process. The attach procedure is started if still needed when timer T3346 expires or is stopped.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GMM parameters GMM state, GPRS update status and GPRS attach attempt counter as specified in 3GPP TS 24.008 [13] for the case when the normal attach procedure is rejected with the GMM cause with the same value.

If the UE is operating in single-registration mode, the UE shall in addition handle the 5GMM parameters as specified in 3GPP TS 24.501 [54] for the case when the initial registration procedure is rejected with the 5GMM cause with the same value.

…

9.2.5.1.4.3	Test description

9.2.5.1.4.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27

UE:

None.

Preamble:

-	The UE is in state Switched OFF (State 0W-A as per TS 38.508-1 [4] Table 4.4A.2-0).

9.2.5.1.4.3.2	Test procedure sequence

Table 9.2.5.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is switched on. | - | - | - |  |
| 2-8 | Steps 1-7 of Table 4.5.2.2-3 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 9 | SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes. (Note 1) | <-- | 5GMM: REGISTRATION REJECT | - | - |
| 10 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |
| 11 | Check: Does the UE initiate IPSec secure tunnel as per 3GPP TS 24.502 [33] clause 7.3.2 within T3346 minutes of Step 9? (Note 1) | - | - | 1 | F |
| 12-18 | Steps 1-7 of Table 4.5.2.2-3 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | 2 | P |
| 19 | SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes. (Note 1) | <-- | 5GMM: REGISTRATION REJECT | - | - |
| 20 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |
| 21 | The UE is made to establish an Emergency PDU session. (Note 2) | - | - | - | - |
| 22-32 | Steps 1-11a1 of Table 4.5.2.2-3 of the generic procedure in TS 38.508-1 [4] are performed, REGISTRATION REQUEST message with IE 5GS registration type set to "Emergency registration" or “Initial registration”. (Note 3) | - | - | 3 | P |
| Note 1: T3346 is set to 3 minutes. This is checked for 3 minutes less tolerance. Note 2: This can be done by an AT/MMI command. Note 3: 5GS registration type can set to either "Initial registration" according to TS 24.501 clause 5.2.2.3.3 or "Emergency registration" according to TS 24.501 clause 5.5.1.2. |  |  |  |  |  |

9.2.5.1.4.3.3	Specific message contents

Table 9.2.5.1.4.3.3-1: REGISTRATION REJECT (steps 9, 19 Table 9.2.5.1.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GMM cause | ’0001 0110’B | Cause #22 (Congestion) |  |
| T3346 Value | ‘00100011’B | 3 minutes |  |

Table 9.2.5.1.4.3.3-2: REGISTRATION REQUEST (step 25 Table 9.2.5.1.4.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.7.1-6, Condition EMERGENCY or INITIAL.

<End of modified section>

<Start of next modified section>

### 9.2.6	De-registration

#### 9.2.6.1	UE-initiated de-registration

##### 9.2.6.1.1	UE-initiated de-registration / switch off

9.2.6.1.1.1	Test Purpose (TP)

(1)

with { the UE in 5GMM-REGISTERED state }

ensure that {

when { the UE is switched off }

then { the UE shall send DEREGISTRATION REQUEST message with De-registration type IE indicated to "Switch off" }

}

(2)

with { the UE supports remove USIM without power down and in 5GMM-REGISTERED state }

ensure that {

when { the USIM is removed from the UE }

then { the UE shall send DEREGISTRATION REQUEST message with De-registration type IE indicated to "Switch off" }

}

(3)

with { the UE in 5GMM-DEREGISTERED-INTIATED state }

ensure that {

when { the first four expiries of the timer T3521 }

then { the UE shall retransmit the DEREGISTRATION REQUEST message and shall reset and restart timer T3521 }

}

(4)

with { the UE in 5GMM-DEREGISTERED-INTIATED state }

ensure that {

when { On the fifth expiry of timer T3521 }

then { the deregistration procedure shall be aborted and the UE perform local detach }

}

9.2.6.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 5.5.2.1, 5.5.2.2.1 and 5.5.2.2.6. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.2.1]

The de-registration procedure is used:

a)	by the UE to de-register for 5GS services over 3GPP access when the UE is registered over 3GPP access;;

b)	by the UE to de-register for 5GS services over 3GPP access, non-3GPP access, or both when the UE is registered in the same PLMN over both accesses;

c)	by the network to inform the UE that it is deregistered for 5GS services over 3GPP access when the UE is registered over 3GPP access;

d)	by the network to inform the UE that it is deregistered for 5GS services over 3GPP access, non-3GPP access, or both when the UE is registered in the same PLMN over both accesses; and

e)	by the network to inform the UE to re-register to the network.

The de-registration procedure with appropriate de-registration type shall be invoked by the UE:

a)	if the UE is switched off; and

b)	as part of the eCall inactivity procedure defined in subclause 5.5.3.

The de-registration procedure with appropriate de-registration type shall be invoked by the network:

a)	if the network informs whether the UE should re-register to the network.

The de-registration procedure with appropriate access type shall be invoked by the UE:

a)	if the UE wants to de-register for 5GS services over 3GPP access when the UE is registered over 3GPP access; or

b)	the UE wants to de-register for 5GS services over 3GPP access, non-3GPP access, or both when the UE is registered in the same PLMN over both accesses.

If the de-registration procedure is triggered due to USIM removal, the UE shall indicate "switch off" in the de-registration type IE.

If the de-registration procedure is requested by the UDM for a UE that has an emergency PDU session, the AMF shall not send a DEREGISTRATION REQUEST message to the UE.

If the de-registration procedure for 5GS services is performed, the PDU sessions, if any, for this particular UE are released locally without peer-to-peer signalling between the UE and the network.

The UE is allowed to initiate the de-registration procedure even if the timer T3346 is running.

NOTE:	When the UE has no PDU sessions over non-3GPP access, or the UE moves all the PDU sessions over a non-3GPP access to a 3GPP access, the UE and the AMF need not initiate de-registration over the non-3GPP access.

The AMF shall provide the UE with a non-3GPP de-registration timer.

[TS 24.501, clause 5.5.2.2.1]

The de-registration procedure is initiated by the UE by sending a DEREGISTRATION REQUEST message (see example in figure 5.5.2.2.1). The De-registration type IE included in the message indicates whether the de-registration procedure is due to a "switch off" or not. The access type included in the message indicates whether the de-registration procedure is:

a)	for 5GS services over 3GPP access when the UE is registered over 3GPP access only;

b)	for 5GS services over non-3GPP access when the UE is registered over non-3GPP access only; or

c)	for 5GS services over 3GPP access, non-3GPP access or both 3GPP access and non-3GPP access when the UE is registered in the same PLMN over both accesses.

If the UE has a valid 5G-GUTI, the UE shall populate the 5GS mobile identity IE with the valid 5G-GUTI. If the UE does not have a valid 5G-GUTI, the UE shall populate the 5GS mobile identity IE with its SUCI.

If the UE does not have a valid 5G-GUTI and it does not have a valid SUCI, then the UE shall populate the5GSmobile identity IE with its PEI.

If the de-registration request is not due to switch off and the UE is in the state 5GMM-REGISTERED or 5GMM-REGISTERED-INITIATED, timer T3521 shall be started in the UE after the DEREGISTRATION REQUEST message has been sent. The UE shall enter the state 5GMM-DEREGISTERED-INITIATED.

If the UE is to be switched off, the UE shall try for a period of 5 seconds to send the DEREGISTRATION REQUEST message. During this period, the UE may be switched off as soon as the DEREGISTRATION REQUEST message has been sent.

[TS 24.501, clause 5.5.2.2.6]

...

c)	T3521 timeout.

On the first four expiries of the timer, the UE shall retransmit the DEREGISTRATION REQUEST message and shall reset and restart timer T3521. On the fifth expiry of timer T3521, the de-registration procedure shall be aborted and the UE proceeds as follows:

9.2.6.1.1.3	Test description

9.2.6.1.1.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27.

UE:

-	None.

Preamble:

-	The UE is in state 3W-A on WLAN Cell 27 according to 38.508-1 [4].

9.2.6.1.1.3.2	Test procedure sequence

Table 9.2.6.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause switch off | - | - | - | - |
| 2 | Check: Does the UE transmit a DEREGISTRATION REQUEST with the De-registration type IE indicating "switch off"? | --> | 5GMM: DEREGISTRATION REQUEST | 1 | P |
| 3 | SS Transmits DEREGISTRATION ACCEPT | <-- | 5GMM: DEREGISTRATION ACCEPT | - | - |
| 4 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |
| 5 | The UE is switched on. | - | - | - | - |
| 6-15 | The registration procedure is completed by executing steps 1-10 of the UE registration procedure in TS 38.508-1 [4] table 4.5.2.2-3. | - | - | - | - |
| 16 | Cause UE to initiate deregistration. | - | - | - | - |
| 17 | The UE transmits a DEREGISTRATION REQUEST message. The UE starts timer T3521. | --> | 5GMM: DEREGISTRATION REQUEST | - | - |
| 18 | The SS does not respond to the DEREGISTRATION REQUEST message. | - | - | - | - |
| 19 | Check: When the timer T3521 expires does the UE re-transmit DETACH REQUEST message. Timer T3421 is re-started (1st expiry). | --> | 5GMM: DEREGISTRATION REQUEST | 3 | P |
| 20 | The SS does not respond to the DEREGISTRATION REQUEST message. | - | - | - | - |
| 21 | Check: When the timer T3521 expires does the UE re-transmit DEREGISTRATION REQUEST message. Timer T3521 is re-started (2nd expiry). | --> | 5GMM: DEREGISTRATION REQUEST | 3 | P |
| 22 | The SS does not respond to the DEREGISTRATION REQUEST message. | - | - | - | - |
| 23 | Check: When the timer T3521 expires does the UE re-transmit DEREGISTRATION REQUEST message. Timer T3521 is re-started (3rd expiry). | --> | 5GMM: DEREGISTRATION REQUEST | 3 | P |
| 24 | The SS does not respond to the DEREGISTRATION REQUEST message. | - | - | - | - |
| 25 | Check: When the timer T3521 expires does the UE re-transmit DEREGISTRATION REQUEST message. Timer T3521 is re-started (4th expiry). | --> | 5GMM: DEREGISTRATION REQUEST | 3 | P |
| 26 | The SS does not respond to the DETACH REQUEST message. | - | - | - | - |
| 27 | When the timer T3521 expires the UE aborts the detach procedure and performs a local detach (5th expiry). | - | - | 4 | P |
| 28 | SS Transmits PDU SESSION MODIFICATION COMMAND | <-- | 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 29 | Check: Does the UE transmit a PDU SESSION MODIFICATION COMPLETE? | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | 4 | F |
| 30 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |
| - | EXCEPTION: Steps 31 to 46 shall be implemented if the UE supports remove USIM without power down: pc_USIM_Removal = TRUE [29] | - | - | - | - |
| 31 | The UE is switched off. | - | - | - | - |
| 32 | The UE is switched on. | - | - | - | - |
| 33-42 | The registration procedure is completed by executing steps 1-10 of the UE registration procedure in TS 38.508-1 [4] table 4.5.2.2-3. | - | - | - | - |
| 43 | Cause removal of USIM from the UE without powering down. | - | - | - | - |
| 44 | Check: Does the UE transmit a DEREGISTRATION REQUEST with the De-registration type IE indicating "switch off"? | --> | 5GMM: DEREGISTRATION REQUEST | 2 | P |
| 45 | SS Transmits DEREGISTRATION ACCEPT | <-- | 5GMM: DEREGISTRATION ACCEPT | - | - |
| 46 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |

9.2.6.1.1.3.3	Specific message contents

Table 9.2.6.1.1.3.3-1: DEREGISTRATION REQUEST (Step 1 and step 44, Table 9.2.6.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| De-registration type |  |  |  |
| Switch off | ‘1’B |  |  |

#### 9.2.6.2	Network-initiated de-registration

##### 9.2.6.2.1	Network-initiated de-registration / De-registration for Non-3GPP access / Re-registration required

9.2.6.2.1.1	Test Purpose (TP)

(1)

with { the UE in 5GMM-REGISTERED state }

ensure that {

when { the SS sends a DEREGISTRATION REQUEST message indicates "re-registration required" and the de-registration request is for non 3GPP access }

then { the UE sends a DEREGISTRATION ACCEPT message to the network and releases the existing NAS signalling connection, then initiates an initial registration and also re-establishes any previously established PDU sessions }

}

9.2.6.2.1.2	Conformance requirements

References: The conformance requirement covered in the present TC is specified in: 3GPP TS 24.501 clauses 5.5.2.3.2. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501 clause 5.5.2.3.2]

NOTE 1:	When the de-registration type indicates "re-registration required", user interaction is necessary in some cases when the UE cannot re-establish the PDU session (s), if any, automatically.

…

Upon sending a DEREGISTRATION ACCEPT message, the UE shall delete the rejected NSSAI as specified in subclause 4.6.2.2.

If the de-registration type indicates "re-registration required", then the UE shall ignore the 5GMM cause IE if received.

If the de-registration type indicates "re-registration not required", the UE shall take the actions depending on the received 5GMM cause value:

#3	(Illegal UE);

#6	(Illegal ME); or

#7	(5GS services not allowed).

The UE shall set the 5GS update status to 5U3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.2.2) and shall delete any 5G-GUTI, last visited registered TAI, TAI list and ngKSI. The UE shall consider the USIM as invalid for 5GS services until switching off or the UICC containing the USIM is removed. The UE shall delete the list of equivalent PLMNs and shall enter the state 5GMM-DEREGISTERED.

If the UE is operating in single-registration mode, the UE shall handle the EMM parameters EMM state, EPS update status, 4G-GUTI, last visited registered TAI, TAI list and eKSI as specified in 3GPP TS 24.301 [15] for the case when a DETACH REQUEST is received with the EMM cause with the same value and with detach type set to "re-attach not required".

NOTE 2:	The possibility to configure a UE so that the radio transceiver for a specific radio access technology is not active, although it is implemented in the UE, is out of scope of the present specification.

If the UE also supports the registration procedure over the other access, the UE shall in addition handle 5GMM parameters and 5GMM state for this access, as described for this 5GMM cause value.

...

#72	(Non-3GPP access to 5GCN not allowed).

The UE shall set the 5GS update status to 5U3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.2.2) and shall delete 5G-GUTI, last visited registered TAI, TAI list and ngKSI. Additionally, the UE shall reset the registration attempt counter and enter the state 5GMM-DEREGISTERED.

NOTE 3:	The 5GMM sublayer states, the 5GMM parameters and the registration status are managed per access type independently, i.e. 3GPP access or non-3GPP access (see subclauses 4.7.2 and 5.1.3).

The UE shall disable the N1 mode capability for non-3GPP access (see subclause 4.9.3).

As an implementation option, the UE may enter the state 5GMM-DEREGISTERED.PLMN-SEARCH in order to perform a PLMN selection according to 3GPP TS 23.122 [5].

9.2.6.2.1.3	Test description

9.2.6.2.1.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27.

UE:

-	None.

Preamble:

-	the UE is in state 3W-A on WLAN Cell 27 according to TS 38.508-1 [4].

9.2.6.2.1.3.2	Test procedure sequence

Table 9.2.6.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | The SS transmits a DEREGISTRATION REQUEST with indicates "re-registration required". | <-- | 5GMM: DEREGISTRATION REQUEST | - | - |
| 2 | Check: Does the UE transmit a DEREGISTRATION ACCEPT message? | --> | 5GMM: DEREGISTRATION ACCEPT | 1 | P |
| 3 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |
| - | EXCEPTION: step 4 describes a behaviour which depends on the UE capability | - | - | - | - |
| 4 | IF NOT pc_Automatic_Re_registration, the user initiates a registration by MMI. | - | - | - | - |
| 6-15 | The registration procedure is completed by executing steps 1-10 of the UE registration procedure in TS 38.508-1 [4] table 4.5.2.2-3. | - | - | 1 | P |

9.2.6.2.1.3.3	Specific message contents

Table 9.2.6.2.1.3.3-1: DEREGISTRATION REQUEST (step 1, Table 9.2.6.2.1.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.7.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| DEREGISTRATION type |  |  |  |
| Switch off | '0’B | Normal de-registration |  |
| Re-registration required | ‘1’B | re-registration required |  |
| Access type | ‘10’B | Non 3GPP access |  |
| 5GMM cause | '0000 0011’B | Cause #3 (Illegal UE) |  |

Table 9.2.6.2.1.3.3-2: REGISTRATION REQUEST (step 8, Table 9.2.6.2.1.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS mobile identity | The valid 5G-GUTI that UE holds |  |  |

##### 9.2.6.2.2	Network-initiated de-registration / De-registration for Non 3GPP access / Re-registration not required

9.2.6.2.2.1	Test Purpose (TP)

(1)

with { the UE in 5GMM-REGISTERED state }

ensure that {

when { the SS sends a DEREGISTRATION REQUEST message indicates no 5GMM cause IE, ""re-registration not required"" and the de-registration request is for npn-3GPP access and 5GMM cause value is not included }

then { the UE deletes 5G-GUTI, TAI list, last visited registered TAI, list of equivalent PLMNs, ngKSI, sends a DEREGISTRATION ACCEPT message enter the state 5GMM-DEREGISTERED for non 3GPP access }

}

(2)

with { the UE in 5GMM-REGISTERED state }

ensure that {

when { the SS sends a DEREGISTRATION REQUEST message indicates no 5GMM cause IE, "re-registration not required" and the de-registration request is for non-3GPP access and 5GMM cause value set to #7	5GS services not allowed }

then { the UE deletes 5G-GUTI, TAI list, last visited registered TAI, list of equivalent PLMNs, ngKSI, consider the USIM as invalid for 5GS services until switching off or the UICC containing the USIM is removed, sends a DEREGISTRATION ACCEPT message enter the state 5GMM-DEREGISTERED for Non 3GPP access }

}

(3)

with { the UE in 5GMM-REGISTERED state }

ensure that {

when { the SS sends a DEREGISTRATION REQUEST message indicates no 5GMM cause IE, "re-registration not required" and the de-registration request is for non-3GPP access and 5GMM cause value set to #72	Non-3GPP access to 5GCN not allowed }

then { the UE deletes 5G-GUTI, TAI list, last visited registered TAI, ngKSI, disable the N1 mode capability for non-3GPP access, sends a DEREGISTRATION ACCEPT message enter the state 5GMM-DEREGISTERED for Non 3GPP access }

}

9.2.6.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501 clauses 5.5.2.3.2 and 5.5.2.3.4. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.2.3.2]

Upon receiving the DEREGISTRATION REQUEST message, if the DEREGISTRATION REQUEST message indicates "re-registration not required" and the de-registration request is for 3GPP access, the UE shall release locally the PDU sessions over 3GPP access, if any. The UE shall send a DEREGISTRATION ACCEPT message to the network and enter the state 5GMM-DEREGISTERED for 3GPP access.

...

If the de-registration type indicates "re-registration not required", the UE shall take the actions depending on the received 5GMM cause value:

...

#7	(5GS services not allowed).

The UE shall set the 5GS update status to 5U3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.2.2) and shall delete any 5G-GUTI, last visited registered TAI, TAI list and ngKSI. The UE shall consider the USIM as invalid for 5GS services until switching off or the UICC containing the USIM is removed. The UE shall delete the list of equivalent PLMNs and shall enter the state 5GMM-DEREGISTERED.

If the UE is operating in single-registration mode, the UE shall handle the EMM parameters EMM state, EPS update status, 4G-GUTI, last visited registered TAI, TAI list and eKSI as specified in 3GPP TS 24.301 [15] for the case when a DETACH REQUEST is received with the EMM cause with the same value and with detach type set to "re-attach not required".

NOTE 2:	The possibility to configure a UE so that the radio transceiver for a specific radio access technology is not active, although it is implemented in the UE, is out of scope of the present specification.

If the UE also supports the registration procedure over the other access, the UE shall in addition handle 5GMM parameters and 5GMM state for this access, as described for this 5GMM cause value.

...

#72	(Non-3GPP access to 5GCN not allowed).

The UE shall set the 5GS update status to 5U3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.2.2) and shall delete 5G-GUTI, last visited registered TAI, TAI list and ngKSI. Additionally, the UE shall reset the registration attempt counter and enter the state 5GMM-DEREGISTERED.

NOTE 3:	The 5GMM sublayer states, the 5GMM parameters and the registration status are managed per access type independently, i.e. 3GPP access or non-3GPP access (see subclauses 4.7.2 and 5.1.3).

The UE shall disable the N1 mode capability for non-3GPP access (see subclause 4.9.3).

As an implementation option, the UE may enter the state 5GMM-DEREGISTERED.PLMN-SEARCH in order to perform a PLMN selection according to 3GPP TS 23.122 [5].

[TS 24.501, clause 5.5.2.3.4]

b)	DEREGISTRATION REQUEST, other 5GMM cause values than those treated in subclause 5.5.2.3.2 or no 5GMM cause IE is included, and the De-registration type IE indicates "re-registration not required".

The UE shall delete 5G-GUTI, TAI list, last visited registered TAI, list of equivalent PLMNs, ngKSI, shall set the 5GS update status to 5U2 NOT UPDATED and shall start timer T3502.

A UE not supporting S1 mode may enter the state 5GMM-DEREGISTERED.PLMN-SEARCH in order to perform a PLMN selection according to 3GPP TS 23.122 [5]; otherwise the UE shall enter the state 5GMM-DEREGISTERED.ATTEMPTING-REGISTRATION.

A UE operating in single-registration mode shall:

-	enter the state 5GMM-DEREGISTERED and attempt to select E-UTRAN radio access technology and proceed with the appropriate EMM specific procedures. In this case, the UE may disable N1 mode capability (see subclause 4.9); or

-	enter the state 5GMM-DEREGISTERED.PLMN-SEARCH in order to perform a PLMN selection according to 3GPP TS 23.122 [5].

A UE operating in single-registration mode shall set the EPS update status to EU2 NOT UPDATED and shall delete the EMM parameters 4G-GUTI, last visited registered TAI, TAI list and eKSI and shall enter the state EMM-DEREGISTERED.

9.2.6.2.2.3	Test description

9.2.6.2.2.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27.

UE:

- 	None

Preamble:

-	The UE is in state 3W-A on WLAN Cell 27 according to TS 38.508-1 [4].

9.2.6.2.2.3.2	Test procedure sequence

Table 9.2.6.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | SS sends a DEREGISTRATION REQUEST message indicates no 5GMM cause IE, "re-registration not required" and the de-registration request is for non 3GPP access | <-- | 5GMM: DEREGISTRATION REQUEST | - | - |
| 2 | Check: Does the UE transmit an DEREGISTRATION ACCEPT message? | --> | 5GMM: DEREGISTRATION ACCEPT | 1 | P |
| 3 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |
| 4 | Cause UE to initiate registration. | - | - | - | - |
| 5-15 | The registration procedure is completed by executing steps 1-10 of the UE registration procedure in TS 38.508-1 [4] table 4.5.2.2-3. | - | - | 1 | P |
| 16 | SS sends a DEREGISTRATION REQUEST message indicates no 5GMM cause IE, "re-registration not required", 5GMM Cause set to #7 (5GS services not allowed) and the de-registration request is for non 3GPP access | <-- | 5GMM: DEREGISTRATION REQUEST | - | - |
| 17 | Check: Does the UE transmit an DEREGISTRATION ACCEPT message? | --> | 5GMM: DEREGISTRATION ACCEPT | 2 | P |
| 18 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |
| 19 | Check for 60 seconds if UE initiates Registration procedure | - | 5GMM: REGISTRATION REQUEST | 2 | F |
| 20 | The UE is switched off. | - | - | - | - |
| 21 | The UE is switched on. | - | - | - | - |
| 22-31 | The registration procedure is completed by executing steps 1-10 of the UE registration procedure in TS 38.508-1 [4] table 4.5.2.2-3. | - | - | - | - |
| 32 | SS sends a DEREGISTRATION REQUEST message indicates no 5GMM cause IE, "re-registration not required", 5GMM Cause set to #72 (Non-3GPP access to 5GCN not allowed) and the de-registration request is for non 3GPP access | <-- | 5GMM: DEREGISTRATION REQUEST | - | - |
| 33 | Check: Does the UE transmit a DEREGISTRATION ACCEPT message? | --> | 5GMM: DEREGISTRATION ACCEPT | 3 | P |
| 34 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5 of TS 38.508-1 [4], takes place performing disconnection of security association. | - | - | - | - |
| 35 | Check for 60 seconds if UE initiates Registration procedure | - | 5GMM: REGISTRATION REQUEST | 3 | F |
| 36 | The UE is switched off. | - | - | - | - |

9.2.6.2.2.3.3	Specific message contents

Table 9.2.6.2.2.3.3-1: Message DEREGISTRATION REQUEST (step 1, Table 9.2.6.2.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| De-registration type |  |  |  |
| Switch off | ‘0’B | Normal de-registration |  |
| Re-registration required | ‘0’B | re-registration not required |  |
| Access type | ‘01’B | 3GPP access |  |
| 5GMM cause | Not Present |  |  |

Table 9.2.6.2.2.3.3-2: Message REGISTRATION REQUEST (step 8, Table 9.2.6.2.2.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ngKSI |  |  |  |
| NAS key set identifier | '111'B | no key is available (UE to network) |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111" |  |
| 5GS mobile identity | The valid SUCI |  |  |
| Last visited registered TAI | Not present |  |  |