3GPP TSG-RAN WG5 Meeting #109	R5-2560434r2

# Dallas, USA, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5497 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of IoT NTN enh TC 22.5.24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | IoT NTN enhancement TC 22.5.24 for enhanced discontinuous coverage is missing and needs to be added as per WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IoT NTN enhancement TC 22.5.24 for enhanced discontinuous coverage has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.24 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.508 CR 1555 TS 36.523-2 CR 1515 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The related 36.508 CR is R5-256045 (CR number 1555). R5-256045 (CR number) defines new NAS IE for enhanced discontinuous coverage. The related applicability CR is R5-256044 (CR number 1515) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Correct the format issue in table Table 22.5.24.3.3-1 and Table 22.5.24.3.3-2. Add a new message content table Table 22.5.24.3.3-6 of TAU accept message for TP3 and add a SIB31 table for Ncell11. Update conformance requirements part to add core spec description for TP3. Update the TP3 since the previous wording is not clear. Update the pre-test condition and main behaviour procedure as per TF160 suggestion. R2: Update the main behaviour procedure to fix typos and only keep one branch for TP3 after offline discussion. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

Table 22.5.23.3.3-3: SystemInformationBlockType31-NB on Ncell 62 (preamble and all steps, Table 22.5.23.3.2-2)

| Derivation Path: TS 36.508 [18], Table 8.1.4.3.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 | Refer to the 2nd cell in TS 36.508 Table 6.3.5.1-1 |  | GSO |
| servingSatelliteInfo-r17 | Refer to the 2nd cell in TS 36.508 Table 6.3.5.2-1 |  | NGSO |
| } |  |  |  |

22.5.24	NB-IoT / NTN / enhanced discontinuous coverage

22.5.24.1	Test Purpose (TP)

(1)

with { the UE is switched-off and supports enhanced discontinuous coverage }

ensure that {

when { UE is powered on }

then { the UE transmits an ATTACH REQUEST message with the EDC bit set to "Enhanced discontinuous coverage supported" in the UE network capability IE }

}

(2)

with { UE in EMM-REGISTERED state and supports enhanced discontinuous coverage }

ensure that {

when { the UE is about to lose satellite coverage }

then { the UE transmits a TRACKING AREA UPDATE REQUEST message with the unavailability type in the Unavailability information IE set to "unavailability due to discontinuous coverage" }

}

(3)

with { UE is not under satellite coverage and MME indicates UE needs to report end of unavailability period in Unavailability configuration IE }

ensure that {

when { the unavailability period duration has ended }

then { the UE transmits a TRACKING AREA UPDATE REQUEST message }

}

22.5.24.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 4.11.4, 5.5.1.2 and 5.5.3.2. Unless and otherwise stated these are Rel-18 requirements.

[TS 24.301, clause 4.11.4]

If the UE and network support enhanced discontinuous coverage, the UE may provide unavailability information to the network in the tracking area updating procedure if the UE is registered over satellite E-UTRAN access (see 3GPP TS 23.401 [10]). The unavailability information includes the unavailability period duration if known, and the start of the unavailability period if known. The support for the enhanced discontinuous coverage is negotiated in the attach procedure or the tracking area updating procedure. The MME may consider the unavailability period duration if provided by the UE to determine the unavailability period duration of the UE. The MME may consider the start of the unavailability period if provided by the UE to determine the start of the unavailability period of the UE. The MME may provide the unavailability period duration or the start of the unavailability period or both to the UE in the TRACKING AREA UPDATE ACCEPT message or the ATTACH ACCEPT message.

If the UE provided unavailability information in the last tracking area updating procedure, the MME considers the UE unreachable until the UE registers for normal service without providing unavailability information. If the UE did not include a start of the unavailability period, the MME shall consider the start of the unavailability period to be the time at which MME received the TRACKING AREA UPDATE REQUEST message from the UE. The MME may determine the values of the periodic tracking area update timer (T3412), extended idle mode DRX cycle parameters and PSM mode configuration parameters provided to the UE based on the discontinuous coverage maximum time offset, the unavailability period duration and the start of the unavailability period, if available. The MME releases the NAS signalling connection after the completion of the tracking area updating procedure in which the UE provided unavailability information without providing the start of the unavailability period.

[TS 24.301, clause 4.11.4]

If the MME sets the EUPR bit in Unavailability configuration IE value to "UE does not need to report end of unavailability period", then upon returning to coverage to a TAI in the TAI list, the UE shall choose an appropriate substate of EMM-REGISTERED and the UE is not required to trigger the tracking area update procedure when the unavailability period duration has ended. If the MME does not provide the Unavailability configuration IE or the MME sets the EUPR bit in Unavailability configuration IE value to "UE needs to report end of unavailability period", when the unavailability period duration has ended and the UE returns in satellite coverage of a TA in the current registration area after being out of satellite coverage due to discontinuous coverage, the UE shall trigger tracking area update procedure and the discontinuous coverage maximum time offset timer has expired, if any.

[TS 24.301, clause 5.5.1.2]

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411.

[TS 24.301, clause 5.5.3.2]

If the UE provided the Unavailability information IE in the TRACKING AREA UPDATE REQUEST message, then:

a1)	the MME shall determine the unavailability period duration value as:

-	a value that was provided by the UE; or

-	a value that was determined by the MME based on satellite coverage availability information; and

the MME shall store the determined unavailability period duration and provide the determined unavailability period duration to the UE by including the Unavailability period duration in the Unavailability configuration IE in the TRACKING AREA UPDATE ACCEPT message;

a2)	the MME shall determine the start of the unavailability period as:

-	a value that was provided by the UE; or

-	a value that was determined by the MME based on satellite coverage availability information; and

the MME shall store the determined start of the unavailability period and provide the determined start of the unavailability period to the UE by including the start of the unavailability period in the Unavailability configuration IE in the TRACKING AREA UPDATE ACCEPT message;

b)	if the UE did not include a start of the unavailability period, the MME shall consider the start of unavailability period to be the time at which MME received the TRACKING AREA UPDATE REQUEST message from the UE. The MME shall consider the UE as unreachable until the UE initiates the tracking area updating procedure for normal service again without providing an unavailability information; and

c)	the MME shall release the NAS signalling connection immediately after the completion of the tracking area updating procedure in which the UE provided unavailability information without providing the start of the unavailability period.

The MME should determine the periodic tracking area update timer, mobile reachable timer and implicit detach timer value based on:

a)	the stored value of the received unavailability period duration or the network determined unavailability period duration;

b)	the stored value of the received start of unavailability period or the network determined start of unavailability period; or

c)	any combination of the above.

If the UE does not provide the Unavailability information IE in the TRACKING AREA UPDATE REQUEST message, the MME shall delete any stored value of the Unavailability information IE if exists.

22.5.24.3	Test description

22.5.24.3.1	Pre-test conditions

System Simulator:

-	System information combination 14 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 are used in Ncell 1 and Ncell 11.

UE:

-	The pre-configured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE shall be in State 1-NB, Switched OFF according to TS 36.508 [18].

22.5.24.3.2	Test procedure sequence

Table 22.5.24.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration T0 indicates the initial conditions for preamble. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 22.5.24.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 [18].

Table 22.5.24.3.2-1: Ncell configuration changes over time

|  | Parameter | Unit | Ncell 1 | Ncell 11 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | NRS EPRE | dBm/15kHz | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | NRS EPRE | dBm/15kHz | -115 | -85 |  |

Table 22.5.24.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Switch on the UE | - | - | - | - |
| 2-4 | Steps 2 to 4 of the generic procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 1. | - | - | - | - |
| 5 | Check: Does the UE transmit an ATTACH REQUEST message with the EDC bit set to "Enhanced discontinuous coverage supported" in the UE network capability IE? | --> | ATTACH REQUEST | 1 | P |
| 6-15 | Steps 5 to 14 of the generic procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 1. | - | - | - | - |
| - | EXCEPTION: If pc_UnavailabilityInfo_DiscontinuousCoverage= TRUE then steps 16 – 22 are applicable | - | - | - | - |
| 16 | SS starts timer T0= 2s (Note 1) | - | - | - | - |
| 17a1 | T0 expires. | - | - | 2 | F |
| 17b1 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with the unavailability type in the Unavailability information IE set to "unavailability due to discontinuous coverage" before T0 expires? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| - | EXCEPTION: Steps 18 – 22 are executed only if Step 17b1 is executed | - | - | - | - |
| 18 | SS sends a TRACKING AREA UPDATE ACCEPT message with the End of unavailability period report in the Unavailability configuration IE set to "UE needs to report end of unavailability period" | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 19 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | NAS: TRACKING AREA UPDATE COMPLETE | - | - |
| 20 | The SS releases the RRC connection. | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 21 | SS adjusts Ncell levels according to row T1 of table 22.5.24.3.2-1. SS starts timer T1= 3s (Note 2) | - | - | - | - |
| 22 | Check: Does the UE transmit TRACKING AREA UPDATE REQUEST after T1 expires? | --> | NAS: TRACKING AREA UPDATE REQUEST | 3 | P |
| 23 | The SS transmits a TRACKING AREA UPDATE ACCEPT message | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 24 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | NAS: TRACKING AREA UPDATE COMPLETE | - | - |
| NOTE 1: T0 is set to same value as t-service-r17 NOTE 2: T1 is set to same value as Unavailability period duration sent by SS in TAU accept message |  |  |  |  |  |

22.5.24.3.3	Specific message contents

Table 22.5.24.3.3-1: ATTACH REQUEST (step 5, table 22.5.24.3.2-2)

| Derivation Path: TS 36.508 [18], Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE network capability |  |  |  |
| Enhanced Discontinuous Coverage (EDC) (octet 11, bit 3) | '1'B | enhanced discontinuous coverage supported |  |

Table 22.5.24.3.3-2: Message TRACKING AREA UPDATE REQUEST (step 17b1, table 22.5.24.3.2-2)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Unavailability information |  |  |  |
| Unavailability type (octet 3 bits 1 to 3) | '001'B | unavailability due to discontinuous coverage |  |
| Start of unavailability period presence indication (SUPPI) (octet 3 bit 5) | '1'B | unavailability period duration present |  |
| Start of unavailability period presence indication (SUPPI) (octet 3 bit 5) | '1'B | start of unavailability period present |  |
| Unavailability period duration (octets 4 to 6) | '000000000000000 000000011'B | This field provides the unavailability period duration in seconds |  |
| Start of unavailability period (octets 7 to 9) | '000000000000000 000000010'B | This field provides the time until the start of the next unavailability period in seconds |  |

Table 22.5.24.3.3-3: SystemInformationBlockType3-NB for Ncell 1 (preamble and all steps, table 22.5.24.3.2-2)

Derivation path: TS 36.508 [18] Table 8.1.4.3.3-2 with condition DiscontinousCoverage

Table 22.5.24.3.3-4: SystemInformationBlockType33-NB for Ncell 1 (preamble and all steps, table 22.5.24.3.2-2)

| Derivation Path: TS 36.508 [18], clause 8.1.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| t-ServiceStartNeigh-r18 | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.5.24.3.3-5: SystemInformationBlockType31-NB for Ncell 1 (preamble and all steps, table 22.5.24.3.2-2)

| Derivation Path: TS 36.508 [18], clause 8.1.4.3.3 with condition NTN enh |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-v1820 SEQUENCE { |  |  |  |
| satelliteId-r18 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.5.24.3.3-6: SystemInformationBlockType31-NB for Ncell 11 (preamble and all steps, table 22.5.24.3.2-2)

| Derivation Path: TS 36.508 [18], clause 8.1.4.3.3 with condition NTN enh |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-v1820 SEQUENCE { |  |  |  |
| satelliteId-r18 | 2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.5.24.3.3-7: Message TRACKING AREA UPDATE ACCEPT (step 18, table 22.5.24.3.2-2)

| Derivation Path: TS 36.508 [18], Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Unavailability configuration |  |  |  |
| End of unavailability period report (EUPR)(octet 3 bit 1) | '0'B | UE does not need to report end of unavailability period |  |
| Unavailability period duration presence indication (UPDPI) (octet 3 bit 2) | '1'B | unavailability period duration present |  |
| Start of unavailability period presence indication (SUPPI) (octet 3 bit 3) | '1'B | start of unavailability period present |  |
| Unavailability period duration (octets 4 to 6) | '000000000000000 000000011'B | This field provides the unavailability period duration in seconds |  |
| Start of unavailability period (octets n to n+2) | '000000000000000 000000010'B | This field provides the time until the start of the next unavailability period in seconds. |  |

22.5.25	NB-IoT / NTN / Attach Procedure / Coarse location information reporting

<< END OF CHANGES >>