3GPP TSG-RAN WG5 Meeting #109	R5-256044r1

# Dallas, USA, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-2 | CR | 1515 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to R18 IoT NTN enh SIG TCs applicability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The applicability needs to be updated for the newly added IoT NTN SIG TC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The applicability for the following TCs has been added: - 22.5.24 NB-IoT / NTN / enhanced discontinuous coverage |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, A.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 36.523-1 CR 5497 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The related 36.523-1 TC CR is R5-256043 (CR number 5497). The new condition “Cxxz” and new PCIS “xy” need to be assigned with hard number by MCC. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Add a new PICS for enhanced discontinuous coverage SIG TC. And update the condition of newly added enhanced discontinuous coverage TC. The affected clauses on the coversheet have been updated. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

Table 4-1: Applicability of tests and additional information for testing

| Clause | TC Title | Release | Applicability Condition | Applicability Comment | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Idle mode operations |  |  |  |  |  |  |  |
| 6.1.1.1 | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  | Either TC 6.1.1.1 or TC 6.1.1.1b shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.1a | PLMN selection / Automatic mode / between FDD and TDD | Rel-8 | C142 | UEs supporting E-UTRA FDD and E-UTRA TDD |  |  |  |  |
| 6.1.1.1b | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / Single Frequency operation | Rel-8 | R | UEs supporting E-UTRA. This test is 'cells on single frequency only' equivalent of TC 6.1.1.1 | pc_eFDD |  | Either TC 6.1.1.1 or TC 6.1.1.1b shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.2 | PLMN selection of "Other PLMN/access technology combinations" / Automatic mode | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  | Either TC 6.1.1.2 or TC 6.1.1.2a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| << Unchanged Parts Skipped >> |  |  |  |  |  |  |  |  |

| 22.4.31 | NB-IoT / NTN / discontinuous coverage | Rel-18 | C441 | UEs supporting NB-IoT and NTN access and discontinuous coverage | pc_NB_FDD pc_NB_ntn_GSO_ScenarioSupport pc_NB_ntn_NGSO_ScenarioSupport pc_NB_ntn_Flexible_TxRx_Separation pc_NB_ntn_Inband_nrNTN | px_NB_OperationMode | Note 22 Note 24 Note 25 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22.4.32 | NB-IoT / NTN / GNSS validity duration reporting / UE-autonomous GNSS measurement | Rel-18 | C445 | UEs supporting NB-IoT and NTN access and autonomous GNSS position fix and ntn uplink Tx extension | pc_NB_FDD pc_NB_ntn_GSO_ScenarioSupport pc_NB_ntn_NGSO_ScenarioSupport |  | Note 22 |  |
| 22.5.1 | NB-IoT / Authentication not accepted by the network, GUTI used / Authentication not accepted by the UE, SQN failure / Authentication not accepted by the UE, non-EPS authentication unacceptable / Network failing the authentication check | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD pc_NB_ntn_only_Connectivity_EPC |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.2 | NB-IoT / NAS Security / Handling of null integrity protection and null ciphering algorithms / NAS count reset to zero / Security mode command with not matching replayed security capabilities / Provision of IMEISV and IMEI | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD pc_NB_ntn_only_Connectivity_EPC |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.3 | NB-IoT / NW initiated detach Re-attach required / UE initiated detach Abnormal case EMM common procedure collision / UE initiated detach Abnormal case Local detach after 5 attempts due to no network response | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD pc_NB_ntn_only_Connectivity_EPC |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.4 | NB-IoT / Attach to new PLMN IMSI / Network reject with Extended Wait Timer / Paging with IMSI / Attach Rejected Illegal ME/UE / Detach upon switch-off | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD pc_NB_ntn_only_Connectivity_EPC |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.5 | NB-IoT / Attach Procedure / Success / List of equivalent PLMNs in the ATTACH ACCEPT message / Attach / Rejected / PLMN not allowed | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.6 | NB-IoT / Attach Abnormal cases / Unsuccessful attach or Repeated rejects for network failures / Change of cell into a new tracking area / EPS services not allowed / Failure due to non integrity protection /UE initiated detach USIM removed from the UE / Detach procedure collision. | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD pc_NB_ntn_only_Connectivity_EPC |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.7a | NB-IoT / Normal tracking area update List of equivalent PLMNs in the TRACKING AREA UPDATE ACCEPT message / Normal tracking area update Rejected (IMSI invalid / Illegal ME / UE identity cannot be derived by the network / UE implicitly detached / PLMN not allowed | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.7b | NB-IoT / Normal tracking area update Rejected ( Tracking area not allowed / No suitable cells in tracking area / Roaming not allowed in this tracking area / Congestion) / UE initiated detach Abnormal case Change of cell into a new tracking area | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.8 | NB-IoT / TRACKING AREA UPDATE REJECT / Change of cell into a new tracking area / Access barred due to access class control or NAS signalling connection establishment rejected by the network / Success or fail after several attempts due to no network response / TA belongs to TAI list and status is UPDATED / Tracking area updating and detach procedure collision. | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.9 | NB-IoT / UE in NB-S1 mode supporting CIoT Optimizations / Paging with not matching identity / Control Plane Service request Rejected (IMSI invalid / Illegal ME / EPS services not allowed / UE identity cannot be derived by the network / UE implicitly detached) | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD pc_NB_ntn_only_Connectivity_EPC |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.10 | NB-IoT / EPS NAS integrity and encryption / SNOW 3G | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD pc_NB_ntn_only_Connectivity_EPC |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.11 | NB-IoT / EPS NAS integrity and encryption / AES | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD pc_NB_ntn_only_Connectivity_EPC |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.12 | NB-IoT / EPS NAS integrity and encryption / ZUC | Rel-13 | C272 | UEs supporting NB-IoT and ZUC algorithms | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.13 | NB-IoT / Attach Procedure / Success / Last visited TAI, TAI list and equivalent PLMN list handling | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.14 | NB-IoT / Attach / Rejected / Tracking Area not allowed / Roaming not allowed in this tracking area / No suitable cells in tracking area | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.15 | NB-IoT / Normal tracking area update / low priority override | Rel-13 | C275 | UEs supporting NB-IoT and LAP and LAP override | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.16 | NB-IoT / Normal tracking area update / Rejected / EPS service not allowed / EPS services not allowed in this PLMN | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.17 | NB-IoT / Attach Success /Normal tracking area update accepted / Periodic tracking area update T3412 Extended Value / PSM | Rel-13 | C423 | UEs supporting NB-IoT and Power Saving Mode | pc_NB_FDD pc_NB_ntn_only_Connectivity_EPC |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.18 | NB-IoT / Attach & Normal tracking area update Procedure / Success / without Idle eDRX parameters / With Idle eDRX parameters / With and without Idle eDRX and PSM parameters | Rel-13 | C424 | UEs supporting NB-IoT and Extended DRX and Power Saving Mode | pc_NB_FDD pc_NB_ntn_only_Connectivity_EPC |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.19 | Void |  |  |  | pc_NB_FDD |  |  |  |
| 22.5.20 | NB-IoT/ UE in NB-S1 mode supporting control plane data back-off timer / Service reject with extended wait time CP data / Release with extended wait time CP data / Attach accept with extended wait time CP data | Rel-14 | C440 | UEs supporting NB-IoT and control plane data back-off timer T3448 | pc_NB_FDD pc_NB_ntn_only_Connectivity_EPC |  |  |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.21 | NB-IoT/APN rate control for MO exception data | Rel-14 | C342 | UEs supporting NB-IoT and APN rate control and additional APN rate control for exception data | pc_NB_FDD |  |  |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.22 | NB-IoT / Tracking area update/Inter-RAT change between NB-IoT and E-UTRA | Rel-14 | C323 | UEs supporting NB-IoT S1 and WB-S1 | pc_NB_FDD |  |  |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.5.23 | NB-IoT / NTN / GNSS position reporting / reject cause #78 "PLMN not allowed to operate at the present UE location" | Rel-17 | C412 | UEs supporting NB-IoT and NTN access in NB-IoT | pc_NB_FDD pc_NB_ntn_GSO_ScenarioSupport pc_NB_ntn_NGSO_ScenarioSupport |  |  |  |
| 22.5.24 | NB-IoT / NTN / enhanced discontinuous coverage | Rel-18 | Cxxz | UEs supporting NB-IoT and NTN access and enhanced discontinuous coverage | pc_NB_FDD pc_NB_ntn_GSO_ScenarioSupport pc_NB_ntn_NGSO_ScenarioSupport pc_UnavailabilityInfo_DiscontinuousCoverage |  | Note 22 |  |
| 22.5.25 | NB-IoT / NTN / Attach Procedure / Coarse location information reporting | Rel-18 | C438 | UEs supporting NB-IoT and NTN access in NB-IoT and reporting coarse location information via NAS | pc_NB_FDD pc_NB_ntn_GSO_ScenarioSupport pc_NB_ntn_NGSO_ScenarioSupport |  |  |  |
| 22.5.26 | NB-IoT / NTN / Normal tracking area update / Coarse location information reporting | Rel-18 | C438 | UEs supporting NB-IoT and NTN access in NB-IoT and reporting coarse location information via NAS | pc_NB_FDD pc_NB_ntn_GSO_ScenarioSupport pc_NB_ntn_NGSO_ScenarioSupport |  |  |  |
| 22.5.27 | NB-IoT / NTN / Periodic tracking area update / Coarse location information reporting | Rel-18 | C438 | UEs supporting NB-IoT and NTN access in NB-IoT and reporting coarse location information via NAS | pc_NB_FDD pc_NB_ntn_GSO_ScenarioSupport pc_NB_ntn_NGSO_ScenarioSupport |  |  |  |
| 22.6.1 | NB-IoT / UE routing of uplinks packets / User Plane / UE requested PDN disconnect procedure accepted by the network | Rel-13 | C290 | UEs supporting NB-IoT, and S1-U Data Transfer | pc_NB_FDD |  |  |  |

<< Unchanged Parts Skipped >>

Table 4-1a: Applicability of tests Conditions

C01 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C01a IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.1/1 AND (A.4.5-2/3 OR A.4.5-2/4) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C01b IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.1/1 AND A.4.5-2/4 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

<<Unchanged Parts Skipped>>

C440 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/259 THEN R ELSE N/A

C441 IF (A.4.4-1/230 OR A.4.4-1/240) AND A.4.4-1/260 THEN R ELSE N/A

C442 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1A/18 THEN R ELSE N/A

C443 IF A.4.1-1/8 AND A.4.4-1/230 AND A.4.4-1/261 THEN R ELSE N/A

C444 IF A.4.1-1/8 AND A.4.4-1/230 AND A.4.4-1/262 THEN R ELSE N/A

C445 IF (A.4.4-1/230 OR A.4.4-1/240) AND A.4.4-1/264 AND A.4.4-1/265 THEN R ELSE N/A

C446 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/266 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C447 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/266 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

Cxxz IF (A.4.4-1/230 OR A.4.4-1/240) AND A.4.4-1/260 AND A.4.4-2/xy THEN R ELSE N/A

<< Unchanged Parts Skipped >>

Table A.4.4-2: Definition of UE implementation capabilities

| Item | Definition of UE implementation capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support EPS attach (with or without pre-configuration) | 24.301 (Note1) | Rel-8 | pc_Attach | UE supports to be configured to initiate EPS attach or will always initiate EPS attach. (pc_PS_voice_centric OR pc_PS_data_centric) shall set this PICS to true. |
| 2 | Support combined EPS/IMSI attach (with or without pre-configuration) | 24.301 | Rel-8 | pc_Combined_Attach | UE supports to be configured to initiate combined EPS/IMSI attach or will always initiate combined EPS/IMSI attach. Implication: ((pc_UTRA OR pc_GERAN) AND [8] pc_CS) OR pc_CS_Fallback OR pc_SMS_SGs OR pc_IMSI_detach OR pc_CS_Em_Call_in_UTRA OR pc_CS_Em_Call_in_GERAN OR pc_CS_PS_voice_centric OR pc_CS_PS_data_centric shall set this PICS to true. |
| 3 | Void |  |  |  |  |
| 4 | Support of CS/PS mode 1 | 24.301 | Rel-8 | pc_CS_PS_voice_centric | UE supports to be configured to consistently behave as a CS/PS Voice centric UE |
| 5 | Support of CS/PS mode 2 | 24.301 | Rel-8 | pc_CS_PS_data_centric | UE supports to be configured to consistently behave as a CS/PS Data centric UE. |
| 6 | Requiring UMI proceeding to paging response | 23.272 | Rel-8 | pc_UMI_ProcNeeded_DuringCSFB | UE requires UMI prior to paging response while CSFB to UTRA |
| 7 | Support of PS mode 1 | 24.301 | Rel-8 | pc_PS_voice_centric | UE supports to be configured to consistently behave as a PS Voice centric UE |
| 8 | Support of PS mode 2 | 24.301 | Rel-8 | pc_PS_data_centric | UE supports to be configured to consistently behave as a PS Data centric UE. |
| 9 | IMS PS voice preferred, CS Voice as secondary | 24.301 | Rel-8 | pc_voice_PS_1_CS_2 | Configured voice domain preference. |
| 10 | Keeps EPS Bearer Context parameters after completion of the normal DETACH procedure | 24.301 cl. 5.5.2.2.2 | Rel-8 | pc_KeepEpsBearerParametersAfterNormalDetach | If the UE supports this, then the next ATTACH after DETACH shall be done using AT command AT+CGATT=1. Otherwise it shall be done using AT+CGDCONT=1,"IP" followed by AT+CGACT=1 |
| 11 | IMS APN as default APN | 23.401 | Rel-8 | pc_IMS_APN_default | Configured with IMS APN as default APN. |
| 12 | XCAP only APN | 23.401 | Rel-8 | pc_XCAP_only_APN | Configured with an APN for XCAP only usage.(Note 2) |
| 13 | Provide IMS APN | 23.401 | Rel-8 | pc_Provide_IMS_APN | Configured to provide IMS APN during initial attach. |
| 14 | Provide IMS as second APN | 23.401 | Rel-8 | pc_Provide_IMS_as_second_APN | Configured to provide IMS APN as the second PDN connection. |
| 15 | Provide Internet as second APN | 23.401 | Rel-8 | pc_Provide_Internet_as_second_APN | Configured to provide Internet as the second PDN connection. |
| 16 | User initiated PDN disconnect | 24.301 | Rel-8 | pc_UE_supports_user_initiated_PDN_disconnect | UE supports user initiated PDN disconnect. |
| 17 | XCAP over Internet PDN | 23.401 | Rel-8 | pc_XCAP_over_Internet_APN | Configured to use internet PDN for XCAP signalling (Note 2) |
| 18 | Dynamically downgrades the GERAN release when the support of EPS is disabled | 24.301, 24.008 | Rel-8 | pc_Dynamic_GERAN_Rel_downgrade | UE may support e.g. from all GERAN Rel-8 features only those related to the interworking with EPS. When EPS is disabled then the Device may comply with a lower than Rel-8 GERAN release requirements. |
| 19 | Provide ProSe APN | 24.334 | Rel-12 | pc_Provide_ProSe_APN | Configured to provide ProSe APN and a PDN connection request. An UE supporting D2D ProSe shall set this PICS to true. |
| 20 | Provisioned FQDN ePDG | 24.302 | Rel-13 | pc_ePDG_FQDN_Provisioned | Configured with an ePDG FQDN provisioned by the home operator. |
| 21 | Operator Identifier FQDN format used for ePDG | 24.302 | Rel-13 | pc_ePDG_FQDN_constructed | Configured to construct the ePDG FQDN in the Operator Identifier FQDN format. |
| 22 | UE supports only NB-S1 mode (i.e. NB-IoT) | 24.301 | Rel-13 | pc_NB_S1_only |  |
| 23 | UE capable of requesting PDN of type "Non-IP" | 24.301 | Rel-13 | pc_NonIP_PDN |  |
| 24 | UE capable of requesting PDN of type "IP" | 24.301 | Rel-13 | pc_IP_PDN |  |
| 25 | The UE supports Non-IP Link MTU parameter | 24.301 | Rel-13 | pc_NonIP_Link_MTU_Parameter |  |
| 26 | The UE supports IPv4 Link MTU parameter | 24.301 | Rel-13 | pc_IPv4_Link_MTU_Parameter |  |
| 27 | The UE supports APN rate control | 24.301 | Rel-13 | pc_APN_RateControl |  |
| 28 | The UE supports Header compression for control plane CIoT EPS optimization | 24.301 | Rel-13 | pc_HCCPCIoT |  |
| 29 | The UE supports a mechanism to provide Daylight Saving Time | 24.301 | Rel-8 | pc_ProvideDST_inUse | Note 3 |
| 30 | The UE does not request IMS PDN connection when IMS VoPS set to ‘0’ | 24.301 | Rel-8 | pc_UE_NoReqIMS_IMSVoPS_0 | Configured not to request IMS PDN connection when IMS VoPS set to ‘0’ |
| 31 | The UE supports additional APN rate control for exception data reporting | 24.301 | Rel-14 | pc_Additional_APN_RateControl |  |
| 32 | The UE is configured to use SMS over IP | 24.167 | Rel-8 | pc_Use_SMS_over_IP | Configured to use SMS over IP |
| 33 | The UE supports a bearer with QCI 66 | 23.203 | Rel-14 | pc_Use_QCI_66 |  |
| 34 | The UE supports a bearer with QCI 67 | 23.203 | Rel-15 | pc_Use_QCI_67 |  |
| xy | The UE transmits a TRACKING AREA UPDATE REQUEST message with the unavailability type in the Unavailability information IE set to "unavailability due to discontinuous coverage" when the UE is about to lose satellite coverage | 24.301 cl 4.11.4 | Rel-18 | pc_UnavailabilityInfo_DiscontinuousCoverage |  |
| Note 1: A UE supporting UTRAN and/or GERAN which is configured to initiate EPS attach considers UTRAN and GERAN cell as candidates for cell selection and cell reselection according to TS 36.304. A UE configured to initiate EPS attach which has selected a UTRAN or GERAN cell may perform registration procedures to the PS and CS domains, or to the PS domain only or to the CS domain only. Note 2: pc_XCAP_only_APN and pc_XCAP_over_Internet_APN are mutual exclusive i.e. shall not be set to true at the same time. Note 3: Shall be set to false when pc_DaylightSavingTime is false. |  |  |  |  |  |

<< END OF CHANGES >>