3GPP TSG-RAN WG5 Meeting #109	R5-256048r1

# Dallas, USA, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 1097 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to TP analysis for ATG TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TP analysis of the newly added ATG TCs are missing and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TP analysis of Carrier leakage and In-band emissions for ATG have been added. Also TP analysis for ACLR ATG TC is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 CR 3565, 3566, 3567 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The related TS38.521-1 test case CRs are R5-256049(CR 3565), R5-256050(CR 3566) and R5-256051(CR 3567). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: 6.5J.2.3 Adjacent channel leakage ratio for ATG TP analysis is added. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

4.1.1.8	FR1 ATG test cases

This section contains information on test point selection for ATG test cases in [2]. The general rules in this section apply to all the NR ATG test cases. Separate analysis is not provided for each single test case unless specific test requirement deviates from the general rules.

General rules of test point selection:

Considering that NR ATG operating bands are defined like FR1 bands, test environment, frequencies, SCSs, channel bandwidths, waveforms, and modulations of a Tx or Rx test for an NR ATG band should be selected based on the same principles of the corresponding single carrier test for an FR1 band.

Table 4.1.1.8-1: NR UE transmitter test point selection for ATG test cases in FR1

| Subclause | Number of test points | Justification in attachment | Comments |
| --- | --- | --- | --- |
| 6.2J.1 UE maximum output power for ATG | 540 | General rules of test point selection apply. Test points of TC 6.2.1 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 6.2J.2 Configured transmitted power for ATG | 30 | General rules of test point selection apply. Test points of TC 6.2.4 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 6.3J.1 Minimum output power for ATG | 45 | General rules of test point selection apply. Test points of TC 6.3.1 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 6.3J.3.1 General ON/OFF time mask for ATG | 180 | General rules of test point selection apply. Test points of TC 6.3.3.2 from TS 38.521-1 can be leveraged. | RAN5#108 |
| 6.3J.3.3 PRACH time mask for ATG |  | General rules of test point selection apply. Test points of TC 6.3.3.4 from TS 38.521-1 can be leveraged | RAN5#105 |
| 6.3J.3.4 SRS time mask for ATG | 30 | General rules of test point selection apply. Test points of TC 6.3.3.6 from TS 38.521-1 can be leveraged | RAN5#105 |
| 6.3J.4.1 Absolute power tolerance for ATG | 6 | General rules of test point selection apply. Test points of TC 6.3.4.2 from TS 38.521-1 can be leveraged. | RAN5#106 |
| 6.3J.4.2 Relative power tolerance for ATG | TBD | General rules of test point selection apply. Test points of TC 6.3.4.3 from TS 38.521-1 can be leveraged. | RAN5#106 |
| 6.3J.4.3 Aggregate power tolerance for ATG | PUCCH: 6 PUSCH: 6 | General rules of test point selection apply. Test points of TC 6.3.4.4 from TS 38.521-1 can be leveraged. | RAN5#106 |
| 6.4J.1 Frequency error for ATG | 5 | General rules of test point selection apply. Test points of TC 6.4.1 from TS 38.521-1 can be leveraged. | RAN5#106 |
| 6.4J.2.2 Carrier leakage for ATG | 3 | General rules of test point selection apply. Test points of TC 6.4.2.2 from TS 38.521-1 can be leveraged. | RAN5#109 |
| 6.4J.2.3 In-band emissions for ATG | 36 | General rules of test point selection apply. Test points of TC 6.4.2.3 from TS 38.521-1 can be leveraged. | RAN5#109 |
| 6.5J.1 Occupied bandwidth for ATG | 10 | General rules of test point selection apply. Test points of TC 6.5.1 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 6.5J.2.2 Spectrum emissions mask for ATG | contiguous allocation: 144 (1681, 1602,3) almost contiguous allocation: 24 | General rules of test point selection apply. Test points of TC 6.5.2.2 from TS 38.521-1 can be leveraged. | RAN5#106 |
| 6.5J.2.3 Adjacent channel leakage ratio for ATG | TBD | General rules of test point selection apply. Test points of TC 6.5.2.3 from TS 38.521-1 can be leveraged. | RAN5#109 |
| 6.5J.3.1 General spurious emissions for ATG | 27 | General rules of test point selection apply. Test points of TC 6.5.3.1 from TS 38.521-1 can be leveraged. | RAN5#102 |
| NOTE 1: For power class 3 UE operating in bands n41, and n79. NOTE 2: UE operating in FDD mode, or in TDD mode in bands other than n41and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41and n79. NOTE 3: UEs supporting pi/2 BPSK DMRS and the corresponding IE [DMRSPi2BPSK] is set to 1 |  |  |  |  |

Table 4.1.1.8-2: NR UE receiver test point selection for ATG test cases in FR1

| Subclause | Number of test points | Justification in attachment | Comments |
| --- | --- | --- | --- |
| 7.3J.2 Reference sensitivity power level for ATG | 45 | General rules of test point selection apply. Test points of TC 7.3.2 from TS 38.521-1 can be leveraged | RAN#105 |
| 7.4J Maximum input level for ATG | 6 | General rules of test point selection apply. Test points of TC 7.4 from TS 38.521-1 can be leveraged limiting the modulation to 64QAM and 256QAM. | RAN5#102 |
| 7.5J Adjacent channel selectivity for ATG | 3 | General rules of test point selection apply. Test points of TC 7.5 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 7.6J.2 In-band blocking for ATG | 3 | General rules of test point selection apply. Test points of TC 7.6.2 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 7.6J.3 Out-of-band blocking for ATG | 3 | General rules of test point selection apply. Test points of TC 7.6.3 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 7.7J Spurious response for ATG | 3 | General rules of test point selection apply. Test points of TC 7.7 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 7.8J.2 Wide band intermodulation for ATG | 3 | General rules of test point selection apply. Test points of TC 7.8.2 from TS 38.521-1 can be leveraged. | RAN5#102 |
| 7.9J Spurious emissions for ATG | 3 | General rules of test point selection apply. Test points of TC 7.9 from TS 38.521-1 can be leveraged. | RAN5#102 |

<< END OF CHANGES >>