3GPP TSG-RAN WG5 Meeting #109	R5-256079r2

# Dallas, USA, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.509 | CR | 0090 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test loop mode for application layer packet |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - For XR PDCP discard TCs, PDU Set marking information is in RTP header extension (application layer) as per TS26.522. If we want to test XR PDCP discard functions, then RTP packet with header extension needs to be constructed for the testing. That means SS needs to send a constructed RTP packet with header extension to UE. After UE receives, UE sends it back without any changes to test XR PDCP discard functions. However, for the current NR conformance testing function spec TS 38.509, there is no test loop mode for application layer packet transmission. Thus a new test loop mode for application layer packet transmission needs to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A new test loop mode for application layer packet transmission has been added so that XR PDCP discard functions can be tested. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.3.1, 5.3.2.1, 5.3.4.4 (new), 6.3.1, 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 CR 5199, 5200, 5201 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The related discussion paper is R5-256071. The related SIG TC CRs are R5-256073 (CR number 5199), R5-256073 (CR number 5200) and R5-256073 (CR number 5201). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: CR impact tick box for ME is set to solve 3GU issue. Correct a typo in cl 5.3.4.4.1 and indicate the newly added application layer loopback mode is not mandatory. R2: Fix the typo of the figure name Figure 5.3.4.4.1-1 in cl 5.3.4.4.1. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

5.2	Activation and deactivation of 5GS test mode bearers (UE test mode procedures)

5.2.1	General

The SS performs, where applicable, activation and deactivation of the conformance test functions in the UE by sending Security Protected NAS Layer 3 messages.

The UE test mode procedures are intended for setting the UE into a test mode where the SS can set up test bearers terminated at a particular point in the 5GS protocol stack as specified in the functional block diagram of each UE test loop mode of operation. The following test bearers are defined in the present specification:

-	data radio bearers (UE test loop mode A),

-	EPS bearers or 5GS QoS flows (UE test loop mode B).

-	MBS radio bearers (UE test loop mode C).

-	NR PC5 QoS flows (UE test loop mode E).

-	Application layer bearers (UE test loop mode J).

<< Unchanged Parts Skipped >>

5.3.1	General

Before a loop functionality can be exercised, the test loop needs to be closed; this is to be understood as the UE being instructed to start looping back received data packets. When looping back received data packets is not any longer required the loop should be re-opened; opening of a loop does not change the type of bearer being established by the UE test mode activation function (subclause 5.2.2).

To limit the number of special test functions, the concept of closing and opening a loop is also used as instruction to the UE to initiate/terminate other actions. An example of this is counting the received packets and reporting the number of received packets back to the SS; in the context of this utilisation closing a loop is to be understood as the UE being instructed to start counting the packets, whereas opening a loop should be understood as stop counting the packets. Other utilisation of the closing/opening a "test loop" test functionality can be specified if appropriate.

The UE test loop functions for 5GS are intended for:

-	NR receiver and transmitter testing to disable application data transfer in downlink and uplink during SS UE measurements.

-	NR layer 2 (MAC, RLC, PDCP, SDAP) and data radio bearer testing to generate data transfer in downlink and uplink.

-	NR layer 2 (MAC, RLC, PDCP, SDAP) and sidelink data radio bearer testing to generate data transfer in sidelink.

-	NR layer 2 (MAC, RLC, PDCP) and MBS radio bearer testing.

-	5GC and NR layer 3 testing to verify data transfer continuation over RRC and 5GC procedures.

-	5GC and NR layer 3 testing to verify data transfer over PC5 interface.

-	5GC NAS user-plane testing to verify uplink QoS flow descriptions handling.

-	PC5-U testing to verify sidelink PC5 QoS flow descriptions handling.

-	Application-layer bearer (RTP, RTP with header extension), NR layer 2 (SDAP, PDCP) testing to verify application-layer payload/header transfer in downlink and uplink.

5.3.2	Close UE test loop

<< Unchanged Parts Skipped >>

5.3.2.1	Reception of CLOSE UE TEST LOOP message by the UE

Same as TS 36.509 [6], subclause 5.4.2.3 with the following exceptions:

…

1>	else if UE test loop mode C has been selected;

2>	if no MBS radio bearer is established or if the UE test mode is not active; or

2>	if UE test loop mode A or UE test loop mode B operation is already closed on one or more data radio bearers; or

2> if TEST_LOOP_MODE_C_ACTIVE = TRUE or TEST_LOOP_MODE_D_ACTIVE = TRUE or TEST_LOOP_MODE_E_ACTIVE = TRUE or TEST_LOOP_MODE_F_ACTIVE = TRUE or TEST_LOOP_MODE_G_ACTIVE = TRUE or TEST_LOOP_MODE_H_ACTIVE = TRUE or TEST_LOOP_MODE_I_ACTIVE = TRUE:

3>	the UE behaviour is unspecified.

2>	otherwise:

3>	set TEST_LOOP_MODE_C_ACTIVE to TRUE

3>	set state variable MBMS_PACKET_COUNTER to zero;

3>	perform the UE actions for UE Test Loop Mode C operation as specified in subclause 5.3.4.2A and

3>	send CLOSE UE TEST LOOP COMPLETE message (the loopback shall be operational prior to the sending of the acknowledgement).

1>	else if UE test loop mode E has been selected;

2>	if UE test loop mode A or UE test loop mode B operation is already closed on one or more data radio bearers; or

2> if TEST_LOOP_MODE_C_ACTIVE = TRUE or TEST_LOOP_MODE_D_ACTIVE = TRUE or TEST_LOOP_MODE_E_ACTIVE = TRUE or TEST_LOOP_MODE_F_ACTIVE = TRUE or TEST_LOOP_MODE_G_ACTIVE = TRUE or TEST_LOOP_MODE_H_ACTIVE = TRUE or TEST_LOOP_MODE_I_ACTIVE = TRUE; or

2> if the UE is unable to read the pre-configured parameters for NR sidelink Communication from the USIM; or

2> when test loop mode E is utilised in NR sidelink out-of-coverage scenarios:

3>	the UE behaviour is unspecified.

2>	otherwise:

3>	set TEST_LOOP_MODE_E_ACTIVE to TRUE

3> if the E0 bit in Communication Transmit or Receive parameter in UE test loop mode E setup IE is set as zero;

4>	set TEST_LOOP_MODE_E_TRIGGER to RECEIVE

4>	set state variable PROSE_COMMUNICATION_MONITOR_N to the number of entities in the list of Group Destination IDs to individually monitor included in the UE test loop mode E setup IE;

4>	if PROSE_COMMUNICATION_MONITOR_N is greater than MAX_ModeE_Monitor_Entities:

5>	the UE behaviour is unspecified.

4>	for SL_ID = 0…(PROSE_COMMUNICATION_MONITOR_N-1):

5>	set state variable PROSE_COMMUNICATION_DEST_ID(SL_ID) to the parameter Group Destination ID #(SL_ID) to monitor of the UE test loop mode E setup IE;

5>	set state variable STCH_PACKET_COUNTER(SL_ID) to zero;

5>	set state variable PSCCH_PACKET_COUNTER(SL_ID) to zero;

5>	set state variable PSSCH_PACKET_COUNTER(SL_ID) to zero;

4>	set state variable STCH_PACKET_COUNTER(PROSE_COMMUNICATION_MONITOR_N) to zero;

4>	set state variable PSCCH_PACKET_COUNTER(PROSE_COMMUNICATION_MONITOR_N) to zero;

4>	set state variable PSSCH_PACKET_COUNTER(PROSE_COMMUNICATION_MONITOR_N) to zero;

4>	perform the UE actions for UE Test Loop Mode E operation as specified in subclause 5.3.4.3.1;

3> if the E0 bit in Communication Transmit or Receive parameter in UE test loop mode E setup IE is set as one;

4>	if the E1 bit in Communication Transmit or Receive parameter in UE test loop mode E setup IE is set as zero;

5>	set TEST_LOOP_MODE_E_TRIGGER to TRANSMIT;

4>	else

5>	set TEST_LOOP_MODE_E_TRIGGER to TRANSMIT_SL_MIMO;

4>	perform the UE actions for UE Test Loop Mode E operation as specified in subclause 5.3.4.3.1;

3>	send CLOSE UE TEST LOOP COMPLETE message (the loopback shall be operational prior to the sending of the acknowledgement).

1>	else if UE test loop mode J has been selected;

2>	if no application layer bearer is established or if the UE test mode is not active; or

2>	if UE test loop mode A or UE test loop mode B operation is already closed on one or more data radio bearers; or

2> if TEST_LOOP_MODE_C_ACTIVE = TRUE or TEST_LOOP_MODE_D_ACTIVE = TRUE or TEST_LOOP_MODE_E_ACTIVE = TRUE or TEST_LOOP_MODE_F_ACTIVE = TRUE or TEST_LOOP_MODE_G_ACTIVE = TRUE or TEST_LOOP_MODE_H_ACTIVE = TRUE or TEST_LOOP_MODE_I_ACTIVE = TRUE or TEST_LOOP_MODE_J_ACTIVE = TRUE:

3>	the UE behaviour is unspecified.

2>	otherwise:

3>	set TEST_LOOP_MODE_J_ACTIVE to TRUE

3>	set state variable APPLICATION_PACKET_COUNTER to zero;

3>	perform the UE actions for UE Test Loop Mode J operation as specified in subclause 5.3.4.4 and

3>	send CLOSE UE TEST LOOP COMPLETE message (the loopback shall be operational prior to the sending of the acknowledgement).

5.3.2.2	Reception of AT Command +CCUTLE by the UE

<< Unchanged Parts Skipped >>

Table 5.3.4.3.1-1: SDAP SDU payload contents for NR sidelink communication transmit operation in UE test loop mode E

| Parameter | Value |
| --- | --- |
| Size (N) | 300 bytes |
| Payload | 00…00 |

5.3.4.4	UE test loop mode J operation

5.3.4.4.1	General

UE test loop mode J provides application-layer loopback functionality for application data packets while UE is operated in standalone NR mode. Prior to closing the UE test loop mode J, thereby requesting the UE to start looping back the received data packets, at least one 5GS test mode bi-directional tunnel over application layer bearer, IP Flow, QoS Flow and data radio bearer shall have been established between SS and UE. This implies that before the procedure for establishing the bi-directional tunnel over application layer bearer, IP Flow, QoS Flow and data radio bearer takes place the SS needs to activate the UE test mode as specified in subclause 5.2.2.

The downlink application layer packets received by the UE are returned on the same bearer without any modification to the packet content, including application layer headers (e.g. RTP header or RTP header with header extension) and payload.

Figure 5.3.4.4.1-1 shows functional block diagrams of UE test loop function for the TMC entity and UE test loop mode J for the case when standalone NR is configured. The Application Layer Packet Counter function is configured to monitor whether the application layer packet is successfully received on each application layer bearer configured by the SS when UE test loop mode J is activated.

UE test loop mode J is optional to all 5GS UEs supporting application layer loopback.

For the Application Layer test loop mode, the processing of an application layer packet (e.g., an RTP packet or an RTP packet with a RTP header extension) through the protocol stack is defined as follows:

- The Application Layer packet is encapsulated with a Transport Layer header (e.g., a UDP header) to form a Transport Layer segment.

- The resulting Transport Layer segment is then encapsulated with a Network Layer header (e.g., an IP header) to form an IP packet.

- This IP packet is processed by the 5G Quality of Service (QoS) framework, where it is mapped to a specific QoS Flow based on its QoS characteristics and the associated QoS rules.

- Within the NG-RAN architecture, the QoS Flow is subsequently mapped to a corresponding Data Radio Bearer (DRB), as per the configured QoS Flow to DRB mapping rules. This operation happens in SDAP layer.

- Finally, at the NR Layer 2 (PDCP) entity associated with the assigned DRB, the IP packet is processed to become a PDCP Service Data Unit (SDU). The PDCP entity then performs its defined procedures (e.g., header compression, ciphering) and forms the PDCP Protocol Data Unit (PDU) for transmission to the lower layers.

NOTE 1:	The number and the order of QoS Flow LB Entities in the UE Test Loop Function in the functional block diagrams are provided for illustration only. No specific order or numbering is precluded.

Figure 5.3.4.4.1-1: Model for Test Mode Control and UE test loop mode J on UE side for application layer packet

5.3.4.4.2	Reception of application layer packets

Upon receiving an application layer packet (e.g., RTP packet or RTP packet with header extension) on the bearer configured for test loop mode J, the UE shall

1>	if UE test loop mode J is active;

2>	increment APPLICATION_PACKET_COUNTER by 1;

2>	loop back the received application layer packet to the SS on the same bearer without any modification.

1>	else:

2>	the UE behaviour is unspecified.

5.4	UE Beamlock test Function (UBF)

<< Unchanged Parts Skipped >>

6.3.1	CLOSE UE TEST LOOP

Same as TS 36.509 [6], subclause 6.1 with the following exception:

-	The supported test modes for 5GS are limited to those specified in subclause 5.3.4.

-	LB Setup DRB#k IE is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z15 | Z14 | Z13 | Z12 | Z11 | Z10 | Z9 | Z8 | octet 1 |
| Z7 | Z6 | Z5 | Z4 | Z3 | Z2 | Z1 | Z0 | octet 2 |
| Reserved |  | Q5 | Q4 | Q3 | Q2 | Q1 | Q0 | octet 3 |

Z15..Z0 = Uplink PDCP SDU size in bits 0.. 12160 (binary coded, Z15 is most significant bit and Z0 least significant bit). See Note 1.
Q5 = 0 for E-UTRA Data Radio Bearers and Q5 = 1 for NR Data Radio Bearers

Q4..Q0 = 0..31 representing DRB-Identity -1, where DRB-Identity identifies the data radio bearer in accordance to TS 36.331 [10] for E-UTRA Data Radio Bearers and TS 38.331 [11] for NR Data Radio Bearers [11] (binary coded, Q4 is most significant bit and Q0 least significant bit).

NOTE 1:	The UL PDCP SDU size is limited to 12160 bits (1520 octets).

NOTE 2:	A "LB Setup DRB IE" is only needed for a DRB if UL PDCP SDU scaling is needed. If there is no "LB Setup DRB IE" associated with a DRB in the CLOSE UE TEST LOOP message, then the same size of the PDCP SDU received in downlink is returned in uplink.

NOTE 3:	The UL PDCP SDU size shall be byte aligned (i.e. multiple of 8 bits) according to TS 36.323 [18] clause 6.2.1 for E-UTRA Data Radio Bearers and TS 38.323 [19] clause 6.2.1 for NR Data Radio Bearers.

-	The Communication Transmit or Receive message in UE test loop mode E setup is as below:

And where UE test loop mode C setup is:

|  | 8 7 6 5 4 3 2 1 |  |
| --- | --- | --- |
|  | MRB ID | octet 1 octet 2 octet 3 |

Where MRB ID is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  | C0 | octet 1 |
| A8 | A7 | A6 | A5 | A4 | A3 | A2 | A1 | octet 2 |
| A0 | Reserved |  |  |  |  |  |  | octet 3 |

A8..A0 = 0..511 representing MRB-Identity -1, where MRB-Identity identifies the Multicast MRB in accordance to TS 38.331 [11] (binary coded, A8 is most significant bit and A0 least significant bit).

A4..A0= 0..31 representing Identity of the logical channel of broadcast MTCH -1, where Identity of the logical channel of broadcast MTCH identifies the Broadcast MRB in accordance to TS 38.321 [23] (binary coded, A4 is most significant bit and A0 least significant bit).

C0=0 is used to trigger the UE to count the MBS packet on Multicast MRB of the MRB-Identity configured in A8..A0; C0=1 is used to trigger the UE to count the MBS packet on Broadcast MRB of the Identity of the logical channel of broadcast MTCH configured in A4..A0.

And where UE test loop mode E setup is:

|  | 8 7 6 5 4 3 2 1 |  |
| --- | --- | --- |
|  | Length of UE test loop mode E Monitor setup contents in bytes | Octet 1 |
|  | Communication Transmit or Receive | Octet 2 |
|  | Monitor list | Octet 3 Octet N+2 or Octet 3*N+2 |

where Communication Transmit or Receive is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  | E1 | E0 | octet 1 |

E0 = 0 is used to trigger the UE to continuously monitor and receive NR sidelink communication message (on STCH, PSCCH and PSSCH), and E0 = 1 is used to trigger the UE to start continuous transmitting NR sidelink communication messages (on STCH).

If E0 = 1, E1 = 1 is used to indicate UE shall transmit with 2-layer SL-MIMO transmission with precoding matrix defined in TS 38.211 [25] subclause 8.3.1.4, and E1 = 0 is used to indicate UE shall transmit with single antenna port; otherwise E1 is used as reserved bit.

And where UE test loop mode J setup is:

|  | 8 7 6 5 4 3 2 1 |  |
| --- | --- | --- |
|  | Application Layer Protocol Type | octet 1 |
|  | Bearer Identity | octet 2 |

Where Application Layer Protocol Type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  | J0 | octet 1 |

J0 = 0 is used to indicate the application layer protocol type is RTP, and J0 = 1 is used to indicate the application layer protocol type is RTP packet with header extension.

Bearer Identity: Identifies the bearer for RTP packet transmission.

6.3.2	CLOSE UE TEST LOOP COMPLETE

<< Unchanged Parts Skipped >>

7	Variables, constants and timers

7.1	State variables

Same as [9] TS 36.509, subclause 7.1 except:

STCH_PACKET_COUNTER(SL_ID)

This 32-bit integer state variable is used to count successful reception of SDAP SDUs while UE test loop mode E is activated and TEST_LOOP_MODE_E_TRIGGER is set to RECEIVE. The counter variable is indexed with SL_ID = 0…( MAX_ModeE_Monitor_Entities). STCH_PACKET_COUNTER(SL_ID) shall be set to zero when UE is switched on. If configured value of variable PROSE_COMMUNICATION_MONITOR_N is set to 0 then UE counts all SDAP SDUs as part of one counter STCH_PACKET_COUNTER(0) irrespective of Group Destination IDs or Destination Layer-2 IDs.

TEST_LOOP_MODE_J_ACTIVE

This boolean state variable is used to indicate if UE test loop mode J is active. TEST_LOOP_MODE_J_ACTIVE shall be set to FALSE when UE is switched on.

APPLICATION_PACKET_COUNTER

This 32-bit integer state variable is used to monitor whether the application layer packet is successfully received on each application layer bearer configured by the SS while UE test loop mode J is activated. The counter variable shall be set to zero when UE is switched on.

7.2	Constants

Same as [9] TS 36.509, subclause 7.2.

<< Unchanged Parts Skipped >>

<< END OF CHANGES >>