3GPP TSG-RAN5 Meeting #109	R5-256215r1

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5500 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of EPS CEN alignments test case 11.3.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Int |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New test case 11.3.10 ("eCall Only mode / Limited service state / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 486 (Busy Here) / UE performs eCall in CS domain / UTRAN or GERAN") needs to be added as per WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 11.3.10 ("eCall Only mode / Limited service state / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 486 (Busy Here) / UE performs eCall in CS domain / UTRAN or GERAN") as per WP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCallCEN-UEConTest WP will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.10 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1518 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-256215r1: 1. Removed "Emergency Registration" from title of the test case. 2. Updated steps 19a1 and 19b1 to include CS emergency call PICS parameter. |  |  |  |  |  |  |  |  |

==============First change==============

### 11.3.10	eCall Only mode / Limited service state / Manual initiation / Abnormal case / IM CN sends a 486 (Busy Here) / UE performs eCall in CS domain / UTRAN or GERAN

11.3.10.1	Test Purpose (TP)

(1)

with { UE in eCALL-INACTIVE.LIMITED-SERVICE state and requested to make a manual eCall over IMS }

ensure that {

when { UE receives 486 Busy Here in response to INVITE }

then { UE establishes the eCall using CS domain (UTRA or GERAN) }

}

11.3.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122, clause 2 and TS 24.229, clauses 5.1.6.11.1 and 5.1.6.11.2.

[TS 23.122 clause 2]

If the MS is in eCall only mode, it attempts to camp on a suitable cell and enters an "eCall inactive" state in which it can only attempt an eCall over IMS, or a call to a non-emergency MSISDN or URI for test or terminal reconfiguration services as specified in 3GPP TS 31.102 [40].

If the MS is in eCall only mode and is unable to find a suitable cell to camp on, it attempts to camp on an acceptable cell in limited service state, and enters an "eCall inactive" state in which it can only attempt an eCall over IMS.

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

…

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

- the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

NOTE 0:	In some countries, the type of emergency service is required, due to national regulations, so a Request-URI with ecall sub-service type is needed to avoid failure in routing, e.g. when the IP-CAN indicates that eCall over IMS is not supported.

2)	if the IP-CAN indicates the eCall over IMS support defined in 3GPP TS 36.331 [19F] or in 3GPP TS 38.331 [19G], the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall act as follows:

iii)	if the UE is in limited service state, perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

iv)	if the UE is not in limited service state, report the response and successful MSD transfer to upper layers; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

NOTE 2:	For case 3-a-iv). upper layers can wait for an incoming call from the PSAP as defined in CEN EN 17184:2024 [300] if incoming IMS calls are possible or if the upper layers can instigate CS fallback to receive incoming calls over CS. If incoming calls cannot be received by the UE, upper layers can perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

11.3.10.3	Test description

11.3.10.3.1	Pre-test conditions

System Simulator:

-	Cell A is set to ''Serving cell''.

-	If (px_RATComb_Tested = EUTRA_UTRA)

-	cell 5 is configured as ''Suitable Neighbour cell''.

-	If (px_RATComb_Tested = EUTRA_GERAN)

-	cell 24 is configured as ''Suitable Neighbour cell''.

-	The PLMN is defined in Table 11.3.10.3.1-1.

Table 11.3.10.3.1–1: PLMN identifier

| Cell | PLMN name | MCC / MNC |
| --- | --- | --- |
| A | PLMN4 | 004 / 31 |

UE:

-	The eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 36.508 [18] Table 4.9.3.5-2.

-	The UE is equipped with a USIM containing values shown in Table 11.3.10.3.1-2.

Table 11.3.10.3.1-2: USIM configuration

| USIM field | Value |
| --- | --- |
| EFFPLMN | PLMN4 |

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

11.3.10.3.2	Test procedure sequence

Table 11.3.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Wait 60s for the UE to camp on Cell A as an acceptable cell. | - | - | - | - |
| 3 | A manual eCall is initiated. (Note 1) | - | - | - | - |
| 4-16 | Steps 2 to 14 of the generic test procedure for IMS Emergency call establishment in EUTRA: Limited Service (TS 36.508 subclause 4.5A.5.3-1) take place. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 17 to 18 below, the behaviour in table 11.3.10.2-2 occurs. | - | - | - | - |
| 17-18 | Steps 15 to 16 of the generic test procedure for IMS emergency call establishment in EUTRA: Limited Service (subclause 4.5A.5.3-1) take place. | - | - | - | - |
| - | EXCEPTION: Steps 19a1 to 19b20 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 19a1 | IF (px_RATComb_Tested = EUTRA_UTRA AND pc_CS_Em_Call_in_UTRA), Check: Does the UE transmit an RRC CONNECTION REQUEST message on Cell 5 with establishment cause set to Emergency Call? | --> | RRC CONNECTION REQUEST | 1 | P |
| 19a2 | The SS transmits an RRC CONNECTION SETUP message. | <-- | RRC CONNECTION SETUP | - | - |
| 19a3 | The UE transmits an RRC CONNECTION SETUP COMPLETE message. | --> | RRC CONNECTION SETUP COMPLETE | - | - |
| 19a4 | Check: Does the UE transmit a CM SERVICE REQUEST with CM service type IE indicating “Emergency call establishment”? | --> | CM SERVICE REQUEST | 1 | P |
| 19a5 | The SS transmits an AUTHENTICATION REQUEST. | <-- | AUTHENTICATION REQUEST | - | - |
| 19a6 | The UE transmits AUTHENTICATION RESPONSE. | --> | AUTHENTICATION RESPONSE | - | - |
| 19a7 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 19a8 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 19a9 | Check: Does the UE transmit an EMERGENCY SETUP message with Emergency Service Category IE bit 6 set to 1 and all other bits are set to 0? | --> | EMERGENCY SETUP | 1 | P |
| 19a10-19a15 | Steps 11 to 16 of the generic test procedure in TS 34.108 subclause 7.2.3.2.3 are performed on Cell 5. NOTE: the CS call setup is completed. | - | - | - | - |
| 19a16 | Traffic channel is kept active for at least 5 seconds. | - | - | - | - |
| 19a17 | The SS transmits DISCONNECT. | <-- | DISCONNECT | - | - |
| 19a18 | The UE transmits RELEASE. | --> | RELEASE | - | - |
| 19a19 | The SS transmits RELEASE COMPLETE. | <-- | RELEASE COMPLETE | - | - |
| 19a20 | The SS transmits an RRC CONNECTION RELEASE message. | <-- | RRC CONNECTION RELEASE | - | - |
| 19a21 | The UE transmits RRC CONNECTION RELEASE COMPLETE. | --> | RRC CONNECTION RELEASE COMPLETE | - | - |
| 19b1 | IF (px_RATComb_Tested = EUTRA_GERAN AND pc_CS_Em_Call_in_GERAN), Check: Does the UE transmit a CHANNEL REQUEST message on Cell 24 with Establishment cause: Emergency call? | --> | CHANNEL REQUEST | 1 | P |
| 19b2 | The SS transmits an IMMEDIATE ASSIGNMENT message. | <-- | IMMEDIATE ASSIGNMENT | - | - |
| 19b3 | Check: Does the UE transmit a CM SERVICE REQUEST with CM service type IE indicating “Emergency call establishment”? | --> | CM SERVICE REQUEST | 1 | P |
| 19b4 | The SS transmits an AUTHENTICATION REQUEST message. | <-- | AUTHENTICATION REQUEST | - | - |
| 19b5 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 19b6 | The SS transmits a CIPHERING MODE COMMAND. | <-- | CIPHERING MODE COMMAND | - | - |
| 19b7 | The UE transmits a CIPHERING MODE COMPLETE. | --> | CIPHERING MODE COMPLETE | - | - |
| 19b8 | Check: Does the UE transmit an EMERGENCY SETUP message with Emergency Service Category IE bit 6 set to 1 and all other bits are set to 0? | --> | EMERGENCY SETUP | 1 | P |
| 19b9-19b15 | Steps 11 to 17 of the generic test procedure in TS 51.010-1 subclause 10.2.3 are performed on Cell 24. NOTE: the CS call setup is completed. | - | - | - | - |
| 19b16 | Traffic channel is kept active for at least 5 seconds. | - | - | - | - |
| 19b17 | The SS transmits DISCONNECT. | <-- | DISCONNECT | - | - |
| 19b18 | The UE transmits RELEASE. | --> | RELEASE | - | - |
| 19b19 | The SS transmits RELEASE COMPLETE. | <-- | RELEASE COMPLETE | - | - |
| 19b20 | The SS transmits CHANNEL RELEASE | <-- | CHANNEL RELEASE | - | - |
| Note 1: The request to originate a manual eCall may be performed by MMI or AT command. |  |  |  |  |  |

Table 11.3.10.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1 describes behaviour that depends on the UE behaviour. | - | - |
| 1 | If initiated by the UE, the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2 | Step 1 defined in annex C.47 of TS 34.229-1 [35] Generic procedure for eCall Setup takes place. (Note 1) | --> | INVITE |
| 3 | SS sends 486 Busy Here response. (Note 2) | <-- | 486 Busy Here |
| 4 | Step 3 defined in annex C.47 of TS 34.229-1 [35] Generic procedure for eCall Setup takes place. (Note 3) | --> | ACK |
| Note 1: Content of INVITE as specified in TS 34.229-1 [35] Step 1 of Annex C.47 using condition A6 instead of A7 and condition A20. Note 2: Content of 486 Busy Here as specified in TS 34.229-1 [35] Annex A.2.21 using condition A1. Note 3: Content of ACK as specified in TS 34.229-1 [35] Annex A.2.7 using condition A4. |  |  |  |

11.3.10.3.3	Specific message contents

Table 11.3.10.3.3-1: Message SystemInformationBlockType1 (Preamble)

Derivation path: 36.508 table 4.4.3.2-3 Condition eCalloverIMS

Table Table 11.3.10.3.3-2: RRC CONNECTION REQUEST (Step 19a1, Table 11.3.10.3.2-1)

| Derivation Path: TS 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Establishment cause | Emergency Call |  |  |

Table Table 11.3.10.3.3-3: CM SERVICE REQUEST (Steps 19a4 and 19b3, Table 11.3.10.3.2-1)

| Derivation Path: TS 24.008[43] Table 9.2.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CM service type | 0010 | Emergency call establishment |  |

Table Table 11.3.10.3.3-4: CHANNEL REQUEST (Step 19b1, Table 11.3.10.3.2-1)

| Derivation Path: TS 44.018 Table 9.1.8.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Establishment cause | 101 | Emergency call |  |

==============End of change==============