3GPP TSG-RAN5 Meeting #109	R5-256236r1

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-2 | CR | 1518 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicability for EPS CEN alignments test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Applicability needs to be added for new EPS CEN alignment test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added applicability of test case 11.3.10. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCallCEN-UEConTest WP will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-1 CR 5500 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Applicability condition Cabc to be resolved by assigning the next available number. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-256236r1: Updated test case title. |  |  |  |  |  |  |  |  |

==============First change==============

# 4	Recommended Test Case Applicability

The applicability of each individual test is identified in Table 4-1. This is just a recommendation based on the purpose for which the test case was written.

The applicability of every test is formally expressed by the use of Boolean expression that are based on parameters (ICS) included in annex A of the present document. The parameters (ICS) shall be set according to the capabilities of the UE on the operating band / band combination under test.

Additional information related to the Test Case (TC), e.g. affecting its dynamic behaviour or its execution may be provided as well.

==============Text Skipped==============

Table 4-1: Applicability of tests and additional information for testing

| Clause | TC Title | Release | Applicability Condition | Applicability Comment | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Idle mode operations |  |  |  |  |  |  |  |
| 6.1.1.1 | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  | Either TC 6.1.1.1 or TC 6.1.1.1b shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| … |  |  |  |  |  |  |  |  |
| 11.3 | eCall over IMS |  |  |  |  |  |  |  |
| 11.3.1 | eCall Only mode / T3444 / eCall inactivity procedure / Removal of eCall only restriction after an eCall over IMS | Rel-14 (Note 7) | C314 | UEs supporting E-UTRA and IMS eCall Only type of emergency services over EPS only and Manual type of eCall initiation | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.2 | eCall Only mode / T3445 / eCall inactivity procedure / Removal of eCall only restriction after a call to URI for test service | Rel-14 (Note 7) | C315 | UEs supporting E-UTRA and IMS eCall Only type of emergency services over EPS and Manual type of eCall initiation and capable of triggering a Test eCall | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.3 | eCall capable / EPS supports IMS voice over PS session / EPS supports emergency service / eCall over IMS is not supported / eCall using the CS domain / emergency call over IMS if eCall using the CS domain is not available / UTRA or GERAN | Rel-14 (Note 7) | C316 | UEs supporting E-UTRA and UTRA or GERAN and IMS eCall type of emergency services over EPS and Automatic type of eCall initiation and IMS emergency call | pc_eFDD |  |  | (Note 7A) |
|  |  |  |  |  | pc_eTDD |  |  | (Note 7A) |
| 11.3.4 | eCall Only mode / EPS supports IMS voice over PS session / EPS does not support emergency service / eCall over IMS is not supported / eCall using CS domain / eCall failure if CS domain is not available | Rel-14 (Note 7) | C317 | UEs supporting E-UTRA and UTRA or GERAN and IMS eCall Only type of emergency services over EPS and Automatic type of eCall initiation | pc_eFDD |  |  | (Note 7A) |
|  |  |  |  |  | pc_eTDD |  |  | (Note 7A) |
| 11.3.5 | eCall Only mode / EPS supports IMS voice over PS session / EPS supports emergency service / eCall over IMS is supported / RACH failure in EUTRA cell / eCall using the CS domain | Rel-14 (Note 7) | C317 | UEs supporting E-UTRA and UTRA or GERAN and IMS eCall Only type of emergency services over EPS and Automatic type of eCall initiation | pc_eFDD |  |  | (Note 7A) |
|  |  |  |  |  | pc_eTDD |  |  | (Note 7A) |
| 11.3.6 | eCall Only mode / Limited service state / Call to URI for test service should not be attempted / eCall over IMS should be attempted | Rel-14 (Note 7) | C315 | UEs supporting E-UTRA and IMS eCall Only type of emergency services over EPS and Manual type of eCall initiation and capable of triggering a Test eCall | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.7 | eCall Only mode / SRVCC Handover to CS domain / UTRAN / MSD Update / Success | Rel-14 (Note 7) | C318 | UEs supporting E-UTRA and UTRA and IMS eCall Only type of emergency services over EPS and Manual type of eCall initiation | pc_eFDD |  |  | (Note 7A) |
|  |  |  |  |  | pc_eTDD |  |  | (Note 7A) |
| 11.3.8 | eCall Only mode / SRVCC Handover to CS domain / GERAN / MSD Update / Success | Rel-14 (Note 7) | C319 | UEs supporting E-UTRA and GERAN and IMS eCall Only type of emergency services over EPS and Manual type of eCall initiation | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.9 | eCall Only mode / EPS supports IMS voice over PS session / EPS supports emergency service / eCall over IMS is not supported / emergency call over IMS | Rel-19 (Note 7) | C314b | UEs supporting E-UTRA and IMS eCall Only type of emergency services over EPS and Manual type of eCall initiation and subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over EPS | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 11.3.10 | eCall Only mode / Limited service state / Manual initiation / Abnormal case / IM CN sends a 486 (Busy Here) / UE performs eCall in CS domain / UTRAN or GERAN | Rel-19 (Note 7) | Cabc | UEs supporting E-UTRA and (UTRA or GERAN) and IMS eCall Only type of emergency services over EPS and Manual type of eCall initiation and subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over EPS | pc_eFDD |  |  | (Note 7A) |
|  |  |  |  |  | pc_eTDD |  |  | (Note 7A) |

==============Next change==============

Table 4-1a: Applicability of tests Conditions

C01 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C01a IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.1/1 AND (A.4.5-2/3 OR A.4.5-2/4) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C01b IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.1/1 AND A.4.5-2/4 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

…

C447 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/266 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

Cabc IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND [45]A.12/55 AND [8]A.10/16 AND [45]A.12/67 THEN R ELSE N/A

==============End of change==============