3GPP TSG-RAN5 Meeting #109	R5-256299r1

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1219 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of a new test case for transmitter power for simultaneous transmission to multiple directions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of transmitter power for simultaneous transmission to multiple directions |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 6.2K |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MIMO_evo_DL_UL-UEConTest work item cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2K |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38905.CR 1100, TS/TR 38.522 CR 0684 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 Correction to power classes and CR correction to other specs |  |  |  |  |  |  |  |  |

38

==============First change==============

### 6.2D.4	Configured transmitted power for UL MIMO

6.2D.4.1	Test purpose

To verify the UE transmitted power PUMAX,f,c is within the range defined prescribed by the specified nominal maximum output power and tolerance.

6.2D.4.2	Test applicability

The requirements of this test are covered in test cases 6.2D.1 UE Maximum output power for UL MIMO, 6.2D.2 UE maximum output power reduction for UL MIMO and 6.2D.3 UE Maximum output power with additional requirements for UL MIMO to all types of NR UE release 15 and forward that supports UL MIMO.

6.2D.4.3	Minimum conformance requirements

For UEs configured for 2-layer transmission as well as UEs configured for single layer uplink full power transmission (ULFPTx), the configured maximum output power PCMAX,c for serving cell c is defined as sum of all streams and is bound by limits set in section 6.2.4.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2D.4.

6.2D.4.4	Test description

This test is covered by clause 6.2D.1 UE Maximum output power for UL MIMO, 6.2D.2 UE maximum output power reduction for UL MIMO and 6.2D.3 UE Maximum output power with additional requirements for UL MIMO.

6.2D.4.5	Test requirements

This test is covered by clause 6.2D.1 UE Maximum output power for UL MIMO, 6.2D.2 UE maximum output power reduction for UL MIMO and 6.2D.3 UE Maximum output power with additional requirements for UL MIMO.

## 6.2K	Transmitter power for simultaneous transmission to multiple directions

Editor's note:	The following aspects are either missing or not yet determined:

Test Procedure is FFS

Test Tolerances are FFS

### 6.2K.1	UE maximum output power for simultaneous transmission to multiple directions

For UEs configured for simultaneous transmission to multiple directions, the maximum output power for each of indicated joint/UL TCI states is specified in clause 6.2K.1.

#### 6.2K.1.0	General

Note: Power classes  are specified based on the assumption of certain UE types with specific device architectures. The UE types can be found in Table 6.2K.1.0-1.

Table 6.2K.1.0-1: Assumption of UE Types

| UE Power class | UE type |
| --- | --- |
| 1 | Fixed wireless access (FWA) UE |
| 2 | Vehicular UE |
| 5 | Fixed wireless access (FWA) UE |
| 6 | High Speed Train Roof-Mounted UE |
| Note: Any power class can be used for Redcap type devices as long as the device can meet the core requirements that are applicable to Redcap devices as defined in clause 4.2.21.1 from TS38.306 [14] |  |

#### 6.2K.1.1	UE maximum output power - EIRP and TRP

Editor's note:	The following aspects are either missing or not yet determined:

-	Measurement Uncertainties and Test Tolerances are FFS.

6.2K.1.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2K.1.1.2	Test applicability

This test case applies to all types of release 18 and UE’s supporting simultaneous transmission to multiple directions.

6.2K.1.1.3	Minimum conformance requirements

6.2K.1.1.3.1	UE maximum output power for power classes 1, 2, 5, and 6

Refer to 6.2.1.1.3 for Power Class 1, 2, 5, and 6

6.2K.1.1.4	Test description

6.2K.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.2.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2K.1.1.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] clause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] clause 4.3.1 |  |  |  |  | Low range, Mid Range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] clause 4.3.1 |  |  |  |  | Lowest, 100 MHz, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw (NOTE 2) | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Default | - | Modulation |  | RB allocation (NOTE 1) |
| 1 | 50 |  |  | DFT-s-OFDM QPSK |  | Inner_Full for PC2, |
| 2 | 100 |  |  |  |  | PC5 and PC6 |
| 3 | 200 |  |  |  |  | Inner_Full_Region1 for |
| 4 | 400 |  |  |  |  | PC1 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC5 and PC6 or Table 6.1-2 for PC1. NOTE 2: The 200MHz and 400MHz bandwidths are not applicable to PC7 RedCap UEs |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.2.1.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2.1.1.4.3

6.2K.1.1.4.2	Test procedure

FFS

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Messages to configure the appropriate uplink modulation in section 6.2.1.1.4.3.

1a.	The side conditions for SSB-based and CSI-RS based L1-RSRP measurements are applied as per clause 6.6.1.3.3.1.1 for PC3 and 6.6.1.3.6.1.1 for PC7.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2.1.1.5-1 to 6.2.1.1.5-4. EIRP test procedure is defined in Annex K.1.3. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

6.	Measure TRP of the transmitted signal for the assigned NR channel with a rectangular measurement filter with bandwidths according to Table 6.5.2.3.5-1. Total radiated power is measured according to TRP measurement procedure defined in Annex K.1.7 and measurement grid specified in Annex M.4. TRP is calculated considering both polarizations, theta and phi.

7.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

6.2K.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] clause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

6.2K.1.1.5	Test requirement

The EIRP derived in step 5 and TRP derived in step 6 shall not exceed the values specified in Table 6.2.1.1.5-1 to Table 6.2K.1.1.5-4.

Table 6.2K.1.1.5-1: UE maximum output test requirements for power class 1

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 35+TT | 55 | 40.0-TT |
| n258 | 35+TT | 55 | 40.0-TT |
| n260 | 35+TT | 55 | 38.0-TT |
| n261 | 35+TT | 55 | 40.0-TT |

Table 6.2K.1.1.5-1a: Test Tolerance (Max TRP for Power class 1)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | TBD TBD | TBD TBD |

Table 6.2K.1.1.5-1b: Test Tolerance (Min peak EIRP for Power class 1)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | TBD TBD | TBD TBD |

Table 6.2K.1.1.5-2: UE maximum output test requirements for power class 2

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 29-TT |
| n258 | 23+TT | 43 | 29-TT |
| n260 |  |  |  |
| n261 | 23+TT | 43 | 29-TT |

Table 6.2K.1.1.5-5: UE maximum output power test requirements for power class 5

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 30.0-TT-MBP,n |
| n258 | 23+TT | 43 | 30.4-TT-MBP,n |
| Note 1: MBP,n = 0 for single band UE. For multi-band UEs, MBP,n is defined in table 6.2.1.1.3.5-4. |  |  |  |

Table 6.2K.1.1.5-5a: Test Tolerance (Max TRP for Power class 5)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | TBD TBD |

Table 6.2K.1.1.5-5b: Test Tolerance (Min peak EIRP for Power class 5)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | TBD TBD |

Table 6.2K.1.1.5-6: UE maximum output power test requirements for power class 6

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257, n261 | 23+TT | 43 | 30.0-TT-MBP,n |
| n258 | 23+TT | 43 | 30.4-TT-MBP,n |
| Note 1: MBP,n = 0 for single band UE. For multi-band UEs, MBP,n is defined in table 6.2.1.1.3.5-4. |  |  |  |

Table 6.2K.1.1.5-6a: Test Tolerance (Max TRP for Power class 6)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | TBD TBD |

Table 6.2K.1.1.5-6b: Test Tolerance (Min peak EIRP for Power class 6)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | TBD TBD |

## 6.3	Output power dynamics

### 6.3.1	Minimum output power

Editor's note:	The following aspects of the clause are for future consideration:

-	Relaxation, Measurement Uncertainties and Test Tolerances are FFS for power class other than 1, 5 and 6

==============End of change==============