3GPP TSG-RAN WG5 Meeting #109	R5-256316r1

# Dallas, US, November 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3659 | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Simplifications on test frequency table for EN-DC and NE-DC band combinations within FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R19-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the discussion paper in R5-256314, the following simplifications are suggested to be applied to the test frequencies for EN-DC band combinations within FR1. - For EN-DC band combinations within FR1, a general rule for deciding test frequencies is setup by using symbolic DC configurations, e.g. DC_XA_nYA could be DC_1A_n3A if X=1 and nY=n3. Any DC configurations not applying the genral rule could be listed separately. - For NE-DC band combinations within FR1, a general rule for deciding test frequencies is setup by using symbolic DC configurations, e.g. DC_nXA_YA could be DC_n28A_3A if nX=n28 and Y=3. Any CA configurations not applying the genral rule could be listed separately. - The specified inter-band EN-DC and NE-DC configurations can be found in 38.521-3 clauses 5.5B.4 and 5.5B.4a respectively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the test frequency table for inter-band EN-DC configurations. - Update the test frequency table for inter-band NE-DC configurations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test frequencies for inter-band EN-DC and NE-DC configurations will remain redundant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.2.1, 4.3.1.1.2.2, 4.3.1.1.2.3, 4.3.1.1.7.1, 4.3.1.4a.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-256316r1: The revision r1 is to correct the spec version to v19.1.1 in the coversheet. |  |  |  |  |  |  |  |  |

<< Start of changes >>

##### 4.3.1.4.1	Inter-band EN-DC configurations within FR1

###### 4.3.1.4.1.1	General

For inter-band EN-DC configurations as listed in this clause, the following apply:

For the E-UTRA band and E-UTRA CA configurations, test frequencies as specified in TS 36.508 [2], clause 4.3.1 are used.

For the NR band and NR CA configurations, test frequencies as specified in clause 4.3.1.1 are used.

For the EN-DC inter-band configuration that includes an EN-DC contiguous configuration (e.g. DC_2A-(n)71AA) the EN-DC contiguous configuration is listed in the NR configuration column and the test frequencies as specified in clause 4.3.1.4.2 are used.

For the secondary NR band in inter-band signalling test cases, the band selected is based on the subset of NR bands supported within the EN-DC configurations specified in Table 4.3.1.4.1.2-1 for NR FR1.

###### 4.3.1.4.1.2	Inter-band EN-DC configurations within FR1 (two bands)

Table 4.3.1.4.1.2-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_XA_nYA | - | XA | nYA | - | - | Yes |
|  | DC_XA_nYA |  |  | XA | nYA |  |
| DC_XA_nYC | - | XA | CA_nYC | - | - | Yes (NR 2CC) |
|  | DC_XA_nYA |  |  | XA | nYA |  |
| DC_XC_nYA | - | CA_XC | nYA | - | - | No |
|  | DC_XA-nYA |  |  | XA | nYA |  |
|  | DC_XC_nYA |  |  | CA_XC | nYA |  |
| DC_XC_nYC | - | CA_XC | CA_nYC | - | - | No |
| DC_XA_nY(2A) | DC_XA_nYA | XA | CA_nY(2A) | XA | nYA | Yes (NR 2CC) |
| DC_XC_nY(2A) | DC_XA-nYA | CA_XC | CA_nY(2A) | XA | nYA | No |
| DC_XA-XA_nYA | DC_XA_nYA | CA_XA-XA | nYA | XA | nYA | No |
| DC_XA-XA_nY(2A) | DC_XA_nYA | CA_XA-XA | CA_nY(2A) | XA | nYA | No |
| DC_XA-XA-XA_nYA | DC_XA_nYA | CA_ XA-XA-XA | nYA | XA | nYA | No |
| DC_XA-XA-XA_nY(2A) | DC_XA_nYA | CA_ XA-XA-XA | CA_nY(2A) | XA | nYA | No |
| DC_XD_nYA | - | CA_XD | nYA | - | - | No |
| DC_XE_nYA | - | CA_XE | nYA | - | - | No |
| DC_1A_n79C | DC_1A_n79A | 1A | CA_n79C | 1A | n79A | FFS (NR 2CC) |
| DC_3A_n79C | DC_3A_n79A | 3A | CA_n79C | 3A | n79A | FFS (NR 2CC) |
| DC_19A_n79C | DC_19A_n79A | 19A | CA_n79C | 19A | n79A | FFS (NR 2CC) |
| DC_21A_n79C | DC_21A_n79A | 21A | CA_n79C | 21A | n79A | FFS (NR 2CC) |
| DC_42A_n79C | - | 42A | CA_n79C | - | - | FFS (NR 2CC) |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. Note 2: The general rule for deciding test frequencies for inter-band EN-DC configurations are represented by symbolic DC configurations using band X and nY as the different bands in the DC Configuration. E.g. DC_XA_nYA could be representing the configuraion DC_1A_n3A by interpreting of X=1 and nY=n3. Any DC configurations not applying the general rule could be listed separately. Note 3: The specified inter-band EN-DC configurations within FR1 with two bands can be found in 38.521-3 clauses 5.5B.4.1. |  |  |  |  |  |  |

###### 4.3.1.4.1.3	Inter-band EN-DC configurations within FR1 (three bands)

Table 4.3.1.4.1.3-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC Configuration |  | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_XA-YA_nZA | DC_XA_nZA |  | CA_XA-YA | nZA | XA | nZA | No |
|  | DC_YA_nZA |  |  |  | YA | nZA |  |
| DC_XA-YA_nZ(2A) |  | DC_XA_nZA | CA_XA-YA | CA_nZ(2A) | XA | nZA | No |
|  | DC_YA_nZA |  |  | YA | nZA |  |
| DC_XA-YA_nZC | DC_XA_nZA |  | CA_XA-YA | CA_nZC | XA | nZA | No |
|  | DC_YA_nZA |  |  |  | YA | nZA |  |
| DC_XA-YC_nZA | DC_XA_nZA |  | CA_XA-YC | nZA | XA | nZA | No |
|  | DC_YA_nZA |  |  |  | YA | nZA |  |
|  | DC_YC_nZA |  |  |  | CA_YC | nZA |  |
| DC_XA-YC_nZ(2A) |  | DC_XA_nZA | CA_XA-YC | CA_nZ(2A) | XA | nZA | No |
|  |  | DC_YA_nZA |  |  | YA | nZA |  |
|  | DC_YC_nZA |  |  | CA_YC | nZA |  |
| DC_XC-YA_nZA |  | DC_XA_nZA | CA_XC-YA | nZA | XA | nZA | No |
|  |  | DC_YA_nZA |  |  | YA | nZA |  |
| DC_XC-YA_nZ(2A) |  | DC_XA_nZA | CA_XC-YA | nZA | XA | nZA | No |
|  | DC_YA_nZA |  |  | YA | nZA |  |
| DC_XA-YD_nZA | DC_XA_nZA |  | CA_XA-YD | nZA | XA | nZA | No |
| DC_XA-YE_nZA | DC_XA_nZA |  | CA_XA-YE | nZA | XA | nZA | No |
| DC_XA-XA-YA_nZA | DC_XA_nZA |  | CA_XA-XA-YA | nZA | XA | nZA | No |
|  | DC_YA_nZA |  |  |  | YA | nZA |  |
| DC_XA-XA-YA_nZ(2A) |  | DC_XA_nZA | CA_XA-XA-YA | CA_nZ(2A) | XA | nZA | No |
|  | DC_YA_nZA |  |  | YA | nZA |  |
| DC_XA-XA-YC_nZA | DC_XA_nZA |  | CA_XA-XA-YC | nZA | XA | nZA | No |
|  | DC_YA_nZA |  |  |  | YA | nZA |  |
|  | DC_YC_nZA |  |  |  | CA_YC | nZA |  |
| DC_XA-YA-YA_nZA |  | DC_XA_nZA | CA_XA-YA-YA | nZA | XA | nZA | No |
|  |  | DC_YA_nZA |  |  | YA | nZA |  |
| DC_XA-YA-YA_nZA(2A) |  | DC_XA_nZA | CA_XA-YA-YA | CA_nZ(2A) | XA | nZA | No |
|  |  | DC_YA_nZA |  |  | YA | nZA |  |
| DC_XA-XA-YA-YA_nZA |  | DC_XA_nZA | CA_XA-XA-YA-YA | nZA | XA | nYA | No |
|  |  | DC_YA_nZA |  |  | XA | nZA |  |
| DC_XA_nYA-nZA |  | DC_XA_nZA | XA | CA_nYA-nZA | XA | nYA | Yes (NR 2CC) |
|  |  | DC_YA_nZA |  |  | XA | nZA | No |
| DC_XC_nYA-nZA |  | DC_XA_nZA | CA_XC | CA_nYA-nZA | XA | nZA | No |
|  |  | DC_YA_nZA |  |  | YA | nZA |  |
|  |  | DC_XC_nZA |  |  | CA_XC | nZA |  |
|  | DC_YC_nZA |  |  | CA_XC | nZA |  |
| DC_XA-(n)YAA |  | DC_XA_nYA | XA | DC_(n)YAA | XA | nYA | No |
|  |  | DC_(n)YAA |  |  | - | DC_(n)YAA |  |
| DC_1A-28A_n5A2 | DC_1A_n5A |  | CA_1A-28A | n5A | 1A | n5A | No |
|  | DC_28A_n5A |  |  |  | 28A | n5A |  |
| DC_7A-28A_n5A2 | DC_7A_n5A |  | CA_7A-28A | n5A | 7A | n5A | No |
|  | DC_28A_n5A |  |  |  | 28A | n5A |  |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. Note 2: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758 – 788 MHz for the DL. Note 3: The general rule for deciding test frequencies for inter-band EN-DC configurations are represented by symbolic DC configurations using band X, Y/nY and nZ as the different bands in the DC Configuration. E.g. DC_XA-YA_nZA could be representing the configuraion DC_1A-3A_n28A by interpreting of X=1, Y=3 and nZ=n28. Any DC configurations not applying the general rule could be listed separately. Note 4: The specified inter-band EN-DC configurations within FR1 with three bands can be found in 38.521-3 clauses 5.5B.4.2. |  |  |  |  |  |  |

###### 4.3.1.4.1.4	Inter-band EN-DC configurations within FR1 (four bands)

Table 4.3.1.4.1.4-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_XA-YA-ZA_nWA | DC_XA_nWA | CA_XA-YA-ZA | nWA | XA | nWA | No |
|  | DC_YA_nWA |  |  | YA | nWA |  |
|  | DC_ZA_nWA |  |  | ZA | nWA |  |
| DC_XA-YA-ZA_nW(2A) | DC_XA_nWA | CA_XA-YA-ZA | CA_nW(2A) | XA | nWA | No |
|  | DC_YA_nWA |  |  | YA | nWA |  |
|  | DC_ZA_nWA |  |  | ZA | nWA |  |
| DC_XA-YA-ZC_nWA | DC_XA_nWA | CA_XA-YA-ZC | nWA | XA | nWA | No |
|  | DC_YA_nWA |  |  | YA | nWA |  |
| DC_XA-YA-ZC_nW(2A) | DC_XA_nWA | CA_XA-YA-ZC | CA_nW(2A) | XA | nWA | No |
|  | DC_YA_nWA |  |  | YA | nWA |  |
| DC_XA-YA-ZD_nWA | DC_XA_nWA | CA_XA-YA-ZD | nWA | XA | nWA | No |
|  | DC_YA_nWA |  |  | YA | nWA |  |
| DC_XA-YC-ZA_nWA | DC_XA_nWA | CA_XA-YC-ZA | nWA | XA | nWA | No |
|  | DC_YA_nWA |  |  | YA | nWA |  |
|  | DC_ZA_nWA |  |  | ZA | nWA |  |
| DC_XC-YA-ZA_nWA | DC_XA_nWA | CA_XC-YA-ZA | nWA | XA | nWA | No |
|  | DC_YA_nWA |  |  | YA | nWA |  |
|  | DC_ZA_nWA |  |  | ZA | nWA |  |
| DC_XA-YC-ZA_nW(2A) | DC_XA_nWA | CA_XA-YC-ZA | CA_nW(2A) | XA | nWA | No |
|  | DC_YA_nWA |  |  | YA | nWA |  |
|  | DC_ZA_nWA |  |  | ZA | nWA |  |
| DC_XA-XA-YA-ZA_nWA | DC_XA_nWA | CA_XA-XA-YA-ZA | nWA | XA | nWA | No |
|  | DC_YA_nWA |  |  | YA | nWA |  |
|  | DC_ZA_nWA |  |  | ZA | nWA |  |
| DC_XA-YA-YA-ZA_nWA | DC_XA_nWA | CA_XA-YA-YA-ZA | nWA | XA | nWA | No |
|  | DC_YA_nWA |  |  | YA | nWA |  |
|  | DC_ZA_nWA |  |  | ZA | nWA |  |
| DC_XA-YA-ZA-ZA_nWA | DC_XA_nWA | CA_XA-YA-ZA-ZA | nWA | XA | nWA | No |
|  | DC_YA_nWA |  |  | YA | nWA |  |
|  | DC_ZA_nWA |  |  | ZA | nWA |  |
| DC_XA-YA_nZA-nWA | DC_XA_nZA | CA_XA-YA | CA_nZA-nWA | XA | nZA | No |
|  | DC_XA_nWA |  |  | XA | nWA |  |
|  | DC_YA_nZA |  |  | YA | nZA |  |
|  | DC_YA_nWA |  |  | YA | nWA |  |
| DC_XA-YC_nZA-nWA | DC_XA_nZA | CA_XA-YC | CA_nZA-nWA | XA | nZA | No |
|  | DC_XA_nWA |  |  | XA | nWA |  |
| DC_XA-YA-(n)ZAA | DC_XA_nZA | CA_XA-YA | DC_(n)ZAA | XA | nZA | No |
|  | DC_YA_nZA |  |  | YA | nZA |  |
|  | DC_(n)ZAA |  |  | - | DC_(n)ZAA |  |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. Note 2: The general rule for deciding test frequencies for inter-band EN-DC configurations are represented by symbolic DC configurations using band X, Y, Z/nZ and nW as the different bands in the DC Configuration. E.g. DC_XA-YA-ZA_nWA could be representing the configuraion DC_1A-3A-7A_n28A by interpreting of X=1, Y=3, Z=7 and nW=n28. Any DC configurations not applying the general rule could be listed separately. Note 3: The specified inter-band EN-DC configurations within FR1 with four bands can be found in 38.521-3 clauses 5.5B.4.3. |  |  |  |  |  |  |

###### 4.3.1.4.1.5	Inter-band EN-DC configurations within FR1 (five bands)

Table 4.3.1.4.1.5-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_XA-YA-ZA-VA_nWA | DC_XA_nWA | CA_XA-YA-ZA-VA | nWA | XA | nWA | No |
|  | DC_YA_nWA |  |  | YA | nWA |  |
|  | DC_ZA_nWA |  |  | ZA | nWA |  |
|  | DC_VA_nWA |  |  | VA | nWA |  |
| DC_XA-YA-ZA-VC_nWA | DC_XA_nWA | CA_XA-YA-ZA-VC | nWA | XA | nWA | No |
|  | DC_YA_nWA |  |  | YA | nWA |  |
|  | DC_ZA_nWA |  |  | ZA | nWA |  |
| DC_XA-YC-ZA-VA_nWA | DC_XA_nWA | CA_XA-YC-ZA-VA | nWA | XA | nWA | No |
|  | DC_YA_nWA |  |  | YA | nWA |  |
|  | DC_ZA_nWA |  |  | ZA | nWA |  |
|  | DC_VA_nWA |  |  | VA | nWA |  |
| DC_XA-YA-ZA_nVA-nWA | DC_XA_nVA | CA_XA-YA-ZA | CA_nVA-nWA | XA | nVA | No |
|  | DC_XA_nWA |  |  | XA | nWA |  |
|  | DC_YA_nVA |  |  | YA | nVA |  |
|  | DC_YA_nWA |  |  | YA | nWA |  |
|  | DC_ZA_nVA |  |  | ZA | nVA |  |
|  | DC_ZA_nWA |  |  | ZA | nWA |  |
| DC_XA-YA-ZC_nVA-nWA | DC_XA_nVA | CA_XA-YA-ZC | CA_nVA-nWA | XA | nVA | No |
|  | DC_XA_nWA |  |  | XA | nWA |  |
|  | DC_YA_nVA |  |  | YA | nVA |  |
|  | DC_YA_nWA |  |  | YA | nWA |  |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. Note 2: The general rule for deciding test frequencies for inter-band EN-DC configurations are represented by symbolic DC configurations using band X, Y, Z, V/nV and nW as the different bands in the DC Configuration. E.g. DC_XA-YA-ZA-VA_nWA could be representing the configuraion DC_1A-3A-7A-20A_n28A by interpreting of X=1, Y=3, Z=7, V=20 and nW=n28. Any DC configurations not applying the general rule could be listed separately. Note 3: The specified inter-band EN-DC configurations within FR1 with five bands can be found in 38.521-3 clauses 5.5B.4.4. |  |  |  |  |  |  |

###### 4.3.1.4.1.6	Inter-band EN-DC configurations within FR1 (six bands)

Table 4.3.1.4.1.6-1: Inter-band EN-DC configurations within FR1 (six bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_XA-YA-ZA-UA_nVA-nWA | DC_XA_nVA | CA_XA-YA-ZA-UA | CA_nVA-nWA | XA | nVA | No |
|  | DC_XA_nWA |  |  | XA | nWA |  |
|  | DC_YA_nVA |  |  | YA | nVA |  |
|  | DC_YA_nWA |  |  | YA | nWA |  |
|  | DC_ZA_nVA |  |  | ZA | nVA |  |
|  | DC_ZA_nWA |  |  | ZA | nWA |  |
|  | DC_UA_nVA |  |  | UA | nVA |  |
|  | DC_UA_nWA |  |  | UA | nWA |  |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. Note 2: The general rule for deciding test frequencies for inter-band EN-DC configurations are represented by symbolic DC configurations using band X, Y, Z, U, nV and nW as the different bands in the DC Configuration. E.g. DC_XA-YA-ZA-UA_nVA-nWA could be representing the configuraion DC_1A-3A-7A-20A_n28A-n78A by interpreting of X=1, Y=3, Z=7, U=20, nV=n28 and nW=n78. Any DC configurations not applying the general rule could be listed separately. Note 3: The specified inter-band EN-DC configurations within FR1 with six bands can be found in 38.521-3 clauses 5.5B.4.5. |  |  |  |  |  |  |

<< Next changes >>

4.3.1.4a	Test frequencies for NE-DC band combinations within FR1

4.3.1.4a.1	Inter-band NE-DC configurations within FR1

4.3.1.4a.1.1	General

For inter-band NE-DC configurations as listed in this clause, the following apply:

For the NR band and NR CA configurations, test frequencies as specified in clause 4.3.1.1 are used.

For the E-UTRA band and E-UTRA CA configurations, test frequencies as specified in TS 36.508 [2], clause 4.3.1 are used.

For the primary NR band in inter-band signalling test cases, the band selected is based on the subset of NR bands supported within the NE-DC configurations specified in Table 4.3.1.4a.1.2-1 for NR FR1.

4.3.1.4a.1.2	Inter-band NE-DC configurations within FR1 (two bands)

Table 4.3.1.4a.1.2-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC Configuration | NR downlink configuration | E-UTRA downlink configuration | NR uplink configuration | E-UTRA uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_nXA_YA | DC_nXA_YA | nXA | YA | nXA | YA | Yes |
| DC_nXA_YC | DC_nXA_YA | nXA | CA_YC | nXA | YA | No |
| Note 1: The general rule for deciding test frequencies for inter-band NE-DC configurations are represented by symbolic DC configurations using band nX and Y as the different bands in the DC Configuration. E.g. DC_nXA_YA could be representing the configuraion DC_n28A_3A by interpreting of nX=n28 and Y=3. Any DC configurations not applying the general rule could be listed separately. Note 2: The specified inter-band NE-DC configurations within FR1 with two bands can be found in 38.521-3 clauses 5.5B.4a.1. |  |  |  |  |  |  |

==============================================================

<< End of changes >>