3GPP TSG-RAN WG5 Meeting # 109	R5-256334r1

# Dallas, US, November 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 2023 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on reference sensitivity test requirement table for PC2 EN-DC configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_CADC_NR_LTE_DC_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | [Editorial Correction] The table headings for reference sensitivity test requirements for PC2 EN-DC configurations should be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the table headings for PC2 EN-DC reference sensitivity test requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The table headings for PC2 EN-DC reference sensitivity test requirements will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-256334r1: Correct the affected clauses in the coversheet. |  |  |  |  |  |  |  |  |

<< Start of changes >>

7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

Editor's note:	The minimum conformance requirements for the following test points are missing in 7.3B.2.0.3.

- DC_40A_n78A (REFSENS_ENDC_2) in Table 7.3B.2.3.4.2.1-6

- DC_48A_n66A (REFSENS_ENDC_1) in Table 7.3B.2.3.4.2.1-6

Editor’s Note: Test configuration table and test requirements in clauses 7.3B.2.1.4.1 and 7.3B.2.1.5 alignment with TP analysis in TR 38.905 (V19.0.0) clause 4.3.3.1 is FFS.

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC within FR1 with 2 DL CCs.

7.3B.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

LTE anchor agnostic approach is not applied.

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.5B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

-------- Skipped unchanged texts --------

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations affected by reference sensitivity exceptions, when test points without NOTE 8, 9 and 11 in Table 7.3B.2.3.4.2.1-6 are tested, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-0, 7.3B.2.3.5-0a, Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-3a, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation for PC3 EN-DC, cross band isolation for PC2 EN-DC, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

For test points with NOTE 8, 9 or 11 in Table 7.3B.2.3.4.2.1-6, reference sensitivity test requirements are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC, and Table 7.3.5-1 in TS 36.521-1 [10] for E-UTRA CC.

Reference sensitivity test requirements for test points in Table 7.3B.2.3.4.2.1-0 are specified in Table 7.3.2.5-1 in TS 38.521-1 [8].

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable two, three, four and five bands operation.

Table 7.3B.2.3.5-0: Reference sensitivity test requirement for PC3 EN-DC

| EN-DC configuration | Test ID | E-UTRA/NR band | UL/DL CBW (MHz) | REFSENS test requirement (dBm) | Exception type |
| --- | --- | --- | --- | --- | --- |
| DC_1A_n3A | 1 | 1 | 20 | REFSENS | Cross band isolation |
|  |  | n3 | 5 | REFSENS | avoided |
|  | 2 | 1 | 5 | REFSENS + 23 | Dual uplink operation |
|  |  | n3 | 5 | REFSENS |  |
|  | 3 | 1 | 20 | REFSENS | Cross band isolation |
|  |  | n3 | 5 | REFSENS + 7.9 |  |
| DC_1A_n8A | 1 | 1 | 5 | REFSENS + 6 | Dual uplink operation |
|  |  | n8 | 5 | REFSENS |  |
| DC_1A_n28A | 1 | 1 | 20 | REFSENS + 5.3 | UL harmonic exception |
|  |  | n28 | 5 | REFSENS |  |
|  | 2 | 1 | 20 | REFSENS | UL harmonic exception |
|  |  | n28 | 5 | REFSENS | avoided |
| DC_1A_n40A | 1 | 1 | 20 | REFSENS | Cross band isolation |
|  |  | n40 | 5 | REFSENS + 6.6 |  |
|  | 2 | 1 | 5 | REFSENS + 9.1 | Cross band isolation |
|  |  | n40 | 100 | REFSENS |  |
| DC_1A_n41A | 1 | 1 | 20 | REFSENS | Cross band isolation |
|  |  | n41 | 10 | REFSENS + 6.1 |  |
|  | 2 | 1 | 5 | REFSENS + 9.1 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_1A_n71A | 1 | 1 | 5 | REFSENS | Harmonic mixing |
|  |  | n71 | 5 | REFSENS + 26.8 |  |
|  | 2 | 1 | 5 | REFSENS | Harmonic mixing |
|  |  | n71 | 20 | REFSENS + 15.6 |  |
|  | 3 | 1 | 5 | REFSENS | Harmonic mixing |
|  |  | n71 | 5 | REFSENS | avoided |
| DC_1A_n77A | 1 | 1 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS +13.8 |  |
|  | 2 | 1 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 10 | REFSENS + 23.9 |  |
|  | 3 | 1 | 5 | REFSENS + 29.8 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 4 | 1 | 5 | REFSENS + 8 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_1A_n78A | 1 | 1 | 5 | REFSENS + 8 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_2A_n41A | 1 | 2 | 5 | REFSENS + 0.6 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_2A_n66A | 1 | 2 | 5 | REFSENS + 20 | Dual uplink operation |
|  |  | n66 | 5 | REFSENS |  |
|  | 2 | 2 | 5 | REFSENS | Dual uplink operation |
|  |  | n66 | 5 | REFSENS + 4 |  |
| DC_2A_n71A | 1 | 2 | 5 | REFSENS + 1.7 | UL harmonic exception |
|  |  | n71 | 5 | REFSENS |  |
|  | 2 | 2 | 5 | REFSENS | Harmonic mixing |
|  |  | n71 | 5 | REFSENS + 15.6 |  |
| DC_2A_n77A | 1 | 2 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 10 | REFSENS + 23.9 |  |
|  | 2 | 2 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS + 13.8 |  |
|  | 3 | 2 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 10 | REFSENS | avoided |
|  | 4 | 2 | 20 | REFSENS + 2.9 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 5 | 2 | 20 | REFSENS | Harmonic mixing |
|  |  | n77 | 10 | REFSENS | avoided |
|  | 6 | 2 | 5 | REFSENS + 26 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 7 | 2 | 5 | REFSENS + 8 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 8 | 2 | 5 | REFSENS + 5 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_2A_n78A | 1 | 2 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS + 13.8 |  |
|  | 2 | 2 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 10 | REFSENS + 23.9 |  |
|  | 3 | 2 | 5 | REFSENS + 26 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
|  | 4 | 2 | 5 | REFSENS + 8 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_3A_n1A | 1 | 3 | 5 | REFSENS | Cross band isolation |
|  |  | n1 | 50 | REFSENS | avoided |
|  | 2 | 3 | 5 | REFSENS + 19.7 | Cross band isolation |
|  |  | n1 | 50 | REFSENS |  |
|  | 3 | 3 | 5 | REFSENS | Dual uplink operation |
|  |  | n1 | 5 | REFSENS + 23 |  |
| DC_3A_n5A | 1 | 3 | 10 | REFSENS + 4 | Dual uplink operation |
|  |  | n5 | 5 | REFSENS |  |
|  | 2 | 3 | 10 | REFSENS | Dual uplink operation |
|  |  | n5 | 5 | REFSENS + 24 |  |
| DC_3A_n7A | 1 | 3 | 5 | REFSENS | Dual uplink operation |
|  |  | n7 | 10 | REFSENS + 10.2 |  |
| DC_3A_n8A | 1 | 3 | 10 | REFSENS | Dual uplink operation |
|  |  | n8 | 5 | REFSENS + 8 |  |
|  | 2 | 3 | 10 | REFSENS + 6.4 | Dual uplink operation |
|  |  | n8 | 5 | REFSENS |  |
| DC_3A_n41A | 1 | 3 | 20 | REFSENS | Cross band isolation |
|  |  | n41 | 10 | REFSENS + 0.7 |  |
|  | 2 | 3 | 5 | REFSENS + 0.6 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
|  | 3 | 3 | 5 | REFSENS + 8.2 | Dual uplink operation |
|  |  | n41 | 10 | REFSENS |  |
| DC_3A_n77A | 1 | 3 | 5 | REFSENS | UL harmonic |
|  |  | n77 | 10 | REFSENS + 23.9 | interference |
|  | 2 | 3 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 10 | REFSENS | avoided |
|  | 3 | 3 | 5 | REFSENS | UL harmonic |
|  |  | n77 | 100 | REFSENS + 13.8 | interference |
|  | 4 | 3 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS | avoided |
|  | 5 | 3 | 5 | REFSENS + 5.7 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 6 | 3 | 20 | REFSENS + 2.3 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 7 | 3 | 5 | REFSENS | Harmonic mixing |
|  |  | n77 | 10 | REFSENS | avoided |
|  | 8 | 3 | 20 | REFSENS | Harmonic mixing |
|  |  | n77 | 10 | REFSENS | avoided |
|  | 9 | 3 | 5 | REFSENS + 26 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 10 | 3 | 5 | REFSENS + 8 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_3A_n78A | 1 | 3 | 5 | REFSENS | UL harmonic |
|  |  | n78 | 10 | REFSENS + 23.9 | interference |
|  | 2 | 3 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 10 | REFSENS | avoided |
|  | 3 | 3 | 5 | REFSENS | UL harmonic |
|  |  | n78 | 100 | REFSENS + 13.8 | interference |
|  | 4 | 3 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS | avoided |
|  | 5 | 3 | 5 | REFSENS + 5.7 | Harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
|  | 6 | 3 | 20 | REFSENS + 2.3 | Harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
|  | 7 | 3 | 5 | REFSENS | Harmonic mixing |
|  |  | n78 | 10 | REFSENS | avoided |
|  | 8 | 3 | 20 | REFSENS | Harmonic mixing |
|  |  | n78 | 10 | REFSENS | avoided |
|  | 9 | 3 | 5 | REFSENS + 26 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
|  | 10 | 3 | 5 | REFSENS + 8 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_5A_n66A | 1 | 5 | 5 | REFSENS + 30 | Dual uplink operation |
|  |  | n66 | 5 | REFSENS |  |
| DC_5A_n77A | 1 | 5 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS+ 2.9 |  |
|  | 2 | 5 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS + 2.9 |  |
|  | 3 | 5 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS | avoided |
|  | 4 | 5 | 5 | REFSENS + 8.3 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 5 | 5 | 5 | REFSENS + 5.5 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_5A_n78A | 1 | 5 | 5 | REFSENS | UL harmonic |
|  |  | n78 | 100 | REFSENS + 2.9 | interference |
|  | 2 | 5 | 5 | REFSENS |  |
|  |  | n78 | 10 | REFSENS + 10.5 |  |
|  | 3 | 5 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS | avoided |
|  | 4 | 5 | 5 | REFSENS + 8.3 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
|  | 5 | 5 | 5 | REFSENS + 5.7 | Harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
|  | 6 | 5 | 5 | REFSENS | Harmonic mixing |
|  |  | n78 | 10 | REFSENS | avoiding |
| DC_7A_n3A | 1 | 7 | 10 | REFSENS + 13 | Dual uplink operation |
|  |  | n3 | 5 | REFSENS |  |
| DC_7A_n5A | 1 | 7 | 10 | REFSENS | Dual uplink operation |
|  |  | n5 | 5 | REFSENS + 12 |  |
| DC_7A_n66A | 1 | 7 | 10 | REFSENS + 15 | Dual uplink operation |
|  |  | n66 | 5 | REFSENS |  |
| DC_7A_n71A | 1 | 7 | 5 | REFSENS+ 9 | UL harmonic exception |
|  |  | n71 | 5 | REFSENS |  |
|  | 2 | 7 | 5 | REFSENS | UL harmonic exception |
|  |  | n71 | 5 | REFSENS | avoided |
| DC_7A_n77A | 1 | 7 | 5 | REFSENS + 7.1 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_7A_n78A | 1 | 7 | 5 | REFSENS + 4.5 | Cross band isolation |
|  |  | n78 | 100 | REFSENS |  |
| DC_8A_n1A | 1 | 8 | 5 | REFSENS | Dual uplink operation |
|  |  | n1 | 5 | REFSENS + 6 |  |
| DC_8A_n3A | 1 | 8 | 5 | REFSENS + 8 | Dual uplink operation |
|  |  | n3 | 10 | REFSENS |  |
|  | 2 | 8 | 5 | REFSENS | Dual uplink operation |
|  |  | n3 | 10 | REFSENS + 6.4 |  |
| DC_8A_n20A | 1 | 8 | 5 | REFSENS | Dual uplink operation |
|  |  | n20 | 5 | REFSENS + 25 |  |
|  | 2 | 8 | 5 | REFSENS + 25 | Dual uplink operation |
|  |  | n20 | 5 | REFSENS |  |
| DC_8A_n41A | 1 | 8 | 5 | REFSENS | UL harmonic exception |
|  |  | n41 | 10 | REFSENS + 10 |  |
|  | 2 | 8 | 5 | REFSENS | UL harmonic exception |
|  |  | n41 | 10 | REFSENS | avoided |
|  | 3 | 8 | 5 | REFSENS + 12.1 | Dual uplink operation |
|  |  | n41 | 10 | REFSENS |  |
| DC_8A_n77A | 1 | 8 | 5 | REFSENS | UL harmonic |
| DC_8A_n78A |  | n77/n78 | 10 | REFSENS + 10.8 | interference |
|  | 2 | 8 | 5 | REFSENS | UL harmonic |
|  |  | n77/n78 | 100 | REFSENS + 3.1 | interference |
|  | 3 | 8 | 5 | REFSENS | UL harmonic exception |
|  |  | n77/n78 | 100 | REFSENS | avoided |
|  | 4 | 8 | 5 | REFSENS + 8.3 | Dual uplink operation |
|  |  | n77/n78 | 10 | REFSENS |  |
| DC_11A_n79A | 1 | 11 | 10 | REFSENS+ 36.3 | Harmonic mixing |
|  |  | n79 | 10 | REFSENS |  |
|  | 2 | 11 | 10 | REFSENS | Harmonic mixing |
|  |  | n79 | 10 | REFSENS | avoided |
| DC_12A_n66A | 1 | 12 | 5 | REFSENS | UL harmonic exception |
|  |  | n66 | 5 | REFSENS + 10 |  |
|  | 2 | 12 | 5 | REFSENS | UL harmonic exception |
|  |  | n66 | 40 | REFSENS + 3.1 |  |
|  | 3 | 12 | 5 | REFSENS | UL harmonic exception |
|  |  | n66 | 40 | REFSENS | avoided |
| DC_12A_n77A | 1 | 12 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS + 2.9 |  |
|  | 2 | 12 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS | avoided |
|  | 3 | 12 | 10 | REFSENS + 28 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 4 | 12 | 10 | REFSENS | Harmonic mixing |
|  |  | n77 | 10 | REFSENS | avoided |
|  | 5 | 12 | 5 | REFSENS + 5.5 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_12A_n78A | 1 | 12 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS + 2.9 |  |
|  | 2 | 12 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS | avoided |
|  | 3 | 12 | 5 | REFSENS + 5.5 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_13A_n77A | 1 | 13 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS + 2.9 |  |
|  | 2 | 13 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS | avoided |
|  | 3 | 13 | 10 | REFSENS + 28 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 4 | 13 | 10 | REFSENS | Harmonic mixing |
|  |  | n77 | 10 | REFSENS | avoided |
|  | 5 | 13 | 5 | REFSENS + 5.5 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_14A_n77A | 1 | 14 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS + 2.9 |  |
|  | 2 | 14 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS | avoided |
|  | 3 | 14 | 10 | REFSENS + 28 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 4 | 14 | 10 | REFSENS | Harmonic mixing |
|  |  | n77 | 10 | REFSENS | avoided |
|  | 5 | 14 | 5 | REFSENS + 5.5 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_18A_n77A | 1 | 18 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS + 2.9 |  |
|  | 2 | 18 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS | avoided |
|  | 3 | 18 | 5 | REFSENS + 8.4 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 4 | 18 | 5 | REFSENS + 4.5 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_18A_n78A | 1 | 18 | 5 | REFSENS + 8.4 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_19A_n77A | 1 | 19 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS+ 2.9 |  |
|  | 2 | 19 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS | avoided |
|  | 3 | 19 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 10 | REFSENS+ 10.5 |  |
| DC_19A_n78A | 1 | 19 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 10 | REFSENS+ 10.5 |  |
|  | 2 | 19 | 5 | REFSENS + 13.6 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
|  | 3 | 19 | 5 | REFSENS + 7.3 | Harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
| DC_19A_n79A | 1 | 19 | 5 | REFSENS + 29.5 | Harmonic mixing |
|  |  | n79 | 10 | REFSENS |  |
|  | 2 | 19 | 5 | REFSENS | Harmonic mixing |
|  |  | n79 | 10 | REFSENS | avoided |
| DC_20A_n3A | 1 | 20 | 5 | REFSENS | Dual uplink operation |
|  |  | n3 | 5 | REFSENS + 4 |  |
|  | 2 | 20 | 5 | REFSENS + 9 | Dual uplink operation |
|  |  | n3 | 5 | REFSENS |  |
| DC_20A_n7A | 1 | 20 | 5 | REFSENS + 12 | Dual uplink operation |
|  |  | n7 | 10 | REFSENS |  |
| DC_20A_n8A | 1 | 20 | 5 | REFSENS + 25 | Dual uplink operation |
|  |  | n8 | 5 | REFSENS |  |
|  | 2 | 20 | 5 | REFSENS | Dual uplink operation |
|  |  | n8 | 5 | REFSENS + 25 |  |
| DC_20A_n78A | 1 | 20 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS | avoided |
|  | 2 | 20 | 5 | REFSENS | UL harmonic |
|  |  | n78 | 10 | REFSENS + 10.8 | interference |
|  | 3 | 20 | 5 | REFSENS | UL harmonic |
|  |  | n78 | 100 | REFSENS + 3.1 | interference |
|  | 4 | 20 | 5 | REFSENS + 11 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_21A_n28A | 1 | 21 | 5 | REFSENS + 2.5 | Dual uplink operation |
|  |  | n28 | 5 | REFSENS |  |
| DC_21A_n79A | 1 | 21 | 15 | REFSENS + 34.5 | Harmonic mixing |
|  |  | n79 | 10 | REFSENS |  |
|  | 2 | 21 | 15 | REFSENS | Harmonic mixing |
|  |  | n79 | 10 | REFSENS | avoided |
|  | 3 | 21 | 5 | REFSENS + 18.4 | Dual uplink operation |
|  |  | n79 | 40 | REFSENS |  |
| DC_25A_n41A | 1 | 25 | 5 | REFSENS + 0.6 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_26A_n41A | 1 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n41 | 100 | REFSENS + 2.7 |  |
|  | 2 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n41 | 10 | REFSENS + 10.3 |  |
|  | 3 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n41 | 10 | REFSENS | avoided |
|  | 4 | 26 | 5 | REFSENS + 15.6 | Dual uplink operation |
|  |  | n41 | 10 | REFSENS |  |
|  | 5 | 26 | 5 | REFSENS + 23.8 | Harmonic mixing |
|  |  | n41 | 10 | REFSENS |  |
|  | 6 | 26 | 15 | REFSENS + 19 | Harmonic mixing |
|  |  | n41 | 10 | REFSENS |  |
|  | 7 | 26 | 15 | REFSENS | harmonic mixing |
|  |  | n41 | 10 | REFSENS | avoiding |
| DC_28A_n40A | 1 | 28 | 5 | REFSENS + 37.8 | Harmonic mixing |
|  |  | n40 | 10 | REFSENS |  |
|  | 2 | 28 | 20 | REFSENS + 30.3 | Harmonic mixing |
|  |  | n40 | 10 | REFSENS |  |
|  | 3 | 28 | 5 | REFSENS | Harmonic mixing |
|  |  | n40 | 10 | REFSENS | avoiding |
| DC_28A_n71A | 1 | 28 | 20 | REFSENS | Cross band isolation |
|  |  | n71 | 5 | REFSENS + 5.7 |  |
|  | 2 | 28 | 5 | REFSENS + 6.5 | Cross band isolation |
|  |  | n71 | 20 | REFSENS |  |
| DC_26A_n77A | 1 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS + 3.1 |  |
|  | 2 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS | avoided |
|  | 3 | 26 | 5 | REFSENS + 11.1 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_26A_n78A | 1 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS + 3 |  |
|  | 2 | 26 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS | avoided |
|  | 3 | 26 | 5 | REFSENS + 11.1 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_26A_n79A | 1 | 26 | 15 | REFSENS + 22.2 | Harmonic mixing |
|  |  | n79 | 10 | REFSENS |  |
|  | 2 | 26 | 15 | REFSENS | Harmonic mixing |
|  |  | n79 | 10 | REFSENS | avoiding |
| DC_28A_n5A | 1 | 28 | 5 | REFSENS + 17.5 | Cross band isolation |
|  |  | n5 | 20 | REFSENS |  |
| DC_28A_n77A | 1 | 28 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS + 2.9 |  |
|  | 2 | 28 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS | avoided |
|  | 3 | 28 | 5 | REFSENS + 5.5 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 4 | 28 | 5 | REFSENS + 31 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
| DC_28A_n78A | 1 | 28 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS + 2.9 |  |
|  | 2 | 28 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS | avoided |
|  | 3 | 28 | 5 | REFSENS + 5.5 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_30A_n66A | 1 | 30 | 10 | REFSENS | Cross band isolation |
|  |  | n66 | 5 | REFSENS + 8.3 |  |
| DC_30A_n77A | 1 | 30 | 5 | REFSENS + 8 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_38A_n78A | 1 | 38 | 5 | REFSENS + 3.3 | Cross band isolation |
|  |  | n78 | 100 | REFSENS |  |
| DC_39A_n41A | 1 | 39 | 20 | REFSENS | Cross band isolation |
|  |  | n41 | 10 | REFSENS + 3.3 |  |
|  | 2 | 39 | 5 | REFSENS + 1.6 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_40A_n41A | 1 | 40 | 5 | REFSENS + 31.4 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_40A_n78A | 1 | 40 | 20 | REFSENS + 9.1 | Harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
|  | 2 | 40 | 20 | REFSENS | Harmonic mixing |
|  |  | n78 | 10 | REFSENS | avoiding |
| DC_41A_n77A | 1 | 41 | 5 | REFSENS + 14.7 | Harmonic mixing |
| DC_41A_n78A |  | n77/n78 | 10 | REFSENS |  |
|  | 2 | 41 | 20 | REFSENS + 9.1 | Harmonic mixing |
|  |  | n77/n78 | 10 | REFSENS |  |
|  | 3 | 41 | 5 | REFSENS + 4.5 | Cross band isolation |
|  |  | n77/n78 | 100 | REFSENS |  |
|  | 4 | 41 | 20 | REFSENS | Cross band isolation |
|  |  | n77/n78 | 10 | REFSENS + 8.3 |  |
| DC_48A_n66A | 1 | 48 | 5 | REFSENS + 27.1 | UL harmonic exception |
|  |  | n66 | 5 | REFSENS |  |
|  | 2 | 48 | 20 | REFSENS + 21.2 | UL harmonic exception |
|  |  | n66 | 5 | REFSENS |  |
|  | 3 | 48 | 20 | REFSENS | UL harmonic exception |
|  |  | n66 | 5 | REFSENS | avoided |
|  | 4 | 48 | 20 | REFSENS | Dual uplink operation |
|  |  | n66 | 5 | REFSENS + 4 |  |
| DC_66A_n2A | 1 | 66 | 5 | REFSENS | Dual uplink operation |
|  |  | n2 | 5 | REFSENS + 20 |  |
|  | 2 | 66 | 5 | REFSENS + 4 | Dual uplink operation |
|  |  | n2 | 5 | REFSENS |  |
| DC_66A_n5A | 1 | 66 | 5 | REFSENS | Dual uplink operation |
|  |  | n5 | 5 | REFSENS + 30 |  |
| DC_66A_n25A | 1 | 66 | 5 | REFSENS | Dual uplink operation |
|  |  | n25 | 5 | REFSENS + 20 |  |
|  | 2 | 66 | 5 | REFSENS + 23 | Dual uplink operation |
|  |  | n25 | 5 | REFSENS |  |
|  | 3 | 66 | 5 | REFSENS + 4 | Dual uplink operation |
|  |  | n25 | 5 | REFSENS |  |
| DC_66A_n41A | 1 | 66 | 5 | REFSENS + 3.5 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_66A_n71A | 1 | 66 | 5 | REFSENS + 5 | Dual uplink operation |
|  |  | n71 | 5 | REFSENS |  |
| DC_66A_n77A | 1 | 66 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS + 13.8 |  |
|  | 2 | 66 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 10 | REFSENS + 23.9 |  |
|  | 3 | 66 | 5 | REFSENS | UL harmonic exception |
|  |  | n77 | 100 | REFSENS | avoided |
|  | 4 | 66 | 5 | REFSENS + 31 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 5 | 66 | 5 | REFSENS + 5 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_66A_n78A | 1 | 66 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 100 | REFSENS + 13.8 |  |
|  | 2 | 66 | 5 | REFSENS | UL harmonic exception |
|  |  | n78 | 10 | REFSENS + 23.9 |  |
|  | 3 | 66 | 5 | REFSENS + 5 | dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_71A_n2A | 1 | 71 | 5 | REFSENS | UL harmonic |
|  |  | n2 | 5 | REFSENS + 1.7 | interference |
|  | 2 | 71 | 5 | REFSENS + 26.8 | Harmonic mixing |
|  |  | n2 | 5 | REFSENS |  |
|  | 3 | 71 | 20 | REFSENS + 17.1 | Harmonic mixing |
|  |  | n2 | 5 | REFSENS |  |
|  | 4 | 71 | 20 | REFSENS | Harmonic mixing |
|  |  | n2 | 5 | REFSENS | avoiding |
| DC_71A_n66A | 1 | 71 | 5 | REFSENS | Dual uplink operation |
|  |  | n66 | 5 | REFSENS + 5 |  |
| NOTE 1: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1a and Table 7.3.2.5-1b of TS 38.521-1 [8] for 2 antenna port NR band, Table 7.3.2.5-2a and Table 7.3.2.5-2b of TS 38.521-1 [8] for 4 antenna port NR band. |  |  |  |  |  |

Table 7.3B.2.3.5-0a: Reference sensitivity test requirement for PC2 EN-DC

| EN-DC configuration | Test ID | E-UTRA/NR band | UL/DL CBW (MHz) | REFSENS test requirement (dBm) | Exception type |
| --- | --- | --- | --- | --- | --- |
| DC_1A_n41A | 2 | 1 | 5 | REFSENS + 12.4 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_1A_n77A | 3 | 1 | 5 | REFSENS + 35.8 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 4 | 1 | 5 | REFSENS + 17.8 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_1A_n78A | 1 | 1 | 5 | REFSENS + 17.8 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_2A_n41A | 1 | 2 | 5 | REFSENS + 1.6 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_2A_n77A | 4 | 2 | 20 | REFSENS + 4.5 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 5 | 2 | 20 | REFSENS | Harmonic mixing |
|  |  | n77 | 10 | REFSENS | avoiding |
|  | 6 | 2 | 5 | REFSENS + 32.1 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 7 | 2 | 5 | REFSENS + 19.1 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 9 | 2 | 5 | REFSENS + 1 | Cross band isolation |
|  |  | n77 | 100 | REFSENS |  |
| DC_3A_n41A | 1 | 3 | 20 | REFSENS | Cross band isolation |
|  |  | n41 | 10 | REFSENS + 0.7 |  |
|  | 2 | 3 | 5 | REFSENS + 2.3 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
|  | 3 | 3 | 5 | REFSENS + 18.4 | Dual uplink operation |
|  |  | n41 | 10 | REFSENS |  |
| DC_3A_n77A | 5 | 3 | 5 | REFSENS + 8.1 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 6 | 3 | 20 | REFSENS + 3.8 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 7 | 3 | 5 | REFSENS | Harmonic mixing |
|  |  | n77 | 10 | REFSENS | avoiding |
|  | 8 | 3 | 20 | REFSENS | Harmonic mixing |
|  |  | n77 | 10 | REFSENS | avoiding |
|  | 9 | 3 | 5 | REFSENS + 31.9 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 10 | 3 | 5 | REFSENS + 18.5 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_3A_n78A | 9 | 3 | 5 | REFSENS + 31.9 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
|  | 10 | 3 | 5 | REFSENS + 18.5 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
| DC_5A_n77A | 4 | 5 | 5 | REFSENS + 18.6 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_7A_n78A | 1 | 7 | 5 | REFSENS + 6.5 | Cross band isolation |
|  |  | n78 | 100 | REFSENS |  |
| DC_12A_n77A | 3 | 12 | 10 | REFSENS + 31 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 4 | 12 | 10 | REFSENS | Harmonic mixing |
|  |  | n77 | 10 | REFSENS | avoiding |
|  | 5 | 12 | 5 | REFSENS + 11.7 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_13A_n77A | 3 | 13 | 10 | REFSENS + 31 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 4 | 13 | 10 | REFSENS | Harmonic mixing |
|  |  | n77 | 10 | REFSENS | avoiding |
| DC_14A_n77A | 3 | 14 | 10 | REFSENS + 31 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 4 | 14 | 10 | REFSENS | Harmonic mixing |
|  |  | n77 | 10 | REFSENS | avoiding |
|  | 5 | 14 | 5 | REFSENS + 11.7 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_18A_n77A | 3 | 18 | 5 | REFSENS + 17.5 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 4 | 18 | 5 | REFSENS + 10.5 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_19A_n78A | 2 | 19 | 5 | REFSENS + 25.3 | Dual uplink operation |
|  |  | n78 | 10 | REFSENS |  |
|  | 3 | 19 | 5 | REFSENS + 9.8 | Harmonic mixing |
|  |  | n78 | 10 | REFSENS |  |
| DC_19A_n79A | 1 | 19 | 5 | REFSENS + 32.5 | Harmonic mixing |
|  |  | n79 | 10 | REFSENS |  |
|  | 2 | 19 | 5 | REFSENS | Harmonic mixing |
|  |  | n79 | 10 | REFSENS | avoiding |
| DC_21A_n79A | 1 | 21 | 15 | REFSENS + 37.5 | Harmonic mixing |
|  |  | n79 | 10 | REFSENS |  |
|  | 2 | 21 | 15 | REFSENS | Harmonic mixing |
|  |  | n79 | 10 | REFSENS | avoiding |
|  | 4 | 21 | 5 | REFSENS + 33.4 | Dual uplink operation |
|  |  | n79 | 10 | REFSENS |  |
| DC_28A_n77A | 3 | 28 | 5 | REFSENS + 19.2 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 4 | 28 | 5 | REFSENS + 34 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
| DC_30A_n77A | 1 | 30 | 5 | REFSENS + 17.6 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
| DC_41A_n77A | 1 | 41 | 5 | REFSENS + 17.7 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 2 | 41 | 20 | REFSENS + 11.9 | Harmonic mixing |
|  |  | n77 | 10 | REFSENS |  |
|  | 3 | 41 | 5 | REFSENS + 6.5 | Cross band isolation |
|  |  | n77 | 100 | REFSENS |  |
| DC_66A_n41A | 1 | 66 | 5 | REFSENS + 12.4 | Cross band isolation |
|  |  | n41 | 100 | REFSENS |  |
| DC_66A_n77A | 4 | 66 | 5 | REFSENS + 34.33 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 5 | 66 | 5 | REFSENS + 11.27 | Dual uplink operation |
|  |  | n77 | 10 | REFSENS |  |
|  | 6 | 66 | 5 | REFSENS + 1 | Cross band isolation |
|  |  | n77 | 100 | REFSENS |  |
| NOTE 1: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1a and Table 7.3.2.5-1b of TS 38.521-1 [8] for 2 antenna port NR band, Table 7.3.2.5-2a and Table 7.3.2.5-2b of TS 38.521-1 [8] for 4 antenna port NR band, Table 7.3.2.5-2c and Table 7.3.2.5-2d of TS 38.521-1 [8] for PC2 UE on NR FDD bands. |  |  |  |  |  |

==============================================================

<< End of changes >>