3GPP TSG-WG5 Meeting #109	R5-256376r1

# Dallas, USA, Nov 17 – 21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3668 | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding test frequency for CA_n78D |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The configuration of CA_n78D is introduced into TS 38.101-1 but the test frequencies are still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the test frequency information CA_n78D |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test frequency information remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.3.78.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Correcting the title of Table 4.3.1.1.3.78.3-1 |  |  |  |  |  |  |  |  |

==============First change==============

4.3.1.1.3.78.2	CA_n78B

Editor's note: Test frequencies for CA_n78B with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.3.78.2-1: NR Intra-Band contiguous CA configuration CA_n78B (PCC=CC1 and SCC=CC2), SCS 15 kHz and ΔFRaster 15 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20+50 | CC1 | 20 | 15 | 106 | Downlink | Low | 3310.02 | 620668 | 3300.48 | 620032 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 1 (6) | 6 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3497.1 | 633140 | 102 |  |  | 7860 | 634656 |  | 4 |  | 2 | 0 (2) | 106 |
|  |  |  |  |  | Uplink | High | 3740.25 | 649350 | 3639.99 | 642666 | 504 |  |  | 8010 | 649056 |  | 6 |  | 2 | 1 (6) | 512 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 3344.76 | 622984 | 3320.46 | 621364 | 0 | 30 |  | 7725 | 621696 |  | 8 |  | 1 | 1 (6) | 7 |
|  |  |  |  |  | & | Mid | 3559.74 | 637316 | 3517.08 | 634472 | 102 |  |  | 7874 | 636000 |  | 4 |  | 3 | 0 (2) | 107 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3659.97 | 643998 | 504 |  |  | 8024 | 650400 |  | 6 |  | 3 | 1 (6) | 513 |
| 50+20 | CC1 | 50 | 15 | 270 | Downlink | Low | 3325.020 | 621668 | 3300.72 | 620048 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 3 | 0 (2) | 5 |
|  |  |  |  |  | & | Mid | 3540 | 636000 | 3497.34 | 633156 | 102 |  |  | 7860 | 634656 |  | 0 |  | 1 | 0 (2) | 105 |
|  |  |  |  |  | Uplink | High | 3755.25 | 650350 | 3640.23 | 642682 | 504 |  |  | 8010 | 649056 |  | 2 |  | 1 | 1 (6) | 511 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3359.76 | 623984 | 3350.22 | 623348 | 0 | 30 |  | 7745 | 623616 |  | 4 |  | 0 | 0 (2) | 2 |
|  |  |  |  |  | & | Mid | 3574.74 | 638316 | 3546.84 | 636456 | 102 |  |  | 7895 | 638016 |  | 0 |  | 2 | 1 (6) | 110 |
|  |  |  |  |  | Uplink | High | 3789.99 | 652666 | 3689.73 | 645982 | 504 |  |  | 8044 | 652320 |  | 2 |  | 2 | 0 (2) | 508 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.3.78.2-2: NR Intra-Band contiguous CA configuration CA_n78B (PCC=CC1 and SCC=CC2), SCS 30 kHz and ΔFRaster 30 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20+50 | CC1 | 20 | 30 | 51 | Downlink | Low | 3310.02 | 620668 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3479.1 | 631940 | 102 |  |  | 7860 | 634656 |  | 4 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3740.25 | 649350 | 3549.63 | 636642 | 504 |  |  | 8010 | 649056 |  | 6 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3344.76 | 622984 | 3320.82 | 621388 | 0 | 30 |  | 7725 | 621696 |  | 20 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3559.74 | 637316 | 3499.08 | 633272 | 102 |  |  | 7874 | 636000 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 |  | 0 | 3 (3) | 1014 |
| 50+20 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3540 | 636000 | 3479.34 | 631956 | 102 |  |  | 7860 | 634656 |  | 12 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3755.25 | 650350 | 3549.87 | 636658 | 504 |  |  | 8010 | 649056 |  | 14 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 3359.76 | 623984 | 3350.58 | 623372 | 0 | 30 |  | 7745 | 623616 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3574.74 | 638316 | 3528.84 | 635256 | 102 |  |  | 7895 | 638016 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3789.99 | 652666 | 3599.37 | 639958 | 504 |  |  | 8044 | 652320 |  | 2 |  | 0 | 1 (1) | 1010 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.3.78.3	CA_n78D

Editor's note: Test frequencies for CA_n78D with mixed numerology with SCS 15kHz, SCS 30 kHz or SCS 60kHz is FFS.

Table 4.3.1.1.3.78.3-1: NR Intra-Band contiguous CA configuration CA_n78D (PCC=CC1, SCC = CC2 and CC3), SCS 30 kHz and ΔFRaster 30 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100+100+100 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 |  | 6 | 0 | 2 | 4 |
|  |  |  |  |  | & | Mid | 3450 | 630000 | 3364.14 | 624276 | 102 | 30 | 7780 | 626976 |  | 12 | 0 | 0 | 204 |
|  |  |  |  |  | Uplink | High | 3550.08 | 636672 | 3319.5 | 621300 | 504 | 30 | 7850 | 633696 |  | 12 | 0 | 2 | 1012 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3449.97 | 629998 | 3400.83 | 626722 | 0 | 30 | 7780 | 626976 |  | 14 | 0 | 0 | 0 |
|  |  |  |  |  | & | Mid | 3549.96 | 636664 | 3464.1 | 630940 | 102 | 30 | 7850 | 633696 |  | 20 | 0 | 2 | 208 |
|  |  |  |  |  | Uplink | High | 3650.04 | 643336 | 3419.46 | 627964 | 504 | 30 | 7919 | 640320 |  | 20 | 0 | 0 | 1008 |
|  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 100 | 30 | 273 | Downlink | Low | 3549.93 | 636662 | 3500.79 | 633386 | 0 | 30 | 7850 | 633696 |  | 22 | 0 | 2 | 4 |
|  |  |  |  |  | & | Mid | 3649.92 | 643328 | 3564.06 | 637604 | 102 | 30 | 7919 | 640320 |  | 4 | 0 | 1 | 206 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 | 30 | 7989 | 647040 |  | 4 | 0 | 3 | 1014 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1, CC2 and CC3 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.3.79	NR Intra-band contiguous configurations CA_n79

4.3.1.1.3.79.1	CA_n79C

==============End of change==============