3GPP TSG-RAN5 Meeting #109	R5-256407r1

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5226 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to testcase 7.1.1.3.21 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Testcase 7.1.1.3.21 needs few clarifications and fix to indexing issue in schedulingRequestResourceId brought into attention during TF160 implementation phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Clarified when part text in TP1 2) Clarified check procedure in step 9 3) Indexing issue fixed in schedulingRequestResourceId |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase can be misunderstood and not implementable as ASN for schedulingRequestResourceId does not allow current indexing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Changed the order of logical channels as per comment from TF160. |  |  |  |  |  |  |  |  |

==============First change==============

##### 7.1.1.3.21	Correct handling of MAC control information / Delay Status / Triggering scheduling request

7.1.1.3.21.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE determines that a new DSR has been triggered for a logical channel and that no scheduling request is currently pending for that DSR and UE has no UL resources allocated for new transmission for this TTI }

then { UE transmits a scheduling request }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE have a pending DSR and all the SDUs associated with the DSR have been discarded when PDCP discardTimer expire }

then { UE cancel the pending DSR }

}

7.1.1.3.21.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.4.9. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.4.9]

The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the running PDCP discardTimers among PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU as specified in clause 7.3 in TS 38.323 [4], and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively.

RRC controls the DSR procedure by configuring the following parameter:

-	remainingTimeThreshold (per LCG): the threshold on remaining time for triggering a DSR for a logical channel within an LCG.

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG:

1>	if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and

1>	if there is no DSR pending for the logical channel:

2>	trigger a DSR for the logical channel.

If there is at least one DSR pending, the MAC entity shall:

…

1>	else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR:

2>	trigger a Scheduling Request.

NOTE 1:	The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same criteria specified in clause 5.4.5.

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR.

A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs.

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

NOTE 2:	It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader.

7.1.1.3.21.3	Test description

7.1.1.3.21.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 with the exception of 2 AM DRBs on NR cell configured according to Table 7.1.1.3.21.3.1-1.

Table 7.1.1.3.21.3.1-1: Logical Channel Configuration Settings

| Parameter | DRB1 | DRB2 |
| --- | --- | --- |
| LogicalChannelIdentity | LCH4(DRB-Identity +3) | LCH5(DRB-Identity +3) |
| Priority | 7 | 6 |
| prioritizedBitRate | 0kbs | 0kbs |
| logicalChannelGroup | 2 (LCG ID#2) | 1 (LCG ID#1) |
| discardTimer | 100ms | 100ms |
| remainingTimeThreshold | 64ms | 32ms |
| schedulingRequestID | 0 | 1 |

7.1.1.3.21.3.2	Test procedure sequence

Table 7.1.1.3.21.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS ignores scheduling requests and does not allocate any uplink grant. |  |  |  |  |
| 2 | The SS transmits a MAC PDU carrying two RLC PDUs, one including PDCP SDU of 32 bytes for LCG#2 (LCH4) and another including PDCP SDU of 16 bytes for LCG#1 (LCH5). | <-- | MAC PDU (RLC PDU on LCH4, RLC PDU on LCH5) | - | - |
| 3 | Wait for 30 ms after step 2. | - | - | - | - |
| 4 | Check: Does the UE transmit a Scheduling Request on following PUCCH occasion(s) only for LCGi = 2? (Note 1) | --> | (SR) | 1 | P |
| 5 | Wait for 30 ms after step 4, | - | - | - | - |
| 6 | Check: Does the UE transmit a Scheduling Request on following PUCCH occasion(s) for LCGi = 1? (Note1) | --> | (SR) | 1 | P |
| 7 | Wait for 50 ms after step 6 for discardTimers to expire for LCH4 and LCH5. | - | - | - | - |
| 8 | The SS sends an uplink grant of size 64 bits. | <-- | (UL grant (C-RNTI)) | - | - |
| 9 | Check: Does the UE transmit MAC PDU containing an DSR MAC CE for indicating remaining time and pending data (‘Buffer size’ field > ‘0’) for any LCGi? | --> | MAC PDU (DSR) | 2 | F |
| Note 1: Depending about the combined timing, waiting periods and delays, UE may miss the first possible occasion for SR. |  |  |  |  |  |

7.1.1.3.21.3.3	Specific message contents

Table 7.1.1.3.21.3.3-1: PUCCH-Config (Preamble)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-112 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-Config ::= SEQUENCE { |  |  |  |
| schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig { | 2 entries |  |  |
| SchedulingRequestResourceConfig[1] | SchedulingRequestResourceConfig1 | entry 1 |  |
| SchedulingRequestResourceConfig[2] | SchedulingRequestResourceConfig2 | entry 2 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.21.3.3-2: SchedulingRequestResourceConfig1 (Table 7.1.1.3.21.3.3-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestResourceId | 1 |  |  |
| schedulingRequestID | 0 |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl10 | 8 | With SCS = kHz15 results in repetition every 10 ms | SCS15 |
| sl20 | 8 | With SCS = kHz30 results in repetition every 10 ms | SCS30 |
| sl80 | 8 | With SCS = kHz120 results in repetition every 10 ms | SCS120 |
| } |  |  |  |
| resource | 0 | ID of the PUCCH resource as configured by PUCCH-Config (Table 4.6.3-112) |  |
| } |  |  |  |

Table 7.1.1.3.21.3.3-3: SchedulingRequestResourceConfig2 (Table 7.1.1.3.21.3.3-1)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestResourceId | 2 |  |  |
| schedulingRequestID | 1 |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl10 | 9 | With SCS = kHz15 results in repetition every 10 ms | SCS15 |
| sl20 | 9 | With SCS = kHz30 results in repetition every 10 ms | SCS30 |
| sl80 | 9 | With SCS = kHz120 results in repetition every 10 ms | SCS120 |
| } |  |  |  |
| resource | 0 | ID of the PUCCH resource as configured by PUCCH-Config (Table 4.6.3-112) |  |
| } |  |  |  |

Table 7.1.1.3.21.3.3-4: SchedulingRequestConfig (Preamble)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-155 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestToAddModList SEQUENCE (SIZE(1..maxNrofSR-ConfigPerCellGroup)) OF SSchedulingRequestToAddMod { | 2 entries |  |  |
| SchedulingRequestToAddMod[1] SEQUENCE { |  | entry 1 |  |
| schedulingRequestId | 0 |  |  |
| sr-ProhibitTimer | Not present |  |  |
| sr-TransMax | n16 |  |  |
| } |  |  |  |
| SchedulingRequestToAddMod[2] SEQUENCE { |  | entry 2 |  |
| schedulingRequestId | 1 |  |  |
| sr-ProhibitTimer | Not present |  |  |
| sr-TransMax | n16 |  |  |
| } |  |  |  |
| } |  |  |  |
| schedulingRequestToReleaseList | Not present |  |  |
| } |  |  |  |

==============End of change==============