3GPP TSG-RAN5 Meeting #109	R5-256411r1

# Dallas, USA, November 17 - 21 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4231 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR SA FR2 RACH-based Intra-frequency LTM PSCell switch test case 7.3.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | FR1 RACH-based intra-frequency LTM PSCell switch test case 6.3.5.1 is not complete |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case has been corrected with the following aspects: - Message content added - Applicability defined in 38.522 - Test case added to Table A.19.3-1 with the FR1-FR2 testability issue |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Typos in table numbers and in the message content exception tables corrected |  |  |  |  |  |  |  |  |

/// Start of changes

#### 7.3.5.1	NR SA FR2 RACH-based Intra-frequency LTM PSCell switch

Editor's Note: This test case is incomplete in following aspects:

TT analysis is missing.

FR1 – FR2 testability is FFS

7.3.5.1.1	Test purpose

To verify the UE's ability to perform NR FR1-NR FR1 RACH-based intra-frequency PSCell switch specified in 38.133 [6] clause 8.20 for both with and without early TCI state activation.

7.3.5.1.2	Test applicability

This test applies to all types of NR UEs supporting NR LTM for SCG with RACH from Release 18 onwards.

The test consists of 2 tests, and UE is required to pass one among Test 1, Test 2.

-     Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18

-     Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r1

7.3.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.3.5.0.

The normative reference for this requirement is TS 38.133 [6] clauses 8.20 and A.7.3.5.1.

7.3.5.1.4	Test description

7.3.5.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 7.3.5.1.4.1-1.

Table 7.3.5.1.4.1-1: NR SA FR2 RACH-based Intra-frequency LTM PSCell switch test configurations

| Config | Description |
| --- | --- |
| 7.3.5.1-1 | PCell: FR1 NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode Source PSCell: FR2 NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target PSCell: FR2 NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.3.5.1-2 | PCell: FR1 NR 15kHz SSB SCS, 10MHz bandwidth, TDD duplex mode Source PSCell: FR2 NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target PSCell: FR2 NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.3.5.1-3 | PCell: FR1 NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode Source PSCell: FR2 NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target PSCell: FR2 NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 7.3.5.1.4.1-2

Table 7.3.5.1.4.1-2: Initial conditions for NR SA FR2 RACH-based Intra-frequency LTM PSCell switch

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 7.3.5.1.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR2 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 7.3.5.1.4.1-3 below.

Table 7.3.5.1.4.1-3: General test parameters for NR SA FR2 RACH-based Intra-frequency LTM PSCell switch

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cell |  | Cell 1, Cell 2 |  |  |  |  |
|  | Neighbouring cell |  | Cell 3 |  |  |  | Cell 3 is the candidate cell |
| Final condition | Active cell |  | Cell 1, Cell 3 |  |  |  |  |
| A3-Offset |  | dB | -30 |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between Cell 2 and Cell 3 |  |  | 0.3 s |  |  |  | RTD between Cell 2 and Cell 3 is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 |  |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  |  | Report candidate cell’s (Cell 3) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DlorJoint TCI.State.0 | DLorJoint TCI.State.2 | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | As specified in clause A.3.16B. In test 1A and 1B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | CandidateTCI-State#2 |  | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | N/A | N/A |  |
| ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | UL TCI.State.0 |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.1 |  |  |  |  |
| T4 |  | s | 0.1 |  |  |  |  |

7.3.5.1.4.2	Test procedure

Three cells are deployed in the test, which are FR1 PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on the same frequency as the PSCell. Test configurations are given in table 6.3.4.3.4.1-1. Both cell switch delay and interruption length are tested by using the parameters in table 7.3.5.1.4.1-2, and 7.3.5.1.4.1-3.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 7.3.5.1.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	SS shall transmit an RRCReconfiguration message on Cell 2, configuring intra-frequency measurement object.

4.	UE shall transmit RRCReconfigurationComplete message.

5.	If the UE transmit an A3 message report including Cell 3, the SS shall switch the power setting from T1 to T2 as specified in Table 7.3.5.1.5-1. T2 starts.

6.	At the start of T2, the SS shall transmit an RRCReconfiguration message with LTM-Candidate-r18 for Cell 3 and SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 3) in PUCCH format 2.

7.	UE shall transmit RRCReconfigurationComplete message.

8.	If the UE transmits a valid L1-RSRP result of Cell 3:

Test 1: The SS shall switch the power setting from T2 to T3 as specified in Table 7.3.5.1.5-1. T3 starts.

Test 2: T3 is skipped. The SS shall switch the power setting from T2 to T4 as specified in Table 7.3.5.1.5-1. T4 starts, go to step10.

Otherwise, skip to step 13.

9.	At the start of T3, the SS shall transmit the candidate cell TCI activation MAC-CE (Refer TS 38.321 [12], clauses 6.1.3.76) for Cell 3. 50ms after the candidate cell TCI state activation MAC-CE transmission, the SS shall switch the power setting from T3 to T4 as specified in Table 7.3.5.1.5-1. T4 starts. Candidate cell TCI activation MAC-CE: CandidateTCI-State#1 is activated.

10.	T4 starts. The start of T4 is the instant when the last TTI containing the LTM cell switch command MAC CE(Refer TS 38.321 [12], clauses 6.1.3.75) is sent by Cell 1 to the UE.

cell switch MAC-CE:

-	Contention-Free Random-Access Resources are indicated and the field of Timing Advance Command is set to FFF

-	Test 1: CandidateTCI-State#2 is indicated.

-	Test 2: CandidateTCI-State#1 is indicated.

11.	If the UE transmits the uplink PRACH channel to Cell 3 less than DLTM from the beginning of time period T4 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

12.	After T4 expires, the SS sends an RRCReconfiguration with reconfigurationWithSync, which includes the release of LTM-Config-r18 to cause UE handover back to Cell 1.

13.	If UE is not in state RRC_CONNECTED with generic procedure parameters Connectivity NR, connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1, switch off and on the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1

14.	Power off Cell 3 and set Cell 3 physical cell identity = ((current Cell 3 physical cell identity + 3) mod1008) for next iteration of the test procedure loop.

15.	Repeat steps 2-14 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.3.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.3.5.1.4.3-1: Common Exception messages (step 3)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.3 FR2, SMTC.1 and Synchronous cells Table H.3.1-4 with A3-offset = -15dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ |

Table 7.3.5.1.4.3-2: Common Exception messages (step 6)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.12.1 with Condition LTM L1-RSRP Table H 3.12-2 with Condition LTM L1-RSRP Table H.3.12-3 for Cell 3 and Condition Candidate config and TCI_INFO Table H.3.12.4 Table H.3.12-5 with pucch-ResourceId = 8 Table H.3.12-6, H.3.12-7 and H.3.12-13 with SSB index = 0 Table H.3.12-8 Table H.3.12-9 for Cell 3 Table H.3.12-10 with Condition Joint AND Early DL Sync for Test 1A Table H.3.12-10 with Condition Seperate for Test 1B and Test 2B Table H.3.12-10 with Condition Joint for Test 2A Table H.3.12-11 |

Table 7.3.5.1.4.3-3: RACH-ConfigCommon for RACH-based Intra-frequency LTM PSCell switch

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric specified in TS 38.508-1 [14] Table 7.3.1-14 with condition FR2 |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| oneFourth | n16 |  |  |
| } |  |  |  |
| rsrp-ThresholdSSB | RSRP_51 |  |  |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| 0 |  |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | kHz 120 |  |  |
| } |  |  |  |

Table 7.3.5.1.4.3-4: RACH-ConfigDedicated for RACH-based Intra-frequency LTM PSCell switch

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF { | 1 entry |  |  |
| ssb[1] | 0 |  |  |
| ra-PreambleIndex[1] | 40 |  |  |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.3.5.1.5	Test requirements

Table 7.3.5.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.3.5.1.5-1: Cell specific test parameters for NR SA FR2 RACH-based Intra-frequency LTM PSCell switch

| Parameter |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- |
|  |  |  |  | T1~T4 |
| NR RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
| TDD configuration |  | Config 1 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 40: NRB,c = 106 |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 40: NRB,c = 106 |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| DRx Cycle |  |  | ms | Not Applicable |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |
|  |  | Config 3 |  | 30 |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |
|  |  | Config 3 |  | 30 |
| PRACH configuration |  |  |  | FR1 PRACH configuration 6 |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |
|  |  | Initial UL BWP |  | ULBWP.0.1 |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98+TT |
| Note2 | Config 1,2 |  | dBm/SCS | -98+TT |
|  | Config 3 |  |  |  |
|  |  |  | dB | 4+TT |
|  |  |  | dB | 4+TT |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94+TT |
|  | Config 3 |  |  | -91+TT |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59+TT |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49+TT |
| Propagation condition |  |  | - | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table 7.3.5.1.5-2: Cell specific test parameters for NR SA FR2 RACH-based Intra-frequency LTM PSCell switch

| Parameter |  |  | Unit | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 ~ T4 | T1 ~ T4 |
| NR RF Channel Number |  |  |  | 2 | 2 |
| Assumption for UE beamsNote 6 |  |  |  | Rough | Rough |
| AoA setup |  |  |  | Setup 1 as defined in clause A.3.15 |  |
| Duplex mode |  |  |  | TDD |  |
| TDD configuration |  |  |  | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |
| PDSCH Reference |  |  |  | SR3.1 TDD |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |
| CP length |  |  |  | Normal |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  |  |  | SSB.3 FR2 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 6 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7+TT |  |
| Note2 |  |  | dBm/SCS | -95.7+TT |  |
|  |  |  | dB | -1.8+TT | 0+TT |
|  |  |  | dB | 6+TT | 7+TT |
| SSB_RP |  |  | dBm/SCS | -88.7+TT | -89.7+TT |
| IoNote3 |  |  | dBm/ 95.04MHz | -56.7+TT | -56.7+TT |
| Propagation condition |  |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 3 in no later than DLTM from the beginning of time period T4.

The rate of correct cell switches observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as DLTM (= Tcmd + TLTM-interrupt), where:

Tcmd = THARQ + 3 ms and is specified in clause 6.3.1.2, TLTM-interrupt is defined in clause 8.20.3 as TLTM-interrupt  = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms.

-	Tfirst-RS + TRS-proc= 0 ms for Test 1A and 1B, Tfirst-RS + TRS-proc= 22 ms for Test 2A and 2B

-	TLTM-IU=20ms,

-	TLTM-RRC-processing = 10 ms if UE does not support ltm-FastProcessingConfig-r18, otherwise TLTM-RRC-processing =0ms

-	TLTM-processing = 10 ms if the UE supports ltm-FastUE-Processing-r18 capability and UE reports 10 ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing = 15 ms if the UE supports ltm-FastUE-Processing-r18] capability and UE reports 15 ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing = 20 ms if the UE does not support ltm-FastUE-Processing-r18 capability.

/// Unchaged sections omitted

## A.19.3	Principle of Testing in SA

For test cases in clause 7 listed in Table A.19.3-1, the following applies:

-	UE does not have to pass the test case.

Table A.19.3-1: Test cases with the E-UTRA/FR1 – FR2 testability issue (SA)

| Clause | Test case slogan |
| --- | --- |
| 7.3.1.1 | NR SA FR1-FR2 Inter-frequency handover; unknown target cell |
| 7.3.1.4 | NR SA FR1-FR2 synchronous DAPS handover |
| 7.3.1.5 | NR SA FR1-FR2 asynchronous DAPS handover |
| 7.3.5.1 | NR SA FR2 RACH-based Intra-frequency LTM PSCell switch |
| 7.5.3.2 | NR SA FR1-FR2 inter-band SCell activation and deactivation delay |
| 7.5.6.1.2 | NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX |
| 7.5.7.1 | NR SA FR2 addition and release delay of known PSCell |
| 7.5.7.2 | NR SA FR2 addition and release delay of unknown PSCell |
| 7.5.12.1 | NR SA FR2 Addition and Release Delay of PSCell |
| 7.5.14.1 | NR SA FR2 PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 |
| 7.6.2.5 | NR SA FR1-FR2 event-triggered reporting in non-DRX |
| 7.6.2.6 | NR SA FR1-FR2 event-triggered reporting in DRX |
| 7.6.2.7 | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection |
| 7.6.2.8 | NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used |
| 7.6.3.6 | NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used |
| 7.7.1.3 | NR SA FR1-FR2 SS-RSRP measurement accuracy |
| Note 1: The list of the test case is based on TS 38.133 [6] Table A.3.13A.3-1 and updated based on the RAN5 testability analysis. |  |

/// End of changes