3GPP TSG-RAN5 Meeting #109	R5-256453r1

# Dallas, Texas, USA, 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3612 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Rel-17 UL Tx switching test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_RF_FR1_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In test case 6.3A.3.3 the IEs uplinkTxSwitching-2T-Mode-r17 and uplinkTxSwitching-DualUL-TxState-r17 are missing in the message exceptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Missing IEs have been added to the message exceptions in Table 6.3A.3.3.4.3-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The message exception will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.3.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: removed updates to the relation between m and n due to overlap with Anritsu CR R5-256255. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 6.3A.3.3	Time mask for switching between two uplink carriers with two transmit antenna connectors

< Unchanged sections omitted >

6.3A.3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO, with the following exceptions.

Table 6.3A.3.3.4.3-1: CellGroupConfig

| Derivation Path: TS 38.508-1[5], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| uplinkTxSwitchingOption-r16 | switchedUL |  | switchedUL OR Both |
|  | dualUL |  | dualUL |
| uplinkTxSwitching-2T-Mode-r17 | enabled |  |  |
| uplinkTxSwitching-DualUL-TxState-r17 | twoT |  | dualUL |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| switchedUL | UE indicated supporting of switchedUL in uplinkTxSwitching-OptionSupport-r16 |
| dualUL | UE indicated supporting of dualUL in uplinkTxSwitching-OptionSupport-r16 |
| Both | UE indicated supporting of both in uplinkTxSwitching-OptionSupport-r16 |

Table 6.3A.3.3.4.3-2: ServingCellConfig

| Derivation Path: 38.508-1[5], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkTxSwitching-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| uplinkTxSwitchingPeriodLocation-r16 | TRUE |  | PL |
|  | FALSE |  | noPL |
| uplinkTxSwitchingCarrier-r16 | carrier1 |  | PCC |
|  | carrier2 |  | SCC |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PL | The location of UL Tx switching period is configured in this carrier |
| noPL | The location of UL Tx switching period is not configured in this carrier |

Table 6.3A.3.3.4.3-3: FrequencyInfoUL-SIB for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  |  |

Table 6.3A.3.3.4.3-4: FrequencyInfoUL for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  |  |

< End of changes >