3GPP TSG-RAN5 Meeting #109	R5-256558r1

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 1045 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MultiRx CQI test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Sweden |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MultiRx WP requires addition of 4Rx TDD FR2 PMI reporting test case using single DCI transmission scheme with simultaneous reception from 2 TRxP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the 4Rx TDD FR2 Single PMI with 2TX TypeI-SinglePanel codebook Single-DCI based transmission scheme with simultaneous reception test case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the work plan will remain unfulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.522 CR 0701 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: added Annex F MU/TT and updated test requirement gamma value taking into account the MU value |  |  |  |  |  |  |  |  |

8.3.2.2.1.5	Test requirement

Table 8.3.2.2.1.5-1: Test requirement (TDD)

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
|  | 1.04 | 1.04 |

==============First change==============

##### 8.3.3.2.2	4Rx TDD FR2 Single PMI with 2TX TypeI-SinglePanel codebook Single-DCI based transmission scheme with simultaneous reception - MultiRx

Editor’s note: This test case is incomplete. Following aspects are pending

-	device positioning procedure

-	minimum test time

-	message exceptions

8.3.3.2.2.1	Test purpose

The purpose of this test is to test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports when PDSCH transmission is done from 2 TRxPs using single DCI based transmission scheme.

8.3.3.2.2.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting simultaneousReceptionDiffTypeD-r16 and singleDCI-SDM-scheme-r16.

8.3.3.2.2.3	Minimum conformance requirements

For the parameters specified in Table 8.3.3.2.2.3-1 and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified in Table 8.3.3.2.2.3-2.

Table 8.3.3.2.2.3-1: Test parameters (dual-layer)

| Parameter |  |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) |  | TRxP #2(Note 1) |
| SSB |  |  |  |  |  | SSB#0 |  | SSB#1 |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #2 |  | N/A |
|  |  |  | CORESETPoolIndex |  |  | 0 |  | N/A |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 4 for CSI-RS resources 1 and 3 l0 = 8 for CSI-RS resources 2 and 4 |  | l0 = 4 for CSI-RS resources 5 and 7 l0 = 8 for CSI-RS resources 6 and 8 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  | 1 for CSI-RS resource 5,6,7,8 |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |  |
|  |  |  | Density |  |  | 3 |  |  |
|  |  |  | CSI-RS periodicity |  | Slots | 160 |  |  |
|  |  |  | CSI-RS offset |  | Slots | 80 for CSI-RS resources 1 and 2 81 for CSI-RS resources 3 and 4 |  | 80 for CSI-RS resources 5 and 6 81 for CSI-RS resources 7 and 8 |
|  |  |  | QCL info |  |  | TCI state #0 |  | TCI state #1 |
| CSI-RS for beam refinement |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1 and 2 |  | k0=1 for CSI-RS resources 3 and 4 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |  | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resources 1,2,3,4 |  |  |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resources 1,2,3,4 |  |  |
|  |  |  | Density |  |  | 3 |  |  |
|  |  |  | CSI-RS periodicity |  | Slots | 160 |  |  |
|  |  |  | CSI-RS offset |  | Slots | 0 for CSI-RS resources 1,2,3,4 |  |  |
|  |  |  | QCL info |  |  | TCI state #2 |  | TCI state #3 |
| Duplex mode |  |  |  |  |  | TDD |  |  |
| Bandwidth |  |  |  |  | MHz | 100 |  |  |
| Subcarrier spacing |  |  |  |  | kHz | 120 |  |  |
| TDD DL-UL configurations |  |  |  |  |  | FR1.120-1 as specified in Annex A.1.3 |  |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  |  | TDLA30-35 |  |  |
| Correlation matrix and antenna configuration (Note 6) |  |  |  |  |  | 4x4 FR2- mTRxP-mRX ρ = -12dB |  |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 4) |  |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |  |
|  | k0 |  |  |  |  | 0 |  |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |  |
|  | Length (L) |  |  |  |  | 12 |  |  |
|  | PRB bundling type |  |  |  |  | Static |  |  |
|  | PRB bundling size |  |  |  |  | 2 |  |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |  |
|  | RBG size |  |  |  |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000 |  | 1002 |
|  | TCI state |  |  |  |  | TCI State #0 |  | TCI State #1 |
|  | DMRS Type |  |  |  |  | Type 1 |  |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |  |
| PTRS configuration (Note 5) | Frequency density (KPT-RS) |  |  |  |  | 2 |  |  |
|  | Time density (LPT-RS) |  |  |  |  | 1 |  |  |
|  | Resource Element Offset |  |  |  |  | ‘offset10’ |  |  |
|  | Max number of ports |  |  |  |  | n1 for Test 1 n2 for Test 2 |  |  |
| TCI State #0 | Type 1 QCL information |  |  | SSB index |  | SSB #0 |  | N/A |
|  |  |  |  | QCL Type |  | Type C |  | N/A |
|  | Type 2 QCL information |  |  | SSB index |  | SSB #0 |  | N/A |
|  |  |  |  | QCL Type |  | Type D |  | N/A |
| TCI State #1 | Type 1 QCL information |  |  | SSB index |  | N/A |  | SSB #1 |
|  |  |  |  | QCL Type |  | N/A |  | Type C |
|  | Type 2 QCL information |  |  | SSB index |  | N/A |  | SSB #1 |
|  |  |  |  | QCL Type |  | N/A |  | Type D |
| TCI State #2 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type A |  | N/A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type D |  | N/A |
| TCI State #3 | Type 1 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type D |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |  |
| Number of HARQ Processes |  |  |  |  |  | 8 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.3 |  |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |  |
|  |  | Density (ρ) |  |  |  | 1 |  |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 4, (8,-) |  |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (13) |  |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #9 | Resource #10 |  |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 2 | 2 |  |
|  |  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 |  |
|  |  | Density (ρ) |  |  |  | 1 | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3, (8,-) | Row 3, (10,-) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (12) | (12) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |  |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 1 |  |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (8,13) |  |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |  |
| csi-ReportMode |  |  |  |  |  | Mode1 |  |  |
| numberOfSingleTRP-CSI-Mode1 |  |  |  |  |  | X=0 |  |  |
| CMR pairing and grouping |  |  |  |  |  | CMR group #1: {NZP CSI-RS resource #9}, with K1=1 CMR group #2: {NZP CSI-RS resource #10}, with K2=1 CMR paring: {NZP CSI-RS resource #9, NZP CSI-RS resource #10} |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 8 |  |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | CodebookType |  |  |  | typeI-SinglePanel |  |  |
|  |  | CodebookMode |  |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (1,1) |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (1,1) |  |  |
|  |  | CodebookSubsetRestriction |  |  |  | 001111 |  |  |
|  |  | RI Restriction |  |  |  | N/A |  |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 1.75 |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  |  |  | Test 1: R.PDSCH.5-7.2 TDD Test 2: R.PDSCH.5-7.3 TDD |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.125 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 4: Randomization of the beam direction per TRxP shall be used as specified in Annex B.2.3.2.3. Note 5: PT-RS configuration in Test 1 uses single port (one TRxP) and Test 2 used dual port (both TRxPs) Note 6: Correlation matrix according to the RFR2-mTxRP-mRX in B.2.3.3. TRxP#1 uses TX antenna indices (1,2) and TRxP#2 uses TX antenna indices (3,4) corresponding to the respective antenna configuration matrix rows |  |  |  |  |  |  |  |  |

Normative reference in clause 8.3.3.2.2 of TS 38.101-4 [5]

Table 8.3.3.2.2.3-2: Minimum requirement

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
|  | 1.14 | 1.14 |

8.3.3.2.2.4	Test description

8.3.3.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2-1 of TS 38.521-2 [8].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

1.	Connection between SS, the faders, AWGN noise source and the UE antenna is shown in TS 38.508-1 [6] Annex A, Figure A.3.3.2 for TE diagram and Figure A.3.4.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 8.1.2-1 and Table 8.3.3.2.2.3-1 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-2 [8].

4.	Propagation conditions for NR cell are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 8.3.3.2.2.4.3.

8.3.3.2.2.4.2	Test procedure

1.	Set the UE in a direction that satisfies the UE positioning procedure specified in Annex H clause TBD. If no direction found, mark the test as inconclusive.

2.	Set the parameters of bandwidth, the propagation condition, antenna configuration, measurement channel and SNR for TRxP#1 and TRxP#2 according to Table 8.3.3.2.2.3-1 as appropriate.

3. 	Set SNR to . The SS shall transmit PDSCH via PDCCH DCI format [1_1] for C_RNTI with randomly selected precoding matrix from codebook (Table 5.2.2.2.1-1 in TS 38.214 [12]) every slot regardless of PMI reports from the UE. Note that each precoding matrix shall be selected in equal probabilities. The precoder shall be updated in each slot (0.125 ms granularity) with equal probability of each applicable i1, i2 combination.

4.	The SS sends downlink MAC padding bits on the DL RMC. SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format 0_1 with aperiodic CSI request triggered. Measure according to Annex G.3.3.

5.	The SS shall transmit PDSCH via PDCCH DCI format [1_1] for C_RNTI to transmit the DL RMC with precoding matrix according to PMI report from the UE. The SS sends downlink MAC padding bits on the DL RMC. SS schedules the UL transmission to carry the PUSCH CQI feedback via PDCCH DCI format 0_1 with aperiodic CSI request triggered. No transport block is sent in parallel to the CQI feedback. Establish and  according to Annex G.3.2.

6.	Calculate . If the ratio  which is specified in table 8.3.2.2.1.5-1, then the test is pass. Otherwise, the test is fail.

7.	Repeat steps from 2 to 6 for each subtest in Table 8.3.3.2.2.3-1 as appropriate.

8.3.3.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2 with the following exceptions:

8.3.3.2.2.4.3_1	Message exceptions

FFS

8.3.3.2.2.5	Test requirement

Table 8.3.2.2.1.5-1: Test requirement (TDD)

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
|  | 1.15 | 1.15 |

==============Next change==============

### F.2.1.3	Measurement of Channel State Information reporting

This clause defines the maximum test system uncertainty for channel state information reporting requirements. The maximum test system uncertainty allowed for the measurement uncertainty contributors are defined in Table F.2.1.3-1.

Table F.2.1.3-1: Maximum measurement uncertainty values for the test system for FR2 (up to 40 GHz) and Channel BW ≤ 400 MHz

| MU contributor |  |  | Unit | Value |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWconfig |  |  | dB | Same as in table F.2.1.2-1 |  |  |  |
| Signal to noise ratio uncertainty |  |  | dB | Same as in table F.2.1.2-1 |  |  |  |
| Impact on non-ideal isolation between branches for the wireless cable mode |  | dB | Same as in table F.2.1.2-1 |  |  |  |  |
| Fading profile power uncertainty |  |  | dB | Same as in table F.2.1.2-1 |  |  |  |

The maximum test system uncertainty for test cases defined in section 8 is defined in Table F.2.1.3-2.

Table F.2.1.3-2: Maximum test system uncertainty for FR2 channel state information reporting test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8.2.2.2.1.1 2 Rx TDD FR2 periodic wideband CQI reporting under AWGN performance for both SA and NSA | ± 1.40 dB | Overall system uncertainty under AWGN conditions comprises three quantities: 1. gNB emulator Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness 3. Impact on non-ideal isolation between branches for the wireless cable mode gNB emulator SNR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x [0.25] effect on the required SNR, so use sensitivity factor of x [0.25] for the uncertainty contribution. Test System uncertainty = SQRT (gNB emulator Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 ) + Impact on non-ideal isolation between branches for the wireless cable mode gNB emulator Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±3.6 dB Impact on non-ideal isolation between branches for the wireless cable mode 0.45 dB for Rank2 and 0.6 for Rank1 |
| 8.2.2.2.2.1 2 Rx TDD FR2 aperiodic wideband CQI reporting under fading performance for both SA and NSA | ± 1.82 dB for Doppler < 100Hz | Overall system uncertainty for fading conditions comprises five quantities: 1. gNB emulator Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time 5. Impact on non-ideal isolation between branches for the wireless cable mode gNB emulator SNR Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (gNB emulator Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2 ) + Impact on non-ideal isolation between branches for the wireless cable mode gNB emulator Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2Tx AWGN flatness and signal flatness ±3.6 dB SNR uncertainty due to finite test time ±0.3 dB Impact on non-ideal isolation between branches for the wireless cable mode 0.6 for Rank1 and 0.45 for Rank2 |
| 8.2.2.2.2.1_1 2Rx TDD FR2 aperiodic wideband CQI reporting under fading performance for both SA and NSA – 256QAM (Rel-16 and forward) | Same as 8.2.2.2.2.1 | Same as 8.2.2.2.2.1 |
| 8.2A.3.1.1 2Rx CQI reporting accuracy under AWGN conditions for CA (2DL CA) | Same as 8.2.2.2.1.1 on each CC. | Same as 8.2.2.2.1.1 |
| 8.2A.3.1.2 2Rx CQI reporting accuracy under AWGN conditions for CA (3DL CA) | Same as 8.2.2.2.1.1 on each CC. | Same as 8.2.2.2.1.1 |
| 8.2A.3.1.3 2Rx CQI reporting accuracy under AWGN conditions for CA (4DL CA) | Same as 8.2.2.2.1.1 on each CC. | Same as 8.2.2.2.1.1 |
| 8.3.2.2.1 2Rx TDD FR2 Single PMI with 2TX TypeI-SinglePanel codebook for both SA and NSA | Same as 8.2.2.2.2.1 | Same as 8.2.2.2.2.1 |
| 8.3.3.2.2 4Rx TDD FR2 Single PMI with 2TX TypeI-SinglePanel codebook Single-DCI based transmission scheme with simultaneous reception - MultiRx | Same as 8.2.2.2.2.1 | Same as 8.2.2.2.2.1 |
| 8.4.2.2.1 2Rx TDD FR2 RI reporting for both SA and NSA | Same as 8.2.2.2.2.1 | Same as 8.2.2.2.2.1 |

==============Next change==============

### F.2.3.3	Measurement of Channel State Information reporting

The derivation of the test requirements for the test cases in section 8 is defined in Table F.2.3.3-1.

Table F.2.3.3-1: Derivation of Test Requirements (FR2 channel state information reporting tests)

| Test | Minimum Requirement in TS 38.101-4 | Test Tolerance (TT) | Test Requirement in TS 38.521-4 |
| --- | --- | --- | --- |
| 8.2.2.2.1.1 2 Rx TDD FR2 periodic wideband CQI reporting under AWGN performance for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 8.2.2.2.2.1 2 Rx TDD FR2 aperiodic wideband CQI reporting under fading performance for both SA and NSA | SNRs as specified  2% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 8.2.2.2.2.1_1 2Rx TDD FR2 aperiodic wideband CQI reporting under fading performance for both SA and NSA | SNRs as specified  2% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 8.2A.3.1.1 2Rx CQI reporting accuracy under AWGN conditions for CA (2DL CA) | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | Test requirement unchanged |
| 8.2A.3.1.2 2Rx CQI reporting accuracy under AWGN conditions for CA (3DL CA) | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | Test requirement unchanged |
| 8.2A.3.1.3 2Rx CQI reporting accuracy under AWGN conditions for CA (4DL CA) | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | Test requirement unchanged |
| 8.3.2.2.1 2Rx TDD FR2 Single PMI with 2TX TypeI-SinglePanel codebook for both SA and NSA | SNRs as specified 1.05 for Test 1 1.05 for Test 2 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 | SNR unchanged  1.04 for Test 1  1.04 for Test 2 |
| 8.3.3.2.2 4Rx TDD FR2 Single PMI with 2TX TypeI-SinglePanel codebook Single-DCI based transmission scheme with simultaneous reception - MultiRx | SNRs as specified 1.15 for Test 1 1.15 for Test 2 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 | SNR unchanged  1.14 for Test 1  1.14 for Test 2 |
| 8.4.2.2.1 2Rx TDD FR2 RI reporting for both SA and NSA | SNRs as specified 1.00 for Test 1 1.05 for Test 2 1.05 for Test 3 | SNR 0 dB 0.01 for Test 1 0.01 for Test 2 0.01 for Test 3 | SNR unchanged  0.99 for Test 1  1.04 for Test 2  1.04 for Test 3 |

==============End of change==============