# 3GPP TSG SA WG5 Meeting #161                 	           S5-252750

# Fukuoka, JAPAN 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.260 | CR | 0443 | rev | 3 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add charging principle for UE-satellite-UE communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-CH |  |  |  |  |  | Date: |  |  | 2025-3-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In order to support the feature of UE-satellite-UE communication, the charging principle and message flow for UE-satellite-UE communication should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add charging principle for UE-satellite-UE communication. Add message flow for UE-satellite-UE communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE-satellite-UE communication feature is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-251645, S5-252707. |  |  |  |  |  |  |  |  |

First change

### 5.1.x	Charging support of duration-based charging for UE-satellite-UE communication

##### 5.1.x.1	General

As specified in the clause 5.4.14 of TS 23.501 [200] and the Annex AE of TS 23.228 [201], the UE-satellite-UE communication supports IMS voice/video services between two UEs belonging to the same PLMN and in a non-roaming scenario.

If UE-Satellite-UE communication in IMS is supported, the IMS-GWF (CTF) sends a charging data request to CHF with charging information about UE-Satellite-UE communication by the trigger events. The following IMS charging information is included:

-	Satellite ID

##### 5.1.x.2	Message flows for satellite change

As specified in the TS 23.228 [208], if the originating network cannot continue, P-CSCF determines IMS AGW relocation and media routing path change due to change of satellites.

The charging message flow of continued optimized media routing procedure, is based on Figure AE.5.2.1-1 description with the differences identified in clause AE.5.2.1 of TS 23.228 [208]:

10ch-a,	If the IMS AS received SIP re-INVITE containing the satellite ID of the target satellite in step10 in clause AE.5.2.1 of TS 23.228 [208], the IMS AS sends the Charging Data Request [Update] may include satellite ID of the target satellite to CHF for the UE-satellite-UE charging.

22ch-a,	If the IMS AS received SIP 200 OK containing source satellite in step 22 in clause AE.5.2.1 of TS 23.228 [208], IMS AS sends the Charging Data Request [Termination] may include satellite ID of the source satellite to CHF for the UE-satellite-UE charging.

End of change