3GPP TSG-SA WG1 Meeting #112 	S1-254248

17-21 November 2025, Dallas, TX, USA	(revision of S1-25xxxx)

Source:	Siemens, EDF

New use case title:	New Use Case on 6G-enabled Decentralized Smart Grid Control – Re-submission

Draft TS/TR:	3GPP TR 22.870

Agenda item:	8.1.8.2

Document for:	Approval

Contact:	Michael Bahr ( bahr et siemens dod com )
Daniel Hauer ( daniel dod hauer et siemens dod com)
Kim Schindhelm ( corina desh kim dod schindhelm et siemens dod com )

Abstract: Re-submitted New Use Case on 6G-enabled Decentralized Smart Grid Control for 3GPP SA1 TR 22.870 Clause 11 Further Use Cases on Industry and Verticals.

In this use case on 6G-enabled Decentralized Smart Grid Control, 6G communication is used for decentralized control during normal operation but especially during failure situations. As renewable energy sources, proactive loads (e.g., EV chargers), and energy communities become increasingly integrated, low-voltage (LV) grids—historically passive and over-provisioned—must evolve into actively managed systems. These grids now require real-time monitoring, control, and coordination to maintain stability, efficiency, and resilience.

Resilience of the communication network is of paramount concern for smart grid operation. The electricity grid is a very critical infrastructure. Low-latency and highly reliable communication is required in order to control the sometimes fast changing dynamics of electrical generation and electrical transmission. In order to account for the criticality of the smart grid and to avoid black-outs, any imaginable failure in the communication network as unlikely as it seems has to be considered and backup mechanisms have to be put in place.

The use case on 6G-enabled Decentralized Smart Grid Control has been elaborated in the SUSTAIN-6G project (https://sustain-6g.eu/).

Acknowledgements: This work is co-funded by the European Union under Grant Agreement 101191936. Views and opinions expressed are, however, those of the author(s) only and do not necessarily reflect those of all SUSTAIN-6G consortium parties nor those of the European Union or the SNS JU (granting authority). Neither the European Union nor the granting authority can be held responsible for them.

The following text changes are proposed to be included in 3GPP TR 22.870 “Study on 6G Use Cases and Service Requirements”.

Start of Change 1

Instruction to TR 22.870 rapporteurs: Please add new references [v1] to [v5] to the list of references in Clause 2.

# 2	References

[…]

[v1]	IEC 61850-9-2:2011, IEC/IEEE International Standard - Communication networks and systems for power utility automation - Part 9-2: Specific communication service mapping (SCSM) - Sampled values over ISO/IEC 8802-3.

[v2]	IEC 61850-9-3-2016 – IEC/IEEE International Standard - Communication Networks and Systems for Power Utility automation – Part 9-3: Precision time protocol profile for power utility automation.

[v3]	IEC 61850-90-5:2012, Use of IEC 61850 to transmit Synchrophasors information according to IEEE C37.118.

[v4]	IEEE Std 1588-2019: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control".

[v5]	3GPP TS 23.180: "Mission critical services support in the Isolated Operation for Public Safety (IOPS) mode of operation".

End of Change 1

Start of Change 2

Instruction to TR 22.870 rapporteurs: The following text is all new text.

## 11.x	Use case on 6G-enabled decentralized smart grid control

### 11.x.1	Description

This use case addresses the transformation of traditional, fossil-fuel-based power grids into intelligent, renewable-powered distribution systems—an evolution that is not only technological but also strategic, given that the smart grid is a critical infrastructure underpinning modern society.

As renewable energy sources, proactive loads (e.g., EV chargers), and energy communities become increasingly integrated, low-voltage (LV) grids—historically passive and over-provisioned—must evolve into actively managed systems. These grids now require real-time monitoring, control, and coordination to maintain stability, efficiency, and resilience of the electrical grid.

Traditionally, grid operation has relied heavily on redundancy and conservative planning to ensure resilience. For example, the N-1 contingency criterion mandates that transformer substations operate well below their physical limits to accommodate necessary resources in case of potential failures. In practice, this results in significant unused capacity, which is increasingly unsustainable as electrification and renewable integration accelerate. Moreover, the grid must remain operational even under communication loss or cyber-attacks. This has led to architectures that assume worst-case scenarios, often resulting in over-provisioning.

This use case involves a hierarchical grid structure, where medium-voltage (MV) networks supply multiple low voltage (LV) grids. Each LV grid is managed by a distributed LV grid computer system (DS), equipped with 6G communication interfaces. These decentral systems coordinate with MV controllers and neighbouring substations to manage grid flexibility—such as load shifting or energy storage utilization—based on real-time demand and grid conditions.

One of the major concerns in the operation of smart grids is resilience against failures. This includes explicitly failures in the communication network leading to disrupted communication services for the smart grid control applications. The electricity grid is a very critical infrastructure. Smart grids incorporate different kinds of renewable energy sources and energy storages in various combinations. In order to control the dynamic electrical load and to be able to react to any changes in such smart grids, low-latency and highly reliable communication between the different entities of the smart grid is necessary. In order to account for the criticality of the smart grid and to avoid black-outs, any imaginable failure in the communication network—as unlikely as it seems—has to be considered and backup mechanisms have to be put in place.

This use case shows a paradigm shift: by enabling fully decentralized, real-time cluster coordination among local grid cells via 6G, meshed DSs can form resilient clusters during fault scenarios. These clusters autonomously optimize local grid behavior, maintaining stability without backend connectivity. This approach allows for more realistic and efficient fallback modes, reducing the need for excessive reserves and unlocking critical latent capacity that is urgently needed to support the energy transition. Figure 11.x.1-1 visualizes the concept of decentralized, real-time cluster coordination among local grid cells. Multiple DSs (DS1-6) are part of one MV grid with a central controller (CS) and each LV grid can manage its own flexibilities. During full connectivity, the CS can optimize the whole MV grid by coordinating all LV grids. If the connection between CS and its DSs is corrupted or fails, the LV grids form clusters with neighbouring LV grids (e.g. DS2 with DS3). Within the clusters the LV grids can optimize themselves and avoid being limited to a worst-case fallback for the case of no connection.

Figure 11.x.1-1: Example of intelligent, renewable-powered distribution systems

Figure 11.x.1-1 shows idealized topologies of both the electrical smart grid and the mobile communication network. However, geographical conditions, but also legal conditions lead to the practical situation that the topology of the mobile communication network does not correspond 1:1 with the topology of the smart grid. Devices (e.g. renewable energy sources, energy storage) in an LV-cell can be connected to different gNBs, especially, they might be connected to a different gNB than their respective DS controlling them. Also a gNB may serve devices from different LV cells and might not serve all devices of an LV cell.

The mobile communication network provides the required communication services between all devices in an LV cell and to their respective DS. Communication for controlling the smart grid devices is required

- Within an LV cell and its DS for distributed control. This is the fundamental, necessary communication in order to provide electricity.

- Between DSes of neighboring LV cells organized in clusters for improved distributed control

- Between all DSes of an MV cell and the CS for central and optimized control of the LV cells by the DSes.

Depending on the location of a (unlikely) failure in the communication network, the 6G system has to setup different mechanisms in operational scenarios as illustrated in Table 11.x.1-1in order to provide backup communication for controling the smart grid.

Table 11.x.1-1 starts from normal mode, being the default operation as supposed to be without any connectivity problems. The other rows show different failure situations for the connectivity of the distributed LV grid controllers to the central MV grid controller. These scenarios have a more or less really low probability to happen. Nevertheless the communication network needs to be able to cope with these situations, even if they will happen only very rarely, in order to sustain the operation of the critical infrastructure of the electrical grid. Some further textual description of the operational scenarios is provided in Table 11.x.1-2.

The mentioned clusters of LV cells are a smart grid concept within the smart grid control application. LV cells with their controlles DS and their smart grid devices (e.g. renewable energy sources, energy storage) are organized and clustered together. The clustering is based on the electrical grid topology and the electrical characteristics of the devices. The goal of the clustering is an improved local and decentralized control of smart grid units e.g. small groups of neighboring LV cells. The clustering can be provided as a communication matrix.

The mentioned backup communication are alternative communication mechanisms in case a failure occurs in some parts of the mobile network. Anticipated mechanisms and potential failures are:

- Backup communication of the gNB in case all associated UEs have still connectivity to the gNB / RAN, but the gNB cannot reach the core network. The backup communication of the gNB will provide service continuity to high-priority smart grid control traffic within its cell.

- In backup device communication, the UEs of the smart grid devices (e.g. renewable energy sources, energy storages, DS of LV cells) will setup local communication based on the communication matrix provided by the clustering of the smart grid application. The clustering might be dynamically reconfigured by the local smart grid application. The local communication is achieved through (in)direct device connectivity and where possible through indirect network connectivity.

- Based on the communication matrix provided by the clustering and depending on the actual topology of the mobile network, some affected devices will setup (in)direct network connections to other gNBs.

Please note, that overlapping gNB coverage and redundant deployments will decrease the need for using backup communications. This is likely to be achieved in urban areas, however, redundancy will be lower in rural areas.

Table 11.x.1-1: Operational scenarios – illustration

|  | Normal mode default operation as supposed to be without any connectivity problems All LVs belong to one MV cell and the LV cells’ DSes are connected through the mobile network to the MV controller (CS) Communication is provided by mobile network. |
| --- | --- |
|  | Full cluster mode MV controller DS lost connectivity. The LV controllers CS operate within the provided cluster structure. No control optimization from the MV controller. Communication between the DSes within MV cell and within the LV cells is provided by the mobile network, some reconfiguration of communication services might be necessary. |
|  | Local cluster mode One or more LV controllers DS lost connectivity to MV cell, connectivity to gNBs is still available. The affected gNBs and selected UEs switch to backup communication in order to provide service continuity to high-priority smart grid control traffic within their LV cell cluster. Goal is to allow for data exchang between as many LV cells as possible. Dynamic reconfiguration of the clustering might be done by local smart grid application on DSes. |
|  | Connectivity Backup Mode Smart gird devices of an LV cell (including DS) lost connectivity to gNB. The affected devices switch to backup device communication. This may require a dynamic reconfiguration of the clustering by the local smart grid application based on the availably connectivity. |

Table 11.x.1-2: Description of operational scenarios

| Scenario | Description | Technical Requirements | Gaps/Challenges |
| --- | --- | --- | --- |
| Normal Operation | Normal Grid operation where all LV grids have full 6G communication with the MV grid controller. The MV grid controller will then collect all LV measurement data and calculate the optimal setpoints. | - Low energy consumption: Support for energy-harvesting sensors. - Coverage: 100% area coverage, including hard-to-reach locations (e.g., basements). - User data rate: ~10 Mbit/s; - Time synchronization: < 1 ms E2E - Interoperability: Standardized onboarding, firmware updates. - Maintenance: Long-term QoS, automated software updates. - Guaranteed QoS: Bandwidth and service levels must be maintained. | - automated sensor onboarding and firmware updates. - Coverage assurance in underground or shielded environments. |
| Full / Local Cluster Mode | Communication to MV grid controller is not available, and affected/all DS form local clusters using communication through available 6G cells for cluster-wise optimizations. | - Cluster Mode: Flexible cluster formation across LV grids and substations. - Guaranteed bandwidth/service levels must be maintained. - Cluster-local communication with minimal latency. - Ultra-precise time sync (< 2 µs) in decentralized clusters | - Ultra-precise time sync (< 2 µs) in decentralized clusters is challenging - Detection of communication loss - Service continuity - Dynamic cluster management protocols for the Smart Grid domain. |
| Connectivity Backup Mode | Smart gird devices of an LV cell (including DS) lost connectivity to gNB. Affected devices set up (in)direct connectivity to other devices, reconfiguration of local clusters might be necessary. | - Switch to (in)direct device connectivity in case of connectivity loss to gNB - Guaranteed bandwidth/service levels must be maintained. - Cluster-local communication with minimal latency. - Ultra-precise time sync (< 2 µs) in decentralized clusters | - Ultra-precise time sync (< 2 µs) in (in)direct device communication - Latency requirements in (in)direct device communication |
| Grid Failure Mode (Grid Failure Scenario) | A failure affects the operation of one LV grid leading to a blackout. Failure protocols and high-resolution measurements have to be transmitted for root cause analysis. | - Time synchronization: < 2 µs for PMUs/differential protection E2E - Backup power: 30 min UPS for communication systems. - High bandwidth: >100 Mbit/s for high-res data. - Spontaneous high data transmission with guaranteed latency. - Dynamic power adaptation of 6G transmitters. | - Energy-efficient ICT operation - Latency guarantees under failure conditions are difficult to maintain. |

Time synchronization is an important requirement for

- the control of the smart grid devices and their dynamic behaviour. In order to keep the electricity grid stable, the smart grid devices have to react very fast within only a few ms. This puts demanding latency requirements on the wireless communication, but also on the accuracy of the time synchronization.

- Time stamping of events in the electrical smart grid, especially for root cause analysis of variations, anomalies, and unlikely potential failures.

The integrity of the timestamps needs to be guaranteed. The timestamps and the time synchronization functionality have to be manipulation-safe.

Potential sustainability impacts

Table 11.x.1-3 lists sustainability effects for this use case of 6G-enabled Decentralized Smart Grid Control. The sustainability effects in Table 11.x.1-3 are grouped into environmental, societal, and economic sustainability. The first rows in green colour describe key values with positive impact, the latter rows in red colour describe key values with negative impact.

Table 11.x.1-3: Sustainability effects

| Environmental |  | Societal |  | Economic |  |
| --- | --- | --- | --- | --- | --- |
| 1st order effects | 2nd order effects | 1st order effects | 2nd order effects | 1st order effects | 2nd order effects |
| Beneficial effects |  |  |  |  |  |
| More local energy generation and consumption, including self-consumption | Higher density of sustainable assets | Cheaper energy | Greater satisfaction in industries and the general population | Reduced need for grid replanning | Less financial burden on low-income households |
| Efficient hardware usage | Minimized infrastructure needs | Sense of doing good | Increased GDP | Lower CAPEX/OPEX | Avoidance of blackouts |
| Higher % of renewables |  | Social safety | Community empowerment |  |  |
| Less CO₂ emissions |  |  |  |  |  |
| Higher density of PV, chargers, heat pumps |  |  |  |  |  |
| Challenging effects |  |  |  |  |  |
| Increased energy consumption from 6G | GHG emissions from production/maintenance | User adoption barriers | Digital divide | High deployment and maintenance costs | Potential job losses in traditional energy sectors |
| Use of rare materials | E-waste from outdated components | Privacy concerns | Need for education and retraining |  |  |

### 11.x.2	Pre-conditions

- Decentralized system units (LV cell controllers, DS), smart grid devices (e.g. renewable energy sources, energy storage)  are deployed and connected to the 6G network; their communication is secured.

- Flexibility-enabled assets (batteries, PV, EV chargers) are available in LV grids.

- 6G network supports cluster-mode fallback scanning and peer discovery.

- Local controller-to-controller communication available as redundant path in absence of backbone.

- Minimum local bandwidth reserved for emergency cluster coordination

### 11.x.3	Service Flows

- MV Grid Controller calculates and sends setpoints to DSs via 6G: The MV controller gathers real-time data from all LV grids and computes target values (setpoints) like power limits. These setpoints guide each DSs to adjust local loads and generation to keep the grid stable and efficient. The instructions are sent over the 6G network for timely and reliable control.

- DSs monitor grid conditions continuously: Local sensors provide real-time measurements to DSs to detect changes or faults in the LV grid. This allows immediate response to variations or anomalies.

- In normal conditions, MV controller maintains full communication with DS: All DSs send updated status and receive new setpoints continuously, ensuring centralized and coordinated control.

- If communication to MV controller is lost, DSs form local 6G clusters: DSs switch to a decentralized mode, coordinating only among themselves within a cluster using 6G cells. This ensures local balancing and resilience despite communication backbone failure.

- If connectivity with the gNB is impacted, affected devices will setup local communication by switching to (in)direct device communication or indirect network connection to neighboring devices.

- DSs within clusters optimize power sharing and load balancing locally: They exchange flexibility information to distribute demand and generation, avoiding overloads and maintaining supply without central guidance.

- Once backbone communication is restored, normal operation resumes: DSs resynchronize with the MV controller, updating setpoints and restoring centralized grid management.

### 11.x.4	Post-conditions

- DSs successfully implement received setpoints, adjusting local loads and generation to meet grid targets. This ensures that energy supply and demand are balanced locally, contributing to overall grid stability and efficiency.

- In case of communication loss to the MV grid controller, DSs formed resilient local clusters that maintain power balance within their geographic area. These clusters enable decentralized coordination, reducing the risk of widespread outages and allowing continued operation despite communication backbone failures.

- Local cluster communication optimizes flexibility sharing, preventing overloads and maintaining voltage/‌frequency within regulatory limits. This adaptive response helps to prevent blackouts and to meet strict power quality and safety standards even during faults or emergencies.

- Once communication with the MV controller was restored, DSs seamlessly resynchronized and reverted to centralized control. This transition minimized disruption and restored optimal grid-wide management.

- Grid resilience and supply security are improved by enabling real-time fault detection and localized response. Faster detection and reaction reduce blackout risks and support compliance with regulatory reliability criteria (e.g., N-1 contingency).

- Efficient use of renewable resources and storage assets is maintained, supporting sustainability goals. The system maximizes clean energy integration and reduces carbon emissions by effectively leveraging distributed flexibility.

.

### 11.x.5	Existing features partly or fully covering the use case functionality

- TS 22.104 [64] contains service performance requirements for electric-power distribution and smart grid and central power generation. Table 5.2-1 lists corresponding service performance requirements for related use cases / scenarios, for instance, on ultra-reliable low latency communications (URLLC). Related use case with corresponding KPI requirements are (a) electrical distribution – distributed automated switching for isolation and service restoration with end-to-end latency < 5 ms and service area of 30 x 20 km (b) primary frequency control, (c) distributed voltage control (d) distributed energy storage (e) distributed energy storage – data collection (f) advanced metering (g) intelligent distributed feeder automation (h) smart distribution – transformer terminal (i) high speed current differential protection, (j) distributed energy resources (DER) and microgrids with e2e latency of 3 ms and communication service availability of 99.9999%, (k) ensuring uninterrupted communication service availability during emergencies.

- TS 22.261 [14]: Service requirements for the 5G system; includes general support for IoT, URLLC, and industrial use cases.

- Support of IEEE 1588 / Precision Time Protocol in TS 22.104 [64] Clause 5.6.1. IEEE 1588 [v4] protocol is used to synchronize clocks across networked devices with sub-microsecond accuracy. It operates over Ethernet and supports high-precision time distribution without requiring GPS at every node.

- User-specific clock synchronicity accuracy level 4 (TS 22.104 [64], Table 5.6.2-1) requires < 1 µs 5GS synchronicity budget for up to 100 UEs.

- Several clock synchronization service performance requirements from IEC 61850-9-2 [v1] are listed in TS 22.104 [64] Table 5.6.2-1. Support for precise time distribution and clock synchronization specific to smart grid applications e.g. IEC 61850-9-3 [v2], IEC 61850-90-5 [v3] is required in TS 22.104 [64] Clause 5.6.1.

- TS 22.261 [14] Clause 6.26.2.2 requires (unreliable) multicast communication with communication types Ethernet and IP in 5G LAN virtual networks.

- The Isolated Operation for Public Safety (IOPS) mode (TS 23.180 [v5]) allows a single base station or a group of base stations to serve UEs independently of the core network when needed. However, this mode of isolated operation needs to be extended beyond public safety for high-priority traffic and needs to support QoS requirements of communication in Smart Grid control.

### 11.x.6	Potential New Requirements needed to support the use case

[PR 11.x.6-1]	Subject to regulatory requirements and operator policy, the 6G system shall be able to support backup communication as a secure, resilient local communication fallback mechanism in case connection to the core network is lost.

[PR 11.x.6-2]	Subject to regulatory requirements and operator policy, the 6G system shall be able to detect loss of connection to the core network.

[PR 11.x.6-3]	Subject to regulatory requirements and operator policy, the 6G system shall be able to provide continuous communication with agreed QoS between UEs when switching to backup communication.

[PR 11.x.6-4]	Subject to regulatory requirements and operator policy, the 6G system shall be able to detect restoration of connection to the core network when in backup communication.

[PR 11.x.6-5]	Subject to regulatory requirements and operator policy, the 6G system shall be able to provide continuous communication with agreed QoS between UEs when switching from backup communication to normal mobile network operation.

[PR 11.x.6-6]	Subject to regulatory requirements and operator policy, the 6G system shall be able to form a group of UEs for high priority communications in backup communication.

NOTE:	An example for such a group of UEs are UEs that manage LV grid flexibility—such as load shifting or energy storage utilization—based on real-time demand and LV grid conditions. The high priority traffic is considered to be the control traffic for managing the LV grids and distributed energy resources.

[PR 11.x.6-7]	Subject to regulatory requirements and operator policy, the 6G system in backup communication shall be able to provide continuous communication with agreed QoS between UEs of a group of UEs for high priority communications for up to 24 h.

[PR 11.x.6-8]	Subject to regulatory requirements and operator policy, the 6G systems shall be able to support indirect and direct device communication as well as indirect network communication in backup communication.

[PR 11.x.6-9]	Subject to regulatory requirements and operator policy, the 6G system in backup communication shall be able to setup direct device communication or indirect network communication between UEs in range in case communication between these UEs is not possible anymore through direct network communication.

[PR 11.x.6-10]	Subject to regulatory requirements and operator policy, the 6G system shall enable dynamic reconfiguration of communication services, the reconfiguration triggered by switching to or back from backup communication.

[PR 11.x.6-11]	Subject to regulatory requirements and operator policy, the 6G system in backup communication shall be able to support high-priority communication with required QoS for a group of UEs.

[PR 11.x.6-12]	Subject to regulatory requirements and operator policy, the 6G system shall be able to reserve communication resources in advance for high-priority traffic with QoS requirements for a group of devices in order to support reconfiguration of communication services triggerd by switching to or back from backup communication.

[PR 11.x.6-13]	Subject to regulatory requirements and operator policy, the 6G system in backup communication shall be able to support service performance requirements as given in TS 22.104 [64] Table 5.2-1 for scenarios related to Distributed Energy Resources (DER) and microgrids and to use cases in TS 22.104 [64] Annex A.4 (see column Remarks in TS 22.104 [64] Table 5.2-1).

[PR 11.x.6-14]	The 6G system shall be able to guarantee <1 ms end-to-end latency.

NOTE:	This requirement applies to a 6G system in backup communication, too.

[PR 11.x.6-15]	Subject to regulatory requirements and operator policy, the 6G system in backup communication shall be able to guarantee sub-microsecond synchronization accuracy (e.g. for control message exchange) in a service area of 20 km x 30 km.

[PR 11.x.6-16]	The 6G system shall be able to support TX (sending) timestamps with accuracies supporting clock synchronization service performance requirements from PR 11.x.6-15 and TS 22.104 [64] Table 5.6.2-1.

NOTE:	This requirement applies to a 6G system in backup communication, too.

[PR 11.x.6-17]	The 6G system shall provide suitable means at a UE to provide TX (sending) timestamps with high accuracies supporting sub-microsecond clock synchronization to an authorized third party

[PR 11.x.6-18]	The 6G system shall be able to ensure the integrity and manipulation-safety of the provided TX (sending) timestamps.

[PR 11.x.6-19]	The 6G system shall be able support periodic reliable L2 multicast.

NOTE:	Support for periodic reliable L2 multicast as described in this requirements is required for both, (in)direct network connections and (in)direct device connections.

End of Change 2