3GPP TSG SA WG 1 Meeting #111 	S1-253046r1

25-29 August 2025, Goteborg, Sweden	(revision of S1-253046)

Source:	China Telecom, ZTE

pCR Title:	Update of 5.6.5 Prevention of signalling storm

Draft Spec:	3GPP TR 22.870 0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Jianzhao Li, lijz@chinatelecom.cn, Yinglin Chen, chenyl37@chinatelecom.cn

Abstract: This is a use case update.

1. Introduction

This contribution removes an Editor’s Note, add new PRs and aligns the wording.

2. Reason for Change

This is a use case update.

3. Conclusions

None.

4. Proposal

It is proposed to agree the following changes.

* * * First Change * * * *

5.6.5	Prevention of signalling storm

5.6.5.1	Description

In 5G, when a network element fails, explicit restoration signalling (e.g., context data, subscription data, policy data) is triggered to its backup element to maintain UE registration status or service sessions. A data-center-level malfunction can generate signalling traffic peaks tens of times higher than normal, potentially causing network congestion or even cascading failures.

For example, a data centre malfunction in a commercial 5GC network resulted in an AMF failure. RAN and other functional network elements detected this failure and simultaneously attempted to restore all affected UEs and sessions. This resulted in a 20-fold surge in registration requests and a 69-fold increase in PDU session establishment requests to the backup AMF—far exceeding normal operational levels, as shown in Figure 5.6.5.1-1 below.

To minimize such signalling, the failure of the network and its backup’s takeover should be transparent to UE, RAN, and other functional network elements, such that no restoration signalling is needed.

Figure 5.6.5.1-1: The number of registration requests and PDU session establishment requests the backup AMF received during a data-centre level malfunction

5.6.5.2	Pre-conditions

The 6GC is deployed in Data Centre A and Data Centre B.

5.6.5.3	Service Flows

1. Data Centre A malfunctions due to unexpected incidents.

2. Data Centre B seamlessly takes over Data Centre A. Messages sent to Data Centre A are automatically routed to Data Centre B. From the perspective of UE, RAN, and other functional network elements, network elements in Data Centre A remain functional. No signalling storm of restoration messages generates.

5.6.5.4	Post-conditions

Messages sent to Data Centre A will be switched back to Data Centre A after Data Centre A is recovered.

5.6.5.5	Existing features partly or fully covering the use case functionality

5G supports restoration procedures, which handle network element failures via its backup, as specified in TS 23.527 [78]. However, these procedures require network element re-selection and session re-establishment (e.g. alternative NF re-selection, PFCP sessions re-establishment.)

Context transfer procedures specified in clause 5.21.4 of TS 23.501 [140] and clause 4.26 of TS 23.502 [30] allow a backup network element to take over the service from the failed network element.

5.6.5.6	Potential New Requirements needed to support the use case

[PR 5.6.5.6-1] The 6G network shall provide a mechanism to minimize the signalling arising from service recovery after a network failure.

NOTE:	Massive signalling can happen e.g. when a network element fails and registrations or data sessions are re-routed to a backup network element.

* * * End of Change * * * *