3GPP TSG-SA WG1 Meeting #111 	S1-253051r1

25-29 August 2025, Goteborg, Sweden	(revision of S1-253051)

Source:	LG Uplus, KT Corp.

New use case title:	New use case on SON improvement for mobility between TN and NTN

Draft TS/TR:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.5

Document for:	Approval

Contact:	Jaehyun Ahn (jaehyun80@lguplus.co.kr)

Abstract: Regarding TN and NTN, mobility protocols are well-defined in 5G. However, basic and essential SON features such as ANR is not clear. In this new use case, an exemplary deployment scene and procedure for ANR between TN and NTN will be described and, correspondingly, requirements are proposed.

---------- Use Case template ----------

x.1	Use case on SON improvement for mobility between TN and NTN

x.1.1	Description

As for NTN in transparent mode, a NTN gNB will be located far from a relevant TN gNB, wherein TN and NTN will require separate AMFs/MMEs for efficiently managing transport network address. This situation is different from legacy TN deployment, where all neighbor relations are sufficiently close within a MME or within neighbor MME. Hence, the neighbor relation request mechanism between TN and NTN should be developed.

For NTN in generative mode, the relationship between TN and NTN would be changed if NTN gNB is moving. NRT (Neighbor Relationship Table) in TN and NTN gNBs would be managed according to time and location. Correspondingly, novel ANR mechanisms should be developed.

x.1.2	Pre-conditions

- Devices (UE #1, UE #2, UE #3) can access and measure TN as well as NTN. In this service scene, it is attached to NTN SAN and gNB for transparent mode and regenerative mode, respectively.

i) NTN in transparent mode

- A TN gNB (TN gNB #1) is newly deployed.

- TN gNB #1 is located within the service coverage of a SAN (Satellite Access Node) #1, which is associated with NTN gNB #1.

- UE #1 is near TN gNB #1 and, hence, can measure TN gNB #1.

ii) NTN in regenerative mode

- A TN gNB (TN gNB #2) is newly deployed in PLACE #1.

- TN gNB #2 is located within the service coverage of a NTN gNB (NTN gNB #2) at time T1.

- UE #2 is near TN gNB #2 and, hence, can measure TN gNB #2.

- A TN gNB (TN gNB #3) is newly deployed in PLACE #2.

- TN gNB #3 is located within the service coverage of NTN gNB #2 at time T2.

- UE #3 is near TN gNB #3 and, hence, can measure TN gNB #3.

x.1.3	Service Flows

i) NTN in transparent mode

1. UE #1 measure TN gNB #1 and report it to NTN gNB #1.

2. NTN gNB #1 detect the new TN cell as a neighbour and proceed a novel TNL (Transport Network Layer) discovery mechanism.

3. TN gNB #1 is autonomously added to NRT of NTN gNB #1.

ii) NTN in regenerative mode

1. At time T1, NTN gNB #2 is over PLACE #1 and UE #2 is attached. UE #2  measures TN gNB #2 in PLACE #1 and report it to NTN gNB #2.

2. NTN gNB #2 detect the new TN cell as a neighbour and proceed a novel TNL discovery mechanism.

3. NTN gNB #2 update NRT for time T1.

4. At time T2, NTN gNB #2 is over PLACE #2 and UE #3 is attached. UE #3  measures TN gNB #3 in PLACE #2 and report it to NTN gNB #2.

5. NTN gNB #2 detect the new TN cell as a neighbour and proceed a novel TNL discovery mechanism.

6. NTN gNB #2 update NRT for time T2.

x.1.4	Post-conditions

- NTN SAN and gNB autonomously updates NRT in efficient way.

x.1.5	Existing features partly or fully covering the use case functionality

Currently, ANR is defined only for TN.

x.1.6	Potential New Requirements needed to support the use case

[PR x.1.6-1]: ANR mechanism shall be specified between TN and transparent mode NTN where NTN SAN is located far from a TN gNB.

[PR x.1.6-2]: ANR mechanism shall be specified between TN and regenerative mode NTN where neighbour relationship between TN and NTN would be changed according to NTN gNBs transversing their orbit

[PR x.1.6-3]: NRT shall be managed in reasonable size for maintaining searching efficiency.

[PR x.1.6-4]: ANR mechanism between TN and NTN shall be specified to be interoperable among different MNO/SNOs.