3GPP TSG SA WG 1 Meeting #110 	S1-253545

25-29 August 2025, Goteborg, Sweden	(revision of S1-253116)

Source:	China Mobile, Lenovo, Motorola Mobility

pCR Title:	Pseudo-CR on update 5.5.4 6G Security requirements

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi, shixiaonan@chinamobile.com;

Xiaoting Huang, huangxiaoting@chinamobile.com;

Sheeba Backia Mary Baskaran (smary@lenovo.com)

Abstract:

1. Introduction

This contribution proposes changes to address the Editor’s Notes remained in clause 5.5.4., issue #24, 25

2. Reason for Change

There are four Editor’s Notes remained in clause 5.5.4 to be addressed.

3. Conclusions

None.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

5.5.4	6G security requirements on trust establishment, security management and digital identity

5.5.4.1	Description

The 6G network will have the competence to adapt to new use cases and new technologies and in tandem be resilient to new attack vectors. The network should be secured and trusted to deliver any services. 6G security should be considered from the following aspects:

Efficient trust establishment and secure communication in inter-PLMNs and intra-PLMNs:

3GPP network security mainly relies on NDS/IP and NDS/AF to provide transport layer security protection mechanisms. The communication network is divided into several security domains, and digital certificates are usually used as identity identifiers and IPSec protocol is used for securing the communication between the domains. This requires both CA (Certificate Authorities) institutions to crossly trust each other. Frequent cross-domain authentication brings about network latency and management cost, as well as threats of single point of failure. What’s more, it is difficult to establish a common root of trust for cross domain communication.

The issue has been identified prominently in the inter-PLMN case, where operators face problems of managing the large scale of CAs in roaming scenarios. Thus, an efficient and decentralized manner of establishing trust in inter-PLMN connection is needed.

For the intra-PLMN case, on one hand, authentication between network elements face similar issues. 6G is expected to provide ubiquitous connectivity and following the network slicing and NPN technologies provided in 5G, there will be more and more local area networks as extension of the PLMN network, which might be deployed in a distributed manner. For these intra-PLMN networks, authentication of network elements in each sub-network should be efficiently performed. Therefore, a distributed and decentralized manner of establishing trust in intra-PLMN is also needed like the inter-PLMN case, in order to ensure the secure interconnection of intra-PLMN networks. On the other hand, as the number and the type of the intra-PLMN networks is increasing, security isolation of these intra-PLMN networks is crucial to avoid cross-attack risks, fine-grained security isolation based on the security needs of different businesses requirements is needed. According to the needs from different vertical industries, the dedicated network resources should be equipped with customized security capabilities for different intra-PLMN networks, this brings the needs of security capability management to orchestrate and dispatch the built-in security capabilities, and to formulate and issue the security policies.Thus the 6G system need to provide flexible orchestration of security capabilities to fulfil customized security needs.

Enhanced network security operation and management:

Cybersecurity is the practice of protecting systems, networks, and programs from digital attacks. It is also fit for the network domain of telecommunication network. The NIST CSF outlines five core functions that serve as a navigation system for managing cybersecurity risk: Identify, Protect, Detect, Respond, and Recover (IDPRR). Existing network security mechanisms can only meet two parts: identify and protect, It has no clear requirements and solutions to cover the rest part, i.e. detect, respond, and recover. So it needs to consider to introduce those three features also in 6G network to enhance network security operation.

Thus, it is necessary to consider providing the abilities: 1) to detect security attacks, security issues and security failure, 2) to analyze them to confirm threats and to make security policies to respond, 3) as well as to implement and deploy such policies to recover normal network and services.

Security as a service for digital identity based on SIM identity and authentication:

Immersive communication is an emerging digital communication method that creates an immersive communication experience for users. The user may have different identities in the virtual world, real world, and physical world. For example, the user may use its digital representatives to transmit audio and video and other multi-modal information to the communication object on the other end according to the intentions of the user. As personal intelligent assistants are expected to rapidly become popular, the user digital representatives need to have its own digital identity associated with the user to ensure their trusted access to the network. An embodied robot may have a SIM-based anchor, but there is a need to ensure the association with a controller/user and for its behaviour control. Thus, it requires a mechanism to identify and establish trust relationships among multiple parties, in order to meet the needs of users to switch and interact in different virtual environments. This requires more flexible and secure identity identification. In addition, in augmented reality scenarios, how to protect user privacy and security is also a concern.

Operators have leveraged the SIM as a trust root in the past few years to authenticate users and build secure connection for the customers. As the core component of mobile communication networks, the security and flexibility of SIM cards make them as a trust anchor, providing a solid security foundation for different forms of digital identities in both virtual world and real world. Operators can leverage the SIM identity as the root ID of user to associate the following digital identity of the user digital representatives, robots, etc., and enable users to seamlessly access various services and resources of the Internet. Accordingly, the operators can associate the user digital representatives with the digital identity and the final root of ID of the SIM. Furthermore, SIM-based self-sovereign identity for authentication and associated attributes for authorization can be used to provide flexible access control of identity access control as well. Certain 3rd party services require user/UE to meet certain eligibility criteria to consume the requested services (e.g., such criteria can be information related to the user e.g., age, location, biometrics etc). In such case, the MNO can act as trust anchor and can provide in addition to the digital identity necessary associated credentials e.g., proof information for the UE/User to support 3rd party service requirements compliance verification.

5.5.4.2	Potential New Requirements

[PR 5.3.4.2-1] The 6G system shall provide efficient mechanisms to support authentication and secure communication in inter-PLMN and intra-PLMN networks.

[PR 5.3.4.2-2] The 6G system shall support fine-grained security isolation based on the security needs of different businesses requirements for intra-PLMN networks.

[PR 5.3.4.2-3] The 6G system shall be able to provide flexible orchestration of security capabilities to ensure customized security requirements from customers.

[PR 5.3.4.2-4] The 6G network shall support identify potential security threats to communication and network elements.

[PR 5.3.4.2-5] The 6G network shall support security analysis and security enhancement policy generation to mitigate attacks and/or security issues in operator management system.

[PR 5.3.4.2-6] The 6G network shall support security response to implement enhanced security policy from management system to network elements, in order to recover network from disturbance.

[PR 5.3.4.2-7] The 6G system shall enable operators using SIM as trust anchor to provide identity and authentication service to the 3rd parties.

[PR 5.3.4.2-8] The 6G system shall support the verification for the association between the SIM identity and the digital identity of the different digital representatives.

[PR 5.3.4.2-9] The 6G system shall authorize the required services of the digital identity of the digital representatives of users and revoke the digital identity if needed.

[PR 5.5.4.2-10] The 6G system shall provide applicable subscriber/user related information to the 3rd party services provider as required for the 3rd party services to allow verification of subscriber/user eligibility to consume the services.

Editor’s Note: Further details of the above requirement PR 5.5.4.2-10 is FFS.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	QIAN LI et all: "6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies" in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022.

[3]	GSMA: "Going green: benchmarking the energy efficiency of mobile networks (Second edition)", Feb 2023.

[4] 	3GPP TR 32.972: "Study on system and functional aspects of energy efficiency in 5G networks".

[5]	3GPP TR 23.700-66: "Study on Energy Efficiency and Energy Saving".

[6] 	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[7]	5GAA: "5GAA MRP input to 3GPP Stage 1 workshop on IMT 2030 Use Cases".

[8]	5GAA: Technical report "Evolution of vehicular communication systems beyond 5G".

[9] 	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication".

[10]	SAE J3016 202104: "Levels of Driving Automation".

[11]	G. Liu, L. Ma, Y. Xue, et al, "SensCAP: A Systematic Sensing Capability Performance Metric for 6G ISAC," IEEE Internet of Things Journal, vol. 11, no. 18, pp. 29438–29454, 2024.

[12]	Igal Bilik, Oren Longman, Shahar Villeval, and Joseph Tabrikian, "The Rise of Radar for Autonomous Vehicles", IEEE signal processing magazine, Volume 36, Issue 5, September 2019.

[13] 	https://knowledge-base.inspire.ec.europa.eu/publications/technical-guidance-implementation-inspire-dataset-and-service-metadata-based-isots-191392007_en.

[14]	3GPP TS 22.261: "Service requirements for the 5G system".

[15] 	Wang et.al, "Effect of Frame Rate on User Experience, Performance, and Simulator Sickness in Virtual Reality", IEEE Transaction on Visualization and Computer Graphics. 2023 Feb 22:PP. doi: 10.1109/TVCG.2023.3247057.

[16] 	https://en.wikipedia.org/wiki/Adreno.

[17] 	https://www.labtekno.com/snapdragon-8-gen-2-vs-apple-a16-bionic/.

[18] 	Algorithms and tools for GPU-accelerated Computer Generated Holography | Ewireless Research Group (https://www.ewireless.eng.ed.ac.uk/projects).

[19] 	https://phlearn.com/tutorial/how-many-exposures-hdr/.

[20]	https://photographylife.com/compressed-vs-uncompressed-vs-lossless-compressed-raw.

[21]	https://www.dotphoton.com/products/jetraw-core.

[22] 	https://www.lambdatest.com/learning-hub/web-performance-testing.

[23] 	Static 3D triangle mesh compression overview, Marcos Aviles, Grupo de Tratamiento de Imágenes, Universidad Politécnica de Madrid, Spain, 15th IEEE International Conference on Image Processing.

[24] 	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[25]	EUSPA: "Report on Aviation and Drones – User Needs and Requirements", 2023.

[26]	3GPP TR 36.763: "Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)".

[27]	Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond".

[28]	3GPP TS 22.156: "Mobile Metaverse Services".

[29] 	ITU-R Recommendation M.2160-0 (11/2023): "Framework and overall objectives of the future development of IMT for 2030 and beyond" (https://www.itu.int/rec/R-REC-M.2160-0-202311-I/en).

[30] 	3GPP TS 23.502: "Procedures for the 5G System".

[31]	Hexa-X-II, "Deliverable D1.2: 6G Use Cases and Requirements", December 2023.

[32]	https://www.easa.europa.eu/en/what-is-uam.

[33]	https://www.autoflight.com/en/air/.

[34]	"Low altitude economy development white paper (2.0) All digital solutions", International digital economy academy (IDEA), 2023.

[35]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP; Stage 1".

[36]	https://www.unmannedairspace.info/latest-news-and-information/shenzhen-reaches-record-number-of-drone-operations-with-600000-flights-in-2023/.

[37]	"C-V2X Use Cases and Service Level Requirements Volume II", 5GAA White Paper, 2023.

[38]	https://www.flyeye.io/drone-acronym-daa/.

[39]	Report ITU-R M.2516-0, "Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond ". (https://www.itu.int/dms_pub/itu-r/opb/rep/R-REP-M.2516-2022-PDF-E.pdf).

[40]				3GPP TR 33.841: "Study on the support of 256-bit algorithms for 5G".

[41]	3GPP TS 35.246: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 1: algorithm specification".

[42]	3GPP TS 35.247: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 2: implementation test data".

[43]	3GPP TS 35.248: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 3: design conformance test data".

[44]	3GPP TR 22.883: "Feasibility Study on Energy Efficiency as service criteria Phase 2".

[45]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[46]	3GPP TS 38.211: "NR; Physical channels and modulation".

[47]	3GPP TS 36.305: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[48]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[49]	3GPP TS 22.856: "Study on Localized Mobile Metaverse Services".

[50]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[51]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

[52]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[53]	3GPP TS 22.179: “Mission Critical Push to Talk (MCPTT); Stage 1”.

[54]	3GPP TS 22.280: “Mission Critical Services Common Requirements (MCCoRe); Stage 1”..

[55]	3GPP TS 22.281: “Mission Critical (MC) video”.

[56]	3GPP TS 22.282: “Mission Critical (MC) data”.

[57]	3GPP TS 22.262: “Message service within the 5G System (5GS); Stage 1”.

[58]	3GPP TS 22.101: “Service aspects; Service principles”.

[59]	3GPP TS 22.173: “IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1”.

[60]	3GPP TS 22.011: “Service accessibility”.

[61]	3GPP TS 22.071: “Location Services (LCS); Service description; Stage 1”.

[62]	3GPP TS 22.268: “Public Warning System (PWS) requirements”.

[63]	3GPP TS 22.153: “Multimedia priority service”.

[64]	3GPP TS 22.104: “Service requirements for cyber-physical control applications in vertical domains”.

[65]	3GPP TS 22.185: “Service requirements for V2X services”.

[66]	3GPP TS 22.186: “Service requirements for enhanced V2X scenarios”.

[67]	3GPP TS 22.263: “Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)”.

[68]	3GPP TS 22.115: “Service aspects; Charging and billing”.

[69]	3GPP TS 22.289: “Mobile communication system for railways”.

[70]	3GPP TS 22.468: “Group Communication System Enablers (GCSE)”.

[71]	3GPP TS 22.368: “Service requirements for Machine-Type Communications (MTC); Stage 1”.

[72]	3GPP TR 22.949: “Study on a generalized privacy capability”.

[73]	3GPP TR 33.849: “Study on subscriber privacy impact in 3GPP”.

[74]	3GPP TS 22.278: “Service requirements for the Evolved Packet System (EPS)”.

[75]	ETSI GR SAI 004 (V1.1.1): "Securing Artificial Intelligence (SAI); Problem Statement".

[76]	ETSI GR SAI 005 (V1.1.1): "Securing Artificial Intelligence (SAI); Mitigation Strategy Report".

[77]	3GPP TR 28.915: "Study on management aspects of Network Digital Twin".

[78]	3GPP TS 23.527: “5G System; Restoration procedures”.

[79]	3GPP TR 29.866: “Study on IMS Disaster Prevention and Restoration Enhancement”.

[80]	Enabling Mobile AI Agent in 6G Era: Architecture and Key Technologies; https://dl.acm.org/doi/abs/10.1109/MNET.2024.3422309

[81]	Wang L, Ma C, Feng X, et al. A survey on large language model based autonomous agents[J]. Frontiers of Computer Science, 2024, 18(6): 186345.

[82]	Hong S, Zheng X, Chen J, et al. Metagpt: Meta programming for multi-agent collaborative framework[J]. arXiv preprint arXiv:2308.00352, 2023.[xx]	ISO/IEC 22989: "Information technology — Artificial intelligence — Artificial intelligence concepts and terminology"

[83] 	GSMA Foundry, “Case Study: Delivering Real-Time Translation”,2023, Reference links for https://www.gsma.com/get-involved/gsma-foundry/5g-new-calling/

[84] 	GSMA Foundry, “Case Study: Remote damage assessment ”,2023, Reference links for https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/remote-damage-assessment/

[85]	 GSMA Foundry, “Case Study: Visualised Video-Calling”,2023, Reference links for  https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/visualised-voice-calling/

[86] 	GSMA Foundry, “Case Study: Adding New Value for Voice Call”,2024, Reference links for https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/adding-new-value-to-voice-calls/

[87]	United Nations Sustainable Development Goals (https://sdgs.un.org/goals).

[88]	United Nations Global Digital Compact (https://www.un.org/global-digital-compact/sites/default/files/2024-09/Global%20Digital%20Compact%20-%20English_0.pdf).

[89]	Zhang Z, Wang S, Hong Y, et al. Distributed dynamic map fusion via federated learning for intelligent networked vehicles[J]. arXiv preprint arXiv:2103.03786, 2021

[90]	AECC: Operational Behavior of a High Definition Map Application – White Paper Version 1.0.0

[91]	 https://spectrum.ieee.org/boats-should-be-sleek-but-only-up-to-a-point

[92]	https://www.marketsandmarkets.com/Market-Reports/evtol-aircraft-market-28054110.html

[93]	https://www.fortunebusinessinsights.com/autonomous-vehicle-market-109045

[94] 	5G-ACIA White Paper, “Use Cases and Requirements for Integrated Sensing and Communication (ISAC) in Connected Industries and Automation”

[95]	ETSI GR ISC-001 V1.0.0 group report, “Integrated Sensing And Communications (ISAC); Use Cases and Deployment Scenarios”

[96]	3GPP TR 37.885 Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR.

[97]	3GPP TR 22.865: “Study on satellite access - Phase 3”.

[98]                   	3GPP TR 22.836: "Study on Asset Tracking Use Cases".

[99]	CHINA AIRSPACE CLASSIFICATION AND PILOT UPDATE https://www.icao.int/APAC/Meetings/2024%20ATMSG12/IP09%20China%20Airspace%20Classification%20and%20Pilot%20Update.pdf#search=AIRSPACE%20CLASSIFICATION.

[100]	3GPP TS 38.171: "NR; Requirements for support of Assisted Global Navigation Satellite System (A-GNSS)".

[101] 	https://cloud.tencent.com/developer/article/1654264.

[102] 	ITU G.114: One-way transmission time.

[103]	S4aV200634: "Traces and Configurations for VR2, CG and AR2".

[104]	ITU-T FG NET-2030, "Representative use cases and key network requirements for Network 2030", 2020.

[105]	Ian F. Akyildiz, Hongzhi Guo, "Holographic-type communication: A new challenge for the next decade", ITU Journal on Future and Evolving Technologies, Volume 3 (2022), Issue 2, Pages 421-442.

[106]	X. Xu, Y. Pan, P. P. M. Y. Lwin and X. Liang, "3D holographic display and its data transmission requirement," 2011 International Conference on Information Photonics and Optical Communications, Jurong West, Singapore, 2011, pp. 1-4, doi: 10.1109/IPOC.2011.6122872.

[107]	P. Ekdahl, T. Johansson, A. Maximov, and J. Yang, "A new SNOW stream cipher called SNOW-V", IACR Transactions on Symmetric Cryptology, vol. 2019, no. 3, pp. 1-42, 2019. doi: 10.13154/tosc.v2019.i3.1-42.

[108]	P. Ekdahl, T. Johansson, A. Maximov, and J. Yang, "SNOW-Vi: an extreme performance variant of SNOW-V for lower grade CPUs ", WiSec '21, pp. 261 – 272, 2021. https://doi.org/10.1145/3448300.3467829

[109]	IEC 62056-6-1:2023 Object Identification System (OBIS).

[110]	IEC 62056-6-2:2023 COSEM interface classes.

[111]	IEC 62056-5-3:2023 DLMS/COSEM application layer.

[112]	Hexa-X-II, “Deliverable D1.4, 6G Value, Requirements, and Ecosystem”, April 2025.

[113] 	ITU-T Recommendation Y.3090 (02/22): "Digital twin network - Requirements and architecture" (https://www.itu.int/rec/T-REC-Y.3090-202202-I).

[114]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[115] 	Urban Air Mobility, https://www.airbus.com/en/innovation/low-carbon-aviation/urban-air-mobility, AIRBUS, accessed in May 2024.

[116] 	Urban Air Mobility (UAM) Concept of Operations, https://www.faa.gov/air-taxis/uam_blueprint, Federal Aviation Administration of U.S. Department of Transportation, Aug. 2023.

[117] 	Urban Air Mobility, https://rotorcraft.arc.nasa.gov/Research/Programs/UrbanAirMobility.html, U.S. NASA, 2024.

[118] 	NASA Urban Air Mobility (UAM) Reference Vehicles, https://sacd.larc.nasa.gov/uam-refs/, U.S. NASA, Jan. 2024

[119] 	Future Mobility, https://www.hyundai.com/worldwide/en/newsroom/detail/0000000093.html, Hyundai Motor Company, Jul. 2022.

[120]	LG Uplus, Near-Future Vertical Applications in Korea: K-UAM, https://nextgalliance.org/wp-content/uploads/2024/05/Near-future-Vertical-Applications-in-Korea-K-UAM_6GF_NGA_Joint_Workshop_JKim_final.pdf, Next G Alliance / Korea 6G Forum Joint Workshop

[121]	A. Baltaci, et. al., “A Survey of Wireless Networks for Future Aerial Communications (FACOM),” IEEE Communications Surveys & Tutorial, Vo. 23, No. 4, Q4 2021. https://ieeexplore.ieee.org/document/9508366.

[122]	3GPP TR 22.887: "Feasibility Study on satellite access – Phase 4".

[123]	Next G Alliance Report “6G Applications and Use Cases”: Use Case 4.6 – High-Speed Wireless Connection in Aerial Vehicle for Entertainment Service.

[124]	Topal, O. A., Schupke, D., Björnson, E., & Cavdar, C. ”Optimal Joint Access Point Placement and Resource Allocation for Indoor mmWave Communications”, ICC 2023, Rome, Italy.

[125]	Hofmann, S.; Duhovnikov, S.; Schupke, D., “Massive Data Transfer from and to Aircraft on Ground: Feasibility and Challenges,” IEEE Aerospace and Electronic Systems Magazine, vol. 36, iss. 5, May 2021.

[126] 	NASA. UTM: Air traffic management for low-altitude drones[EB/OL] (2015-10) (https://www.nasa.gov/sites/default/files/atoms/files/utm-factsheet-11-05-15.pdf)

[127] 	SESAR. U-space blueprint[EB/OL]. (2017-06-09). (https://www.sesarju.eu/sites/default/files/documents/reports/U-Space%20Blueprint%20brochure%20final.pdf)

[128] 	AOPA China. Regulations for the operation of light and small UAVs [EB/OL] (2018-04-19). (http://www.aopa.org.cn/usr/1/upload/file/20180419/15241267234030958.pdf)

[129] 	HIROYUKI U. UTM project in Japan[EB/OL]. (2017-06-26)[2025-01-20]. (https://gutma.org/montreal-2017/wp-ontent/uploads/sites/2/2017/07/UTM-Project-in-Japan_METI.pdf).

[130] 	G. Wagner and H. Choset, “A complete multirobot path planning algorithm with performance bounds,” Proc. In IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS), San Francisco, CA, USA, Sept. 2011. Online Link.

[131]	B. Hu, S. Xu, and Z. Cao, “Multi-Robot Path Planning for Each Robot with Several Jobs in a Single Trip,” IFAC-PapersOnline, Volume 53, Issue 5, 2020, Pages 279-284. Online Link.

[132]	S. Lin, A. Liu, J. Wang and X. Kong, “A Review of Path-Planning Approaches for Multiple Mobile Robots,” Section of Automation and Control Systems, MDPI Machines, 10(9), 773, Sept. 2022. Online Link.

[133]	A New Approach to Data Sharing - Open data sharing and collaboration for data, analytics and AI, DataBricks eBook 3rd Ed. Online Link.

[134]	K.-D. Lee, “A Smart Network of Service Robots: Technical Challenges and Design Considerations,” IEEE Communications Magazine, pp. 28-34, vol. 59, Iss. 8, Aug. 2021. Online Link.

[135]	Network-Enabled Robotic and Autonomous Systems, Next G Alliance, May 2023. Online Link.

[136]	3GPP TR 22.916: "Study on Network of Service Robots with Ambient Intelligence; Stage 1".

[137]	3GPP TS 23.548: “5G System Enhancements for Edge Computing; Stage 2”.

[138]	3GPP TS 22.228:  “Service requirements for the Internet Protocol (IP) multimedia core network subsystem (IMS); Stage 1”.

[139]	3GPP TS 28.105: “Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management”.

[140]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”.

[141]	3GPP TS 23.503: “	Policy and charging control framework for the 5G System (5GS); Stage 2”.

[142]	3GPP TS 23.228: “IP Multimedia Subsystem (IMS); Stage 2”.

[X]                      NIST CyberSecurity Framework “The CSF 1.1 Five Functions”,https://www.nist.gov/cyberframework/getting-started/online-learning/five-functions?ref=hackernoon.com.

* * * End of Changes * * * *