3GPP TSG SA WG 1 Meeting #112 	S1-254295

17-21 November 2025, Dallas, Texas, USA	(revision of S1-25xxxx)

Source:	6G Study Rapporteurs

pCR Title:	Merged Proposed changes on Clause Y.1.11 (Ubiquitous)

Draft Spec:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.10

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: Rapporteur’s collection of proposed changes to CPRs in Clause Y.1.11

1. Introduction

The following TDOCs submitted into SA1 # 112 have been collected into this document to faciliitate discussion and approval of CPRs.

S1-254001, 2nd Draft Agenda for SA1#112 was used as the basis of this contribution.

- S1-54020 (Clause Y.1.11 from zip file)

- S1-254120  and S1-254121 (CATT) – DP and Y.1.11 (Tables Y.1.11-1, -2, New Table Y.1.11-3)

- S1-254163 (ZTE Corporation, CSCN) – Y.1.11 (Tables Y.1.1-1 and -2)

- S1-254250 (Qualcomm)

- S1-254300r1

Editor’s Note: If some of these are approved, changes to CPR Tables from other clauses  may need to be aligned.

2. Reason for Change

There are several proposals to consolidate functional requirements. This combines the proposals to facilitate discussions in the meeting.

3. Proposal

It is proposed to disscuss and agree the following changes for inclusion into Clause Y in 3GPP TR 22.870 v0.4.1.

* * * First Change: Y.11 and Table Y.1.11-1 – NTN /satellite access communication* * *

### Y.1.11	Ubiquitous Connectivity

Editor’s Note: This clause contains consolidated potential requirements for 6G that include (but not limited to): support for non-terrestrial networks (NTN) and support for positioning services and capabilities. This includes (but not limited to) PRs from clause 8.

NEW Editor’s Note: PR 8.2.6-1 and PR 8.17.6-1 have Editor’s Notes associated with them, so they are not included for consolidation into Table Y.1.11-1 at this time.

NEW Editor’s Note: PRs that only state a requirement to meet KPIs in a “Table” will be revised to point to the appropriate consolidated tables in Y.2.

Proposed Comments/Changes:

S1-254121 (CATT) uses “Table Y.1.11-1 – NTN Communication” as the Table Title.

S1-254250 (Qualcomm) proposes to change Table Y.1.11-1 – Satellite based communication and Table Y.1.11-2 – Satellite-based Positioning

S1-254250 (Qualcomm): [QC] no need to include functional CPRs pointing to KPI, so I removed those.

Editor’s Note: these PRs are: Y.1.11-1-15 and -16, Y.1.11-2-5, -6, -7, and -8.

S1-254300r1 (Huawei): adds 2 ENs and a new Table Y.1.11-3 – Other aspects related to ubiquitous communication

Editor’s NOTE: How to consolidate computing service PRs (i.e. [PR 8.9.6-1], [PR 8.15.6-2]) in the context of ubiquitous communication need to consider, e.g. whether/how compelling/relevant is spaceborne SHE.

Editor’s NOTE: For Ubiquitous, more input will be provided regarding CPRs as well as KPI values.

Table Y.1.11-1 – NTN /satellite accesscommunication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-1-1 | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimize the coverage per LEO satellite. | PR 8.3.6-1 PR 8.3.6-3 | LEO Coverage Flex configs Proposed to be changed by S1-254121 From S1-254250 [QC} this PR may need some further rewording / clarification. Our stage-2 folks cannot understand the meaning of it. |
| Y.1.11-1-2 | The 6G system with satellite access shall support service continuity when UE is within the optimized coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 | LEO Service continuity Proposed to be changed by S1-254121 Proposed to be changed by S1-254163 Proposed to be changed by S1-254250 |
| Y.1.11-1-3 | The 6G system with satellite access shall support service continuity within NTN, and between TN and NTN, minimising the interruption to the UE during the transition | PR 8.4.6-1 PR 8.8.6-1 | Service continuity TN-NTN Proposed to be changed by S1-254121 Proposed to be changed by S1-254163 Proposed to be changed by S1-254250 |
| Y.1.11-1-4 | The 6G system with satellite access, shall support SMS delivery to a high density of UEs i.e. up to [1000] UE per km2. | PR 8.4.6-2 | KPI (Y.2) high density # of UEs SMS Proposed to be changed by S1-254121 Proposed to be changed by S1-254163 Proposed to be changed by S1-254250 |
| Y.1.11-1-5 | Subject to regulatory requirements, the 6G system with satellite access, shall support text messaging services and PWS [62] for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 PR 8.3.6-3 | PWS Text Messaging Adverse propagation conditions Proposed to be changed by S1-254121 Proposed to be changed by S1-254163 |
| Y.1.11-1-6 | The 6G system with satellite access, shall be able to provide eMBB service to UE mounted on SWAP constrained moving platforms (e.g. flying drone, small vessels mounted). | PR 8.6.6-1 | eMBB Proposed to be changed by S1-254121 Proposed to be changed by S1-254163 Proposed to be changed by S1-254250 |
| Y.1.11-1-7 | The 6G system with satellite access shall be able to support communication service for UEs (e.g. UAV) with variant altitudes (e.g. from 0 to 3 km [99]) with satellite access and/or terrestrial access. | PR 8.9.6-2 | Communication service UEs at variant altitudes Proposed to be changed by S1-254121 Proposed to be changed by S1-254163 Proposed to be changed by S1-254250 |
| Y.1.11-1-8 | The 6G system shall support communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network. | PR 8.13.6-1 | UE-UE direct communications Proposed to be changed by S1-254121 Proposed to be changed by S1-254163 Proposed to be changed by S1-254250 |
| Y.1.11-1-9 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 | Satellite backhaul links selection/switching Proposed to be changed by S1-254121 Proposed to be changed by S1-254163 Proposed to be changed by S1-254250 |
| Y.1.11-1-10 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization Proposed to be changed by S1-254121 Proposed to be changed by S1-254163 Proposed to be changed by S1-254250 |
| Y.1.11-1-11 | The 6G system using satellite access based on regenerative satellites shall be able to support the transfer of computing information (e.g. pre-processed data within the service hosting environment) between satellites over a given area. | PR 8.15.6-2 | Satellite-Satellite transfer of computing info Proposed to be changed by S1-254121 Proposed to be changed by S1-254163 Proposed to be changed by S1-254250 Proposed to be deleted by S1-254300r1 |
| Y.1.11-1-12 | Subject to operator’s policy, the 6G network with satellite access shall support onboard Service Hosting Environment(s) and mechanisms to - Minimise the necessary bandwidth of the inter-satellite and feeder links, - Modify the path for routing data traffic between a UE and the Service Hosting Environment to minimize service interruption considering the movement of UE and/or satellite, - provide a computing service via a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities, and - subject to user consent, support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) provided by Service Hosting Environment on aboard satellite. | PR 8.15.6-1 PR 8.9.6-1 PR 8.9.6-3 PR 8.9.6-4 | Onboard Service Hosting Environment Modification of data routing Computing service UE Data Sharing Non-3GPP sensing Proposed to be changed by S1-254121 Proposed to be changed by S1-254163 Proposed to be deleted by S1-254300r1 |
| Y.1.11-1-13 | The 6G system using satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions, e.g. using signalling optimization, codec rate adaptation. | PR 8.18.6-1 | QoS Improvement Proposed to be changed by S1-254121 Proposed to be changed by S1-254163 |
| Y.1.11-1-14 | The 6G system with satellite access shall be able to provide eMBB service with KPIs from Table 8.6.6-1: KPIs of the 6G satellite access network to support the use case disaster relief [112]. | PR 8.6.6-2 | KPI eMBB Proposed to be changed by S1-254121 Proposed to be changed by S1-254163 Proposed to be deleted by S1-254250 |
| Y.1.11-1-15 | The KPIs in Table 8.8.6-1: KPIs for global mobile video should be met for global mobile video service. | PR 8.8.6-2 | KPI Global mobile video service Proposed to be deleted by S1-254250 |

Company Proposals:

- Y.1.11-1-1:

  - S1-254121: (CATT)

| Y.1.11-1-1 | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimize the coverage per LEO satellite for sparse satellite constellation. | PR 8.3.6-1 PR 8.3.6-3 | [Staellite access] LEO Coverage Flex configs |
| --- | --- | --- | --- |

- Y.1.11-1-2:

  - S1-254121: (CATT)

| Y.1.11-1-2 | The 6G system with satellite access shall support service continuity when UE is within the optimized coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 | [Satellite access] Service continuity LEO |
| --- | --- | --- | --- |

  - S1-254163 (ZTE/CSCN)

|  |  |  | [ZTE] merged into PR 8.4.6-1 |
| --- | --- | --- | --- |

  - S1-254250 (Qualcomm)

| Y.1.11-1-2 | The 6G system with satellite access shall support service continuity when UE is within the coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 | LEO Service continuity |
| --- | --- | --- | --- |

- Y.1.11-1-3:

  - S1-254121: (CATT)

| Y.1.11-1-3 | The 6G system with satellite access shall support service continuity (with minimum service interruption) for UEs moves between different non-terrestrial access networks (e.g. satellite, HAPS), or between terrestrial access network and non-terrestrial access network. | PR 8.4.6-1 PR 8.8.6-1 | [Satellite access] Service continuity TN-NTN |
| --- | --- | --- | --- |

  - S1-254163 (ZTE/CSCN)

| Y.1.11-1-2 | The 6G system with satellite access shall support service continuity with minimum interruption for UEs within NTN, and between TN and NTN, and within the optimized coverage of the LEO constellation, (e.g. in the sparse LEO scenarios, minimising the interruption to the UE during the transition). | PR 8.4.6-1 PR 8.8.6-1 PR 8.3.6-2 | Service continuity TN-NTN, NTN, LEO |
| --- | --- | --- | --- |

  - S1-254350 (Qualcomm)

|  |  |  | [already part of existing 5GS requirements |
| --- | --- | --- | --- |

- Y.1.11-1-4:

  - S1-254121: (CATT): moves this PR to Y.2.

  - S1-254163 (ZTE/CSCN): suggested to merged into UC KPI table 8.x.2-1

  - S1-254250 (Qualcomm): proposes to delete ([part of KPIs])

- Y.1.11-1-5:

  - S1-254121: (CATT)

| Y.1.11-1-4 | Subject to regulatory requirements, the 6G system with satellite access, shall support text messaging services and PWS for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 PR 8.3.6-3 | [Satellite access] PWS Text Messaging |
| --- | --- | --- | --- |

  - S1-254163 (ZTE/CSCN)

| Y.1.11-1-3 | Subject to regulatory requirements, the 6G system with satellite access, shall support text messaging services and PWS [62] for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 | PWS Text Messaging Adverse propagation conditions |
| --- | --- | --- | --- |

- Y.1.11-1-6:

  - S1-254121: (CATT)

| Y.1.11-1-5 | The 6G system with satellite access, shall be able to provide broadband communication service to UEs mounted on Size, Weight and Power constrained moving platforms (e.g. flying drone, small vessels mounted). | PR 8.6.6-1 | [Satellite access] eMBB |
| --- | --- | --- | --- |

  - S1-254163 (ZTE/CSCN)

|  |  |  | [ZTE] It is suggested to merge it with KPI table 8.6.6-1 |
| --- | --- | --- | --- |

  - S1-254250 (Qualcomm)

|  |  |  | [QC] unclear what is not already supported by 5GS requirements. Maybe converted in eMBB type of KPIs |
| --- | --- | --- | --- |

- Y.1.11-1-7:

  - S1-254121: (CATT): moves this PR to Y.2 and inserts PR 11.5.6.2.

| Y.1.11-1-7 | The 6G system with satellite access shall provide means to obtain and expose service interruption information (e.g. estimated interruption time) to authorized third party to minimize interruption for immersive media services during UE moves between terrestrial access and satellite access network. | PR 11.5.6-2 | [Satellite access] Information exposure for Service continuity |
| --- | --- | --- | --- |

  - S1-254163 (ZTE/CSCN)

| Y.1.11-1-4 | The 6G system with satellite access shall be able to support communication service for UEs (e.g. UAV) with variant altitudes (e.g. from 0 to 3 km [99]) with satellite access and/or terrestrial access. | PR 8.9.6-2 | Communication service UEs at variant altitudes |
| --- | --- | --- | --- |

  - S1-254250 (Qualcomm)

|  |  |  | [QC] unclear what is not already supported by 5GS requirements. |
| --- | --- | --- | --- |

- Y.1.11-1-8:

  - S1-254121: (CATT)

| Y.1.11-1-8 | The 6G system shall support communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network. | PR 8.13.6-1 | [Other non-terrestrial access] UE-HAS-UE communications |
| --- | --- | --- | --- |

  - S1-254163 (ZTE/CSCN)

| Y.1.11-1-5 | The 6G system shall support communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network. | PR 8.13.6-1 | UE-UE direct communications |
| --- | --- | --- | --- |

  - S1-254250 (Qualcomm)

|  |  |  | QC] unclear what is not already supported by 5GS requirements (TN and SAT UE-UE comm.). |
| --- | --- | --- | --- |

- Y.1.11-1-9:

  - S1-254121: (CATT)

| Y.1.11-1-9 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 | [Satellite backhaul] links selection/switching |
| --- | --- | --- | --- |

  - S1-254163 (ZTE/CSCN)

| Y.1.11-1-6 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 | Satellite backhaul links selection/switching |
| --- | --- | --- | --- |

  - S1-254250 (Qualcomm)

| Y.1.11-1-6 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 | Satellite backhaul links selection/switching [QC] should be moved out of this table (about SAT access), e.g. can go in “others” |
| --- | --- | --- | --- |

- Y.1.11-1-10:

  - S1-254121: (CATT): moves to Y.11-3

  - S1-254163 (ZTE/CSCN)

| Y.1.11-1-8 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization |
| --- | --- | --- | --- |

  - S1-254250 (Qualcomm)

| Y.1.11-1-10 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization [QC] prefer to leave this TBD, and discuss more about “time-synchronization” |
| --- | --- | --- | --- |

- Y.1.11-1-11:

  - S1-254121: (CATT) moves to Y.1.11-3

  - S1-254250 (Qualcomm): [QC] can go in “others” (together with SAT BH)

- Y.1.11-1-12:

  - S1-254121: (CATT) moves to Y.1.11-3

  - S1-254163 (ZTE/CSCN)

| Y.1.11-1-9 | Subject to operator’s policy, the 6G network with satellite access shall support onboard Service Hosting Environment(s) to m inimise the necessary bandwidth of the inter-satellite and feeder links, and provide a computing service via to UEs (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities . | PR 8.15.6-1 PR 8.9.6-1 | Onboard Service Hosting Environment Modification of data routing Computing service UE Data Sharing Non-3GPP sensing [ZTE] It is suggested to seperate which are targeting different aspects. |
| --- | --- | --- | --- |
| Y.1.11-1-10 (new) | Subject to operator’s policy and user’s consent, the 6G system with satellite access shall be able to support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) , provided by Service Hosting Environment on aboard satellite. | PR 8.9.6-3 |  |
| Y.1.11-1-11 (new) | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimize service interruption considering the movement of UE and/or satellite. | PR 8.9.6-4 |  |

- Y.1.11-1-13:

  - S1-254121: (CATT)

| Y.1.11-1-6 | The 6G system using satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions, e.g. using signalling optimization, codec rate adaptation. | PR 8.18.6-1 | [Satellite access] QoS Improvement |
| --- | --- | --- | --- |

  - S1-254163 (ZTE/CSCN)

| Y.1.11-1-13 | The 6G system using satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions,. | PR 8.18.6-1 | QoS Improvement [ZTE] remove the examples which are solution oriented. |
| --- | --- | --- | --- |

- Y.1.11-1-14:

  - S1-254163 (ZTE/CSCN)

| Y.1.11-1-14 | The 6G system with satellite access shall be able to provide eMBB service to UE mounted on SWAP(Size, Weight and Power) constrained moving platforms (e.g. flying drone, small vessels mounted) with KPIs from Table 8.6.6-1: KPIs of the 6G satellite access network to support the use case disaster relief [112]. | PR 8.6.6-2 | KPI eMBB |
| --- | --- | --- | --- |

- Y.1.11-1-15:

  - S1-254121: (CATT) adds a new PR

| Y.1.11-1-10 | Subject to operator’s policy, the 6G network shall support mechanisms to connect the base stations (onboard satellite) and the ground core networks using 3GPP technology. | PR 8.17.6-1 | [Satellite backhaul] New connection method |
| --- | --- | --- | --- |

NEW Editor’s Note: PR 8.17.6-1 has an Editor’s Note associated with it, so it is not included for consolidation into Table Y.1.11-1 at this time.

* * * Second Change: Table Y.1.11-2 – NTN-based Positioning * * *

Table Y.1.11-2 – NTN-based Positioning

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-2-1 | Subject to regulatory requirements, operator policy, and user consent, the 6G system with satellite access shall be able to determine and provide the UE location meeting the regulatory requirements for both sparse and dense satellite constellations. | PR 8.3.6-4 PR 8.6.6-1 | UE Location Regulatory Support Proposed to be changed by S1-254121 |
| Y.1.11-2-2 | The 6G system with satellite access shall be able to provide a mechanism for energy constrained devices to determine its position using 3GPP positioning technology. | PR 8.7.6-1 | Energy constrained devices Proposed to be changed by S1-254121 Proposed to be changed by S1-254250 |
| Y.1.11-2-3 | The 6G system with satellite access shall be able to provide a positioning service supported by the combination of NTN and GNSS positioning technologies, including Network-based Assisted GNSS. | PR 8.11.6-1 | Support of NTN & GNSS positioning technologies Proposed to be changed by S1-254250 |
| Y.1.11-2-4 | The 6G system with terrestrial and satellite access shall be able to provide integrity for a positioning service. | PR 8.16.6-1 | Positioning integrity |
| Y.1.11-2-5 | The 6G system with satellite access shall be able to provide the UE with positioning service with 3GPP technologies, independently of non-3GPP positioning technologies (e.g. GNSS) with the KPIs in Table 8.5.6-1: Performance requirements for satellite-based positioning services | PR 8.5.6-1 | KPI Proposed to be changed by S1-254121 Proposed to be deleted by S1-254250 |
| Y.1.11-2-6 | The 6G system with satellite access shall be able to provide positioning service following the KPIs in Table 8.7.6-1: Performance requirements for satellite-based positioning services for energy constrained devices from [338] | PR 8.7.6-2 | KPI Energy constrained devices Proposed to be deleted by S1-254250 |
| Y.1.11-2-7 | The 6G system with terrestrial and satellite access shall be able to provide positioning service for UE type e.g. Airplane Mounted and UAV Mounted with high availability, by using 3GPP positioning technologies and fulfilling the KPIs in Table 8.10.6-1: Performance requirements for positioning services based on hybrid TN and NTN positioning technologies. | PR 8.10.6-1 | KPI Proposed to be changed by S1-254121 Proposed to be deleted by S1-254250 |
| Y.1.11-2-8 | The 6G system with satellite access shall be able to provide positioning service with the KPIs from Table 8.11.6-1: Performance requirements for positioning services with satellite access | PR 8.11.6-2 | KPI Proposed to be deleted by S1-254250 |

Company Proposals:

- Y.1.11-2-1:

  - S1-254121: (CATT)

| Y.1.11-2-1 | The 6G system with satellite access shall be able to provide the positioning service in compliant with regulatory requirements to UEs under the coverage of sparse or dense satellite constellations. | PR 8.3.6-4 | [General] Regulatory Support |
| --- | --- | --- | --- |

- Y.1.11-2-2:

  - S1-254121: (CATT) merges to (new) PR-Y.1.11-2-3

  - S1-254250 (Qualcomm)

| Y.1.11-2-2 | The 6G system with satellite access shall be able to provide a mechanism for a energy constrained UE to determine its position using 3GPP positioning technology. | PR 8.7.6-1 | Energy constrained devices |
| --- | --- | --- | --- |

- Y.1.11-2-3:

  - S1-254121: (CATT)

| Y.1.11-2-4 | The 6G system with satellite access shall be able to provide a positioning service to a UE using satellite access, supported by the combination of 3GPP satellite-based positiniong method and GNSS-based positioning technologies (e.g. pure GNSS, Network-based Assisted GNSS). | PR 8.11.6-1 | Support of NTN & GNSS positioning technologies |
| --- | --- | --- | --- |

  - S1-254250 (Qualcomm)

| Y.1.11-2-3 | The 6G system with satellite access shall be able to provide a positioning service supported by the combination of 3GPP satellite and GNSS positioning technologies, including Network-based Assisted GNSS. | PR 8.11.6-1 | Support of NTN & GNSS positioning technologies |
| --- | --- | --- | --- |

- Y.1.11-2-4:

  - S1-254121: (CATT)

| Y.1.11-2-2 | The 6G system with satellite access shall be able to determine the integrityof a positioning service and provide the assistance information related integrity to a UE. | PR 8.16.6-1 | [General] Positioning integrity |
| --- | --- | --- | --- |

- Y.1.11-2-7:

  - S1-254121: (CATT) merges to (new) PR-Y.1.11-2-3

- Y.1.11-2-5:

  - S1-254121: (CATT)

| Y.1.11-2-3 | The 6G system with satellite access shall be able to provide different positioning service to different types of UE (e.g. energy constrained device, UAV, airplane) using satellite access, supported by single satellite based positioning method or hybrid 3GPP methods. positioning NOTE: hybrid 3GPP positioning methods include the combination of 3GPP satellite based positioning technology and various legacy RAT dependent positioning methods such as U- TDOA, E-OTD, etc. | PR 8.5.6-1 PR 8.7.6-1 PR 8.10.6-1 | Positioning Service based on satellite-based positioning technology |
| --- | --- | --- | --- |

- Y.1.11-2-1:

  - S1-254121: (CATT) adds a new (separate) requirement from Y.1.11-2-1.

| Y.1.11-2-5 | Subject to regulatory requirements, operator policy, and user’s consent, the 6G system with satellite access shall be able to determine UE location based on 3GPP positioning technologies for UE only using satellite access. | PR 8.5.6-2 | Determine UE location using satellite-based positioning |
| --- | --- | --- | --- |

* * * Third Change: New Table Y.1.11-3 * * *

- Y.1.11.-3:

  - S1-254121 (CATT): add the following to Table Y.1.5-1 with some of the PRs from Table Y.1.11-1

Table Y.1.11-3 – Other NTN capabilities

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-3-1 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization |
| Y.1.11-3-5 | The 6G system with satellite access shall be able to support the transfer of computing information (e.g. pre-processed data) of a computing task between Service Hosting Environments on board satellites over a given area. | PR 8.15.6-2 | Data transfer between onboard computing resources |
| Y.1.11-3-3 | Subject to operator’s policy and user’s consent, the 6G system with satellite access shall be able to share the data (e.g. the processed result based on the non-3GPP sensing data from multiple UEs) provided by the Service Hosting Environment on board satellite among multiple UEs (e.g. UAV). | PR 8.9.6-3 | Computing data sharing |
| Y.1.11-3-4 | Subject to operator’s policy, the 6G network with satellite access shall be able to support modifying the path for routing data traffic between a UE and Service Hosting Environment on board satellites to minimize service interruption considering the movement of UE and/or satellite. | PR 8.9.6-4 | Routing modification |
| Y.1.11-3-2 | Subject to operator’s policy and agreement with 3rd party, the 6G network with satellite access shall be able to provide a computing service via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities, or minimise the necessary bandwidth of the inter-satellite and feeder links. | PR 8.15.6-1 PR 8.9.6-1 | Onboard Computing |

  - S1-254300r1 (Huawei) added a new Table with the following PRs.

Table Y.1.11-3 – Other aspects related to ubiquitous communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-3-1 | Subject to operator’s policy, the 6G network with satellite access may support onboard Service Hosting Environment(s) and mechanisms to - Minimise the necessary bandwidth of the inter-satellite and feeder links, - Modify the path for routing data traffic between a UE and the Service Hosting Environment to minimize service interruption considering the movement of UE and/or satellite, - provide a computing service via a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities, and - subject to user consent, support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) provided by Service Hosting Environment on aboard satellite. This implies some need to transfer computing information in/related to Service Hosting Environment (e.g. pre-processed data within the service hosting environment) between satellites over a given area. | PR 8.15.6-1 PR 8.9.6-3 PR 8.9.6-4 PR 8.15.6-2 | Onboard Service Hosting Environment Modification of data routing Computing service UE Data Sharing Non-3GPP sensing and transfer of computing service related info related to SHE Spaceborne SHE, how compelling? |
| Y.1.11-3-2 | Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access may be able to provide to logistic companies computing service in a given area via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities. | PR 8.9.6-1 | Spaceborne SHE, how compelling? |
|  | [PR 8.9.6-1] Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access shall be able to provide a computing service via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities | [PR 8.9.6-1] |  |

* * * End of Changes * * * *