3GPP TSG-SA WG1 Meeting #112	S1-254364

17-21 November 2025, Dallas, Texas, USA	(revision of S1-254157r1, S1-253322)

Source:	Ericsson

pCR Title:	Solving two editor’s note’s in 6.38 AI for disability

Draft Spec:	3GPP TR 22.870 0.4.1

Agenda item:	8.1.3.1 Artificial Intelligence Editor note solving

Document for:	Agreement

Contact:	Daniel.lonnblad at ericsson.com

Abstract: This pCR introduce some discussion and explanation about the KPI’s in the table with the intent to solve the Editors notes.

1. Introduction

Two editors notes where added to the use case AI for disability support, Intention is to add some extra information how the KPI’s are estimated.

2. Reason for Change

The first Editor’s Note “The value of Application round trip latency are for FFS”.

First of all we propose to clarify that the latency intended here is the extra latency that the disability function will introduce. Therefore we have updated the heading in the latency column to be “Additional latency for disability service”

This will be dependent on what type of support the user would need. If voice should be enhanced in a voice call the overall latency of the voice call including the enhancement should not be larger than 300 to 400ms to not break the overeall experience of the voice call. 300-400 ms can be seen as a upper bound for voice. This needs to take both the latency in the network (roundtrip) and the latency for the enhancement into acocunt. Estimating network latency to be 10ms to 50 ms in one direction leaving 200-300ms for the voice enhancement application.

If it instead is disability enhancement application to explain surrondings from a video feed somewhat higher latency can be expected. Latencies up to 1000ms would be accepted, this then need in the same way to cover video uplink, application and information of the content of the video in down stream

For the second Editor’s Note “ The value of Bit rate uplink are FFS”

The UL data rate will be dependent on the type of support service the user intends to use. If a video stream should be analysed you need to  use  at least two greyscale cameras with 60fps to capture all the movements. This would result in 1920x1080x16x60 giving 2Gpbs uncompressed, compressed dependent on compression rate but 1-50Mbps is expected.

3. Conclusions

The two Editor Notes can be removed

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

We also suggest to remove the now empty box at the top of the table. It is not possible to do with change bars.

* * * First Change * * * *

### 6.38.6	Potential New Requirements needed to support the use case

The following requirement for AI for Disability support are derived from the use case above.

[PR 6.38.6-1] Based on operator policy and user consent, the 6G system should enable the IMS services enhanced by AI capability, to enhance the audio and video stream for disability support.

NOTE:	The disability support can be e.g. describing and explaining the surroundings, audio aid (modification of the audio stream i.e. enhancements, clarifications, translations, speech to text, text to speech, etc), visual aid (modification of the visual stream).

[PR 6.38.6-2] Based on operator policy, user consent and regulatory requirements, the 6G system may be able to enhance the IMS emergency communication service with AI capability, to enhance the audio and video stream for disability support.

[PR 6.38.6-3] The following KPIs in Table 6.38.6-1 should be supported, in Dense Urban, Urban, and Rural indoor/outdoor scenario:

Table 6.38.6-1: Performance requirements for AI for Disability support

| Profile |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit rate down link | Bit rate up link | Compute latency | Communication service availability | Overall user density | Activity factor | UE speed |
| AI for Disability support | 1-2 Mbit/s | 1-50 Mbit/s (NOTE 1) | 200-300ms for audio support (NOTE 2) [400- 800] ms for video support (NOTE 4:) | Dense Urban 99.9% Urban 99.9% Rural[99% | Dense Urban 2 500 / km2 Urban 1 000 / km2 Rural 10 / km2 (NOTE 3) | 50% | 5 km/h |
| NOTE 1: Compression ratio may change dependent on use case NOTE 2: Estimating network latency to be 10ms to 50 ms in one direction leaving 200-300ms for the additional voice support in order to stay within the range 300-400 ms in total. This latency is in addition to the “IMS call” latency NOTE 3: It is assumed that 10% of all users are using AI for Disability support NOTE 4: Video support can be “Fast” (e.g., instance sign language interpretation, scene interpretation) and “Slow” (e.g., orientation, object recognition) |  |  |  |  |  |  |  |