3GPP SA WG2#170	S2-2506289

Goteborg, Sweden, 25-29 August, 2025	(revison of S2-250xxxx)

Source:	OPPO

Title:	New solution to KI#1 about UE proximity area as target

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes a new solution to KI#1 on UE proximity area as target for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

This paper proposes for a generic procedure for UE proximity area as target scenarios addressing KI#1.

Based on the requirements from TS 22.137, one of the most import scenario for the use cases is that the sensing target is the environment in a target object’s surroundings. If this target object can be identified by a UE, the target area is UE proximity area of this UE. This solution provides the general sensing procedure for UE proximity area as target.

This solution has the following assumptions:

1)	The sensing service consumer provides UE ID and a proximity indicator to identicate that the sensing target is the UE’s proximity area.

2)	The gateway function initiates positioning procedure to get the UE’s location.

3)	The gateway function initiates the sensing procedure to SF with the UE’s location (e.g. as target area) and the UE’s ID.

4)	The SF discovers and selects the sensing entities according to the UE’s location.

5)	The SF subscribes to the UE’s location information and can reselect the sensing entities according to the UE’s current location info.

Based on the assumptions above, the following procedures are provided.

2	Proposal

It is proposed to include the following changes in TR 23.700-14 V0.2.0.

* * * * Start of Changes (All text new) * * * *

6.X	Solution #X: UE proximity area as target

6.X.0	High-level solution Principles

The solution is based on the following general principles to support sensing service:

1)	The sensing service consumer provides UE ID and a proximity indicator to identicate that the sensing target is the UE’s proximity area.

2)	The gateway function initiates positioning procedure to get the UE’s location.

3)	The gateway function initiates the sensing procedure to SF with the UE’s location (e.g. as target area) and the UE’s ID.

4)	The SF discovers and selects the sensing entities according to the UE’s location.

5)	The SF subscribes to the UE’s location information and can reselect the sensing entities according to the UE’s current location info.

6.X.1	Description

This solution resolves KI#1 for UE proximity area as target.

6.X.2	Procedures

Figure 6.x.2-1: Procedure for UE proximity area as target

- Sensing Consumer sends sensing service request indicating a UE proximity area (identified by a UE ID and proximity indicator) as target and related sensing requirements (e.g. sensing mode, sensing QoS) to the Gateway Function.

- Gateway Function initiates a UE positioning procedure to get the UE’s location.

- Gateway Function selects Sensing Function(s) for the sensing service request according to the UE’s location.

4.	Gateway Function sends sensing service request to the selected Sensing Function(s) with the UE’s location, UE ID, proximity indicator and sensing requirements.

5.	Sensing Function selects Sensing Entities based on the UE’s location information received in step 4.

6.	Sensing Function and selected Sensing Entities perform Sensing measurement and sensing result calculation.

7.	Sensing Function subscribes to LMF (via GMLC) about the UE’s location notification.

8. If the UE’s location changes, Sensing Function may re-select the Sensing Entities according to the UE’s current location information.

6.X.3	Impacts on services, entities and interfaces

The solution has impacts on the following entities:

Gateway Function:

-	Support to receive the sensing service request with UE proximity area as target (i.e. identified with UE ID and proximity indicator).

-	Support to query the UE’s location.

-	Support to the invoke the Sensing Function’s service operation with UE’s location and UE ID.

Sensing Function:

-	Support to select the Sensing Entities from the available Sensing Entities according to the UE’s location.

-	Support to receive the sensing request from the Gateway Function with UE ID and UE’s location for UE proximity area as sensing target.

-	Support to subscribe to the UE’s location and re-select Sensing Entities according to the UE’s current location.

* * * * End of Changes * * * *