3GPP TSG-SA WG2 Meeting #170	S2-2506549

Gothenburg, Sweden, 25 – 29 August 2025

Source:	Ericsson

Title:	Key Issue WT#1 and WT#2: Support the SMS delivery to emergency centre realization

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution proposes one key issue for WTs in agree SID.

# 1.	Discussion

The agreed SID for the support for SMF to emergency centre in SP-250837 has following aspects, copied below.

# 4 Objective The study item aims at investigating how to specify architectural and procedural enhancements to support Short Message Service to Emergency Response Centre in both EPS and 5GS, based on the requirements specified in TS 22.101. The study includes the following work tasks: - WT-1: How to identify a Short Message to Emergency Response Center based on the use of emergency numbers. Address aspects when serving PLMN does not support this feature, but UE supports it. - WT-2: Whether and how to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls. - WT-3: How to route a Short Message to Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location). NOTE 1: Depending on the type of emergency service, Short Messages can be routed to different emergency response centres. NOTE 2: This study assumes that Short Messages initiated by the Emergency Response Center in response to the Short Message to the Emergency Response Center are treated as regular Short Messages. For EPS and 5GS, 3GPP access (including TN and NTN) is in scope. UE in limited service state will not be in scope of this study item. The architectures for SMS over NAS (see TS 23.272, TS 23.401, TS 23.501, TS 23.502) and SMS over IP (see TS 23.204), and the architectural principles of Emergency Services (see TS 23.167) will be used as a basis. SMS over IP can be applied for both 3GPP access and non-3GPP access (either over EPS or 5GS).

It is proposed to have a key issue for capability realization and identification of SMS for emergency centre based on WT#1 and WT#2 from the SID.

# 2.	 Proposal

It is proposed to update TR 23.700-65 with below changes:

* * * Start of change * * * *

## 5.1	Key Issue #Z: Support SMS delivery to emergency centre realization

To support SMS delivery to emergency centre, following aspects are included as part of the KI:

-	Study how SMS delivery to emergency centre is realized and synched between UE and network for both SMS via NAS and over IP.

* * * End of change * * * *