3GPP TSG-WG SA2 Meeting #170	            S2-2506663

Göteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250XXXX)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0316 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Security related protocol-specific parameters for N6 delay measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As clarified in our LS reply S2-2506661, “security related parameters” are contained in the "protocol-specific configuration parameters", but it’s not clear in the specification. It is proposed to clarify this issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reference the example of protocol-specific configuration parameters to TS 23.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The "protocol-specific configuration parameters" is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 6370 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.2.3.4	EAS Deployment Information Management

6.2.3.4.1	General

EAS Deployment Information management refers to the capability to create, update or remove EAS Deployment Information from AF and the distribution to the SMF or to the I-SMF in case I-SMF based Local Offloading Management applies. The NEF is in charge of the management of EAS Deployment Information which may be stored in UDR.

The EAS Deployment Information indicates how edge services are deployed in each Local part of the DN, the description of EAS Deployment Information is shown in Table 6.2.3.4-1.

Table 6.2.3.4-1 Description of EAS Deployment Information

| Parameters | Description |
| --- | --- |
| AF ID | Addressing information of Application Function responsible for the DNAI in the record. [Optional]. See NOTE 1. |
| DNN | DNN for the EAS Deployment Information. [optional] |
| S-NSSAI | S-NSSAI for the EAS Deployment Information. [optional] |
| External Group Identifier/Internal Group Identifier | Group ID for the EAS Deployment information. [optional]. See NOTE 2. |
| Application ID | Identifies the application for which the EAS Deployment Information corresponds to. [optional] |
| FQDN(s) | Supported FQDN(s) for application(s) deployed in the Local part of the DN. |
| DNAI(s) | DNAI(s) for the EAS Deployment information. [optional] |
| DNS Server Information | list of DNS server identifier (consisting of IP address and port) for each DNAI. [optional] |
| EAS IP address range Information | IP address(es) of the EASs in the Local part of the DN or the IP address ranges (IPv4 subnetwork(s) and/or IPv6 prefix(es) of the Local part of the DN where the EAS is deployed for each DNAI. [optional] |
| N6 traffic routing information | Information about how to forward edge traffic in the local part of DN corresponding to DNAI. [optional] |
| N6 delay measurement assistance information | Includes the following parameters per DNAI and/or per EAS to enable N6 delay monitoring: - measurement endpoint address information (consisting of IP address and optionally port number). - measurement protocol(s) supported by the application measurement endpoint, - protocol-specific configuration parameters. [optional]. See NOTE 4, NOTE 5, NOTE 6. |
| NOTE 1: When an AF ID is provided, all DNAI(s) correspond to the same EHE provider. NOTE 2: The AF may provide External Group Identifier, and NEF can map the External Group Identifier into Internal Group Identifier according to information received from UDM. For HR-SBO roaming scenario, External Group Identifier and Internal Group Identifier, cannot be used by AF in VPLMN. NOTE 3: AF ID can be used in case of AF(s) involving different EHE providers, and the source EHE is unaware of other/target EHE specific deployment details. NOTE 4: A measurement endpoint can be used for delay measurements representing a single EAS IP address or a range (IPv4 subnet or IPv6 prefix) of EAS IP addresses. SMF provides the measurement endpoint information together with Network Instance information to the candidate UPF(s) to perform N6 delay measurements. NOTE 5: The N6 delay measurement assistance information is utilized when AF traffic influence request includes indication of considering N6 delay as defined in clause 5.6.7.1 of TS 23.501 [2]. NOTE 6: Some examples of the measurement protocol(s) considered for N6 delay measurements and corresponding protocol-specific configuration parameters are listed in clause 5.8.2.23 of TS 23.501 [2]. |  |

The EAS Deployment Information management procedures are described in this clause, the procedures are independent of any PDU Session, including:

-	The procedure for EAS Deployment Information management from AF via the NEF.

-	The procedure for EAS Deployment Information management in the SMF or in the I-SMF when I-SMF based Local Offloading Management applies.

-	The procedure for BaselineDNSPattern management in the EASDF.

NOTE 1:	In order to support EAS discovery when the Edge Hosting Environment is provided by a partner, an SLA is needed between current operator and the partner to provide e.g. the Address(es) and credentials for the DNS servers if the partner hosts the DNS server(s) for the related DNS resolution.

NOTE 2:	BaselineDNSPattern management in the EASDF can be performed by I-SMF when I-SMF based Local Offloading Management applies.

End of CHANGES