3GPP TSG SA SA WG2 Meeting #170	S2-2506728

25 – 29 Aug, 2025, Goteborg, SE

Source:	NTT DOCOMO

Title:	[WT#5, all topics] Study 6G Data Framework

Document for:	Agreement

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes scope and key issues of the “WT#5: Study data framework for all aspects related to efficient and scalable data handling including” of the 6G SID. It reviews the 5GC data collection mechanism, its limitations to support 6G use-cases and requirements; to address the issues, it is proposed to study all the data handling aspects as a unified framework in 6G.

# 1. Justifications

Data-driven operation and control are becoming increasingly important not only to effectively manage complex and dynamic networks, such as 5G systems, but also enable other applications and use cases via exposing the data to the consumers. Using data collected from the network allows operators to make better decisions, optimize resources more efficiently, and automate network tasks.

In 5G, traditionally, data collection mechanism is specified mainly to derive network analytics by the NWDAF; and recently, solutions for UE data collection and handling sensing data are also under study. The following figure shows the currently specified architecture and also under-study options (the dashed lines) for data handling in 5GC.

Figure 1: 5G Core data handling (to be) architecture

As it is shown, the current architecture is centralized, and it does not support inter-NF data exchange which would be needed in AI-native core to be studied in WT#3. Fragmented solutions have been developed (and also under study now) per use-case that makes the system operation and maintenance complex. Moreover, the current architecture is developed mainly for the network analytics by NWDAF but recently SA1 in TR 22.870 (Table 5.9.2.1-1) identified various data types and corresponding use-case as shown below. The same data handling steps would be needed for all data types, and therefore a unified framework is needed to avoid duplication in the architecture.

Table 1 (TR 22.870 Table 5.9.2.1-1): Heterogeneous 6G System Data

| Use case category | Data types | Data provider | Data Consumer | Data volume |
| --- | --- | --- | --- | --- |
| Sensing data collection/sensing result exposure (e.g. measurement) | Sensing measurement data requires further study and discussion. The potential sensing measurement can be channel estimation results, delay, Doppler, angle, signal strength, etc. | UE, RAN | Sensing Function | Related to use cases: e.g. data rate of measurement is about 10Mbps for objects creating hazards on roads |
| AI/ML data collection / analytics exposure (e.g. AI/ML for air interface, AI/ML for core network) | - For RAN data collection for Air interface, see TR 38.843 [243] - For core network data collection, see the data collection framework in clause 6.2, TS 23.288 [114], and the detailed data type are defined from clause 6.3 to clause 6.21, TS 23.288 [114]. | CN, RAN, UE, AF | CN, RAN, UE | Typically, the internal data collection for AI model training yields tens of thousands of samples. Related to AI model: e.g. the data volume is about 10k bytes to 100M bytes for AI model of beam management |
| Energy Efficiency (EE information exposure to AF) | still under the discussion in SA2 R19 EnergySys WID | CN, OAM | EIF (Energy Information Function) | Related to use cases: e.g. could be similar to data volume for charging |
| Positioning, | See clause 6, TS 23.273 [240] | UE, RAN | LMF, UE | Related to use cases |
| Network Exposure in core network | See clause 5. 20, TS 23.501 [140], and clause 4.15, TS 23.502 [30] | 5GC NF, AF | 5GC NF, AF | Related to use cases |

Hence, today's standardized 5G system does not provide a unified framework for data handling (including data collection, data storage, data process, data sharing/exposure and data archiving/disposal). Fragmented solutions have been developed in 5G per case, for example data collection by NWDAF for the analytics, UE data collection data collection by MDAF, etc. Moreover, new kinds of data such as sensing data and also data collected from the UE are emerging in 6G that need to be managed properly by the network. Specifying, implementing, deploying, and operating multiple (overlapping) mechanisms will be quite complex and difficult to operate. To resolve the issues, it is proposed to develop a data framework in 6G to be act as a logically centralized entity to handle all the aspects of data.

In the current 5GC design, to derive the analytics, the control plane related data is collected by the NWDAF using the event exposure services provided by other NFs. Therefore, the data is implicitly identified by the NF type (that provides the service operation) and the event ID. In the case of storing and retrieval of data from ADRF, when the NWDAF stores the data in the ADRF, the ADRF returns “Storage Transaction Identifier” or “Data SetTag” which can be used the NWDAF to retrieve the data from the ADRF. However, in the case of centralized data management framework, the entity that provides data (e.g., an NF) is not the same as the entity that consume the data (e.g., NWDAF), this implies that some mechanisms are needed to identify the data in a unique way. The data identification is an issue should be resolved.

**** First Change ****

# Annex A.X. WT#5 Scope

The scope of the WT includes the following aspects of data handling:

- Data Generation: Identify the data (e.g., control plane events, sensing data, etc.) to be generated and the procedures to initiate the data generation

- Data Collection/Transfer: Identify the data (e.g., control plane events, sensing data, etc.) to be collected and the procedures to transfer the data

- Data Distribution in 6G system: Provide the collected data to the data consumers in 6G system

- Data Exposure: Expose the collected data to the authorized data consumers out of 6G system while maintaining security/privacy and also charging requirements

- Data Governance, Security, and Compliance: Adhere to security, privacy, governance, and compliance guidelines defined by local regulations (e.g., GDPR compliance), and operator policies.

**** Next Change ****

# 5.X. Key Issue #X: Data Handling Framework Overall Architecture

This KI will study the following aspects of the Data handling in 6G:

- Which data types (e.g., control plane information, sensing, etc.) are managed by the framework

- Which domains' (e.g., 6GC, RAN, UE, etc.) data is managed by the framework

- Whether and how to design overall architecture of the data handling framework from SA2 perspective

NOTE: This KI needs coordination with other WGs including SA5, SA, RAN WGs.

**** Next Change ****

# 5.X. Key Issue #X:  Data Collection & Processing

This KI will study the following aspects of the Data handling in 6G:

- Whether and how to identify data for collection

- Whether and how to initiate/instruct/configure/... data collection in the data source

- Whether and how to transfer data from different data sources for different data types

- Whether and how to maintain efficiency (e.g., overhead, scalability, delay, etc.) of data collection

- Whether and how to store collected data

- Whether and how to process the collected data

**** Next Change ****

# 5.X. Key Issue #X:  Data Distribution & Exposure

This KI will study the following aspects of the Data handling in 6G:

- Whether and how to identify data for distribution and exposure

- Whether and how the data consumers in 6G system retrieve data from the framework

- Whether and how to maintain efficiency (e.g., overhead, scalability, delay, etc.) of data  distribution and exposure

- How the authorized data consumers out of 6G system retrieve data from the framework

- Whether and how to support charging aspects of data exposure

NOTE: This aspect needs coordination with SA5.

**** Next Change ****

# 5.X. Key Issue #X:  Data Security, Privacy and Consent

This KI will study the following aspects of the Data handling in 6G:

- How to maintain security, privacy and consent aspect of data to be collected

- How to maintain security, privacy and consent aspect of data to be distributed in 6GC

- How to maintain security and privacy, user consent of data to be exposed out of 6GC

NOTE: This KI needs coordination with SA3.

**** End of Changes ****