# SA WG2 Meeting #170	S2-2506957

25 - 29 August, 2025, Goteborg, Sweden	(Revision of S2-25xxxxx)

Title:	[Draft] Reply LS on the path information in 5G ProSe multi-hop discovery procedures

Response to:	S2-2506119/C1-253724

Release:	Release 19

Work Item:	5G_ProSe_Ph3

Source:	[ZTE will be] SA2

To:	CT1

Cc:	-

Contact Person:

Name:	Hao Dong

E-mail Address:	dong dot hao1 at zte dot com dot cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.304 (CR#0561)

1. Overall Description:

SA2 thanks CT1 for the questions related to path information in 5G ProSe multi-hop discovery procedures.

SA2 has discussed the questions and provides the following answers:

"For 5G ProSe multi-hop UE-to-UE relay discovery procedure with reference to TS 23.304 clause 5.8.6.3 (excerpted below), the following two questions to SA2 are raised:

Question 1: For both model A and model B discovery, given that in the path to a 5G ProSe multi-hop layer-3 end UE, the 5G ProSe multi-hop layer-3 end UE's first 5G ProSe layer-3 multi-hop UE-to-UE relay inherently stores the source layer-2 ID of the 5G ProSe multi-hop layer-3 end UE, why does the path information need to include the source layer-2 ID of the 5G ProSe multi-hop layer-3 end UE?"

SA2 answer 1: The source layer-2 ID of the discoverer UE in the path information is used by the last hop relay when sending the response message in 5G ProSe multi-hop UE-to-UE relay discovery procedure.

Question 2: In the 5G ProSe UE-to-UE relay discovery announcement message (for model A discovery), it specifies the path information includes the "source layer-2 ID of the discoverer ProSe end UE (yellow highlighted texts as below)". However, the term "discoverer ProSe end UE" pertains to model B discovery. Why does the announcing relay UE indicate this source layer-2 ID of the discoverer ProSe end UE in the path information, given that model A does not involve a "discoverer" end UE?

SA2 answer 2: It should be "5G ProSe End UE" other than "discoverer Prose end UE". The correction is made in the CR attached.

"

#### 5.8.6.3	Identifiers for 5G ProSe Multi-hop UE-to-UE Relay discovery of non-IP PDU type

The 5G ProSe UE-to-UE Relay Discovery Announcement message (Model A) is sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay and contains the following parameters for multi-hop relaying in addition to as described in clause 5.8.4:

-	For each user info in the Direct Discovery set:

…

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE.

… …

The 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC and includes the following parameters for multi-hop relaying in addition to as described in clause 5.8.4:

…

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer UE.

The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC includes the following parameters for multi-hop relaying in addition to as described in clause 5.8.4:

…

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE.

The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the target 5G ProSe End UE matching the RSC includes the following the following parameters for multi-hop relaying in addition to as described in clause 5.8.4:

…

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE.

"

For 5G ProSe multi-hop UE-to-network relay discovery with reference to step 4a in clause 6.3.2.5.3 of TS 23.304 (excerpted below), the following third question to SA2 is raised.

Question 3: Why are the Layer-2 IDs included in the Multi-hop path info IE, together with the list of the User info IDs?

"

4a.	A 5G ProSe Intermediate UE-to-Network Relay sends the Solicitation message, it additionally includes its own User Info ID in the message. i.e. the message contains the path information which is an (ordered) list of User Info ID and Layer-2 ID of Relays in the path that has relayed the Solicitation message. … …

"

SA2 answer 3: The inclusion of list of Layer-2 ID of relays in path information is unnecessary since the adjacent relays already stores related Layer-2 IDs and can manage response routing without propagating it further. The procedure is modified in the CR attached.

2. Actions:

To RAN3

ACTION: 	SA2 kindly asks CT1 to take above answers into consideration and updates corresponding specs applicable.

3. Date of Next SA2 Meeting:

SA2#171   				13 – 17 October, 2025			       Wuhan, CN

SA2#172   				17 – 21 November, 2025			       Dallas, US