SA WG2 Meeting #170	S2-2506962

25 – 29 Aug, 2025, Goteborg , SE

Source:	TCL

Title:	[WT #1.1, 6G NAS] Discussion and key issues for distributed NAS

Document for:	Agreement

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This CR raises key issues in the field of distributed NAS to support the study of WT#1.1.

1. Discussion

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Architectural Motivation: To support more new non-access stratum functionalities, 6G network shall support direct communication between UE and core network functions. 6G network doesn’t want to increase the complicity of network function such as AMF due to new functionalities.

Gap Analysis: Currently the NAS communication is transferred by AMF between UE and core network functions, which cause following gaps:

- Lack of flexibility of the network topology

- Risk of single point failure and overload on AMF

- Latency impact due to the longer routing path

- Implementation and Protocol evolve cost to add new non-access stratum functionalities

Technical Impact: The new non-access stratum architecture shall be able to provide direct connection between UE and core network functions without help of other core network functions. The new architecture may introduce new procedures for UE registration and mobility management to keep service continuity and UE reachability. The UE status management in the core network functions may not be limited in the AMF and the status management method might be changed.

**** First Change ****

5.X. Key Issue #X: Direct Connection between UE and Core Network

The 6G network will introduce more core network services which can be provided to UE, such as sensing, computing, AI and so on. All of these services need their own non-access stratum connections. In current architecture, these connections are concentrated on AMF. Thus, it can be foreseen that the implementation of AMF will become more complicated and the traffic can be heavier and cause higher single-point failure risk. It will be preferred that 6G network can provide direct connection between UE and core network functions providing services. Follow issue shall be solved:

- How to improve RAN-Core network interface to connect UE directly to any core network functions without the help of AMF?

- How to support UE discover the core network functions to acquire services? How to route the UE uplink service requests?

- For downlink messages, how to keep UE reachable if there is no core network anchor such as AMF?

5.X. Key Issue #X: UE status and mobility management across Core Network Functions

The non-access stratum connections between UE and different NFs can be in different status at the same time. The status depend on which service the UE is acquiring. However, for each core network functions providing service to the UE, the mobility information of the UE shall be synchronized to keep service continuity. Following issues shall be solved:

- How to support UE mobility management among multiple core network functions? How to synchronize UE mobility and location information among multiple core network functions?

- How to manage, transfer and synchronize UE contexts between network functions during UE mobility when the UE status is different on each NF?

**** End of Changes ****