SA WG2 Meeting #170	S2-2506969

# 25 – 29 Aug, 2025, Goteborg , SE

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6392 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Energy Saving Indicator as a new filtering criteria for member UE selection assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc., Lenovo, Oracle |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_ ESI_MUESA |  |  |  |  |  | Date: |  |  | 2025-07-21 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some Application Functions may want to select UEs based on the UEs Energy Saving Indicator. For example, an application that sends and receives AIML traffic (e.g., Federated Learning), may not want to select UEs that may undergo Energy Saving during the training session, to avoid impacting their performance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a description on how UEs Energy Saving Indicator can be used to assist Application Function in Member UE Selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Member UE selection assistance feature cannot filter UEs based on their Energy Saving Indicator |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.46.2, 5.51.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>START OF CHANGES<<<<

### 5.46.2	Member UE selection assistance functionality for application operation

5G System may support Member UE selection assistance functionality to assist the AF to select member UE(s) that can be used in application operations such as AI/ML based applications (e.g. Federated Learning) as specified in clause 5.46.1 according to the AF's inputs.

The Member UE selection assistance functionality shall be hosted by NEF and the features of the Member UE selection assistance functionality hosted by the NEF include (see clause 4.15.13 of TS 23.502 [3] for details of Member UE selection procedures):

-	Receiving a request from the AF that shall include a list of target member UE(s) (which may not necessarily be a part of the subsequent updated request), optionally (a) time window(s) and one or more filtering criteria as specified in Table 4.15.13.2-1 of TS 23.502 [3] (e.g. UE current location, UE historical location, UE direction, UE separation distance, QoS requirements, DNN, preferred access/RAT type, Desired end-to-end data volume transfer time performance, Service Experience, or Energy Saving Indicator).

-	Referring to the filtering criteria provided by the AF and then interacting with 5GC NFs using existing services in order to have the corresponding data for all the UEs in the list of target member UE from relevant 5GC NFs (e.g. PCF, NWDAF, AMF, SMF, UDM) to derive the list(s) of candidate UE(s) (i.e. UE(s) among the list of target member UE(s) provided by the AF) which fulfil the filtering criteria.

-	Providing the AF with the Member UE selection assistance information, including one or more lists of candidate UE(s) and optionally other additional information (e.g. one or more recommended time window(s) for performing the application operation, QoS of each target UE, UE(s) location, Access/RAT type, Service Experience, or Energy Saving Indicator). NEF may also provide the number of UEs per each filtering criterion that do not fulfil the corresponding filtering criterion.

The Member UE selection assistance information provided by the NEF may be used by the AF to select member UE(s) used in application AI/ML operation. (See clause 5.2.6.32 of TS 23.502 [3] for details of parameters).

NOTE:	The AF can decide whether to use the Member UE selection assistance functionality provided by NEF.

Without using the Member UE selection assistance functionality, AF in either trusted or untrusted domain can select the Member UE(s) for e.g. participating in federating learning operation, by collecting the corresponding data using network exposure information as described in clause 4.15 of TS 23.502 [3], e.g. UE location reporting from the AMF, user plane information from the UPF and network data analytics from NWDAF.

# >>>>2nd CHANGES<<<<

### 5.51.x	Member UE Selection Assistance aspects

During Member UE selection assistance procedure, the AF may avoid including UEs with certain ESI values, thus assuring the desired connectivity quality needed to run FL training operations. The AF may request the 5GC to only select UEs that have no Energy Saving Indicator or only select UEs whose Energy Saving Indicator is set to a certain value (see clause 4.15.13.2 of TS 23.502 [3]). If the Energy Saving Indicator filtering criterion is included in the AF request, then the NEF queries the UDM to check the UEs’ Energy Saving Indicator and the NEF determines whether to include a UE in the list of candidate UEs depending on the result obtained from the UDM and the filtering criterion value provided by the AF.

# >>>>END OF CHANGES<<<<