SA WG2 Meeting #170	S2-2506978

25 – 29 Aug, 2025, Goteborg , SE

Source:	CSCN

Title:	[WT#7, NTN] Clarification on 6G NTN work task scope

Document for:	Approval

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the scope of WT#7, including Core Network NFs onboard satellites, QoS framework, AI and computing for 6G NTN.

1. Justifications

At the TSG SA#108 held in June 2025, the 6G system architecture study item (FS_6G_ARC) was approved, marking the official launch of architectural study for 6G. As outlined in the SID, this study includes multiple high-level WTs, but the current descriptions of these WTs remain relatively brief, lacking detailed scope definitions. To advance the study effectively, the current meeting aims to collect input papers that provide justifications and clarifications on the scope of each WT, thereby refining and enriching the content of the work tasks. To refine the depth of WT#7, new study directions need to be added in line with the technical characteristics and requirements of 6G NTN.

Core Network NFs onboard satellites: 6G NTN demands rethinking core network deployment. 6G NTN will support various services, including UE-SAT-UE, AI, computing, etc. However, if all the core network functions are located on the ground,it may lead to inefficiency or even unavailability of these services. Therefore, it is critical to study which core network NFs are necessary to be deployed on satellites, with interface and procedure enhancements.

QoS framework for 6G NTN: 6G NTN introduces unique architectural challenges such as dynamic satellite link characteristics (e.g. dynamic ISL, orbit-induced variability). Existing architectures lack mechanisms to address satellite-specific QoS dynamics, such as real-time parameter adaptation to link fluctuations. This WT addresses these architectural gaps by focusing on structural solutions for end-to-end QoS assurance in NTN scenarios.

AI and computing: It is difficult to directly apply AI and computing frameworks to NTN because of the constraints including satellite network constraints (e.g. limited feeder link bandwidth, satellite mobility, and long propagation delays) and resource constraints. Therefore, it is essential to study how to adapt AI and computing to 6G NTN.

**** First Change ****

Annex A.X. WT#7 Scope

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects, including:

-	WT#7.1: Study which core network NFs are necessary to be deployed on satellites, with interface and procedure enhancements.

-	WT#7.2: Study the QoS framework for 6G NTN, considering dynamic characteristics of the satellite link.

-	WT#7.3: Study on adapting the AI framework to 6G NTN (e.g. delivery of AI services over satellite networks), considering constraints such as limited feeder link bandwidth, satellite mobility, and long propagation delays.

-	WT#7.4: Study on supporting computing services for 6G NTN, considering the dynamic characteristic of both network and computing resources.

NOTE 8:	The detailed scope for WT#7 will be coordinated and aligned with RAN

**** End of Changes ****