SA WG2 Meeting #170	S2-2506995

25 – 29 Aug, 2025, Goteborg, SE

Source:	Huawei, Hisilicon, SK Telecom, China Unicom

Title:	[WT#6, Computing] Computing Enabler service, coordination of communication and computing service

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes to introduce the computing enable service, which coordinates the computing service and communication service, into 6G network. The justification on defining this service and related GAP analysis are given. Per analysis the related KI are proposed for WT#6.

1. Justifications

1.1 Introduction

The approved 6G SID include the following WT, i.e. WT#6

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE 7: Application layer mechanism and exposure framework may require coordination with SA6.

To support the emerging computing intensive service, we see the necessity to offload the computing task from UE to network. Similarity also exists at the network side. When offloading the computing task to computing resource, there are some common processing, e.g., computing resource discovery/selection, task delivery. To avoid duplicate same work on different computing task handling, these common processing can be derived and defined as a computing enabler service. Then it may be questioned on whether existing EC mechanism can be re-used for this purpose. Per GAP analysis the computing resource controlled by the 6G CN and scheduled together with communication service need be supported, i.e., coordination of communication and computing service. This is different comparing to EC mechanism. Hence we suggest the corresponding WT scope and KI.

1.2 Architectural Motivation

1.2.1 Why need support computing task offloading?

With the development of new technology, more and more computing intensive work are emerging, e.g., AI training, AI inference, Sensing, XR rendering. How to support those computing intensive work at terminal need be considered. One mechanism is to offload the computing task outside of terminal. The reason why computing task offloading is needed can be seen as following:

- GAP between task required capability and task supported capability: Figure 1 shows that the computing power offered by the mobile GPU is hardly to satisfy the computing power required by the emerging service requirement since 2025.

Figure 1:  Computing power offering and request [Fig 9.2.1-1 of TR22.870]

Similarity also exists for memory supported by the UE considering more and more large size AI model to be supported for AI services. It is challenging for UE to support the execution of AI model size more than 10B.

- Battery consumption: The emerge of AI model has led to an exponential increase demand for computing power. And the battery may drop dramatically when the computing task is executed at the UE side. Hence even if the computing power requested is supported by the UE, the UE may still prefer offload the computing task outside.

In short, due to the limited computing capability and constrains on battery consumption of terminals, offloading computing task outside the UE is necessary.

Similarity also exist at the network as more and more computing related task is to be executed at the network side, e.g. AI4Network, if the computing task can be offloaded from the required NF, the computing resource can be aggregated and shared among NFs. This is benefit for network deployment and operation.

Observation 1: Due to the limitation on the terminal, it should be possible to offload the computing task from the terminal. Similarities also exist at the network side.

1.2.2 Why offload to public cloud is not enough?

One possibility for computing task offloading is to offload task to public cloud. This is an application layer mechanism and transparent to network.  Besides this mechanism, it is worthwhile to think about whether other mechanism is needed, e.g., the network need be aware the computing task offloading?

From our view, 3GPP needs consider to support the computing task offloaded to network, the reason can be seen as following:

- New revenue and network resource usage efficiently: with the introduction of more and more AI technology into network, i.e. AI4Network, it is expected that more and more computing resources are invested to network. To bring more revenue to operator and utilize those computing resource efficiently, it’s meaningful to enable operator to introduce new services by providing computing resource to its subscriber, e.g., offloading the computing task to mobile network.

- Support computing task which is needed or better to be executed at the network:

- Tasks need be executed in the network only: some computing tasks can only be executed at the network, e.g.  tasks are executed with the assistance of the data provided by network. One example, to improve the sensing result accuracy, combining the sensing data provided by the UE and RAN side are required. Hence when the UE want to perform the environment reconstruction via AI inference, UE offloads the task to the network. Since the sensing raw data cannot be exposed outside network, this computing task can only be executed at the network.

Another example is that for the network collected DATA handling, e.g., Data processing, this task can only be done at the network due to the collected data cannot be transferred outside network.

- Task is better to be executed in the network: some computing task is better to be executed at the network as they need the service assistance from network. One example is AI model training, e.g. FL, with the network help to filter some special UEs. If this task is not executed in the network, multi-interaction between AF and network is needed. This is in-efficient or restricted. Hence this task is better to be executed at the network.

- Joint control of communication and computing service: some computing task needs to ensure the related performance KPI, e.g. E2E latency. In this case it needs to consider the latency of the communication and computing service together. For example, if the communication latency is increased, the processing delay should be reduced, e.g. by adding more computing resources. If the computing task is offloaded to public cloud and transparent to the network, two independent system (communication system and computing processing system) cannot coordinate with each other to achieve E2E performance. More information refer to clause 1.3.

- “N6” interface issue:  the “N6” interface between the network and computing resource is not under the operator’s control. Hence the latency of N6 path transmission is uncontrollable. This brings negative impacts to E2E performance, especially for the offloading task with low latency requirements.

SA1 requirement:

The SA1 definition about the computing service can be found as below:

- 6G Computing Service: A service provided by 6G network utilizing computing resources in Service Hosting Environment, which can be used by a subscriber (via UE)/3rd party. (TR 22.870)

- Service Hosting Environment: the environment, located inside of 5G network and fully controlled by the operator, where Hosted Services are offered from. (TS 22.261)

From the SA1 definition the 6G computing service is to use the computing resource located at the environment controlled by the operator, i.e., computing task is not executed at public cloud.

Observation 2:  Offloading the computing task to public cloud without network awareness is not enough and cannot meet the requirement from SA1. It need consider how to offload the computing task to the computing resource controlled by the operator.

1.2.3 What is the Computing enabler Service?

As indicated above, TR22.870 has defined the 6G computing service. From outside network SA1 view, the computing service consumer is defined as UE or 3rd party. However, the service consumer of computing service is not restricted to UE or 3rd party AF. Any computing task requester wanting to use the computing resource controlled by the operator can be the computing service consumer. And the computing resource is not restricted to pure hardware, e.g. xPU (e.g. GPU, CPU, NPU..), Memory. It can be any resource supporting the execution of the required computing task and exposed by operator, e.g.  operator provides MaaS (Model as a Service) to its subscriber to support the required computing task.

As seen in TR22.870, the computing intensive task offloaded to network can be, e.g., AI inference or XR split rendering. In these two cases, the task can be split into following steps:

- Service requirement decomposition: the requirement(s) to the network are derived, e.g. which type of computing resource is to be discovered/selected, performance requirement, ….

- Computing resource discovery/selection: Discover and select a computing resource per requirement.

- Computing task delivery: Transfer the computing task from the consumer to the selected computing resource.

- Computing task execution: Execute the required Computing task, i.e., AI inference or XR split rendering.

- Computing result return: Return the result of the Computing task back to the consumer.

Step 1/4 is related to the corresponding service work and service specific.

Step 2/3/5 is related to computing resource discovery/selection and task request/result delivery, which can be defined as one common service offered by network.  For this common process handling, it can be named as Computing enabler Service.  Then this service can be invoked for any above layer computing service provided by operator.

Observation 3: The service consumer of the computing service is not restricted to UE or 3rd party AF. The computing resource can be any resource, which supports the execution of the required computing task and exposed by operator.

Proposal 1: Define a computing enabler service to enable the computing task offloaded to the computing resource controlled by the operator.

Proposal 2: the computing enabler service is related to 1) how to discover and select one suitable computing resource. 2) Computing task management, e.g. task delivery and result reception between the service consumer and the selected computing resource.

1.3 Gap Analysis

1.3.1 What is the GAP of the existing mechanism?

To offload the computing task to the environment controlled by the operator, whether the existing 5GC EC mechanism e.g., as defined in TS23.548, is enough?  This is also the question raised from the previous offline discussion.

In the existing 5G EC mechanism, the EAS is outside of mobile network and AF provides the EAS deployment information to the network. Per the DNS query message handling result, 5GC routes the related UE dedicated traffic to the EAS selected by DNS server. If we consider EC as one potential mechanism to provide computing enabler service, then the FQDN is the required computing resource, EAS is the selected computing resource. Comparing to the requirement as described in TR22.870, at least there are following GAPs:

- Different type of requester for the computing enabler service: the computing task offloading is not restricted to be only requested by UE. For example, in TR22.870 the AF want to offload the XR split rendering service to the network. The EC design only support the UE initiated task request. For the AF or other requester, e.g. NF, it is not supported.

- Computing resource status management:  the computing resource status may be changed, e.g. on/off. The status of the computing resource in the EC mechanism is managed by the DNS system and not be able to be updated by the 5GC. For example, if the transport path between the 5GC and EAS is broken, it is impossible for 5GC to update the DNS DB as from the EAS itself view it may still be operable. Then in this case the DNS server may still return this EAS as candidate EAS to UE.

- Diversity of target computing resource to be discovered/selected: per the requested offloading task the computing enabler service should support the discovery/selection of different type of computing resource. For example, in case AI inference computing task offloading, one may request the selected computing resource need support one dedicated AI model, other may only request the dedicated hardware and the AI model will be provided by the requester itself. Hence the computing enabler service need support different type of computing resource requested. Due to this diversity of target computing resource, the parameter for computing resource discovery/selection need be extensible and flexible.

The EC design is based on DNS mechanism. Hence the required computing resource need be reflected as one related FQDN and configured at the DNS database and also be aware by the UE. Considering the above extensible/flexible requirement and synchronization between UE and network, we do not see the EC can be extended to support this mechanism.

- Computing task differentiation: for different computing task to be offloaded, it is nature that they may have different characteristic, e.g. different performance KPI requirement. For example, even for the same image recognition AI inference task, one is for Robot control it have stringent latency requirement, another is for object recognition it does not need stringent latency requirement.

In the EC mechanism, it is impossible for UE to indicate different service requirement.

- Different type of computing resource support: the offering computing resource can be special hardware, e.g.  xPU resource for remote access. In this case no AF provides the related resource information to the CN as it is not dedicated to one specific AF. Also for this type task offloading there are no computing service context need be stored at the computing resource. Hence each time when a new computing task is offloaded to network, different computing resource can be selected, e.g. per UE location.

This is not achievable for DNS mechanism due to its cache mechanism, i.e. before cache is expired the old computing resource is always selected. Hence the EC mechanism does not support above type computing resource.

- Service performance fulfilment: to satisfy user’s experience, E2E latency is one of the important factors to be considered for computing task offloading, especially for the high value service. Besides the device handling latency, the E2E latency includes other two parts, i.e. communication latency + computing processing delay. To fulfil service performance following two aspect need be considered:

f.1) Computing aware path selection/reselection:

In EC work the EAS selection and the UE path selection are executed sequently. It is possible that the selected EAS is the best one from DNS server view but not the best one from E2E system view.

One example can be found in  https://datatracker.ietf.org/meeting/116/materials/slides-116-cats-03-a-cats-ps-usecases-01.pdf

Figure 2: E2E delay for different edge DC

In the above example, it can be seen that either the DNS server returns the EAS in site 1(light load) or EAS in site 2(closet one), all are not the best one from the E2E delay view. The EC mechanism is expected to encounter similar situation because it executes in the similar way.

When the mobility is triggered, the DNAI/UPF selection is done by SMF and the selected target DNAI is notified to AF. The target EAS selection is done by AF per notified target DNAI. Therefore, both selections are also done by the SMF and AF sequently. Then similar issue happens as mentioned above.

f.2) Achievement from joint schedule:

If the communication latency is increased, it can consider to reduce the processing delay. This is achievable only via the joint schedule.

One of typical computing tasks to be offloaded to network is AI inference. It is possible to split AI inference work part on the UE side and other part in network. Depending on how the AI inference work is split, different processing delay/computing power is required on two sides. If the computing enabler service support schedule the communication and computing together, it gives more flexibility to the upper layer, i.e. computing service layer, on how to split the AI work in different side. For example, the AI inference layer give different option on how to split inference work on UE and network side. Then computing enabler service layer determines which option is supported at the network after considering the communication and computing latency together.

In EC mechanism, this joint schedule is impossible due to communication and computing are controlled separately.

Observation 4: computing enabler service as a general enabler service, it needs support different type of requested computing task offloading. Comparing to required enabler service requirement the EC mechanism cannot fulfill the related requirement.

1.4 Way forward

From the analysis in clause 1.3, the key problem is that computing resource is not seen and controlled by the CN, i.e. out of operator control. Due to this constraint the discovery/selection of the computing resource use external DNS mechanism and impossible to be considered together with the discovery/selection of communication resource (UPF).

To achieve good performance of computing task offloading, it is natural that the computing service should cooperate with communication service tightly, like joint schedule. Also, to overcome the above problem it is proposed that in the computing service WT#6, the computing resource be seen and controlled by the 6G core network, i.e., In-network computing. When the computing service is required by the consumer, the computing network resource and communication resource can be scheduled by 6GC together. This can be achieved via the computing enabler service.

Proposal 3: In WT#6, the computing enabler service should be able to coordinate the communication and computing service, i.e., computing resources awareness and controlled by the CN should be supported.

1.5 Technical Impact

For WT#6 as explained above the work is to define a computing enabler service, which is a general enabler service to support the computing service. The potential work on this area, i.e. impact to system, can be listed as following:

- How the 6G CN be aware the available computing resource(s), which is under operator’s control, its profile and status?

- How the UE be aware the computing task can be offloaded to network?

- How the network provision the computing resource per request from the computing service consumer, e.g. UE or 3rd party, including the corresponding architecture and procedures, e.g. discover/select computing resource?  The procedures also need consider that the computing task to be offloaded can have different requirement, e.g. E2E latency requirement.

- How does the network support to relocate or update the computing resource if needed, e.g. in case of the UE mobility or AF triggered?

2. Reference

1.   https://datatracker.ietf.org/meeting/116/materials/slides-116-cats-03-a-cats-ps-usecases-01.pdf

**** 1st Change ****

Annex A.1. WT#1 Scope

- NAS: Support the interaction between UE and network for computing enabler service.

- UP:  Support computing task delivery and result return.

- Network Exposure framework: Support the AF can invoke the computing enabler service.

- Network internal Interface: Support interaction among NFs for computing enabler service.

****2nd Change****

Annex A.6. WT#6 Scope

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE 7: Application layer mechanism and exposure framework may require coordination with SA6.

This work task is to define a general computing enabler service, it includes following aspects to be studied:

-	Study how the 6G CN be aware the available computing resource(s), which is under operator’s control, its profile and status.

-	Study how the UE be aware the computing task can be offloaded to network.

-	Study how the network provision the computing resource per request from the computing service consumer, e.g. UE or 3rd party.

-	Study how does the network support to relocate or update the computing resource if needed, e.g. in case of the UE mobility or AF triggered.

****3rd Change****

5.6. Key Issue #6: Architecture support for computing service

To support computing service executed at network, one general enabler service need be defined, i.e., computing enabler service. This key issue addresses the system architecture and procedure for this computing enabler service, especially considering the following aspects

- How the 6G CN be aware the available computing resource(s), which is under operator’s control, its profile and status?

- How the UE be aware the computing task can be offloaded to network?

- How the network provision the computing resource per request from the computing service consumer, e.g., UE or 3rd party, including the corresponding architecture and procedures, e.g. discover/select computing resource? The procedures also need consider that the computing task to be offloaded can have different requirement, e.g. E2E latency requirement.

- How does the network support to relocate or update the computing resource if needed, e.g. in case of the UE mobility or AF triggered?

**** End of Changes ****