SA WG2 Meeting #170	S2-2507047

25 – 29 Aug, 2025, Goteborg, SE

Source:	vivo

Title:	KI#1: proposal for agreed principles

Document for:	Agreement

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes to agree the principles for KI#1 for FS_EnergySys_Ph2.

1.	Introduction

Key issue#1 studies whether and how to enhance the collection, determination and exposure of energy related information in the 5GS.

Sol#1, 2, 8,9,10,11,12,22 propose the solutions for this Key issue.

Table 1: Brief summary of solutions for Key issue#1

| Sol# | Brief summary of the solution | Highlights |
| --- | --- | --- |
| 1 | Two event IDs for EIF exposure API are proposed. - Energy consumption percentile (or percentile band): - Energy consumption per bit: | New parameters for EIF exposure: - Energy consumption percentile - Energy consumption per bit |
| 8 | This solution proposes to calculate the renewable energy consumption at the required granularity. This solution assumes that EIF can collect the ratio of Renewable Energy at node level from OAM. This solution relies on the SA5 feedback on whether the ratio of the renewable energy information can be provided. | New parameters for EIF exposure: - Renewable energy consumption information. Note: have dependency on SA5. |
| 9 | In this solution, the EIF calculates the share of the renewable energy. The share of the renewable source energy may be represented as a percentage of the full consumption energy. This solution is assumed that the 5G system can obtain renewable energy information from the energy providers. How to obtain this information is out of scope. | New parameters for EIF exposure: - the share of the renewable energy Note: have some similarities with sol#8. |
| 12 | This solution proposes that the EIF exposes the additional information on the List(s) of top contributors that contribute the most to the energy consumption. - For UE level energy consumption, a list of applications that contribute the most to the energy consumption for the UE can be included. - For PDU Session level energy consumption information, a list of applications that contribute the most to the energy consumption for the PDU Session can be included. | New parameters for EIF exposure: - the List(s) of top contributors that contribute the most to the energy consumption |
| 22 | In this solution, the EIF obtains the energy resource usage information per slice or for a set of slices from OAM. The SMF can be notified by EIF whether renewable energy per slice is used and then the SMF is triggered to query NRF by considering whether to reselect a UPF or not for the slice related to the existing PDU Sessions. | New parameters for EIF exposure: - Renewable energy information. Note: have some similarities with sol#8. |
| 10 | This solution proposes to enhance the mathematical approach of energy consumption information for the required granularity defined in Rel-19. It aims to reduce the impact of the energy consumption for maintaining the basic operation. For the time interval T, the EIF calculates the per bit energy consumption at the network node by using the ratio of the energy consumption difference to the total data volume difference between the two time intervals. Then, the EIF applies the per bit energy consumption as the coefficient to calculate the energy consumption of the required granularity at the node (i.e., UPF and gNB) over the time interval T. This solution is based on the information provided by the OAM which has been defined in R19. | Energy consumption calculation enhancements |
| 11 | This solution also proposes to enhance the mathematical approach of energy consumption information for the required granularity defined in Rel-19. It is proposed that the EIF collects the basic power consumption of the node periodically or based on operator’s configuration. And then the EIF excludes (deducts) the (estimation of) node basic power consumption from the total energy consumption of the node to calculate the energy consumption at the required granularities. This solution requires the OAM to provide the basic energy consumption of a node (i.e., UPF and gNB) per scaling level. | Energy consumption calculation enhancements Note: have dependency on SA5. |
| 2 | In this solution, the PCF collects slice level EnC from OAM and compare the collected EnC to the thresholds in the policy data. And then the PCF apply/revoke the AM policy, SM policy and/or BDT policy by monitoring the EnC according the EnC policy per slice. | Enhancements are related with PCF to collect energy information from OAM and make the policy control. This can be further considered in key issue 2. |

According to the current solutions for KI#1, the following analysis are proposed:

Option 1: New parameters for EIF exposure:

The following new parameters can be exposed by the EIF:

-  Energy consumption percentile

-  Energy consumption per bit

-  Renewable energy information

-  List(s) of top contributors that contribute the most to the energy consumption

Proposal 1: These parameters help the application server to get more energy related information for its applications or UEs. It is proposed to consider these parameters for EIF exposure.

Option 2: More energy consumption calculation examples at EIF

More examples on energy consumption calculation at EIF can be captured in Annex T of TS 23.501[2] in order to reduce the impact of the energy consumption for maintaining the basic operation of the node. These examples are focusing on the following two aspects to improve the robustness for the data volume based energy consumption calculation model:

The following energy consumption calculation examples are proposed:

-    If the basic energy consumption information of the node can be provided by OAM, this information can be excluded from the total energy consumption of the node to calculate the energy consumption at the required granularities.

-   The per bit energy consumption at the network node can be calculated by using the ratio of the energy consumption difference to the data volume difference between two time intervals. This per bit energy consumption information can be further used to calculate the energy consumption at the required granularities.

These two calculation examples aim to reduce the impact of the energy consumption for maintaining the basic operation of the node and improve the robustness and accuracy of the calculation.

Proposal 2: It is proposed to capture these two calculation examples in Annex T of TS 23.501[2].

2. Text Proposal

It is proposed to add the following changes for KI#1 in TR 23.700-67.

# * * * * First change (all new Text) * * * *

7.1.1	Agreed Principles for KI#1

The following principles are agreed for KI#1:

-	Based on the AF request, the EIF exposes energy related information to the AF to help the application server to get more energy related information for its applications or UEs. The following parameters or event ID(s) can be considered:

-  Energy consumption percentile

-  Energy consumption per bit

-  Renewable energy information

-  List(s) of top contributors that contribute the most to the energy consumption

NOTE 1:	The renewable energy information has dependency on SA5 and can be further updated based on SA5 feedback in the normative phase.

-    More examples on energy consumption calculation at EIF can be captured in Annex T of TS 23.501[2] in order to reduce the impact of the energy consumption for the basic operation of the node. These examples are focusing on the following two aspects to improve the robustness for the data volume based energy consumption calculation model:

-    If the basic energy consumption information of the node can be provided by OAM, this information can be excluded from the total energy consumption of the node to calculate the energy consumption at the required granularities.

-   The per bit energy consumption information of the network node can be calculated by using the ratio of the energy consumption difference to the data volume difference between two time intervals. This information can be further used to calculate the energy consumption at the required granularities.

NOTE 2:	The energy consumption calculation based on basic energy consumption information provision by OAM has dependency on SA5 and can be further updated based on SA5 feedback in the normative phase.

# * * * * End of change * * * *