3GPP TSG-WG SA2 Meeting #170	S2-2507262

# Goteborg, SE, 25th Aug – 29th Aug, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0052 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Device Filtering Information for single AIoT Devices and all AIoT Devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At present, the Device Filtering information can be used to target a single device or multiple devices during inventory, and the AIoT Device does not to differentiate between single or multiple device inventory, i.e., it compares the received Filtering Information against its identity and responses if it matches, although this is not noted in the specification. Therefore, clarification that the Filtering Information can identify a single device should be made clearer. The note that clarified that it is expected that the Filtering Information is not expected to contain more than one element for the ID Type is not correct, as the ID Type is a set of flags/indicators, and it may need to apply filtering to non-contiguous flags/indicators in the ID Type field. During the inventory procedure the “information about the target AIoT Device(s)” is an optional parameter. It is not clear that when this parameter is absent the target is all AIoT Devices, which can also be indicated to NG-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Make it clearer that the Filtering Information can target a single device. Clarify the AIOTF handling when “information about the target AIoT Device(s)” is not provided and clarify that AIoT Identification Information can indicate all AIoT Devices. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear the filtering can be used during the inventory procedure for a single device, and incorrect understand for inventory all AIoT Devices cases and incorrect handling if information about the target AIoT Device(s) is not provided. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2, 5.8, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.7	Identifiers

### 5.7.1	General

An AIoT Device is identified by AIoT Device Permanent Identifier, see clause 5.7.2.

# * * * * Second change * * * *

## 5.8	Filtering Information

The filtering information is used to identify or filter a single AIoT Device or multiple AIoT Devices, and is constructed by one or multiple components (i.e., ID Type, PLMN Identifier, NID, third party identifier, and Identification Information) of the AIoT Device Permanent Identifier as defined in clause 5.7.

The Filtering Information is a list of filtering elements. Each filtering element includes:

-	Information indicating a component of the AIoT Device Permanent Identifier that is used to match the bitstring.

-	A bitstring which is used to compare with the component.

-	An Offset from the beginning of the indicated component of the AIoT Device Permanent Identifier, which indicates the start location in the indicated component to be used to compare with the corresponding bitstring.

When the indicated component is PLMN ID, NID, or Third Party Identifier, the Offset is not included in the filtering information, and the whole component is compared with the bitstring, otherwise the Offset is always included.

The Filtering Information shall contain 1 or more filtering elements indicating components of the AIoT Device Permanent Identifier.

There may be multiple filtering elements within the Filtering Information for the Identification Information component of the AIoT Device Permanent Identifier.

Each bitstring is corresponding to one component of the AIoT Device Permanent Identifier.

NOTE 1:	It is assumed that the AIOTF includes at most one filtering element information within the Filtering Information for PLMN ID, NID or Third Party Identifier. It is assumed that the AIoT Device does not validate the number of filtering element(s) for any indicated component within the Filtering Information.

NOTE 2:	The maximum number of the filtering element for Identification Information is left to stage3.

Editor's note:	Whether and how to secure the filtering information is up to SA WG3.

Figure 5.8-1: The example of the comparing the bitstring to the component

To determine whether an AIoT Device Permanent Identifier matches the filtering information, it is compared with every filtering element information within filtering information by comparing the whole bitstring within a filtering element information with the indicated component of its AIoT Device Permanent Identifier. If all the compared bitstrings match the AIoT Device Permanent Identifier then an AIoT Device Permanent Identifier matches the filtering information. If an AIoT Device Permanent Identifier does not contain an indicated component then it does not match the filtering information.

# * * * * Third change * * * *

### 6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [External Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval]) service operation request to the NEF.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s). If no Information about the target AIoT Device(s) is provided by the AF then the target is all AIoT Devices.

The approximate number of AIoT Devices, if provided, is used to determine the number of AIoT Devices expected to respond to this inventory service operation, which is sent by AIOTF to the NG-RAN in the assistance information for NG-RAN in step 7 for proper radio resource allocation.

The time interval, if provided, is described in clause 5.9.

2.	The NEF may further authorize the AF request as specified in clause 5.6.

The NEF determines the Target Area information from the External Target Area information, and selects one or multiple AIOTF(s) to handle the request as specified in clause 5.3.1. The Target Area information is specified in clause 5.3.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval]) service operation towards to the selected AIOTF(s).

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF may perform authorization as specified in clause 5.6. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a Correlation ID corresponding to this AF service operation request.

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, to identify a single AIoT Device or multiple AIoT Devices, or can indicate all AIoT Devices.

AIOTF performs Reader Selection, see clause 5.3.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4, taking into account the parameters provided in the service request.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the Correlation ID, the AIoT Identification Information to be included in the paging message, and assistance information to the selected NG-RAN as specified in TS 38.413 [10].

8.	The NG-RAN sends an Inventory Response to the AIOTF with the Correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly as specified in TS 38.413 [10].

9.	Upon reception of the Inventory Request message from the AIOTF, the RAN Reader(s) will execute the inventory operation as specified in TS 38.300 [5] and TS 38.391 [11]. The RAN Reader(s) broadcast the paging message that includes the AIoT Identification Information.

The AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.8.

If an AIoT device matches the AIoT Identification Information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT identity.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

10.	NG-RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s) as specified in TS 38.413 [10]. The NG-RAN may aggregate multiple Inventory Report messages based on the assistance information before reporting the response to the AIOTF as described in clause 5.9. The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s).

NOTE:	When to erase the stored mapping between the Reader ID and AIoT device ID(s) is up to implementation and local configuration.

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	Optionally, if the NG-RAN detects that no more AIoT Devices will respond to the inventory procedure, the NG-RAN informs the AIOTF that the procedure is complete and the last inventory result. After the procedure has completed NG-RAN will not send any further Inventory Reports for this requested Inventory.

Editor's note:	The details about completion of the procedure need to be aligned with RAN.

13.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the Naiotf_AIoT_Notify message including a list of AIoT Device Permanent Identifier (s). The AIOTF may send multiple reports. The AIOTF in the final Naiotf_AIoT_Notify message indicates it is the last report for this operation. If multiple AIOTFs are involved in the procedure, the NEF may receive the AIoT_Notify from multiple AIOTFs.

14.	When receiving the Naiotf_AIoT_Notify message from AIOTF, the NEF informs the AF of the outcome of the AIoT_Inventory request by sending the Nnef_AIoT_Notify message(s) including the AIoT Device Permanent Identifier(s). The NEF in the final Nnef_AIoT_Notify message indicates that it is the last report for this operation.

# * * * * End of changes * * * *