3GPP TSG-SA WG2#170	S2-2507278

Gothenburg, Sweden, 25-29 August 2025	(was S2-250xxxx)

Source:	Qualcomm

Title:	[WT#5] Scope for work task on data framework

Document for:	Discussion and approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes an updated scope for WT#5 (data framework).

# 1.	Discussion

## 1.1	Overview

SA2’s 6G SID [1] contains the following description for WT#5:

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework. NOTE 6: The work split with SA3, SA5 and RAN WGs will require TSG coordination.

The remaining sections will discuss the various aspects captured in the WT: data collection, distribution, processing, storage, data access/data exposure and user consent/privacy.

## 1.2	Data collection

It is important to discuss and analyze the system entities from which data can be collected: CN NFs, RAN and UEs:

- Data collection from CN NFs

  - Procedures for data collection from CN NFs have already been specified for analytics purposes (see TS 23.288 and TS 23.502).

  - The same can be done for new 6G NFs and hence it is not obvious why anything new is needed for data collection from CN NFs.

  - The typical consumer so far is NWDAF but the respective APIs for exposure are generic and may hence also be used by other NFs.

- Data collection from RAN

  - SA2 asked RAN1/RAN2/RAN3 and RAN plenary whether there was a need for data collection from RAN for AI/ML purposes [2] to which RAN3 replied that there was no need [3].

  - While there may be a need for interaction between RAN and the core network for sensing related data, the details of that are currently studied as part of the 5G sensing study.

- Data collection from UEs

  - Data collection from UEs is currently being studied in the context of UE side model training.

  - It is not obvious that there will be other use cases that require data collection from UEs.

  - Data collection from UEs for purposes other than UE side model training have traditionally been done using MDT procedures, which are in scope of RAN2.

- Data collection from AF

  - Procedures for data collection from CN NFs have already been specified for analytics purposes (see TS 23.288 and TS 23.502).

  - The same can be done for new 6G NFs: if any enhancements are required, those can be discussed in other WTs.

- Data collection from OAM

  - Procedures for data collection from OAM have already been specified for analytics purposes (see TS 23.288) and can be reused also for 6G.

For the reasons given above, the authors see no need

- for a new, generic data framework for data collection and related functionality such as distribution, storage, etc. from CN NFs;

- to study generic data collection from RAN in SA2; instead, the authors continue to see generic data collection from RAN in scope of SA5 and RAN3;

- to study generic data collection from UEs in SA2 unless SA2 is asked explicitly by RAN WGs.

## 1.3	Data distribution

TS 23.288 [4] defines procedures for data collection. Since in 5GS model training and inference are always performed by NWDAF, the NWDAF is the sole consumer for any data collected from CN NFs in 5GS. To enable distribution of data also to other NFs, one area worth exploring for 6G is to enable all 6G CN NFs to act as data consumer for data exposed by CN NFs by generalizing the exposure functionality already defined in TS 23.288 [4]. Similarly, Data Collection Coordination Function (DCCF) as defined in TS 23.288 [4] clause 5A can be generalized to allow all CN NFs to request data from a Data Source.

## 1.4	Data processing

From the author’s perspective data processing is typically use case-specific. Therefore, the value of studying generic processing functionality beyond the already existing model training is not obvious and hence not warranted.

## 1.5	Data storage

Data storage is already supported by Analytics Data Repository Function (ADRF) as defined in TS 23.288 [4] clause 4.2.1 and can be reused also for 6G.

## 1.6	Data exposure

Data analytics exposure is already supported based on the procedures defined in TS 23.288 [4] clause 6.1 and can be reused also for 6G.

## 1.7	User consent/privacy

User consent for analytics has already been addressed as per TS 23.288 [4] clause 6.2.9 and can be reused also for data collection from new and modified CN NFs. Also, any user consent framework is in scope of SA3.

# 2	Conclusion and proposal

In line with the observations and conclusions made in the previous section, this paper proposes to focus WT#5 on the following:

-	Study whether and how to generalize the exposure functionality already defined in TS 23.288 to enable all 6G CN NFs to act as data consumers for data exposed by CN NFs.

-	Study whether and how to generalize the Data Collection Coordination Function (DCCF) defined in TS 23.288 clause 5A to allow all CN NFs to request data from a data source.

-	Study whether there is a need for additional data exposure for data collection from new and modified CN NFs based on the same principles for data collection as defined in TS 23.288.

# 3.	References

[1]	TSG SA: Study on Architecture for 6G System. SP-250806.

[2]	SA WG2: LS on AI/ML Core Network enhancements. S2-2311921.

[3]	RAN WG3: Reply to LS on AI/ML Core Network enhancements. R3-237745.

[4]	TS 23.288: Architecture enhancements for 5G System (5GS) to support network data analytics services.

# 4.	Proposed changes

This paper proposes to apply the following changes to TR 23.801-01.

**** First Change ****

# Annex A.X. WT#5 Scope

WT#5: Data framework

.

This work task will address the following:

-	Study whether and how to generalize the exposure functionality already defined in TS 23.288 to enable all 6G CN NFs to act as data consumer for data exposed by CN NFs.

-	Study whether and how to generalize the Data Collection Coordination Function (DCCF) defined in TS 23.288 clause 5A to allow all CN NFs to request data from a Data Source.

-	Study whether there is a need for additional data exposure for data collection from new and modified CN NFs based on the same principles for data collection as defined in TS 23.288.

**** End of Changes ****