3GPP TSG-SA WG2 Meeting #170	S2-2507347

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6415 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on energy consumption information collection from OAM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For the energy consumption information collection from OAM, the “UPF data volume” should be the data volume handled by the UPF not the data volume consumed at UPF. It is proposed to clarify this description in the Table. - In NOTE 2, it aims to explain that the node level information collected from OAM can be used for all the UEs being served by the node. However, it only mentions that the Node-level Energy Consumption information collected from the OAM can be used for all the UEs served by the node, so it is proposed to add “Node-level data volume information” received from OAM can also be used for all the UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 5.51.2.2.3, it is proposed to change the description of the “UPF data volume” to the “The UL/DL Data volume handled by a UPF over the configured time interval T.” - For NOTE 2, it is proposed to add “Node-level data volume information” in the description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description for energy consumption information collection from OAM is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.51.2.2.3	Energy Consumption information collection from OAM

Table 5.51.2.2.3-1 provides the list of information received from OAM. EIF requests the energy consumption and data volume information per NF level from OAM by providing the serving gNB ID(s) and (I-)UPF ID(s) received from SMFs.

NOTE 1:	The management consumer in EIF is configured with information to retrieve the energy related information from OAM as defined in TS 28.554 [209].

The node-level reporting interval from the OAM is the network-wide configurable starting time and interval T which is the same as data volume reporting interval from SMFs.

Table 5.51.2.2.3-1: Information from OAM for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| gNB energy consumption | The Energy consumed by a gNB over the configured time interval T based on clause 6.7.3.4.2 of TS 28.554 [209]. |
| gNB data volume | The UL/DL data volume handled by a gNB over the configured time interval T based on clause 6.7.1.1 of TS 28.554 [209]. |
| UPF energy consumption | The Energy consumed by a UPF over the configure time interval T based on clause 6.7.3.1 of TS 28.554 [209]. |
| UPF data volume | The UL/DL Data volume handled by a UPF over the configured time interval T. |

When the serving gNB and/or the (I-)UPF(s) of the UE are changed, the serving gNB ID and (I-)UPF ID(s) will be updated to the EIF through SMF(s). And then, the EIF requests the energy consumption and data volume information from OAM for the updated serving gNB ID and (I-)UPF ID(s).

NOTE 2:	The Node-level Energy Consumption information and Node-level data volume information received from OAM could be used by EIF for all the UEs being served by the NF Node. Also, if the EIF has subscriptions from one or more Consumer NFs for the same UE, the Node-level Energy Consumption information and Node-level data volume information received from OAM could be used by EIF for one or more Consumer NFs that subscribed to the EIF.

NOTE 3:	In this Release, only the energy related information of user plane communication is supported and that of control plane signalling is not supported.

The EIF acts as the enforcement point for user consent for collection and processing of energy-related information depending on local regulations. For support of user consent (for collection and processing of energy-related information), the EIF communicates with the UDM as specified in Annex V of TS 33.501 [29].

# * * * * End of changes * * * *