3GPP SA WG2 Meeting #171	S2-2508216

13 – 17 Oct, 2025, Wuhan, China

Source:	NEC

Title:	[WT#1.3] Scope and key issue on Non-3GPP Access

Document for:	Agreement

Agenda Item:	20.6.1.3

Work Item / Release:	FS_6G_ARC/Rel-20

This contribution proposes scope and key issue to Work task 1.3 of the 6G study on Non-3GPP Access.

1. Justifications

This contribution discusses the scope and proposes a corresponding key issue to facilitate work on Work Task #1.3 of the approved 6G SID (SP-250806):

WT#1.3: Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

The contribution builds on the baseline of S2-2508127, which was postponed at the end of SA2#170.

1.1 Architectural Motivation

There are common interests that 3GPP core network can be accessed via Non-3GPP Access, e.g. WiFi, Wireline. The WiFi calling based on IMS has been commercially deployed in the real market. 6G system should consider whether and how non-3GPP Access can access to the core network of 6G system.

1.2 Gap Analysis

3GPP has specified multiple non-3GPP Access solutions in different generations, i.e. Interworking-WLAN(3G), Trusted/untrusted(ePDG) non-3GPP Access(4G), N3IWF based and Wireline access(5G). For the new generation 3GPP system i.e. 6G, it is suggested to study whether and how a new non-3GPP access solution can be specified, while the different already specified solutions may be compared and re-considered including to simplify the network architecture and UE implementation.

During the SA2#170 discussion, there is an opinion that only WiFi is considered as the non-3GPP access in the first release of 6G system, other non-3GPP accesses may be considered in the following releases(s).

The service continuity between 3GPP access and untrusted non-3GPP access may not be needed in all cases, it is suggested to study whether the service continuity between 3GPP access and untrusted non-3GPP access is needed.

1.3 Technical Impact

There are some opinions the current non-3GPP Access solutions are too complicated to be deployed, it is suggested to simplify the network architecture and UE implementation for non-3GPP Access.

Based on the gap analysis, WT#1.3 scope should address the following key issues:

- Whether only one or some specific non-3GPP access-es are considered in the first release of 6G system.

- Compared with previous generations, how to simplify the network architecture and UE implementation for non-3GPP Access connected to the core network of 6G system.

2. Proposal

This contribution proposes to update the annex of TR 23.801-01 to include the scope of WT#1.3 and the related Key Issue.

**** Tracked changes on top of S2-2508127 ****

**** First Change – all text new ****

Annex A.X. WT#1.3 Non-3GPP Access

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

It proposes to add several sub-WTs to address the non-3GPP access scenario and multi-access data connections.

1.3.	Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

For non-3GPP access:

-	Study how to support untrusted non-3GPP access in 6G System architecture.

-	Study whether and how to support non-3GPP wireline access in 6G System architecture.

NOTE 1:	For wireline access, co-ordination with BBF and CableLabs is needed.

-	Whether and how to support service continuity between 3GPP access and untrusted non-3GPP access.

NOTE 2:	Interworking aspects related to non-3GPP access will be studied as part of WT#2.

NOTE 3:  WT#1.2 aspects related to QoS, Policy, and UP applicable to non-3GPP access are studied under this WT with a checkpoint to align with WT#1.2 at TBD.

-	Whether and how to support Non-Seamless WLAN Offload (NSWO).

For the multi-access:

-	Study whether and how to support the multi-access data connectivity (e.g. traffic steering, switching and splitting) between 3GPP access and non-3GPP access for different type of traffic (e.g. IPv4/v6, UDP, TCP, Ethernet) in 6G.

NOTE 4:  The services on untrusted non-3GPP access are specific to the operator services over WiFi, e.g., WiFi calling on IMS.

**** Second Change – all new text ****

5.X. Key Issue #X: Support of Non-3GPP Access

- How to support non-3GPP access in 6G system.

NOTE: It may be decided before the solution phase that whether only one or some specific non-3GPP access-es and service(s) are considered in the first release of 6G system.

**** End of Changes ****