3GPP TSG-WG SA2 Meeting #171	S2-2508235

Wuhan, CN, 13th – 17th Oct., 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon, China Telecom

Title:	On the open issues of sensing and way forward proposal

Document for:	Agreement

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-19

Abstract: The paper discusses the open issues in the current specification and provide the proposals.

# 1. Introduction

The paper discusses the open issues for KI#1 to KI#6. The proposals are summarized in clause 3 and in other conclusion proposals in S2-2508236, S2-2508237, S2-2508238, S2-2508239, S2-2508240, S2-2508241.

# 2. Discussion

## 2.1	Open issue for KI#1 System Architecture to Support Sensing

### 2.1.1	Dedicated Sensing Function

As per the current TR, all solutions assume that there will be dedicated NF for sensing. Considering the forward compatibility, and the sensing functionality is separate from other existing NFs, it is proposed to have dedicated NF(s) for sensing.

Proposal 1.1: Dedicated NF(s) should be used for sensing.

### 2.1.2	Connection between Sensing Function and gNB

There are two options for interactions between the sensing function (NF for sensing) and gNB in the TR, namely a direct connection and indirect connection. The details for the interfaces will be discussed in clause 2.1.3. Following Figure 2.1.2-1 provides the two options in TR.

Figure 2.1.2-1: Direct connection (left) and Indirect connection (right).

For the direct connection option: the request for sensing data, and provisioning of sensing data (by the gNB) does not need an intermediary 5GC NF(s).

For indirect connection option: intermediary 5GC NF(s) (e.g., AMF) is needed for requesting and providing sensing data.

The direct connection is better since:

-	No intermediary 5GC NF(s) (e.g., AMF) is needed, thereby enabling easy-deployment.

-	Sensing is more for localized services, no need a roundabout path for Sensing configuration/exposure which causes additional latencies.

-	In indirect connection, the only functionality used for sensing is for N2 message routing, therefore having a full-fledged AMF in between is not needed.

Proposal 1.2: The architecture for sensing service should support a direct connection between the Sensing Function and the gNB.

### 2.1.3	Interfaces for Sensing data reporting

This issue will be discussed together with KI#4 in clause 2.4.

### 2.1.4	Control and Processing Split of the Sensing Function

Several solutions in the TR assumes the Sensing Function can be further divided into a Sensing Control Function (SCF) and a Sensing Processing Function (SPF). The messages between SCF and SPF includes Sensing Process Configuration (Initiate, Update or Release) and Sensing Result Report. Figure 2.1.4-1 depicts the options with further splitting compared with the non-splitting one.

-	Issue of single centralized SF: Single-point failure of the control function, the load for processing sensing data could be high. Moreover, centralized deployment may cause high latency.

-	Issue of distributed SFs: Need to deploy many sensing functions in which the control plane parts may not be frequently used, and it might be needed to define the interactions between local Sensing Function and Sensing Function in higher layer; If the higher layer Sensing Function also performs sensing data processing, it still has scalability issues.

-	Centralized SCF with multiple SPFs: Similar as C-U split, the processing for Sensing result will be done by the localized processing function, while the interaction with gNBs will be done by the centralized control function. When needed, the Sensing result can be also exposed by processing function for a lower latency. The disadvantages of centralized SF and distributed SF can be overcome.

Figure 2.1.4-1: Centralized Sensing Functions (left), distributed Sensing Functions (right), and Centralized SCF with multiple SPFs (bottom).

For the interface between SCF and SP, considering time limitation, in this release, it is assumed the SCF and SPF to be deployed by the same vendor, thus the protocol between SCF and SPF can leave to implementation.

Proposal 1.4: Centralized SCF with multiple SPFs should be supported to enable scalability. Considering time limitation, the interface between SCF and SPF is not specified in this release and leave to implementation.

### 2.1.5	Storage of Authorization Data and Sensing Result

Several solutions assume that a dedicated sensing data storage (or sensing data management) can be used for storing the data for sensing, e.g., authorization, Sensing Result. Though storing the data is useful for reusing the Sensing Result, the following aspects need to be considered if a dedicated NF is introduced in the standard:

-	Standardize the stored data for sensing, including the data for 1) authorization (e.g., authorized target sensing service area); 2) format of the sensing result for reusing;

-	Standardize the interface for the interaction between Sensing (Control) Function and sensing data storage;

The following Figure 2.1.5-1 shows the above-mentioned aspects.

Figure 2.1.5-1: Sensing data storage.

However, the storage can be implemented as the internal logic of Sensing Control Function. Since the authorization information is mainly location-dependent (e.g., whether the sensing is allowed for a certain area as further clarified in clause 2.2.1), having the storage as the internal logic of SCF will not cause duplicated information stored by different SCFs.

Proposal 1.5: Sensing data storage is considered as a part of SCF/SPF in this release.

### 2.1.6	Sensing Gateway Function

Several solutions introduce the sensing gateway function, in which it is mainly for:

-	Authorization, i.e., whether the AF requested parameters is valid;

-	Selection of Sensing Control Function (SCF);

-	Unified entrance point for the external request;

As for many other features that use the exposure framework, the NEF is responsible for selecting the internal nodes according to the service requirement. As such the NEF can perform the selection of SCF and so no additional functionality is needed.

Once the SCF is selected, any additional authorization e.g., in relation to sensing service parameters, etc, can be performed by the SCF. This service level authorization is discussed in 2.2.1.

When there are multiple SCFs involved, either the AF or the NEF can be used to aggregate the Sensing Result from the associating SCF(s).

Proposal 1.6: Dedicated Gateway Function is not needed. Functionalities for authorization and SCF selection can be implemented as part of Sensing Control Function and NEF, respectively.

## 2.2	Open issue for KI#2 Authorization and Revocation to Support Sensing Service

### 2.2.1	Entities involved in authorization and related parameters

This is related to the discussion in clause 2.1.6.

Besides the "parameter-level" sensing request authorization, the 5GC also needs to authorize the AF as the requester for the sensing services ("AF-level"). As such, the two-level authorization is proposed and:

-	NEF is in charge of authorizing the AF as the sensing service requester, following the same paradigm of existing NEF logic as defined in TS 23.501 [1].

-	Sensing (Control) Function is in charge of authorizing the requested parameters, given that the parameter-level authorization is quite sensing service-specific. The possible parameters considered for authorization at Sensing (Control) Function are as follows:

-	Whether all or part of the Target Sensing Service Area is within the Allowed Sensing Service Area;

-	Whether all or part of the Target Sensing Service Area is outside of the non-Allowed Sensing Service Area;

-	SCF and NEF locally store the information for authorizing the request from AF, and the details are implementation-specific, as explained in clause 2.1.5.

-	Authorization of the gNB is not needed since the gNB is authorized by definition given that they are deployed by operator (and as such only discovery / selection is needed).

Some solutions propose the authorization considering the privacy related to the detected object, but if the object detected is not associated to any specific UE ID, then there is no privacy associated risk, since there is not association to a known and identifiable user. Moreover, there is no stage 1 requirement for such scenario.

The following Figure 2.2-1 depicts the procedure for authorization.

Figure 2.2.1-1: Procedure for authorization.

Proposal 2.1: Sensing (Control) Function is assumed to authorize the requested parameters from AF, it locally stores the information for authorizing, and the Target Sensing Service Area is the main authorization criteria.

### 2.2.2	Revocation procedures

From the description of KI#2, it also includes the revocation of granted authorization of the sensing service. The revocation is mainly triggered by the change of the subscription. In summary:

-	Triggered by NEF: the NEF determines that the AF is no longer allowed to use the sensing service due to e.g., SLA changes.

-	Triggered by Sensing Control Function: The change of Subscription may invalidate the current granted request. E.g., the previously authorized service area is no longer authorized and so the sensing service has to be stopped.

The following Figure 2.2.2-1 depicts the procedure for the revocation of the granted authorization.

Figure 2.2.2-1: Framework for revocation.

Procedure of revocation triggered by NEF and Sensing Control Function starts from step 1a and 1b respectively. For NEF triggered revocation, step 1b does not apply.

Proposal 2.2: Revocation can be triggered by NEF and Sensing (Control) Function for different reasons.

### 2.2.3	Issues of AF in trusted/untrusted domain

There is another issue discussed during the meeting that whether the sensing service is only for AF in trusted domain, or can additionally accommodate the request from AF in untrusted domain.

From SA1 TS 22.137 [2]:

-	"The 5G network shall limit the exposure of the sensing results only to a trusted third-party authorized to receive that sensing results."

A trusted third-party means either an AF in a trusted domain, or an AF in untrusted domain but has been authorized by the PLMN (i.e., NEF). This is the common understanding in general 5G requirement in TS 22.261 [3]. For example:

-	"Based on MNO policy, the 5G network shall provide suitable APIs to allow a trusted third-party to add/remove an authorized UE to/from a specific 5G LAN-VN managed by the trusted third-party."

This done by TS 23.502 [4] that

-	"Provisioning capability allows an external party to provision the information, such as expected UE behaviour and service specific parameters or the 5G VN group information to 5G network functions."

"An external party" is the AF in the untrusted domain. Therefore, the trusted third-party should also contain the case that authorized AF in untrusted domain.

Proposal 2.3: AF in both domains (trusted and untrusted) should be supported.

## 2.3	Open issue for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection

### 2.3.1a	Area-related terms for Sensing

The coverage area for Sensing is different from the one for communication, and for Sensing services the area-related definition should consider the following requirements:

-	Cell-like sub-area within a gNB to support the finer-granular sensing service for a smaller area, since the efficient handlings for radio resources need to be considered;

-	TA-like area within/across gNB(s) to re-use the selection/management within the CN side; Note that this TA-like area is not for any tracking purpose for the UE.

Therefore, reusing the similar conceptions as defined for communication is a straightforward way. For sensing it can have the following area-related information (the name can be re-checked):

-	Cell-like sub-area: Sensing cell. A logical concept of an area where the Sensing Entity (i.e., gNB in this release) can obtain the 3GPP Sensing Data by itself. For each gNB, it may associate with several Sensing cells;

-	TA-like area: Sensing Tracking Area. This definition is comparable to a special Tracking Area, and is an area that can be across gNB(s). One or more gNB can belong to the same Sensing Tracking Area. When the gNB establishes the NG setup connection with Sensing (Control) Function, the gNB can include the supported Tracking Area for the later selection.

The example of the Sensing Tracking Area and Sensing cell is as the following Figure 2.3.1a-1 shows.

Figure 2.3.1a-1: Example of Sensing cell and Sensing Tracking Area.

To denote the area for sensing, the following definitions can be used:

-	For gNB:

-	Sensing cell: A logical concept of an area where the Sensing Entity (i.e., gNB in this release) can obtain the 3GPP Sensing Data by itself. For each gNB, it may associate with several Sensing cells;

-	Sensing Tracking Area: Comparable to a "specific Tracking Area", which will be reported via NGAP as current Sensing Tracking Area for gNB/SPF/SCF discovery and selection;

-	For SPF, the SPF Sensing Service Area can be denoted as an area consisting of one or more Sensing Tracking Area, and within which the generated 3GPP Sensing Data by gNB(s) can be sent to the SPF directly;

-	For SCF, the SCF Sensing Service Area can be described as the collection of SPF Sensing Service Areas of all SPFs which can be controlled by one SCF;

The example of the relationship among those three terms are as the following Figure 2.3.1a-2 shows.

Figure 2.3.1a-2: Example of area-related definitions for gNB, SPF and SCF.

In summary, the Sensing cell and Sensing Tracking Area are proposed for the purpose of 1) reuse the current area management for communication, and 2) efficient control of radio resources. In addition, the SPF sensing service area and SCF sensing service area are introduced comparable to UPF/SMF service area.

Proposal 3.1: It is proposed to have the Sensing cell and Sensing Tracking Area denoted for area-related terms at gNB and comparable to cell and TA; and define SPF sensing service area and SCF sensing service area for SPF and SCF respectively linked to Sensing Tracking Area for discovery and (re-)selection.

### 2.3.2	Capability provisioning for discovery

For Sensing (Control/Processing) Function capability registration, there are following current mechanisms can be used:

-	Pre-configuration at the NEF. E.g., the necessary addresses and capabilities are pre-configured at the NFs;

-	CN NF discovery via NRF, i.e.:

-	Service provider NF (e.g., Sensing Control Function) registers the NF profile with additional sensing information into the NRF, or

-	NRF may be configured by OAM with a set of NF discovery policy. Each NF discovery policy includes a score Metric for the to be evaluated information in the NF profile;

-  Node level association.

It is proposed to reuse them for sensing-related discovery and selection:

-	Information provisioning for Sensing (Control) Function discovery:

-	Pre-configure the NF profile of Sensing (Control) Function to the NEF, or

-	Define NF profile of Sensing (Control) Function, and NEF detects the Sensing (Control) Function using NRF via current mechanism;

-	Information provisioning for Sensing Processing Function discovery: This may not be further specified, since as proposed in clause 2.1.4 that the interface between SCF and SPF is not specified in this release and leave to implementation.

For the connection between gNB and CN, the current procedures are:

-	NG setup initiated by the gNB;

It is proposed to have the similar logic for gNB discovery and selection:

-	Using NG setup-like procedure to the Sensing (Control) Function by the gNB. More specifically:

-	gNB is configured with the addresses of Sensing (Control) Function;

-	gNB initiate the NG setup procedure to Sensing (Control) Function with optionally including the Sensing Tracking Area supported by the gNB, and other necessary information for setting up the association. However whether only the NG setup procedure is used or other NG signaling after NG setup is used will be coordinated with RAN WG3 in the normative phase.. Or alternatively, the Sensing Tracking Area is preconfigured at Sensing (Control) Function;

NOTE:	The protocol between gNB and Sensing (Control) Function will be determined by RAN WGs.

Proposal 3.2: It is proposed to either pre-configure the information for the discovery and selection of Sensing (Control) Function and SPF at NEF or SCF, or using NRF by current existing mechanism; for gNB discovery and selection, NG setup-like procedure or other NG signaling after NG setup is used to provide Sensing Service area and other configuration parameter to the Sensig Control Function.

### 2.3.3	Discovery and selection criteria

When need to discover the gNB (as Sensing Entity) and the Sensing (Control/ Processing) Function, there are several aspects mentioned in the current TR.

The NEF maps the requested area from AF to the Sensing cell list or Sensing Tracking Area list as the Target Sensing Service Area, and selects the Sensing Control Function based on local configuration or query the NRF which matches the following criteria:

-	SCF Sensing Service Area of the Sensing (Control) Function that matches with the Target Sensing Service Area;

-	The requested Sensing Service Type and the supported Sensing Service Type (as defined in TS 22.137 [2]) of the Sensing (Control) Function;

NOTE 1:	The detailed list of criteria for selection of the Sensing (Control) Function will be determined during the normative phase.

NOTE 2:	There might be more than one Sensing (Control) Function to be selected by the NEF for a specific sensing request.

NOTE 3:	NEF is not required if AF is in trusted domain. Sensing cell list and Sensing Tracking Area list shall only be used by AFs who reside in trust domain, and when the AFs are aware of such information.

NOTE 4:	(Copied from clause 2.1.6) When there are multiple SCFs involved, either the AF or the NEF can be used to aggregate the Sensing Result from the associating SCF(s).

The Sensing Control Function selects the Sensing Processing Function based on local configuration or query the NRF which matches the following criteria:

-	SPF Sensing Service Area of the Sensing Processing Function that matches with the Target Sensing Service Area;

-	The requested Sensing Service Type and the supported Sensing Service Type (as defined in TS 22.137 [2]) of the Sensing Processing Function;

The Sensing (Control) Function selects one or more the gNB(s) to satisfy the Sensing Service Request based on local configuration that match the following criteria:

-	Sensing Service Area (i.e., supported Sensing Tracking Area) of the gNB that matches with the Target Sensing Service Area;

The selection of gNB as Sensing Entity is based on the information configured in the Sensing Control function via NG setup-like by the gNB or pre-configuration as described in clause 2.3.2.

Proposal 3.3: For Sensing (Control) Function, and Sensing Processing Function selection, the criteria for discovery and selection is Sensing Service Area and Sensing Service Type; for gNB discovery and selection, the criteria is Sensing Service Area.

### 2.3.4	Framework for discovery and selection

As a summary of 2.3.2 and 2.3.3, the following Figure 2.3.4-1 depicts the framework for discovery and selection.

Figure 2.3.4-1: Framework for discovery and selection.

## 2.4	Open issue for KI#4 Sensing Data and the Associated Information Collection and Transport

### 2.4.1	Information to be provided by gNB

Here is the list of necessary information that is useful for the Sensing result calculation:

-	The description related to the object data, e.g., reflected points, and that could be the one in the Geographical Area Description (GAD) as defined in TS 23.032 [5].

-	Timestamp when gNB collects the data;

-	And the Coordinates of the gNB should be available at the Sensing (Control) Function.

It is preferred to use Point Cloud Data (PCD) (or similar as "Intermediate sensing results per detected point" mentioned by RAN1 [FL1][H] Proposal 7.1-1-rev3) to represent the object reported by the gNB, since it results in relatively lower transmission overhead while still effectively capturing the original characteristics of the measurements. A typical example of the point cloud can be denoted as follows:

-	List of Point, for each point, the following information can be considered as the starting point:

-	Timestamp;

-	Coordinate information;

-	3D-point relative to origin in reference system;

-	Velocity information;

-	Others

The rationale for using PCD is as follows:

-	Forward compatibility.

-	There will be more information sources (i.e., non-3GPP sensing data, UE-provided data) for a more precise Sensing calculation at the Sensing Function in the future releases. If gNB provides Sensing Result directly, it will affect Sensing Function obtaining a good result.

-	No need to upgrade the gNB for future potential supported sensing types, especially when the number of gNBs are large, while upgrade the Sensing Functions would be much simpler.

-	Strike a good balance between lower transmission overhead and effectively capturing the original characteristics of the measurements.

Note that the information to be provided by the gNB for 3GPP Sensing Data should be coordinated with RAN WG3. Especially what information can be provided and whether it is feasible to do so. However, due to the time plane issue that RAN WG3 starts the sensing study since October, it is hard to provide a timely information to SA2.

Proposal 4.1: It is proposed to adopt the Point Cloud Data as the gNB reported information, and the details will be further coordinate with RAN WG3 in the normative phase.

### 2.4.2	Sensing data report granularity

When gNB provides PCD to the SPF, the granularity should be considered.

In the TR there are two granularities for sensing data report from gNB to Sensing (Processing) Function, namely, the Node-level (or Area-level) granularity, and Service-level granularity:

-	Node-level granularity: The 3GPP Sensing Data for a certain Sensing Service Area will be sent to Sensing (Processing) Function only once. In other words, for different sensing service, the 3GPP Sensing Data will be "shared/reused" for the on-going sensing.

-	To support the node-level granularity, the Sensing (Control) Function should be able to reuse part or all of the on-going-collected 3GPP Sensing Data for the newly-requested sensing service. To this end, the Sensing (Control) Function can further update the on-going collection for fulfilling the new requirement. Note that the support of this granularity would be the internal logic of SCF.

-	Service-level granularity: The 3GPP Sensing Data will be sent by gNB in a per-Sensing service manner, and the related 3GPP Sensing Data will be reported for each Sensing service.

The following Figure 2.4.1-1 provides an example for Node-level granularity:

Figure 2.4.2-1: An example of Node-level granularity.

It is preferred to support the Node-level granularity for its efficient resource usage.

Proposal 4.2: For different sensing service, the 3GPP Sensing Data will be shared/reused.

### 2.4.3	Establishing the tunnel for sensing data report

As per the explanation of 2.4.1 and 2.4.2, when PCD and node-level granularity are used, current NGAP message is not forward compatible due to limited size of DTLS over SCTP (16KB at maximum) as defined in RFC 6083 [6]:

-	"The supported maximum length of SCTP user messages MUST be at least 2^14 + 2048 + 13 = 18445 bytes (2^14 + 2048 is the maximum length of the DTLSCiphertext.fragment, and 13 is the size of the DTLS record header). In particular, the SCTP implementation MUST support fragmentation of user messages. Every SCTP user message MUST consist of exactly one DTLS record."

Note that the message fragmentation was once discussed in IETF however it has been expired. See details on https://datatracker.ietf.org/doc/draft-ietf-tsvwg-dtls-over-sctp-bis/.

Therefore, the data connected needs to be defined for transmitting the PCD and current GTP-U is a good candidate considering it has been well-defined and can be re-used.

NOTE:	The protocol between gNB and Sensing Processing Function will be determined by RAN WGs.

Proposal 4.3: GTP-U like way should be used for 3GPP Sensing Data report considering the reporting efficiency.

## 2.5	Open issue for KI#5 Sensing Result Exposure

### 2.5.1	Input parameters

The input parameters of sensing request are in fact related to the scenarios to be supported. There are following typical scenarios mentioned in the TR solutions:

-	Map generation, i.e., Sensing for list of objects in target sensing service area;

-	Intruder detection, i.e., checking whether there is any new object move to the target sensing service area;

-	Object tracking, i.e., a path of some certain objects in a given time span;

-	Sensing for the surrounding area of a certain UE.

Among those scenarios, the first three scenarios are related, and the second (intruder detection) and third (object tracking) scenarios are based on the result of the objects within the Target Sensing Service Area. For example, by comparing objects in the area in two subsequent time slot, one can obtain the intruder detection result as following Figure 2.5.1-1 demonstrates.

Figure 2.5.1-1: An example of the scenarios.

The fourth object is not the identified scenario in TS 22.137 [2] for UAV Object detection and tracking. And it is not expected to be supported by SA1 in this release.

Proposal 5.1: Support the scenario that Sensing Result contains a list of objects within a target sensing service area, for other use cases based on this, they can be determined in the normative phase.

## 2.6	Open issue for KI#6 Configuration of Parameters for Sensing Entities

### 2.6.1	Parameters contained in the request to gNB

The input parameters of sensing request from Sensing (Control) Function to the selected gNB(s) is used to provide the requirement for obtaining the Sensing data for result calculation, and need the coordination with RAN, while the information should at least contain the following information:

-	Quality requirement for the sensing data, e.g., Refreshing Rate,

-	Address for Sensing data reporting;

-	Target Sensing Service Area;

Proposal 6.1: Parameters for Sensing (Control) Function reporting to gNB should at least contain Quality requirement for the sensing data and Target sensing service area, and will coordinate with RAN WG3.

# 3. Conclusion and proposal(s)

In summary, the proposals highlighted in this document are summarized as follows:

Proposal 1.1: Dedicated NF(s) should be used for sensing.

Proposal 1.2: The architecture for sensing service should support a direct connection between the Sensing Function and the gNB.

Proposal 1.4: Centralized SCF with multiple SPFs should be supported to enable scalability. Considering time limitation, the interface between SCF and SPF is not specified in this release and leave to implementation.

Proposal 1.5: Sensing data storage is considered as a part of SCF/SPF in this release.

Proposal 1.6: Dedicated Gateway Function is not needed. Functionalities for authorization and SCF selection can be implemented as part of Sensing Control Function and NEF, respectively.

Proposal 2.1: Sensing (Control) Function is assumed to authorize the requested parameters from AF, it locally stores the information for authorizing, and the Target Sensing Service Area is the main authorization criteria.

Proposal 2.2: Revocation can be triggered by NEF and Sensing (Control) Function for different reasons.

Proposal 2.3: AF in both domains (trusted and untrusted) should be supported.

Proposal 3.1: It is proposed to have the Sensing cell and Sensing Tracking Area denoted for area-related terms at gNB and comparable to cell and TA; and define SPF sensing service area and SCF sensing service area for SPF and SCF respectively linked to Sensing Tracking Area for discovery and (re-)selection.

Proposal 3.2: It is proposed to either pre-configure the information for the discovery and selection of Sensing (Control) Function and SPF at NEF or SCF, or using NRF by current existing mechanism; for gNB discovery and selection, using NG setup-like procedure is used.

Proposal 3.3: For Sensing (Control) Function, and Sensing Processing Function selection, the criteria for discovery and selection is Sensing Service Area and Sensing Service Type; for gNB discovery and selection, the criteria is Sensing Service Area.

Proposal 4.1: It is proposed to adopt the Point Cloud Data as the gNB reported information, and the details will be further coordinate with RAN WG3 in the normative phase.

Proposal 4.2: For different sensing service, the 3GPP Sensing Data will be shared/reused.

Proposal 4.3: GTP-U like way should be used for 3GPP Sensing Data report considering the reporting efficiency.

Proposal 5.1: Support the scenario that Sensing Result contains a list of objects within a target sensing service area, for other use cases based on this, they can be determined in the normative phase.

Proposal 5.2: Reuse of SBI for sensing result report in this release.

Proposal 6.1: Parameters for Sensing (Control) Function reporting to gNB should at least contain Quality requirement for the sensing data and Target sensing service area, and will coordinate with RAN WG3.

It is proposed to adopt the above-mentioned proposals into the conclusions as in <our conclusion paper>

# 4. References

[1]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[2]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 23.032: " Universal Geographical Area Description (GAD)".

[6]	IETF RFC 6083: " Datagram Transport Layer Security (DTLS) for Stream Control Transmission Protocol (SCTP)".